3GPP TSG RAN WG2 Meeting #129	 R2-2501156

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	8.6.4

Source: 	ITL

Title: 	Discussion on Conditional intra-CU LTM

Document for:	Discussion and decision

Introduction

In the previous RAN2 meeting [1], agreements related to conditional intra-CU LTM are as follows:

Agreements on C-LTM:

- The triggering condition of conditional LTM can be based on L3 measurement.

- CondEventA3 and CondEventA5 conditions can be baseline for the conditional LTM execution.

- The L1 execution condition of a candidate cell is associated to only one triggering event.

- For L3 execution condition, it may consist of one or two triggering condition(s). If there are two triggering conditions associated with the same candidate cell, the UE shall consider the execution condition is fulfilled only when both triggering conditions are met. Only single RS type is supported and at most two different trigger quantities can be configured simultaneously for the evaluation of execution condition of a single candidate cell.

- To support initial and subsequent conditional LTM, the following items can be considered for the configuration of execution condition:

- The CLTM configuration of each candidate cell shall include the execution condition for initial conditional LTM, which is generated by the initial source cell to trigger the CLTM for the candidate cell.

- The CLTM configuration of each candidate cell may include execution conditions for subsequent conditional LTM, which is generated by the candidate cell to trigger the CLTM for other candidate cells when the candidate cell becomes a serving cell.

- The network can configure measurement reports e.g., L1 periodic, semi-persistent, aperiodic and event triggered report, or L3 measurement reports for conditional LTM, e.g., to trigger PDCCH ordered early RACH.

- For CLTM, the Candidate Cell TCI States Activation/Deactivation MAC CE is re-used for the early activation/deactivation of TCI state(s) of a CLTM candidate configuration.

- The Early TA is signalled to the UE from the source cell (i.e., not from the candidate cell directly to the UE). This agreement will be included in the LS to RAN1/3/4.

- The network can inform the candidate cell’s TA information to UE via new MAC CE, which is the TA value when UE switches to that candidate cell during CLTM.

- Candidate cell TA is maintained by a new timer.

- For L1-based conditional LTM the condition evaluation is at MAC level and for L3-based conditional LTM the condition evaluation is at RRC level.

In this contribution, we discussed the conditional LTM for Intra-CU LTM.

Discussion

For Conditional LTM, the network can configure conditions for candidate cell(s). If the UE determines that any of these conditions are met, it initiates an LTM cell switch to the corresponding candidate cell.

In Conditional LTM, since the UE triggers an LTM Cell switch without an LTM Cell switch command MAC CE, the UE needs to receive or derive the information that would typically be included in the LTM Cell switch command MAC CE before performing the C-LTM.

Therefore, we consider it a key point of discussion to determine what information the UE needs to derive or receive, as well as how it should do so.

Figure 1. LTM Cell switch command MAC CE

In Figure 1, the LTM Cell Switch Command MAC CE contains information that enables the UE to perform either RACH-less or RACH-based LTM:

- Target Config ID: Indicates the candidate cell ID.

- Timing Advance Command: Specifies whether the timing advance (TA) is valid. Based on this, the UE determines whether to perform RACH-based or RACH-less LTM.

- TCI State ID: Indicates and activates the TCI state for the LTM target cell.

- Random Access Related Information (preamble index, SSB index, PRACH mask index, C, S/U, repetition number): Provides RACH-related information for CBRA and CFRA procedures.

On the below, we show our views on some aspects of the information that should be used and how it should be received by the UE to support both RACH-less and RACH-based Conditional LTM.

RACH-less Conditional LTM

In RAN2#127bis, it was agreed to support Conditional LTM using both RACH-based and RACH-less methods. For RACH-less Conditional LTM, the CG-based option was agreed, but the DG-based option is still remaining FFS.

In Rel-18/19, RACH-less Intra/Inter-CU LTM can support DG-based mechanisms, as the network transmits the LTM Cell Switch Command via MAC CE to the UE to initiate the cell switch. In other words, since the source cell can inform the target cell that the UE has initiated LTM, the target cell can allocate DG through the PDCCH.

- CG-based RACH-less LTM: The first uplink transmission occurs on a configured grant that is configured before performing LTM.

- DG-based RACH-less LTM: The first uplink transmission occurs on a dynamic grant sent by the target cell

For the DG-based RACH-less C-LTM, the C-LTM candidate cell(s) are not aware of whether the UE has triggered the C-LTM or not. Therefore, to enable the DG-based RACH-less C-LTM, two possible options can be considered:

- Pre-configured DG allocation: The C-LTM candidate cell(s) can pre-allocate DG resources based on predefined criteria to ensure availability when the UE initiates C-LTM.

- Dynamic DG allocation via network assistance: The source cell can notify the target cell when the UE triggers C-LTM, allowing the target cell to allocate DG resources dynamically through PDCCH.

Since the C-LTM candidate cell(s) don’t know whether the UE will trigger C-LTM, they might need to pre-allocate DG resources just in case. We think that this leads to resource underutilization, especially if the UE does not switch to that cell. Additionally, network coordination for DG allocation would cause extra signaling overhead and increase latency. Therefore, we think that CG-based RACH-less C-LTM is sufficient.

Proposal 1: The DG-based RACH-less C-LTM is not supported

In R18 LTM, the TCI state ID indicated in the LTM Cell Switch Command MAC CE is used to determine the validity of a CG occasion. In other words, if any of the associated SSB(s) in the CG occasion match the SSB corresponding to the TCI state ID, the UE considers the CG occasion valid and forwards the SSB information to the PHY.

Therefore, we propose discussing how the UE verifies the validity of a CG occasion for CG-based RACH-less conditional LTM. In our understanding, several options can be considered as follows:

- Option 1: Reuse any suitable beam(s) from the triggered conditional LTM event.

- Option 2: Evaluate the RSRP of the SSB(s) associated with the CG occasion.

For Option 1, the UE reuses the RS that was evaluated and met the conditions during the triggered conditional LTM event. For example, if the beam satisfying the conditional LTM event is SSB X, the UE considers the CG occasion associated with SSB X as valid.

For Option 2, similar to CG-SDT, if at least one SSB configured for CG has an SS-RSRP above the threshold, the CG occasion is considered valid.

Proposal 2: RAN2 is kindly asked to discuss the following options to check the CG validity

- Option 1: Re-use the RS that satisfies the triggered conditional LTM event

- Option 2: Check the RSRP of associated SSB(s)

Timing advance command (TAC)

In R18 LTM, if the Timing Advance Command (TAC) indicated in the LTM cell switch command MAC CE is valid (i.e., not FFF), the UE can perform RACH-less LTM. Therefore, we think that the UE should obtain TAC information for candidate cell(s) to determine whether to proceed with RACH-less or RACH-based conditional LTM.

In RAN2#128, it was agreed that a new MAC CE is introduced to indicate TA value of candidate cell(s).

- The network can inform the candidate cell’s TA information to UE via new MAC CE, which is the TA value when UE switches to that candidate cell during CLTM.

If the UE has a valid TA value for the C-LTM candidate cell(s), it can perform RACH-less C-LTM to the target cell when the L1 event conditions are met.

We think that at least following contents can be considered for new MAC CE.

- C-LTM candidate cell ID(s)

- TA value(s)

We think that the C-LTM candidate cell ID can either be the same as the existing LTM candidate cell ID (LTM-CandidateId) or serve as a separate identifier. In other words, if LTM and C-LTM coexist within the LTM configuration, the existing LTM candidate cell ID can be used. Conversely, if they do not coexist, a new identifier will be used.

Proposal 3: A new MAC CE for the TA value of C-LTM candidate cells includes the following information:

- C-LTM candidate cell ID(s)

- TA value(s)

Conditional LTM failure recovery

In TS 38.300, for the Handover failure recovery, methods for not performing the RRC re-establishment procedure in certain cases are defined as follows:

- When DAPS handover fails, the UE falls back to the source cell configuration, resumes the connection with the source cell, and reports DAPS handover failure via the source without triggering RRC connection re-establishment if the source link has not been released.

- When initial CHO execution attempt fails or HO fails, the UE performs cell selection, and if the selected cell is a CHO candidate and if network configured the UE to try CHO after handover/CHO failure, then the UE attempts CHO execution once, otherwise re-establishment is performed.

- When LTM execution attempt triggered by LTM cell switch command MAC CE fails, the UE performs cell selection and if the selected cell is an LTM candidate cell and if network configured the UE to try LTM after LTM execution failure, then the UE attempts RACH-based LTM execution once, otherwise re-establishment is performed.

In other words, after an LTM failure, if attemptLTM-Switch is configured and the selected cell is one of the LTM candidate cells, the UE can attempt the LTM cell switch again. And if attemptLTM-Switch is not configured and the selected cell is not one of the LTM candidate cells, the UE perform the RRC re-establishment procedure.

For the C-LTM failure case, if the UE selects an Intra-CU cell, we think that the above scheme for attempting LTM can be reused. Additionally, if C-LTM is configured for the selected Intra-CU cell and the TA is valid, we think that C-LTM attempt can also be performed.

Proposal 4: If the UE selects an intra-CU LTM candidate cell after C-LTM failure, R18 LTM failure handling is reused

Proposal 5: If the UE selects an intra-CU LTM candidate cell where C-LTM is configured and the TA is valid after C-LTM failure, the UE attempts C-LTM

In R18, after an LTM failure, only RACH-based LTM execution was supported. Unlike in R18, in C-LTM, the UE has the TA value for the C-LTM candidate cells, allowing it to perform RACH-less based C-LTM. In our understanding, RACH-less based C-LTM can be applied to both LTM failure and C-LTM failure.

Proposal 6: Introduce RACH-less based C-LTM attempt after the (C)-LTM failure

- If the UE selects an intra-CU LTM candidate cell where C-LTM is configured and the TA is valid after (C)-LTM failure, the UE attempts RACH-less based C-LTM execution once

Conclusion

In this contribution, we discussed the conditional LTM for Intra-CU LTM. According to discussion in section 2, we have the following proposals:

Proposal 1: The DG-based RACH-less C-LTM is not supported

Proposal 2: RAN2 is kindly asked to discuss the following options to check the CG validity

- Option 1: Re-use the RS that satisfies the triggered conditional LTM event

- Option 2: Check the RSRP of associated SSB(s)

Proposal 3: A new MAC CE for the TA value of C-LTM candidate cells includes the following information:

- C-LTM candidate cell ID(s)

- TA value(s)

Proposal 4: If the UE selects an intra-CU LTM candidate cell after C-LTM failure, R18 LTM failure handling is reused

Proposal 5: If the UE selects an Intra-CU LTM candidate cell where C-LTM is configured and the TA is valid after C-LTM failure, the UE attempts C-LTM

Proposal 6: Introduce RACH-less based C-LTM attempt after the (C)-LTM failure

- If the UE selects an intra-CU LTM candidate cell where C-LTM is configured and the TA is valid after (C)-LTM failure, the UE attempts RACH-less based C-LTM execution once

Reference

RAN2#128 meeting report