3GPP TSG RAN WG2 Meeting #129                          	   R2-2501165

Athens, Greece, Feb. 17th – 21st, 2025

Agenda item:	8.5.4

Source:	Qualcomm Incorporated

Title:	Discussion on Adaptation of Common Signal/Channel Transmissions

WID/SID:	Netw_Energy_NR_enh-Core – Release 19

Document for:	Discussion

# Introduction

The objectives for adaptation of common signal and channel transmissions are outlined with the follows [1].

The objectives of the work item are the following: - Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g., adapting periodicity - Adaptation of PRACH in time domain - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown

In this contribution, we discuss scenarios or options for adaptation of SSB in time domain, adaptation of PRACH in time domain and adaptation of paging occasions, and the potential impact to RAN2’s specification work.

# Discussion

## Adaptation of SSB in time domain

RAN4 sent the following LS (R1-2409350) related to on-demand SSB SCell operation and SSB adaptation. The last three questions are related to SSB adaptation in time.

1. Overall Description: In RAN4#112bis meeting, RAN4 discussed the impacts on RRM requirements for on-demand SSB in Obj. 1 and SSB adaptation in Obj. 3. During the discussion, RAN4 identified the following questions. [RAN4 Questions]: Obj. 1 related to on-demand SSB SCell operation_ Based on the discussion, RAN4 identified that the relation between always-on SSB and on-demand SSB (i.e. Case#2 as agreed in RAN1) has impacts on UE’s behavior and requirements in SCell. RAN4 would like RAN1 to clarify at least the following aspects: - Q1: What is the relation in terms of periodicity between always-on SSB and OD-SSB? - Q2: What is the relation in terms of time location between always-on SSB and OD-SSB? - Q3: What is the relation in terms of frequency location between the always-on SSB and OD-SSB? - Q4: What is the spatial relation between the always-on SSB and OD-SSB? Obj.3 related to SSB adaptation in time domain For the SSB adaption in the objective of adaptation of common signal/channel transmits, RAN4 would also like RAN1 to clarify the relation for SSB adaptation at least for following aspects. - Q1: What is the relation in terms of time location before and after SSB adaptation? - Q2: What is the relation in terms of frequency location before and after SSB adaptation? - Q3: What is the spatial relation before and after SSB adaptation? 2. Actions: To RAN1: RAN4 respectfully asks RAN1 to answer the questions above.

At RAN1 #119, the following agreements were made for responding RAN4’s questions on SSB adaptation in time [2].

Agreement Reply to Q1(What is the relation in terms of time location before and after SSB adaptation?): - RAN1 agreed that at least SSB burst periodicity is adapted. There are no RAN1 agreements to adapt the time location of the SSB burst other than the periodicity but RAN1 is still discussing other options.

Agreement Reply to Q2(What is the relation in terms of frequency location before and after SSB adaptation?): - The frequency location is same before and after SSB adaptation.

Agreement Reply to Q3(What is the spatial relation before and after SSB adaptation?): - There is no change to spatial relation (in terms of QCL assumption) for the same SSB index before and after SSB adaptation. - At least the case where there is no change to actually transmitted SSBs within a burst before and after SSB adaptation is supported. Further update to be made based on RAN1#119 progress if any.

Based on RAN1’s responses to RAN4’s questions regarding SSB adaptation in time, the SSB adaptation in time with periodicity is similar to the on-demand SSB transmission cases (e.g., the Combination 2, where both always-on SSB and OD-SSB are NCD-SSB and off sync and are allocated at the same frequency, refer to our contribution R2-2410xxx for details).

Observation 1. The SSB adaptation in time (periodicity) may follow the same framework of on-demand SSB Case 2, where the frequency locations of always-on SSB an adapted SSB are the same.

Given the commonality between on-demand SSB transmission and SSB transmission with periodicity adaptation, a unified MAC CE may be used to indicate both on-demand SSB transmission and SSB transmission with periodicity adaptation.

Observation 2.  With the SSB adaptation in the same way as the OD-SSB, the same design (such as MAC CE indication, L3 measurement) may be adopted for SSB adaptation in time.

Proposal 1. For SSB adaptation in time, RAN2 discuss a common framework of the following design subjects for both OD-SIB1 and SSB adaptation

- MAC CE indication

- L3 measurement

## Adaptation of PRACH in time domain

For adaptation of PRACH in time domain, the following agreement wwas made at RAN1 #116 for further study [3].

Agreement For adaptation of PRACH in time-domain, consider the following adaptation mechanisms for further study. - Adaptation based on configuration of additional[/different] PRACH resources for NES-capable UEs in addition to PRACH resources for legacy UEs (if any) - Note: NES-capable UEs can use both additional PRACH resources and PRACH resources for legacy UEs - For the additional PRACH resources, - Adaptation of PRACH resource periodicity/PRACH occasion - Adaptation at PRACH configuration/association period/association pattern period level and SSB to RO mapping cycle - Adaptation based on extending cell DRX operation for PRACH. - Concentrating ROs in time domain - Other options are not precluded

Additionally, the following agreements were made at RAN1 #116bis [4].

Agreement For adaptation of PRACH in time-domain, support at least the following: - Adaptation based on additional PRACH resources for NES-capable UEs in addition to PRACH resources for legacy UEs (if any) - Note: NES-capable UEs can use both additional PRACH resources and PRACH resources for legacy UEs - Configuration of additional PRACH resources is provided by semi-static signalling - FFS: details including whether there is overlap of additional PRACH resources and PRACH resources for legacy UEs - FFS: adaptation mechanism for additional PRACH resources - Note: No change to the existing PRACH configuration tables in 38.211 Agreement For adaptation of PRACH in time-domain, support the following: - SSB-RO mapping for the additional PRACH resources is separate from the SSB-RO mapping of the PRACH resources for legacy UEs (if any) - FFS: whether/how to handle SSB-RO mapping if the additional PRACH resources overlap in both time and frequency with the PRACH resources for legacy UEs - Note: SSB-RO mapping of the PRACH resources for legacy UEs is not impacted if Rel-19 UE uses these PRACH resources - FFS: SSB-RO mapping for the additional PRACH resources Agreement Support adaptation mechanisms of PRACH in time-domain for following: - UE in idle/inactive mode - UE in connected mode

Furthermore, following agreements were made at RAN1 #117 for the configuration of additional PRACH resources [5]:

Agreement For adaptation of PRACH in time-domain, support at least the following case(s) - Case 1: no time-domain overlap between the additional PRACH resources for NES-capable UEs and the PRACH resources for legacy UEs - Case 2: time-domain overlap but no overlap in frequency domain between the additional PRACH resources for NES-capable UEs and the PRACH resources for legacy UEs - Case 3: additional PRACH resources for NES-capable UEs and legacy PRACH resources overlap neither in time nor frequency domains - FFS: whether additional conditions are needed to support the above cases - FFS: Additional case whether full/partial overlap in both time and frequency is allowed - Above does not preclude discussion for the case where the configuration for additional PRACH resources contains legacy PRACH resources Agreement For adaptation of PRACH in time-domain, the additional PRACH resources are configured based on at least: - a PRACH configuration index - FFS: whether the PRACH configuration index is same and/or different from the PRACH configuration index for the legacy PRACH resources Study further the following - When the PRACH configuration index for the additional PRACH resources is same as the PRACH configuration index for the legacy resource, - Additional parameter(s) for determining the additional PRACH resources e.g. - Scaled/adjusted PRACH configuration period - Additional timing offset - Adjusting the parameters (e.g., (x, y) value and slot number) of the PRACH configuration - Muting/masking ROs - When the PRACH configuration index for the additional PRACH resources is different from the PRACH configuration index for the legacy resource - Additional mechanisms (if any) for determining the additional PRACH resources e.g. - Muting/masking ROs (e.g. for the case when the PRACH configuration index for the additional PRACH resources contains legacy resources) - Additional parameters to facilitate condensed/cluster RACH resources in time-domain (including whether needed)

For configuration of additional PRACH resources, the following agreement was made in RAN1#118 [6].

Agreement For adaptation of PRACH in time-domain, select at least one from the following alternatives for configuration of the additional PRACH resources - Alt 1: The PRACH configuration index for the additional PRACH resources is same as the PRACH configuration index for the legacy resources and - Discuss further additional mechanism(s) for determining the additional PRACH resources, e.g. - Opt 1-1: Scaled/adjusted PRACH configuration period - Opt 1-2: Adjusting the parameters (e.g., (x, y) value and slot number) of the PRACH configuration - Opt 1-3: Muting/masking ROs - Opt 1-4: additional timing offset(s) - Alt 2: The PRACH configuration index for the additional PRACH resources is different from the PRACH configuration index for the legacy resources, - Discuss further additional mechanism(s) for determining the additional PRACH resources, e.g. - Opt 2-1: Muting/masking ROs (e.g. for the case when the PRACH configuration index for the additional PRACH resources contains legacy resources) - Opt 2-2: Additional timing offset(s) - FFS: Additional parameters to facilitate condensed/cluster RACH resources in time-domain (including whether needed) - FFS: Additional frequency domain parameter(s) (e.g., freq. starting offset)

At RAN1 #118bis, more agreements were made for supporting adaptation of PRACH in time-domain [7].

Agreement For adaptation of PRACH in time-domain, the same PRACH preamble format is used for the additional RACH resources and legacy PRACH resources. Agreement For adaptation of PRACH in time-domain, support both of the following - Alt 1: The PRACH configuration index for the additional PRACH resources is same as the PRACH configuration index for the legacy resources - Alt 2: The PRACH configuration index for the additional PRACH resources is different from the PRACH configuration index for the legacy resources - FFS: Additional details Working Assumption For DCI-based adaptation for additional PRACH resources, at least DCI format 1_0 can carry the adaptation indication for UEs in idle/inactive and connected mode. - P-RNTI is used Agreement For adaptation of PRACH in time-domain, the frequency domain resources for the additional PRACH resources and legacy PRACH resources can be same or different - FFS: applicable case(s) (i.e., case(s) from the RAN1#117 agreement). - Discuss further following options for signaling - Option 1: at least the following parameter(s) can be configured separately for the additional PRACH resources. - msg1-FrequencyStart at least for 4-step RACH - FFS: other applicable legacy frequency domain parameter(s) - Option 2: Offset to legacy frequency domain parameter(s) are configured for the additional PRACH resources - Note: Offset to legacy frequency domain parameter(s) is a new parameter - FFS: applicable legacy frequency domain parameter(s) Agreement For adaptation of PRACH in time-domain, for determining the additional PRACH resources in time-domain, - For Alt 1 (same PRACH configuration index for additional and legacy PRACH resources), select one or more of the following additional mechanism(s), - Opt 1-2a: up to N1 additional value(s) of (x, y) - FFS: value of N1(>=1) - Opt 1-2b: up to N2 additional value(s) of y - FFS: value of N2 (>=1) - Opt 1-3: Muting/masking ROs - Opt 1-4: - up to N4_1 additional timing offset(s) at frame-level - up to N4_2 additional timing offset(s) at slot-level - FFS: values of N4_1(>=1) and N4_2(>=1) - Opt 1-5: No additional mechanism is selected. - Note: x and y refer to the parameters from the random-access configuration tables (from 38.211) - For Alt 2 (different PRACH configuration index for additional and legacy PRACH resources), select one or more of the following additional mechanism(s), - Opt 1-2a: up to N1 additional value(s) of (x, y) - FFS: value of N1(>=1) - Opt 1-2b: up to N2 additional value(s) of y - FFS: value of N2 (>=1) - Opt 1-3: Muting/masking ROs - Opt 1-4: - up to N4_1 additional timing offset(s) at frame-level - up to N4_2 additional timing offset(s) at slot-level - FFS: values of N4_1(>=1) and N4_2(>=1) - Opt 1-5: No additional mechanism is selected. - Note: x and y refer to the parameters from the random-access configuration tables (from 38.211)

At RAN1 #119, more agreements were made for supporting adaptation of PRACH in time-domain [2].

Agreement At least msg1-FrequencyStart can be configured separately for the additional PRACH resources at least for 4-step RACH. Agreement For DCI-based adaptation for additional PRACH resources, select only from the following alternatives: - Alt 1: DCI-based adaptation to indicate whether the additional PRACH resources provided by semi-static signalling are available or not - [DCI payload size = 1 bit] - FFS: A single PRACH mask provided by semi-static signalling is used to identify the subset of the additional PRACH resources - FFS: details Agreement Confirm the following working assumption from RAN1#118bis. Working Assumption For DCI-based adaptation for additional PRACH resources, at least DCI format 1_0 can carry the adaptation indication for UEs in idle/inactive and connected mode. - P-RNTI is used

Proposal 2. For PRACH adaptation in time domain, RAN2 waits for RAN1’s further decisions.

# Conclusion

In this contribution, we discussed scenarios or options for adaptation of SSB in time domain, adaptation of PRACH in time domain and adaptation of paging occasions and concluded with the following observations and proposals.

Adaptation of SSB in time domain

Observation 1. The SSB adaptation in time (periodicity) may follow the same framework of on-demand SSB Case 2, where the frequency locations of always-on SSB an adapted SSB are the same.

Observation 2.  With the SSB adaptation in the same way as the OD-SSB, the same design (such as MAC CE indication, L3 measurement) may be adopted for SSB adaptation in time.

Proposal 1. For SSB adaptation in time, RAN2 discuss a common framework of the following design subjects for both OD-SIB1 and SSB adaptation

- MAC CE indication

- L3 measurement

Adaptation of PRACH in time domain

Proposal 2. For PRACH adaptation in time domain, RAN2 waits for RAN1’s further decisions.

# References

[1] RP-242354 “Revised WID: Enhancements of network energy savings for NR”, RAN Meeting #105, Melbourne, Australia, Sep 9-12, 2024.

[2] “Chair notes RAN1#119 eom0”, RAN1 #119, Orlando, US, November 18th – 22nd, 2024

[3] “Chair notes RAN1#116 eom0” RAN1 #116	 Athens, Greece, February 26th – March 1st, 2024

[4] “Chair notes RAN1#116bis eom0” RAN1 #116bis	 Changsha, China, April 15th – 19th, 2024

[5] “Chair notes RAN1#117 eom0” RAN1 #117 Fukuoka, Japan, May 20th – 24th, 2024, 2024

[6] “Chair notes RAN1#118 eom0” RAN1 #118, Maastricht, NL, August 19th – 23rd, 2024

[7] “Chair notes RAN1#118bis eom0” RAN1 #118bis, Hefei, China, October 14th – 18th, 2024