3GPP TSG RAN WG2 Meeting #129                          	   R2-2501166

Athens, Greece, Feb. 17th – 21st, 2025

Agenda item:	8.12.2

Source:	Qualcomm Incorporated

Title:	Discussion on UL only mTRP

WID/SID:	NR_MIMO_Ph5-Core – Release 19

Document for:	Discussion

# Introduction

For asymmetric DL sTRP/UL mTRP deployment scenarios, the updated subject (addition of two TA) was outlined in the WID of NR MIMO Phase 5” [1].

- Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2 - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP. - Two TAs through reusing Rel-18 specification of two TAs for multi-DCI-based multi-TRP and removing the restriction that coresetPoolIndex needs to be configured, assuming legacy PRACH resources.

In this contribution, we discuss potential RAN2 work for supporting asymmetric DL sTRP/UL mTRP.

# Discussion

## MAC CE for PL offset

At RAN1 #116bis, the following agreements were made for PL offset [2].

Agreement For FR1, a joint TCI state can be associated with a PL offset. - When a joint TCI state associated with a PL offset is applied for the PUSCH/PUCCH/SRS transmission, the UE shall calculate the Tx power of the PUSCH/PUCCH/SRS based on the DL PL RS and PL offset associated with this joint TCI state. - Reuse the legacy uplink power control formulation by replacing legacy PL with a PL which is derived from the DL PL RS and the PL offset. - FFS: The UE can update UL PL in a way that new UL PL = current UL PL + an update delta indicated by the NW.

Agreement Support applying PL offset on PDCCH-order PRACH towards a UL TRP in FR1. - Note: The DL reference timing determination for PDCCH-order PRACH transmission to an UL TRP is still based on the DL RS defined in current RAN4 specification - Above is subject to a separate UE capability signaling

At RAN1 #117, the following agreements were made for PL offset [3].

Agreement For the association between PL offset and joint/UL TCI state, support the following Alt1b: One PL offset value is configured in a joint or UL TCI state by RRC, where different PL offset values can be configured to different joint or UL TCI states. A MAC CE can update the PL offset value(s) for joint or UL TCI state(s).

At RAN1 #118, the following agreements were made for PL offset [4].

Agreement For indicating a PL offset for PDCCH-order PRACH transmission at least for FR1, support Alt3: - Alt3: The PL offset associated with one of the indicated joint/UL TCI state for UL TRP in unified TCI framework is applied on the PDCCH-order PRACH transmission - FFS the detailed design of DCI format: e.g., how to indicate one of the indicated joint/UL TCI states or whether to apply the PL offset in the indicated TCI state or not. Agreement For the asymmetric DL sTRP/UL mTRP scenarios, the value range of PL offset includes at least [-10, 60] dB - FFS: Extending the range to lower than -10dB Step size is 4dB

Based on RAN1’s agreement, a PL offset value is configured for a joint or UL TCI state by RRC and can be updated via a MAC CE. However, it is still not clear if the PL offset value updated via a MAC CE is associated to the active TCI state or configured TCI state for SRS.

At RAN2 #128, the following agreements were made to the MAC CE indicating PL offset [6].

Agreement - New MAC CE is introduced for PL offset update for asymmetric DL sTRP/UL mTRP. This new MAC CE is identified by new eLCID. - Absolute value of PL offset is indicated in the new MAC CE. For the offset value, the value range is [-12, 60] dB and the step size is 4dB. - In the MAC CE, PL offset value can be updated for any configured TCI states with RRC configured PL offset, i.e., not limited to the activated TCI states.

Based on the above RAN1 and RAN2 agreements, at least the following fields may be considered for the PL offset MAC CE design for unified TCI state, as shown in Figure 1 with TCI state IDs.

- Cell ID,

- BWP ID,

- TCI state IDs or TCI state bitmap, and

- PL offset absolute values.

Figure 1. Example MAC CE for unified TCI state

Proposal 1. At least the following fields are supported for the PL offset MAC CE design for TCI states with RRC configured PL offset values:

- Cell ID,

- BWP ID,

- TCI state IDs or TCI state bitmap, and

- PL offset absolute values.

As agreed at RAN1, the PL offset MAC CE supports joint DL/UL TCI states and separate DL/UL TCI states. Additionally, the PL offset MAC CE also supports both Release 17 and Release 18 unified TCI states.

To support joint DL/UL TCI states and separate DL/UL TCI states, a common MAC CE supporting both joint and separate DL/UL TCI states or separate MAC CEs respectively supporting joint and separate DL/UL TCI states may be applied.

Proposal 2. RAN2 discusses whether a common MAC CE or separate MAC CEs for supporting joint DL/UL TCI states and separate DL/UL TCI states.

To support Release 17 and Release 18 unified TCI states, a common MAC CE supporting both Release 17 and Release 18 unified TCI states or separate MAC CEs respectively supporting Release 17 and Release 18 unified TCI states may also be applied.

Proposal 3. RAN2 discusses whether a common MAC CE or separate MAC CEs for supporting Release 17 and Release 18 unified TCI states

## Two TAs

As shown in Figure 1, a UE may have two TAs where one TA (e.g., TA1) is for the uplink transmission to DL sTRP and the other TA (e.g., TA2) is for the uplink transmission to UL mTRP.

With initial RACH procedure, a UE may acquire an initial Timing Advance Command (TAC) via RAR.

Figure 1. Two TAs for asymmetric DL sTRP and UL mTRP

At RAN1 #118bis, the following agreements were made for 2TA [5].

Agreement Support 2TA for the asymmetric DL sTRP/UL mTRP deployment scenarios: - Remove the restriction that coresetPoolIndex needs to be configured for the 2TA feature. - One downlink reference timing is supported and applied to both TAGs. - (FFS) Note: UE autonomous TA adjustment is only applicable to the first TAG - One single n-TimingAdvanceoffset is configured and applied to both TAGs. - Any of the TCI states can be associated with any one of the two TAGs. - The RAR carrying TA adjustment for those 2 TAGs is reused for Rel-19 2TA - The MAC CE based TA adjustment for 2 TAGs is reused for Rel-19 2TA. - Introduce the optional UE capability of “Overlapping UL transmission reduction” for Rel-19 2TA - If UE does not report this UE capability, UE does not expect two UL transmissions associated with different TAGs are overlapped. - FFS: UE does not expect that in intra-slot TDM PUSCH type-B repetition transmission, two consecutive repetitions associated with different TAGs are overlapped.

Observation 1. RAN1’s agreements have outlined the basics of two-TA design considerations.

Furthermore, at RAN1 #119, the following agreements were made for 2TA [7].

Agreement For a UE supporting the UE capability of “Overlapping UL transmission reduction” of Rel-19 2TA: The overlapping duration of the latter one of two overlapping UL transmissions is reduced, where the two overlapped UL transmissions are associated with two TAGs.

For asymmetric DL sTRP/UL mTRP deployment scenarios, the RACH for UL mTRP can be PDCCH order RACH procedure which can be used for acquiring the initial TA for UL mTRP.

Observation 2. To acquire the initial TA for UL mTRP, PDCCH order procedure can be used.

Based on RAN1’s agreements above, RAN2 may start discussing the spec impact with the supporting of two TAs. RAN2 may start discussing PDCCH order RACH procedure for TA acquisition.

Proposal 4. Based on RAN1’s agreements on two TAs, RAN2 starts to discuss spec impact with two TAs.

Proposal 5. PDCCH order RACH procedure may be the starting point for TA acquisition.

# Conclusion

In this contribution, we discussed potential RAN2 work for supporting asymmetric DL sTRP/UL mTRP and concluded with the following observations and proposals.

MAC CE for PL offset

Proposal 1. At least the following fields are supported for the PL offset MAC CE design for TCI states with RRC configured PL offset values:

- Cell ID,

- BWP ID,

- TCI state IDs or TCI state bitmap, and

- PL offset absolute values.

Proposal 2. RAN2 discusses whether a common MAC CE or separate MAC CEs for supporting joint DL/UL TCI states and separate DL/UL TCI states.

Proposal 3. RAN2 discusses whether a common MAC CE or separate MAC CEs for supporting Release 17 and Release 18 unified TCI states.

Two TAs

Observation 1. RAN1’s agreements have outlined the basics of two-TA design considerations.

Observation 2. To acquire the initial TA for UL mTRP, PDCCH order procedure can be used.

Proposal 4. Based on RAN1’s agreements on two TAs, RAN2 starts to discuss spec impact with two TAs.

Proposal 5. PDCCH order RACH procedure may be the starting point for TA acquisition.

# References

[1] RP-242394, “WID revision: NR MIMO Phase 5”, RAN Meeting #105, Melbourne, Australia, September 9-12, 2024.

[2] “Chair notes RAN1#116bis eom0” RAN1 #116bis	 Changsha, China, April 15th – 19th, 2024

[3] “Chair notes RAN1#117 eom0” RAN1 #117 Fukuoka, Japan, May 20th – 24th, 2024, 2024

[4] “Chair notes RAN1#118 eom0” RAN1 #118, Maastricht, NL, August 19th – 23rd, 2024

[5] “Chair notes RAN1#118bis eom0” RAN1 #118bis, Hefei, China, October 14th – 18th, 2024

[6] R2-2410912, “Report from session on Rel-18 MIMO, Rel-19 MIMO, LPWUS, and SBFD”, RAN2 #128, Orlando, US, November 18th – 22nd, 2024

[7] “Chair notes RAN1#119 eom0”, RAN1 #119, Orlando, US, November 18th – 22nd, 2024