3GPP TSG-RAN WG2 Meeting #129	R2-2501187

Athens, Greece, 17 – 21 February 2025

Agenda item:	6.1.3.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on inconsistent reporting of NR SA capabilities

WID/SID:	NR_newRAT-Core, TEI17

Document for:	Discussion and Decision

# 1	Introduction

We have encountered the following inter-operability issue: an eNB triggers an inter-RAT handover to a cell in an NR band indicated as supported for NR SA in the UE’s LTE capabilities; however, the handover is rejected by the target gNB because the UE does not indicate in its NR capabilities any band combination where that same NR band is present with both UL and DL bandwidth classes, i.e. the gNB does not perceive that the UE supports standalone PCell operation in that band. Although this behaviour is undesirable, it is our understanding that the specifications do not have any restriction in place to prevent this issue. This paper discusses the issue in more detail and presents a few options to address it for further discussion in RAN2.

# 2	Discussion

## 2.1	Background of the issue

TS 36.331 defines the field supportedBandListNR-SA within UE-EUTRA-Capability for reporting supported (target) NR bands for inter-RAT handover/redirection from LTE to NR:

SupportedBandListNR-SA Includes the NR bands supported by the UE in NR-SA (for handover and redirection). The field is included in case the UE supports NR SA as specified in TS 38.331 [32] and not otherwise. The presence of this field also indicates that the UE can perform both NR SS-RSRP and SS-RSRQ measurement in the included NR band(s) as specified in TS 38.215 [89].

Then, with respect to (LTE and NR) capability reporting from the UE, TS 38.331 states the following in clause 5.6.1.4 (see complete copy of the clause in the Annex):

The UE invokes the procedures in this clause if the NR or E-UTRA network requests UE capabilities for nr, eutra-nr or eutra. This procedure is invoked once per requested rat-Type (see clause 5.6.1.3 for capability enquiry by the NR network; see TS 36.331 [10], clause 5.6.3.3 for capability enquiry by the E-UTRA network). The UE shall ensure that the feature set IDs are consistent across feature sets, feature set combinations and band combinations in all three UE capability containers that the network queries with the same fields with the same values, i.e. UE-CapabilityRequestFilterNR, UE-CapabilityRequestFilterCommon, rrc-SegAllowed and fields in UECapabilityEnquiry message (i.e. requestedFreqBandsNR-MRDC, requestedCapabilityNR, eutra-nr-only flag, requestedCapabilityCommon and rrc-SegAllowed) as defined in TS 36.331 [10], where applicable.

<other lines omitted…>

The UE shall:

1>	compile a list of "candidate band combinations" according to the filter criteria in capabilityRequestFilterCommon (if included), only consisting of bands included in frequencyBandListFilter, and prioritized in the order of frequencyBandListFilter (i.e. first include band combinations containing the first-listed band, then include remaining band combinations containing the second-listed band, and so on), where for each band in the band combination, the parameters of the band do not exceed maxBandwidthRequestedDL, maxBandwidthRequestedUL, maxCarriersRequestedDL, maxCarriersRequestedUL, ca-BandwidthClassDL-EUTRA or ca-BandwidthClassUL-EUTRA, whichever are received;

<other lines omitted…>

1>	if the requested rat-Type is nr:

2>	include into supportedBandCombinationList as many NR-only band combinations as possible from the list of "candidate band combinations", starting from the first entry;

<other lines omitted…>

1>	else, if the requested rat-Type is eutra-nr:

2>	include into supportedBandCombinationList and/or supportedBandCombinationListNEDC-Only as many E-UTRA-NR band combinations as possible from the list of "candidate band combinations", starting from the first entry;

According to the yellow highlighted part above, a UE may omit some band combinations it supports when compiling supportedBandCombinationList in the NR and/or MRDC capability containers, e.g. due to excessive capability container sizes. In particular, when building supportedBandCombinationList, it is possible that a UE could remove all band combinations where the BandParameters associated with a particular NR band include both the DL and UL bandwidth classes (ca-BandwidthClassDL-NR and ca-BandwidthClassUL-NR, respectively), while still reporting a band combination where that band only has one of ca-BandwidthClassDL-NR or ca-BandwidthClassUL-NR in its BandParameters. In this case the gNB would only understand that the UE supports that NR band for SCell configuration and would not consider the band as a candidate for PCell, including standalone PCell operation.

What we have observed is some cases where UEs are indicating certain NR bands, e.g. n28, in supportedBandListNR-SA at the LTE side, while excluding the same band as a PCell candidate (i.e. without both ca-BandwidthClassDL-NR and ca-BandwidthClassUL-NR) when indicating their supportedBandCombinationList at the NR side.

As a consequence of this, what we have seen is a scenario where an eNB triggers inter-RAT handover to an NR cell in such a band (based on the eNB’s understanding of the UE capabilities reported in supportedBandListNR-SA) but on the gNB side the handover gets rejected due to that band not being supported for standalone PCell operation (based on the gNB’s understanding of the UE capabilities reported in supportedBandCombinationList).

Observation 1: There are cases where UEs can report a band as supported for NR standalone operation in the LTE capabilities while the same band is not reported as supported for NR standalone PCell operation in the NR or MRDC capabilities.

Observation 2: The above described scenario may lead to inter-RAT handovers being rejected by a gNB.

Since this is technically compliant with the specifications (although an unintended outcome) we think it is an issue that should be discussed further by RAN2.

## 2.2	How to address the issue

In our view, there could be a few different ways to address this issue.

Option A: Add a normative clarification about ensuring consistency across LTE and NR capabilities when reporting bands supported for NR standalone PCell operation.

Since the issue stems from an inconsistency between the LTE and NR (or MRDC) capability containers, one way to address it would be to add a statement about reporting bands supported for NR standalone consistently across LTE and NR at the beginning of clause 5.6.1.4 of TS 38.331, similar to what has been specified in the green highlighted part above for reporting feature sets consistently. For example, the statement could look something like:

“The UE shall ensure that for each band reported by the UE in supportedBandListNR-SA within UE-EUTRA-Capability (as defined in TS 36.331 [10]) and requested by the network in frequencyBandListFilter at least one band combination with both ca-BandwidthClassDL-NR and ca-BandwidthClassUL-NR indicated for that band is included within supportedBandCombinationList.”

The advantage of this approach is that it is clarified at the beginning of the clause without interruption to the procedural text. Also, since this first paragraph of the clause is applicable to both LTE and NR (and also MRDC), there is no need to add multiple clarifications throughout the different procedural branches. It is also follows earlier principles where we have added clarifications about reporting capabilities consistently to the beginning of the clause.

Option B: Add a normative clarification about ensuring not to remove all band combinations with NR band in PCell role when building the supportedBandCombinationList when that NR band is indicated for NR standalone operation in LTE.

This clarification would associated with the yellow highlighted part above, and could look something like the following in the case of the NR branch of the procedure (while a similar clarification could be added to the MRDC branch):

1>	if the requested rat-Type is nr:

2>	include into supportedBandCombinationList as many NR-only band combinations as possible from the list of "candidate band combinations", starting from the first entry, and (if UE includes supportedBandListNR-SA within UE-EUTRA-Capability as specified in TS 36.331 [10]) ensure that for each band reported by the UE in supportedBandListNR-SA and requested by the network in frequencyBandListFilter at least one band combination is included with both ca-BandwidthClassDL-NR and ca-BandwidthClassUL-NR indicated for that band;

Although Option B is quite explicit about what requirements must be taken when compiling the supportedBandCombinationList, this option seems quite disjointed compared to Option A because it cross-references another specification directly within the procedural steps and requires the same clarification to be made twice in the procedure: once in the NR branch and once in the MRDC branch.

Option C: Add a NOTE recommending consistency when reporting NR bands for standalone operation within the LTE and NR capabilities.

For this option the intention of the NOTE would be to clarify an expected “good behaviour” from the UE to report the capabilities consistently, like the following:

NOTE:	If the UE cannot include all candidate band combinations in the supportedBandCombinationList, for consistency, the UE should ensure that for each band reported by the UE in supportedBandListNR-SA within UE-EUTRA-Capability (see TS 36.331 [10]) and requested by the network in frequencyBandListFilter at least one band combination with both ca-BandwidthClassDL-NR and ca-BandwidthClassUL-NR indicated for that band is included within supportedBandCombinationList.

Since a NOTE is not normative, it would be understood that there is no requirement to maintain this consistency, and hence there could still be cases where inter-RAT handovers are rejected. On the other hand, a NOTE might be seen as preferable by some companies since Option A and Option B would likely enforce new requirements on UEs already in the field.

Regarding the UE impact, although it is noted that this issue could technically occur since Rel-15, we have only recently started to observe it as capability sizes have become more excessive in later releases, e.g. Rel-17. From our perspective, it would be sufficient to address this from Rel-17, understanding that the issue is not likely to occur with legacy devices. Given that, Option A or B might be considered more feasible if it would only be required to support from Rel-17. (On the other hand, legacy devices could still optionally support the restriction without any compatibility issues.)

Proposal 1: RAN2 discuss how to address this issue:

-	Option A: Add a normative clarification about ensuring consistency across LTE and NR capabilities when reporting bands supported for NR standalone PCell operation.

-	Option B: Add a normative clarification about ensuring not to remove all band combinations with NR band in PCell role when building the supportedBandCombinationList when that NR band is indicated for NR standalone operation in LTE.

-	Option C: Add a NOTE recommending consistency when reporting NR bands for standalone operation within the LTE and NR capabilities.

Proposal 2: Clarification can be made starting from Rel-17, understanding that there would be no compatibility issues for Rel-15/16 UEs implementing the change.

Text proposals based on Options A, B, C are shown in the Annex.

# 3	Conclusion

This document has made the following observations:

Observation 1: There are cases where UEs can report a band as supported for NR standalone operation in the LTE capabilities while the same band is not reported as supported for NR standalone PCell operation in the NR or MRDC capabilities.

Observation 2: The above described scenario may lead to inter-RAT handovers being rejected by a gNB.

And proposed the following:

Proposal 1: RAN2 discuss how to address this issue:

-	Option A: Add a normative clarification about ensuring consistency across LTE and NR capabilities when reporting bands supported for NR standalone PCell operation.

-	Option B: Add a normative clarification about ensuring not to remove all band combinations with NR band in PCell role when building the supportedBandCombinationList when that NR band is indicated for NR standalone operation in LTE.

-	Option C: Add a NOTE recommending consistency when reporting NR bands for standalone operation within the LTE and NR capabilities.

Proposal 2: Clarification can be made starting from Rel-17, understanding that there would be no compatibility issues for Rel-15/16 UEs implementing the change.

# Annex			TS 38.331

#### 5.6.1.4	Setting band combinations, feature set combinations and feature sets supported by the UE

The UE invokes the procedures in this clause if the NR or E-UTRA network requests UE capabilities for nr, eutra-nr or eutra. This procedure is invoked once per requested rat-Type (see clause 5.6.1.3 for capability enquiry by the NR network; see TS 36.331 [10], clause 5.6.3.3 for capability enquiry by the E-UTRA network). The UE shall ensure that the feature set IDs are consistent across feature sets, feature set combinations and band combinations in all three UE capability containers that the network queries with the same fields with the same values, i.e. UE-CapabilityRequestFilterNR, UE-CapabilityRequestFilterCommon, rrc-SegAllowed and fields in UECapabilityEnquiry message (i.e. requestedFreqBandsNR-MRDC, requestedCapabilityNR, eutra-nr-only flag, requestedCapabilityCommon and rrc-SegAllowed) as defined in TS 36.331 [10], where applicable. The UE shall ensure that for each band reported by the UE in supportedBandListNR-SA within UE-EUTRA-Capability (as defined in TS 36.331 [10]) and requested by the network in frequencyBandListFilter at least one band combination with both ca-BandwidthClassDL-NR and ca-BandwidthClassUL-NR indicated for that band is included within supportedBandCombinationList.

NOTE 1:	Capability enquiry without frequencyBandListFilter is not supported.

NOTE 2:	In (NG)EN-DC, the gNB needs the capabilities for RAT types nr and eutra-nr and it uses the featureSets in the UE-NR-Capability together with the featureSetCombinations in the UE-MRDC-Capability to determine the NR UE capabilities for the supported MRDC band combinations. Similarly, the eNB needs the capabilities for RAT types eutra and eutra-nr and it uses the featureSetsEUTRA in the UE-EUTRA-Capability together with the featureSetCombinations in the UE-MRDC-Capability to determine the E-UTRA UE capabilities for the supported MRDC band combinations. Hence, the IDs used in the featureSets must match the IDs referred to in featureSetCombinations across all three containers. The requirement on consistency implies that there are no undefined feature sets and feature set combinations.

NOTE 3:	If the UE cannot include all feature sets and feature set combinations due to message size or list size constraints, it is up to UE implementation which feature sets and feature set combinations it prioritizes.

NOTE 3a:	If the UE cannot include all candidate band combinations in the supportedBandCombinationList, for consistency, the UE should ensure that for each band reported by the UE in supportedBandListNR-SA within UE-EUTRA-Capability (see TS 36.331 [10]) and requested by the network in frequencyBandListFilter at least one band combination with both ca-BandwidthClassDL-NR and ca-BandwidthClassUL-NR indicated for that band is included within supportedBandCombinationList.

The UE shall:

1>	compile a list of "candidate band combinations" according to the filter criteria in capabilityRequestFilterCommon (if included), only consisting of bands included in frequencyBandListFilter, and prioritized in the order of frequencyBandListFilter (i.e. first include band combinations containing the first-listed band, then include remaining band combinations containing the second-listed band, and so on), where for each band in the band combination, the parameters of the band do not exceed maxBandwidthRequestedDL, maxBandwidthRequestedUL, maxCarriersRequestedDL, maxCarriersRequestedUL, ca-BandwidthClassDL-EUTRA or ca-BandwidthClassUL-EUTRA, whichever are received;

1>	for each band combination included in the list of "candidate band combinations":

2>	if the network (E-UTRA) included the eutra-nr-only field, or

2>	if the requested rat-Type is eutra:

3>	remove the NR-only band combination from the list of "candidate band combinations";

NOTE 4:	The (E-UTRA) network may request capabilities for nr but indicate with the eutra-nr-only flag that the UE shall not include any NR band combinations in the UE-NR-Capability. In this case the procedural text above removes all NR-only band combinations from the candidate list and thereby also avoids inclusion of corresponding feature set combinations and feature sets below.

2>	if it is regarded as a fallback band combination with the same capabilities of another band combination included in the list of "candidate band combinations", and

2>	if this fallback band combination is generated by releasing at least one SCell or uplink configuration of SCell or SUL according to TS 38.306 [26]:

3>	remove the band combination from the list of "candidate band combinations";

NOTE 5:	Even if the network requests (only) capabilities for nr, it may include E-UTRA band numbers in the frequencyBandListFilter to ensure that the UE includes all necessary feature sets needed for subsequently requested eutra-nr capabilities. At this point of the procedure the list of "candidate band combinations" contains all NR- and/or E-UTRA-NR band combinations that match the filter (frequencyBandListFilter) provided by the NW and that match the eutra-nr-only flag (if RAT-Type nr is requested by E-UTRA). In the following, this candidate list is used to derive the band combinations, feature set combinations and feature sets to be reported in the requested capability container.

1>	if the requested rat-Type is nr:

2>	include into supportedBandCombinationList as many NR-only band combinations as possible from the list of "candidate band combinations", starting from the first entry, and (if UE includes supportedBandListNR-SA within UE-EUTRA-Capability as specified in TS 36.331 [10]) ensure that for each band reported by the UE in supportedBandListNR-SA and requested by the network in frequencyBandListFilter at least one band combination is included with both ca-BandwidthClassDL-NR and ca-BandwidthClassUL-NR indicated for that band;

3>	if srs-SwitchingTimeRequest is received:

4>	if SRS carrier switching is supported;

5>	include srs-SwitchingTimesListNR and srs-SwitchingAffectedBandsListNR for each band combination;

4>	set srs-SwitchingTimeRequested to true;

2>	include, into featureSetCombinations, the feature set combinations referenced from the supported band combinations as included in supportedBandCombinationList according to the previous;

2>	compile a list of "candidate feature set combinations" referenced from the list of "candidate band combinations" excluding entries (rows in feature set combinations) with same or lower capabilities;

2>	if uplinkTxSwitchRequest is received:

3>	include into supportedBandCombinationList-UplinkTxSwitch as many NR-only band combinations that supported UL TX switching as possible from the list of "candidate band combinations", starting from the first entry;

4>	if srs-SwitchingTimeRequest is received:

5>	if SRS carrier switching is supported;

6>	include srs-SwitchingTimesListNR and srs-SwitchingAffectedBandsListNR for each band combination;

5>	set srs-SwitchingTimeRequested to true;

3>	include, into featureSetCombinations, the feature set combinations referenced from the supported band combinations as included in supportedBandCombinationList-UplinkTxSwitch according to the previous;

NOTE 6:	This list of "candidate feature set combinations" contains the feature set combinations used for NR-only as well as E-UTRA-NR band combinations. It is used to derive a list of NR feature sets referred to from the feature set combinations in the UE-NR-Capability and from the feature set combinations in a UE-MRDC-Capability container.

2>	if sidelinkRequest is received:

3>	for a sidelink band combination the UE included in supportedBandCombinationListSidelinkEUTRA-NR, supportedBandCombinationListSL-RelayDiscovery or supportedBandCombinationListSL-NonRelayDiscovery:

4>	if the UE supports partial sensing for a band of the sidelink band combination, include the partial sensing capabilities for the band using the sl-TransmissionMode2-PartialSensing-r17;

3>	set sidelinkRequested to true;

2>	include into featureSets the feature sets referenced from the "candidate feature set combinations" and may exclude the feature sets with the parameters that exceed any of maxBandwidthRequestedDL, maxBandwidthRequestedUL, maxCarriersRequestedDL or maxCarriersRequestedUL, whichever are received;

1>	else, if the requested rat-Type is eutra-nr:

2>	include into supportedBandCombinationList and/or supportedBandCombinationListNEDC-Only as many E-UTRA-NR band combinations as possible from the list of "candidate band combinations", starting from the first entry, and (if UE includes supportedBandListNR-SA within UE-EUTRA-Capability as specified in TS 36.331 [10]) ensure that for each band reported by the UE in supportedBandListNR-SA and requested by the network in frequencyBandListFilter at least one band combination is included into supportedBandCombinationList with both ca-BandwidthClassDL-NR and ca-BandwidthClassUL-NR indicated for that band;

3>	if srs-SwitchingTimeRequest is received:

4>	if SRS carrier switching is supported;

5>	include srs-SwitchingTimesListNR, srs-SwitchingTimesListEUTRA and srs-SwitchingAffectedBandsListNR for each band combination;

4>	set srs-SwitchingTimeRequested to true;

2>	include, into featureSetCombinations, the feature set combinations referenced from the supported band combinations as included in supportedBandCombinationList according to the previous;

2>	if uplinkTxSwitchRequest is received:

3>	include into supportedBandCombinationList-UplinkTxSwitch as many E-UTRA-NR band combinations that supported UL TX switching as possible from the list of "candidate band combinations", starting from the first entry;

4>	if srs-SwitchingTimeRequest is received:

5>	if SRS carrier switching is supported;

6>	include srs-SwitchingTimesListNR, srs-SwitchingTimesListEUTRA and srs-SwitchingAffectedBandsListNR for each band combination;

5>	set srs-SwitchingTimeRequested to true;

3>	include, into featureSetCombinations, the feature set combinations referenced from the supported band combinations as included in supportedBandCombinationList-UplinkTxSwitch according to the previous;

1>	else (if the requested rat-Type is eutra):

2>	compile a list of "candidate feature set combinations" referenced from the list of "candidate band combinations" excluding entries (rows in feature set combinations) with same or lower capabilities;

NOTE 7:	This list of "candidate feature set combinations" contains the feature set combinations used for E-UTRA-NR band combinations. It is used to derive a list of E-UTRA feature sets referred to from the feature set combinations in a UE-MRDC-Capability container.

2>	include into featureSetsEUTRA (in the UE-EUTRA-Capability) the feature sets referenced from the "candidate feature set combinations" and may exclude the feature sets with the parameters that exceed ca-BandwidthClassDL-EUTRA or ca-BandwidthClassUL-EUTRA, whichever are received;

1>	include the received frequencyBandListFilter in the field appliedFreqBandListFilter of the requested UE capability, except if the requested rat-Type is nr and the network included the eutra-nr-only field;

1>	if the network included ue-CapabilityEnquiryExt:

2>	include the received ue-CapabilityEnquiryExt in the field receivedFilters;