3GPP TSG-RAN WG2 Meeting #129	R2-2501191

Athens, Greece, 17 – 21 February 2025

Agenda item:	8.19

Source:	Nokia

Title:	Capability signalling for overlapping FDD-FDD inter-band CA

WID/SID:	NR_CADC_SUL_R19 - Release 19

Document for:	Discussion and Decision

# 1	Introduction

RAN4 sent an LS to RAN2 in R2-2500041 (R4-2420410 LS on UE capability for FDD-FDD inter-band CA simultaneous Rx/Tx). In the LS RAN4 explained that the 2UL/2DL FDD-FDD inter-band CA combination CA_n5-n8 can be supported using non-concurrent n5 DL and n8 UL since the frequency ranges of n8 UL partially overlaps with n5 DL. Typically, in FDD-FDD inter-band CA all band pairs support simultaneous Rx/Tx (i.e. for band pair nA+nB, (nAUL, nBDL) and (nADL, nBUL) are both supported), but this is not the case for CA_n5-n8, where simultaneous (n5UL, n8DL) is supported but simultaneous (n8UL, n5DL) is not required to be supported.

As this is the first time RAN4 has specified an inter-band FDD-FDD CA combination with band pairs that are not required to support concurrent Rx/Tx operation, RAN4 asked RAN2 to specify the necessary capability signalling for this scenario in a future proof manner, while also taking into account the earlier capability signalling for non-simultaneous Rx/Tx for TDD-TDD band pairs and FDD-TDD band pairs.

In this contribution, we present our views on how the requested capability signalling could be specified in a future proof manner.

# 2	Discussion

## 2.1	Non-simultaneous TDD-TDD, FDD-TDD

So far, RAN2 has only specified signalling for non-simultaneous Rx/Tx for inter-band TDD-TDD and TDD-FDD band pairs based on the capability simultaneousRxTxInterBandCAPerBandPair (see Annex A below).

Since TDD uses the same operating bandwidth for DL and UL (i.e. UE Rx/Tx), for TDD-TDD band pairs, there is no need to differentiate the cases where the respective TDD bands are transmitting or receiving – if the UE does not support (nADL,nBUL) then it also does not support (nAUL,nBUL), and vice versa; hence only one bit per band pair was needed to indicate non-simultaneous Rx/Tx.

Observation 1: For TDD-TDD band pairs, simultaneousRxTxInterBandCAPerBandPair addresses all cases of non-simultaneous Rx/Tx.

On the other hand, for TDD-FDD band pairs, there could be cases where the TDD band only overlaps with (or is close to) the DL or UL operating bandwidth (but not both) of the FDD band (i.e. for TDD-FDD, UE not supporting (nADL,nBUL) does not necessarily imply (nAUL,nBUL) cannot be supported). However, there is no way to differentiate these cases with the current UE capability signalling, as simultaneousRxTxInterBandCAPerBandPair only allows the UE to indicate whether or not the TDD-FDD band pair is supported concurrently for both Rx/Tx and Tx/Rx. This is a limitation with the current capability signalling, then RAN2 could consider fixing at the same time as we address the case of non-simultaneous Rx/Tx for inter-band FDD-FDD CA.

Observation 2: For TDD-FDD band pairs, simultaneousRxTxInterBandCAPerBandPair cannot differentiate between cases where the TDD band would interfere with only one of the Rx or Tx parts (but not both) of the FDD band.

## 2.2	Non-simultaneous FDD-FDD

Now going back to the case of FDD-FDD discussed in RAN4, Figure 2.2-1 shows the example of CA_n5-n8 given in the LS, where n5 DL and n8 UL are not supported concurrently. Since the operating bandwidths of the bands (partially) overlap, the UE’s DL filter for n5 overlaps with n8; so, despite the configured bandwidths of n5DL and n8UL not overlapping, it is still possible that a UE could experience self-interference in n5 reception when it was transmitting on n8. In this case, the UE can always support n5UL with n8DL, but n5DL cannot be supported concurrently with n8UL.

Figure 2.2-1: FDD-FDD inter-band CA_n5-n8

If simultaneousRxTxInterBandCAPerBandPair is applied to FDD-FDD CA_n5-n8, the UE could only indicate to the network that n5 and n8 cannot be used concurrently, but then the network would never configure n5 and n8 together, despite both of the configurations shown @t1 and @t2 in Figure 2.2-1 being valid.

To further illustrate the point, we can consider a hypothetical example with more overlapping bands. For example, FDD-FDD CA_nA-nB-nC-nD where band B partially overlaps with bands C and D as shown in Figure 2.2-2. (Since RAN4 already supports band combinations with many bands and is expected to support even larger band combinations in the future, we think this type of scenario will occur in the future.) In this case neither simultaneous (nBUL, nDDL) nor simultaneous (nBDL nCUL) are supported, even though simultaneous (nBDL, nDUL) and simultaneous (nBUL, nCUL) are supported. If the legacy capability simultaneousRxTxInterBandCAPerBandPair was applied to this band combination, the network would understand that band B can only be configured in CA_nA-nB, even though other configurations including band B could be valid.

| A UL |  | A DL |  |  |  |  |  |  | B UL |  | B DL |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | D UL |  |  | D DL |  |  |  |  |  | C UL |  |  | C DL |
| A UL |  | A DL |  |  |  |  |  |  | B UL |  | B DL |  |  |  |  |  |
|  |  |  |  | D UL |  |  | D DL |  |  |  |  |  | C UL |  |  | C DL |

Figure 2.2-2: FDD-FDD inter-band CA_nA-nB-nC-nD

Observation 3: For FDD-FDD band pairs, simultaneousRxTxInterBandCAPerBandPair cannot differentiate between cases where a UE supports simultaneous Rx/Tx of a first/second band but does not support simultaneous Tx/Rx of the first/second band.

Proposal 1: Introduce new capability signalling to indicate when band pairs of an FDD-FDD inter-band CA combination do not support concurrent operation.

As mentioned above, there is a similar limitation for TDD-FDD band pairs, so to make the signalling more future proof, RAN2 should consider whether to enhance the signalling for simultaneous TDD-FDD operation.

Proposal 2: Generalize the capability signalling for TDD-FDD band pairs where only one of the DL or UL of an FDD band cannot be operated simultaneously with the TDD band.

## 2.3	Required signalling

In this section, we present and analyse two different capability signalling schemes. Both schemes involve similar bitmap structures as the legacy capability simultaneousRxTxInterBandCAPerBandPair.

Scheme 1:

As a starting point, one might consider using two bitmaps of length L * (L – 1) / 2, where L is the number of bands in the band combination (L * (L – 1) / 2 is the number of band pair combinations). For each band pair (x,y) in the band combination, one bitmap could be used to indicate that the UE supports simultaneous Tx/Rx on band x/y (i.e. xUL+yDL) and an additional bitmap could indicate that the UE supports simultaneous Rx/Tx on band x/y (i.e. xDL+yUL). The total number of bits that need to be signalled in Scheme 1 is L * (L – 1).

The resulting bitmaps for CA_n5-n8 and CA_nA-nB-nC-nD (from Figure 2.2-2) are shown in Table 2.3-1 and Table 2.3-2, respectively.

Table 2.3-1: Scheme 1 bitmap for CA_n5-n8

| Bitmap | Value | Meaning |
| --- | --- | --- |
| Bitmap 1 “Simultaneous Tx on band x, Rx on band y” | 1 | UE can Tx on n5 while Rx on n8 |
| Bitmap 2 “Simultaneous Rx on band x, Tx on band y” | 0 | UE cannot Rx on n5 while Tx on n8 |

Table 2.3-1: Scheme 1 bitmap for CA_nA-nB-nC-nD

| Bitmap | Value | Meaning |
| --- | --- | --- |
| Bitmap 1 “Simultaneous Tx on band x, Rx on band y” | 111101 | 111 indicates that UE can Tx on band A while Rx on bands B,C,D. 10 indicates that UE can Tx on B while Rx on C, but UE cannot Tx on B while Rx on D. 1 indicates UE can Tx on C while Rx on D. |
| Bitmap 2 “Simultaneous Rx on band x, Tx on band y” | 111011 | 111 indicates that UE can Rx on band A while Tx on bands B,C,D. 01 indicates that UE cannot Rx on B while Tx on C, but UE can Rx on B while Tx on D. 1 indicates UE can Rx on C while Tx on D. |

Obviously, the number of bits signalled increases for band combinations involving more bands. As an (extreme) example, consider a band combination with 32 bands (the largest band combination supported by simultaneousRxTxInterBandCAPerBandPair), but with only one band pair partially overlapping, e.g. n5 and n8 with 30 other non-overlapping bands. This would require 32 * 31 = 992 bits to be signalled, even though the first bitmap would have 496 1s (and no 0s) and the second bitmap would have a single 0 (corresponding to Rx on n5 while Tx on n8) and the remaining 495 bits would be 1. This motivates us to consider another scheme.

Scheme 2:

This scheme would use three bitmaps, but less bits in total for most cases. Bitmap 1 focuses on bands that cannot simultaneously TxRx in all UL and DL cases, and has a length of L (equal to the number of bands).

Then, only for bands with value 0 in bitmap 1, the UE could send additional bitmaps 2 and 3. Bitmaps 2 and 3 in Scheme 2 would be encoded like bitmaps 1 and 2 of Scheme 1, respectively, i.e. bitmap 2 indicates that the UE supports simultaneous Tx/Rx on band x/y, and bitmap 3 indicates that the UE supports simultaneous Rx/Tx on band x/y. The length of each of bitmaps 2 and 3 is K = M * (M – 1) / 2, where M is number of zeros in bitmap 1, i.e., the length of each of bitmaps 2 and 3 is the number of band pair combinations of the “problematic bands” indicated in bitmap 1. Hence the total number of bits that need to be signalled in Scheme 2 is L + M * (M – 1).

The resulting bitmaps for CA_n5-n8 and CA_nA-nB-nC-nD) are shown in Table 2.3-3 and Table 2.3-4, respectively.

Table 2.3-3: Scheme 2 bitmap for CA_n5-n8

| Bitmap | Value | Meaning |
| --- | --- | --- |
| Bitmap 1 “Bands that cannot simultaneously TxRx in all UL and DL CA configurations” | 00 | Bands n5 and n8 each have some issue simultaneously TxRx with some other band |
| Bitmap 2 “Simultaneous Tx on band x, Rx on band y” | 1 | UE can Tx on n5 while Rx on n8 |
| Bitmap 3 “Simultaneous Rx on band x, Tx on band y” | 0 | UE cannot Rx on n5 while Tx on n8 |

Table 2.3-4: Scheme 2 bitmap for CA_nA-nB-nC-nD

| Bitmap | Value | Meaning |
| --- | --- | --- |
| Bitmap 1 “Bands that cannot simultaneously TxRx in all UL and DL CA configurations” | 1000 | 1 indicates band A has no problems 000 indicates bands B,C,D each have some issue simultaneously TxRx with at least one other band |
| Bitmap 2 “Simultaneous Tx on band x, Rx on band y” | 101 | 10 indicates that UE can Tx on B while Rx on C, but UE cannot Tx on B while Rx on D. 1 indicates that UE can Tx on C while Rx on D. |
| Bitmap 3 “Simultaneous Rx on band x, Tx on band y” | 011 | 01 indicates that UE cannot Rx on B while Tx on C, but UE can Rx on B while Tx on D. 1 indicates that UE can Rx on C while Tx on D. |

Considering again the “extreme case” mentioned above, with 32 bands and only one partially overlapping band pair, e.g. (n5, n8): the bitmap 1 would require 32 bits (one for each band) and two of those bits would be set to 0 for the two bands that overlap. Then, for each of bitmaps 2 and 3, only a single bit needs to be signalled, e.g. corresponding to the band pairs (n5UL,n8DL) and (n5DL,n8UL). Hence, Scheme 2 would only require 34 bits compared to the 992 bits of Scheme 1.

With Scheme 2, the total bits saved is [L * (L – 1)] – [L + M * (M – 1)] = L2 – 2L – M2 + M. For a fixed number of bands L in a band combination, the bit savings are shown in Table 2.3-5.

Table 2.3-5: Scheme 2 bit savings for a fixed number of bands L in a band combination

| Number of overlapping bands M | Bits saved |
| --- | --- |
| M = L (see NOTE 1) | – L |
| M = L – 1 | L – 2 |
| M = L – 2 | 3L – 6 |
| M = L – 3 | 5L – 12 |
| M = L – 4 | 7L – 20 |
| … | … |
| M = L – 30 | 59L – 930 |
| NOTE 1: M = L is the case where every band in the band combination overlaps with at least one other band. This is the only case where Scheme 1 requires less bits to be signalled. |  |

Observation 4: Scheme 2 requires less bits to be signalled except when every band in the band combination overlaps with at least one other band.

This means that Scheme 2 would require 2 extra bits relative to Scheme 1 to indicate non-concurrent CA_n5-n8; however, when in the future RAN4 specifies higher order band combinations with n5 and n8, there would be immediate bit savings. Plus, when higher order BCs involving n5 and n8 are specified, we expect there would be no need to separately indicate CA_n5-n8 due to the fallback principle for band combinations.

Observation 5: When higher order BCs are specified, the limitation on Scheme 2 can be avoided by the fallback principle.

We also note that both schemes can be applied directly to the case of (partially) overlapping or close by TDD-FDD band pairs and are able to differentiate the cases where the TDD band overlaps with or is close to only the DL or UL of the FDD band, overcoming the limitation of the legacy capability simultaneousRxTxInterBandCAPerBandPair when applied to TDD-FDD inter-band CA.

Observation 6: Schemes 1 and 2 can be applied to TDD-FDD inter-band CA, and overcome the limitation from Observation 2.

Based on the above observations we propose to adopt Scheme 2 for inter-band CA combinations with partially overlapping or close by FDD-FDD or TDD-FDD band pairs. Examples of how this could be specified in TS 38.306 and TS 38.331 are shown in Annex B.

Proposal 3: RAN2 introduces new capability signalling applicable to inter-band CA with partially overlapping FDD-FDD band pairs or partially overlapping TDD-FDD band pairs according to the following scheme (Scheme 2):

| Bitmap | Length | Description |
| --- | --- | --- |
| Bitmap 1 | L L is the number of bands in the band combination | Bit x is set to "1" if the UE supports simultaneous TxRx for all band pairs (x, y), in all UL and DL CA configurations, where x and y are the indices of the band entries in the band combination. |
| Bitmap 2 | M * (M – 1) / 2 M is the number of bits set to “0” in Bitmap 1 | Bit K is set to "1" if the UE supports simultaneous Tx on band x and Rx on band y for band pair (x, y), where x and y are the band entries corresponding to mth and nth bits set to "0" in bitmap 1, respectively, m < n, and K = m*(2*M – m – 1)/2 + n – m – 1. |
| Bitmap 3 | M * (M – 1) / 2 M is the number of bits set to “0” in Bitmap 1 | Bit K is set to "1" if the UE supports simultaneous Rx on band x and Tx on band y for band pair (x, y), where x and y are the band entries corresponding to mth and nth bits set to "0" in bitmap 1, respectively, m < n, and K = m*(2*M – m – 1)/2 + n – m – 1. |

## 2.4	Generalization to 2DL+1UL and 1DL+2UL

Lastly we consider another scenario that could be worth addressing at the same time. Referring to the inter-band reference sensitivity exceptions of TS 38.101-1, Table 7.3A.5-2 shows a number of cases where intermodulation interference can be caused by two bands in the UL can seriously desensitize the DL of another band, as shown in Figure 2.4.1 below.

Figure 2.4-1: IMD Interference from UL CA in two bands can seriously desensitize DL in another band

For example, a 31.5 dB n77 maximum sensitivity degradation (MSD) is allowed for band n3DL with UL CA in bands n1 and n77, as specified for CA_n1-n3-n77 in Table 7.3A.5-2, rendering the band n3DL unusable together with UL CA in bands n1 and n77, even though the UE might otherwise support n3DL with n1UL or n77UL separately (i.e. when n1 and n77 are not configured with 2UL). However, there would be no way to indicate this with the current capability signalling.

Observation 7: There are cases where UE could separately support simultaneous Rx/Tx in bands A/B and simultaneous Rx/Tx in A/C, but would not support Rx in band A when Tx is configured in bands B and C at the same time.

RAN2 should consider adding capability signalling for these types of scenarios at the same time. Scheme 1 presented above cannot be adapted to this case; however, Scheme 2 can naturally be extended by adding two additional (optional) bitmaps, Bitmap 4 and Bitmap 5. Bitmap 4 would indicate that the UE supports simultaneous transmission on bands x and y (at the same time) with reception on band z. Bitmap 5 would indicate that the UE supports simultaneous reception on bands x and y (at the same time) with transmission on band z. In this case the length of bitmaps 4 and 5 would be M * M * (M – 1) / 2, i.e. the number of “problematic bands” in Bitmap 1 times the number for two band combinations from the problematic bands in Bitmap 1.

For example, for the case of CA_n1-n3-n77, a UE might indicate the following based on the MSD specified in TS 38.101-1:

Table 2.4-1: Scheme 2 extended bitmap for CA_n1-n3-n77

| Bitmap | Value | Meaning |
| --- | --- | --- |
| Bitmap 1 “Bands that cannot simultaneously TxRx in all UL and DL CA configurations” | 000 | 000 indicates that each band n1, n3, n77 has an issue simultaneously TxRx with at least one other band |
| Bitmap 2 “Simultaneous Tx on band x, Rx on band y” | 111 | 11 indicates that UE can Tx on n3 or n77 while Rx on n1. 1 indicates that UE can Tx on n77 while Rx on n3. |
| Bitmap 3 “Simultaneous Rx on band x, Tx on band y” | 111 | 11 indicates that UE can Rx on n3 or n77 while Tx on n1. 1 indicates that UE can Rx on n77 while Tx on n3. |
| Bitmap 4 “Simultaneous Tx on band x plus y, Rx on band x or y or z” | 110101011 | 110 indicates that UE can Tx on n1 plus n3 or n1 plus n77 while Rx on n1, but cannot Tx on n3 plus n77 while Rx on n1. 101 indicates that UE can Tx on n1 plus n3 or n3 plus n77 while Rx on n3, but cannot Tx on n1 plus n77 while Rx on n3. 011 indicates that UE can Tx on n1 plus n77 or n3 plus n77 while Rx on n77, but cannot Tx on n1 plus n3 while Rx on n77. |
| Bitmap 5 “Simultaneous Rx on band x plus y, Tx on band x or y or z” | 111111111 | 111 indicates that UE can Rx on n1 plus n3 or n1 plus n77 or n3 plus n77 while Tx on n1. 111 indicates that UE can Rx on n1 plus n3 or n1 plus n77 or n3 plus n77 while Tx on n3. 111 indicates that UE can Rx on n1 plus n3 or n1 plus n77 or n3 plus n77 while Tx on n77. |

Proposal 4: RAN2 extends new capability signalling to indicate cases where simultaneous 2UL+1DL or 2DL+1UL are not supported for inter-band FDD-FDD/TDD-FDD CA while simultaneous RxTx are supported for the band pairs separately.

# 3	Conclusion

This document has made the following observations:

Observation 1: For TDD-TDD band pairs, simultaneousRxTxInterBandCAPerBandPair addresses all cases of non-simultaneous Rx/Tx.

Observation 2: For TDD-FDD band pairs, simultaneousRxTxInterBandCAPerBandPair cannot differentiate between cases where the TDD band would interfere with only one of the Rx or Tx parts (but not both) of the FDD band.

Observation 3: For FDD-FDD band pairs, simultaneousRxTxInterBandCAPerBandPair cannot differentiate between cases where a UE supports simultaneous Rx/Tx of a first/second band but does not support simultaneous Tx/Rx of the first/second band.

Observation 4: Scheme 2 requires less bits to be signalled except when every band in the band combination overlaps with at least one other band.

Observation 5: When higher order BCs are specified, the limitation on Scheme 2 can be avoided by the fallback principle.

Observation 6: Schemes 1 and 2 can be applied to TDD-FDD inter-band CA, and overcome the limitation from Observation 2.

Observation 7: There are cases where UE could separately support simultaneous Rx/Tx in bands A/B and simultaneous Rx/Tx in A/C, but would not support Rx in band A when Tx is configured in bands B and C at the same time.

And proposed the following:

Proposal 1: Introduce new capability signalling to indicate when band pairs of an FDD-FDD inter-band CA combination do not support concurrent operation.

Proposal 2: Generalize the capability signalling for TDD-FDD band pairs where only one of the DL or UL of an FDD band cannot be operated simultaneously with the TDD band.

Proposal 3: RAN2 introduces new capability signalling applicable to inter-band CA with partially overlapping FDD-FDD band pairs or partially overlapping TDD-FDD band pairs according to the following scheme (Scheme 2):

| Bitmap | Length | Description |
| --- | --- | --- |
| Bitmap 1 | L L is the number of bands in the band combination | Bit x is set to "1" if the UE supports simultaneous TxRx for all band pairs (x, y), in all UL and DL CA configurations, where x and y are the indices of the band entries in the band combination. |
| Bitmap 2 | M * (M – 1) / 2 M is the number of bits set to “0” in Bitmap 1 | Bit K is set to "1" if the UE supports simultaneous Tx on band x and Rx on band y for band pair (x, y), where x and y are the band entries corresponding to mth and nth bits set to "0" in bitmap 1, respectively, m < n, and K = m*(2*M – m – 1)/2 + n – m – 1. |
| Bitmap 3 | M * (M – 1) / 2 M is the number of bits set to “0” in Bitmap 1 | Bit K is set to "1" if the UE supports simultaneous Rx on band x and Tx on band y for band pair (x, y), where x and y are the band entries corresponding to mth and nth bits set to "0" in bitmap 1, respectively, m < n, and K = m*(2*M – m – 1)/2 + n – m – 1. |

Proposal 4: RAN2 extends new capability signalling to indicate cases where simultaneous 2UL+1DL or 2DL+1UL are not supported for inter-band FDD-FDD/TDD-FDD CA while simultaneous RxTx are supported for the band pairs separately.

# Annex A: Existing Capability Signalling

#### 4.2.7.4	CA-ParametersNR

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| … |  |  |  |  |
| simultaneousRxTxInterBandCA Indicates whether the UE supports simultaneous transmission and reception in TDD-TDD and TDD-FDD inter-band NR CA. If this field is included in ca-ParametersNR-ForDC, it indicates the UE supports simultaneous transmission and reception between any UL/DL band pair within a cell group and across MCG and SCG in TDD-TDD and TDD-FDD inter-band NR-DC. It is mandatory for certain TDD-FDD and TDD-TDD band combinations defined in TS 38.101-1 [2], TS 38.101-2 [3] and TS 38.101-3 [4]. This capability does not apply to the following components within TDD-TDD and TDD-FDD inter-band NR-CA or NR-DC combinations: - Intra-band NR-CA or NR-DC component - Inter-band NR-CA or NR-DC component where the frequency range of one TDD band is a subset of the frequency range of the other NR TDD band (as specified in TS 38.101-1 [2]). | BC | CY | N/A | N/A |
| simultaneousRxTxInterBandCAPerBandPair Indicates whether the UE supports simultaneous transmission and reception in TDD-TDD and TDD-FDD inter-band NR CA for each band pair in the band combination. Encoded as a bitmap with size L * (L – 1) / 2, and bit N (leftmost bit is indexed as bit 0) is set to "1" if the UE supports simultaneous transmission and reception for band pair (x, y), where L is the number of band entries in the band combination, x and y are the indices of the band entry in the band combination (the first band entry is indexed as 0), x < y, and N = x*(2*L – x – 1)/2 + y – x – 1. If this field is included in ca-ParametersNR-ForDC, each bit of this field indicates whether the UE supports simultaneous transmission and reception between each band pair, within a cell group and across MCG and SCG in TDD-TDD and TDD-FDD inter-band NR-DC. The UE does not include this field if the UE supports simultaneous transmission and reception for all applicable band pairs in the band combination (in which case simultaneousRxTxInterBandCA is included) or does not support for any band pair in the band combination. It is mandatory for certain band pairs as specified in TS 38.101-1 [2], TS 38.101-2 [3] and TS 38.101-3 [4]. The UE shall consistently set the bits which correspond to the same band pair. | BC | CY | N/A | N/A |
| … |  |  |  |  |

# Annex B: Example text proposals

## TS 38.301

#### 4.2.7.4	CA-ParametersNR

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| … |  |  |  |  |
| simultaneousRxTxInterBandCAPerBandPair-v1900 Indicates whether the UE supports simultaneous transmission and reception in FDD-FDD and TDD-FDD inter-band NR CA for each band pair in the band combination. Encoded as five bitmaps: - Bitmap 1: has length L, where L is the number of band entries in the band combination. Bit x (leftmost bit is indexed as bit 0) is set to "1" if the UE supports simultaneous transmission and reception for all band pairs (x, y), in all UL and DL CA configurations where x and y are the indices of the band entry in the band combination (the first band entry is indexed as 0). - Bitmap 2: has length M * (M – 1) / 2, where M equals the number of bits set to "0" in bitmap 1. Bit K (leftmost bit is indexed as bit 0) is set to "1" if the UE supports simultaneous transmission on band x and reception on band y for band pair (x, y), where x and y are the band entries corresponding to mth and nth bits set to "0" in bitmap 1, respectively, m < n, and K = m*(2*M – m – 1)/2 + n – m – 1. - Bitmap 3: has length M * (M – 1) / 2, where M equals the number of bits set to "0" in bitmap 1. Bit K (leftmost bit is indexed as bit 0) is set to "1" if the UE supports simultaneous reception on band x and transmission on band y for band pair (x, y), where x and y are the band entries corresponding to mth and nth bits set to "0" in bitmap 1, respectively, m < n, and K = m*(2*M – m – 1)/2 + n – m – 1. - Bitmap 4: has length M * M * (M – 1) / 2, where M equals the number of bits set to "0" in bitmap 1. Bit K (leftmost bit is indexed as bit 0) is set to "1" if the UE supports simultaneous transmission on bands x and y and reception on band z for band triple (x, y, z), where x and y and z are the band entries corresponding to mth and nth and oth bits set to "0" in bitmap 1, respectively, m < n, and K = o*M + m*(2*M – m – 1)/2 + n – m – 1. - Bitmap 5: has length M * M * (M – 1) / 2, where M equals the number of bits set to "0" in bitmap 1. Bit K (leftmost bit is indexed as bit 0) is set to "1" if the UE supports simultaneous reception on bands x and y and transmission on band z for band triple (x, y, z), where x and y and z are the band entries corresponding to mth and nth and oth bits set to "0" in bitmap 1, respectively, m < n, and K = o*M + m*(2*M – m – 1)/2 + n – m – 1. The UE does not include this field if the UE supports simultaneous transmission and reception for all applicable band pairs in the band combination (in which case simultaneousRxTxInterBandCA is included for TDD-FDD) or does not support for any band pair in the band combination. It is mandatory for certain band pairs as specified in TS 38.101-1 [2], TS 38.101-2 [3] and TS 38.101-3 [4]. The UE shall consistently set the bits which correspond to the same band pair. | BC | CY | N/A | N/A |
| … |  |  |  |  |

## TS 38.331

#### –	CA-ParametersNR

The IE CA-ParametersNR contains carrier aggregation and inter-frequency DAPS handover related capabilities that are defined per band combination.

CA-ParametersNR information element

-- ASN1START

-- TAG-CA-PARAMETERSNR-START

CA-ParametersNR ::=                 SEQUENCE {

dummy                                         ENUMERATED {supported}      OPTIONAL,

parallelTxSRS-PUCCH-PUSCH                     ENUMERATED {supported}      OPTIONAL,

parallelTxPRACH-SRS-PUCCH-PUSCH               ENUMERATED {supported}      OPTIONAL,

simultaneousRxTxInterBandCA                   ENUMERATED {supported}      OPTIONAL,

simultaneousRxTxSUL                           ENUMERATED {supported}      OPTIONAL,

diffNumerologyAcrossPUCCH-Group               ENUMERATED {supported}      OPTIONAL,

diffNumerologyWithinPUCCH-GroupSmallerSCS     ENUMERATED {supported}      OPTIONAL,

supportedNumberTAG                            ENUMERATED {n2, n3, n4}     OPTIONAL,

...

}

<fields omitted>

CA-ParametersNR-v1900 ::=           SEQUENCE {

simultaneousRxTxInterBandCAPerBandPair-v1900  SimultaneousRxTxPerBandPairExt    OPTIONAL

}

-- TAG-CA-PARAMETERSNR-STOP

-- ASN1STOP

#### –	SimultaneousRxTxPerBandPairExt

The IE SimultaneousRxTxPerBandPairExt contains the simultaneous Rx/Tx UE capability for each band pair in a FDD-FDD or TDD-FDD band combination.

SimultaneousRxTxPerBandPair information element

-- ASN1START

-- TAG-SIMULTANEOUSRXTXPERBANDPAIR-START

SimultaneousRxTxPerBandPairExt ::=  	SEQUENCE {

impactedBands									BIT STRING (SIZE (2..32))

simultaneousTxWhileRx							BIT STRING (SIZE (1..496))

simultaneousRxWhileTx							BIT STRING (SIZE (1..496))

simultaneous2TxWhileRx							BIT STRING (SIZE (1..496))		OPTIONAL,

simultaneous2RxWhileTx							BIT STRING (SIZE (1..496))		OPTIONAL

}

-- TAG-SIMULTANEOUSRXTXPERBANDPAIREXT-STOP

-- ASN1STOP