3GPP TSG RAN WG2#129	R2-2500008

Athens, Greece, 17th - 21st February 2025

3GPP TSG RAN WG1 #119	R1-2410758

Orlando, US, November 18th – 22nd, 2024

Title:	LS to RAN2 on RRC and MAC impacts for Rel-19 NR MIMO Ph5

Response to:

Release:	Rel-19

Work Item:	NR_MIMO_Ph5

Source:	RAN1

To:	RAN2

Cc:

Contact Person:

#### Name:	Eko Onggosanusi

E-mail Address:	eko.o@samsung.com

1. Overall Description:

Pertaining to the Rel-19 NR MIMO Ph5 WI, RAN1 would like to inform RAN2 about the following:

- A revised list of RRC parameters (excel spreadsheet attached)

- Additional agreements requiring MAC support:

Agreement

Support 2TA for the asymmetric DL sTRP/UL mTRP deployment scenarios:

- Remove the restriction that coresetPoolIndex needs to be configured for the 2TA feature.

- One downlink reference timing is supported and applied to both TAGs.

  - (FFS) Note: UE autonomous TA adjustment is only applicable to the first TAG

- One single n-TimingAdvanceoffset is configured and applied to both TAGs.

- Any of the TCI states can be associated with any one of the two TAGs.

- The RAR carrying TA adjustment for those 2 TAGs is reused for Rel-19 2TA

- The MAC CE based TA adjustment for 2 TAGs is reused for Rel-19 2TA.

- Introduce the optional UE capability of “Overlapping UL transmission reduction” for Rel-19 2TA

  - If UE does not report this UE capability, UE does not expect two UL transmissions associated with different TAGs are overlapped.

- FFS: UE does not expect that in intra-slot TDM PUSCH type-B repetition transmission, two consecutive repetitions associated with different TAGs are overlapped.

“RAN2 could check whether this RAN1 agreement would cause any MAC impact”

2. Actions:

To: RAN2

ACTION: 	RAN1 respectfully asks RAN2 to take the above into account.

3. Date of Next TSG-RAN WG1 Meetings:

TSG RAN WG1 Meeting #120	17th – 21st February, 2025	Athens, Greece