3GPP TSG RAN WG2#129	R2-2500042

Athens, Greece, 17th - 21st February 2025

# 3GPP TSG-RAN WG4 Meeting # 113 	R4-2420476

Orlando, US, 18th – 22nd November, 2024

Title:	Reply LS to RAN1 and RAN2 on FR2-NTN inclusion to specifications

Response to:	(R1-2407406) LS on FR2-NTN inclusion to specifications

Release:	Rel-18

Work Item:	NR_NTN_enh-Core

Source:	RAN4

To:	RAN1, RAN2

Cc:

Contact Person:

Name: Shuai Zhou

E-mail Address: shuai.zhou@vivo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN4 would like to thank RAN1 for LS on FR2-NTN inclusion to specification.

RAN4 would like to confirm that NOTE2 in Table 5.1-1 of 38.101-5 needs to be updated. FR2-NTN bands are regarded as FR2-1 bands when references from other specifications unless otherwise stated. The following change is made to the notes in Table 5.1-1 of 38.101-5:

Table 5.1-1: Definition of NTN frequency ranges

| Frequency range designation | Corresponding frequency range |
| --- | --- |
| FR1-NTN (Note 1) | 410 MHz – 7125 MHz |
| FR2-NTN (Note 2) | 17300 MHz – 30000 MHz |
| NOTE 1: NTN bands within this frequency range are regarded as a FR1 band when references from other specifications. NOTE 2: NTN bands within this frequency range are regarded as a FR2-1 band when references from other specifications unless otherwise stated. |  |

Besides, we also find the similar note specified in Table 8.1C.1-2 in TS 38.133 and Table 5.1-1 in TS 38.108, we believe the same adaptation can be made.

Table 8.1C.1-2: Maximum number of RLM-RS resources NRLM

| Carrier frequency range of PCell |  | Maximum number of RLM-RS resources, NRLM |
| --- | --- | --- |
| FR1, ≤ 3 GHzNote | 4 | 2 |
| FR1, > 3 GHzNote | 8 | 4 |
| FR2-NTNNote 2 | 64 | 8 |
| NOTE: For unpaired spectrum operation with Case C - 30 kHz SCS, 3GHz is replaced by 1.88GHz, as specified in clause 4.1 in TS 38.213 [3]. NOTE 2: NTN bands within this frequency range are regarded as a FR2-1 band when references from other specifications unless otherwise stated. |  |  |

Table 5.1-1: Definition of NTN frequency ranges

| Frequency range designation | Corresponding frequency range (MHz) |
| --- | --- |
| FR1-NTN (NOTE 1) | 410 – 7125 |
| FR2-NTN (NOTE 2) | 17300 – 30000 |
| NOTE 1: NTN bands within this frequency range are regarded as a FR1 band when references from other specifications. NOTE 2: NTN bands within this frequency range are regarded as a FR2-1 band when references from other specifications unless otherwise stated. |  |

RAN4 kindly asks RAN1 and RAN2 to consider the above clarification in RAN4 specifications TS 38.101-5, TS 38.108 and TS 38.133.

2. Actions:

To RAN1:

ACTION: RAN4 respectfully asks RAN1 and RAN2 to take the above information into account.

3. Date of Next TSG WG RAN4 Meetings:

TSG-RAN WG4 Meeting #114	17 - 21 Feb. 2024	Athens, GR

TSG-RAN WG4 Meeting #114-bis	7 - 11 Apr. 2024	China, CN