3GPP TSG-RAN WG2 Meeting #129bis	R2-2501765

Wuhan, China, April 7-11, 2025

Agenda item:	8.7.6

Source:	Qualcomm Incorporated

Title:	Discussion on XR rate control

WID/SID:	NR_XR_Ph3-Core

Document for:	Discussion and decision

# Introduction

In this paper, we discuss a few design issues for uplink rate control, e.g. granularity of rate indication, UL rate query, and bit rate table design.

# Discussion

Granularity of rate indication

For the bit indication between RAN and UE, there can be two options:

- Option A. Bit rates of each QoS flow enabled with rate control can be included in the rate control MAC CE.

- Option B. Only the bit rate for the whole DRB/LCH is included in the rate control MAC CE. After receiving this total bit rate, UE derives the bit rates for different QoS flows based on some RRC configured parameters, e.g. split ratio between the flows.

During the online discussion at RAN2#128, there were concerns about Option A by both infra vendors and UE makers. For example, some RAN implementations may not be able to estimate rates for individual QoS flows. Some UE makers explained the challenges they may face in managing multiple QoS flows within the MAC layer. If rates are indicated dynamically for individual flows, it becomes difficult because, in theory, the MAC layer does not have any visibility of QoS flows.

Considering the variations in network and UE implementations, and the differing impacts of per QoS flow indication on the existing implementations, we think an optional UE capability can serve as a good compromise for moving forward. More specifically,

- UEs support Option B as the baseline, while Option A can be a UE capability;

- Given a UE’s capability, it is up to network implementation to choose which option it applies.

Proposal 1.	Per-QoS-flow rate indication by RRC configuration is the baseline. Support for per-QoS-flow rate indication within MAC CE is an optional UE capability.

Proposal 2.	If rate indication for a DRB is based on RRC configuration, network configures UE with a ratio for allocating the indicated rate among QoS flows in the DRB.

It is to be discussed how to determine the ratio for allocating the indicated rate among QoS flows in the DRB. A possible approach is that CN obtains a split ratio between traffic flows from the application server. CN then configures RAN or UE with a ratio between corresponding QoS flows based on PCC rules. But this discussion is outside RAN2’s scope and should be handled by SA2/SA4/RAN3 instead. Therefore, RAN2 should inform SA2/SA4/RAN3 of RAN2’s design (Proposal 2) and request them to discuss and specify any remaining design details, if applicable.

Proposal 3.	Send an LS to SA2/SA4/RAN3 to inform them about Proposal 2 and request them to discuss and specify any remaining design details, if applicable.

Rate query procedure

At RAN2#129, the following working assumptions are made:

- Working assumption: - Support rate query MAC CE with the target to use same design that we will agree for rate indication MAC CE. - The rate query MAC CE is configurable by the network, i.e. the network may turn it off completely (same as legacy).

In the following, we discuss additional aspects in supporting UL rate query.

In the legacy Recommended Bit Rate procedure, UE may either query network for available bit rate or request a desired bit rate from network. We think both scenarios are still relevant for XR applications. Rate query is useful for application-layer rate adaptation algorithms. Rate request is useful when an application attempts to change its operating state (e.g. due to scene change).

Proposal 4.	UE can either query for a recommended bit rate or request a desired bit rate in a UL rate query.

As to the granularity of rate queries, we think it makes the most sense for it to match the granularity of the rate indication from network. Namely, if rate indication from network is based on RRC configuration, then rate query MAC CE from UE should also be for a DRB/LCH. After obtaining a response from network, UE can use the same rate split ratio configured by network to derive the recommended bit rate for a QoS flow.

Proposal 5.	Rate queries by UE follow the same granularity as rate indications from network.

In the legacy Recommended Bit Rate procedure, network may configure a prohibit timer for rate query per LCH. We think the same design can be reused for XR applications. The only difference is that XR rate control may require finer granularity for the timer, as the rate query can be for a QoS flow. We think it makes sense to apply the same granularity as rate query (Proposal 4), so that QoS flows that require frequent queries do not penalize those in the same DRB.

Proposal 6.	Prohibit timer for rate query is configured at the same granularity for rate indication as configured by network.

Bit rate table design

At RAN2#129, the following agreements were made:

- Specify a new table for XR rate control. FFS distribution (exponential, linear), codepoints etc. - We will try to design a table first and check whether it is possible to meet the required range/granularity. Afterwards, we can check whether multipliers are needed

In the reply LS from RAN4 (S4-242182), it has been indicated that

- The range of uplink bit rate varies from 100 Kbps to 40 Mbps;

- The desired granularity can be as small as 1% but coarser granularity such as 10% may also be sufficient.

With these two requirements, we can evaluate which type of encoding scheme and how many code points should be used.

In legacy Recommended Bit Rate MAC CE, bit rates are encoded with a piecewise linear distribution. We first check if linear or piecewise linear encoding is also suitable for XR rate control.

If linear encoding (i.e. constant step size) is used, the largest quantization error is associated with the smallest code point. If we apply the largest quantization error required by SA4, which is 10%, then the quantization step size = 100 Kbps x 10% = 10 Kbps. Consequently, the number of required code points = (40,000 – 100)/10 +1 = 3991.

If piecewise linear encoding is used, suppose M is the end point of the 1st segment (which is also the start of the 2nd segment) and p = quantization error. Then the total number of code point = (M - 100)/(100 x p) + 1 + (40000 – M) / (M x p) +1. It is not difficult to derive the optimal value of M that minimizes the number of code points from this equation, which is 2,000 Kbps. The corresponding number of code points = 38/p. Therefore, the total number of code point is 380 at p=10%, 760 at 5% and 3800 at 1%.

From the calculations above, we may observe that

Observation 1. 	If linear or piecewise linear distribution is used to encode the bit rates provided by SA4, it would require excessive number of code points to achieve the required granularity of 10% or less.

If exponential encoding is used, then for a quantization error p, the number of code points N can be found from the equation Rmax = Rmin x (1 + p)^N, where Rmin and Rmax are the minimum and maximum rates, respectively. The table below lists N as a function of p:

| Q.E. | # of code points | # of bits |
| --- | --- | --- |
| 1% | 603 | 10 |
| 2% | 303 | 9 |
| 3% | 203 | 8 |
| 4% | 153 | 8 |
| 5% | 123 | 7 |
| 6% | 103 | 7 |
| 7% | 89 | 7 |
| 8% | 78 | 7 |
| 9% | 70 | 7 |
| 10% | 63 | 6 |

We may observe from the table that the resulting encoding is much mor efficient than linear or piecewise linear ones. For example, quantization error of only 3% can be achieved with only 203 code points (or 8 bits). And even 6 bits is sufficient to achieve quantization error of 10%.

Observation 2.	Exponential encoding can achieve a good balance between low quantization error and low overhead.

Therefore, we propose to use exponential distribution to encode the range of bit rates required by SA4. In addition, with exponential distribution, the full range of the bit rates can be covered by a single field. Hence no rate multiplier similar to the one in the legacy Recommended Bit Rate MAC CE is needed.

But we think the choice for the number of codes point can be tentatively postponed until all other fields in the MAC CE are determined. The reason is that it is related to the length of the bit rate field, whose choice may also depend on the choice and design of other fields in the MAC CE.

Another issue could be FFS is how to round off the rates associated with each code point, as exponential encoding will likely result in uneven lower digitals. It would be more practical for applications if the rates have more even low digitals.

Proposal 7.	Apply exponential distribution to encode the bit rates required by SA4. FFS The number of code points (e.g. after all fields in the MAC CE are finalized) and how to round off rates associated with code points.

Proposal 8.	No rate multiplier is needed.

MAC CE design

One of the special characteristics of XR applications is that it generates multiple flows with different QoS requirements, e.g. video, audio, haptic, control, etc. When an application performs rate adaptation, it typically adjusts the rates of multiple flows simultaneously. Although it is possible to send multiple MAC CEs (one for each flow or LCH) at once in this case, it is more efficient and customary (like many other MAC CEs) to include all of them in a single MAC CE.

This enhancement is applicable to both rate indication MAC CE from gNB or rate query MAC CE from UE.

Proposal 9. 	The rate indication MAC CE (DL) and rate query MAC CE (UL) can include bit rate indications for multiple QoS flows or logical channels. The exact granularity of the rate indications can follow the granularity of rate indication configured by network.

So far, there are at least several differences in the necessary fields required by XR rate control and the legacy Recommend Bit Rate procedure. For example, XR rate control require per-flow indication. The rate indication field likely to be longer, but no rate multiplier probably is needed. And rate indication/query MAC CE may include information of multiple QoS flows or LCHs. Therefore, in our opinion, it is simpler to design a new MAC CE for XR rate control instead of extending the legacy Recommended Bit Rate MAC CE.

Proposal 10.	Introduce a new MAC CE for XR rate control, instead of enhancing the legacy Recommended Bit Rate MAC CE.

# Conclusion

Based on the above analysis, we respectively request RAN2 to discuss and agree to the following proposals:

Granularity of rate indication

Proposal 1.	Per-QoS-flow rate indication by RRC configuration is the baseline. Support for per-QoS-flow rate indication within MAC CE is an optional UE capability.

Proposal 2.	If rate indication for a DRB is based on RRC configuration, network configures UE with a ratio for allocating the indicated rate among QoS flows in the DRB.

Proposal 3.	Send an LS to SA2/SA4/RAN3 to inform them about Proposal 2 and request them to discuss and specify any remaining design details, if applicable.

Rate query procedure

Proposal 4.	UE can either query for a recommended bit rate or request a desired bit rate in a UL rate query.

Proposal 5.	Rate queries by UE follow the same granularity as rate indications from network.

Proposal 6.	Prohibit timer for rate query is configured at the same granularity for rate indication as configured by network.

Bit rate table design

Observation 1. 	If linear or piecewise linear distribution is used to encode the bit rates provided by SA4, it would require excessive number of code points to achieve the required granularity of 10% or less.

Observation 2.	Exponential encoding can achieve a good balance between low quantization error and low overhead.

Proposal 7.	Apply exponential distribution to encode the bit rates required by SA4. FFS The number of code points (e.g. after all fields in the MAC CE are finalized) and how to round off rates associated with code points.

Proposal 8.	No rate multiplier is needed.

MAC CE design

Proposal 9. 	The rate indication MAC CE (DL) and rate query MAC CE (UL) can include bit rate indications for multiple QoS flows or logical channels. The exact granularity of the rate indications can follow the granularity of rate indication configured by network.

Proposal 10.	Introduce a new MAC CE for XR rate control, instead of enhancing the legacy Recommended Bit Rate MAC CE.