3GPP TSG-RAN WG2 Meeting #129bis                                                              R2-2501771

Wuhan, China, April 7th – 11th, 2025

Agenda item:	7.0.2.17

Source:	vivo

Title:	Discussion on RAN1 and RAN4 Reply LS on Soft Satellite Switch with Re-sync

Document for:	Discussion and Decision

- Introduction

Recently, RAN1 and RAN4 have sent back the reply LS on soft satellite switch with re-sync, providing the RAN1 and RAN4 considerations on the support of soft satellite switch with re-sync based on temporarily shifted SSB [1][2].

In this contribution, we would like to provide our view on the further work for soft satellite switch with re-sync.

- Discussion

Based on the offline discussion [3], Figure 1 illustrates the propagation of the normal SSB from the source satellite/target satellite and the temporarily shifted SSB from the target satellite during [t-ServiceStart, t-Service].

Figure 1: Soft satellite switch with re-sync based on temporarily shifted SSB

To achieve the soft satellite switch with re-sync based on temporarily shifted SSB, the UE has to first search and maintain the timing from the target satellite based on the SSB time offset (for detecting temporarily shifted SSB) during [t-ServiceStart, t-Service]. Then, the UE has to adjust the timing to the target satellite by additionally detecting the normal SSB from the target satellite instead of the temporarily shifted SSB. The remaining issue should be focused on whether RAN1 and RAN4 specs can support this in Rel-18.

From RAN4 reply LS, we understand that such a scenario is not considered in RAN4. As a result, the corresponding requirement is not defined. It is not sure whether the related requirement is needed or not. What’s worse, RAN4 indicates that the temporary SSB pattern shift and shifting back may require additional UE processing for shifting. Also, it may have an impact on UE performing initial access during [t-ServiceStart, t-Service]. It seems RAN4 cannot make sure the UE can successfully maintain the pre-acquired DL synchronization after t-Service. In this sense, we think RAN2 should ask RAN4 whether the current Rel-18 spec can make sure the UE can maintain the pre-acquired DL synchronization started between [t-ServiceStart, t-Service] with target satellite if the SSB pattern is changed at t-Service. For the remaining RAN4 question, we think RAN2 can confirm that Option 1 given by RAN4 is aligned with RAN2 considerations. Further, RAN2 should also list all the candidate values of SSB time offset (i.e. 0..159 in units of subframes) to RAN4. For question 2) (i.e. how the half-frame indication and SFN indication bits are configured during [t-ServiceStart, t-Service]), we think it is a pure RAN1 issue regarding cell search and SSB design. So RAN2 may not answer it.

Regarding the RAN1 reply LS, we think the RAN2 original intention is to ask whether the existing Rel-18 RAN1 specification (taking RAN2 procedure into account) can successfully maintain the pre-acquired DL synchronization for soft satellite switch with re-sync or not. Based on this, we think RAN2 should ask RAN1 whether UE timing can be maintained based on the existing RAN1/2 specs. Meanwhile, RAN2 should also respond that the RAN1 assumptions are aligned with RAN2 considerations.

Further, considering Rel-18 has been frozen for a long time. If further spec change is needed in RAN1 and RAN4 to support soft satellite switch with re-sync based on temporarily shifted SSB, RAN2 should further carefully discuss whether to support soft satellite switch with re-sync based on temporarily shifted SSB in Rel-18 (input from RAN1 and RAN4 may be needed). Generally, we do not do big enhancements for a frozen release.

Proposal 1: RAN2 responds to RAN4 that

- Option 1 is aligned with RAN2 considerations;

- value range of ssb-TimeOffset-r18 is (0..159) given in integer number of subframe;

- respectfully asking whether a Rel-18 UE (based on the current spec) can maintain the pre-acquired DL synchronization started between [t-ServiceStart, t-Service] with target satellite if the SSB pattern is changed at t-Service.

Proposal 2: RAN2 responds to RAN1 that

- respectfully asking whether a Rel-18 (based on the current spec) UE can maintain the pre-acquired DL synchronization started between [t-ServiceStart, t-Service] with target satellite if the SSB pattern is changed at t-Service;

- RAN1’s assumptions are aligned with RAN2 considerations.

Proposal 3: If further spec change is needed in RAN1 and RAN4, RAN2 further discusses whether the soft satellite switch with re-sync based on temporarily shifted SSB should be supported in Rel-18.

3 Conclusion

In this contribution, we have provided our view on the further work for soft satellite switch with re-sync. All the proposals are summarized below.

Proposal 1: RAN2 responds to RAN4 that

- Option 1 is aligned with RAN2 considerations;

- value range of ssb-TimeOffset-r18 is (0..159) given in integer number of subframe;

- respectfully asking whether a Rel-18 UE (based on the current spec) can maintain the pre-acquired DL synchronization started between [t-ServiceStart, t-Service] with target satellite if the SSB pattern is changed at t-Service.

Proposal 2: RAN2 responds to RAN1 that

- respectfully asking whether a Rel-18 (based on the current spec) UE can maintain the pre-acquired DL synchronization started between [t-ServiceStart, t-Service] with target satellite if the SSB pattern is changed at t-Service;

- RAN1’s assumptions are aligned with RAN2 considerations.

Proposal 3: If further spec change is needed in RAN1 and RAN4, RAN2 further discusses whether the soft satellite switch with re-sync based on temporarily shifted SSB should be supported in Rel-18.

4 Reference

[1] R1-2501606, Reply LS on soft satellite switch with re-sync, RAN1.

[2] R4-2502701, Reply LS on soft satellite switch with re-sync, RAN4.

[3] R2-2410971, Report of [AT128][301][R18 NR NTN] SMTC issues, Huawei.