3GPP TSG-RAN WG2 Meeting #129bis                                                              R2-2501773

Wuhan, China, April 7th – 11th, 2025

Agenda item:	7.0.2.20

Source:	vivo

Title:	Discussion on RAN4 LS on IoT NTN and NR NTN Inband Operation

Document for:	Discussion and Decision

- Introduction

RAN4 sent an LS to ask for RAN2 confirmation on the RAN4 assumptions for IoT NTN and NR NTN inband operation is aligned with RAN2 specification without impact on RAN2 [1].

In this contribution, we would like to discuss how to support inband operation for NB-IoT NTN and NR-NTN from the RAN2 point of view and the potential RAN2 impacts.

- Discussion

In the previous RAN4 meeting, it was agreed to support inband operation between NB-IoT NTN and NR NTN (i.e. NB-IoT NTN inband operation in NR-NTN) in Rel-18 with the following principles applied [2] [3]:

- The NB-IoT NTN UE is only required to support the same operation mode for anchor and non-anchor carriers;

Table 1: Anchor and non-anchor carrier deployment combinations in NB-IoT NTN inband operation

|  | Anchor Carrier |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | In-band | Guard-band | Standalone |
| Non-Anchor Carrier | In-band | Yes (Rel-19) | Yes (Rel-19) | NO (Rel-18) |
|  | Guard-band | NO | YES (Rel-18) | NO (Rel-18) |
|  | Standalone | NO | NO | YES (Rel-18) |

- RRC signalling should indicate guardband-r13 in operationModeInfo-r13 in MasterInformationBlock-NB;

- UE is not expected to access a cell indicating inband-SamePCI-r13 or inband-DifferentPCI-r13 in operationModeInfo-r13 in MasterInformationBlock-NB;

- UE is not expected to be aware of where the configured NB-IoT UL carrier is within the NTN NR carrier. It is presumed that operators will ensure, through network configuration during deployment, that there is sufficient guardband between NB-IoT carrier and operating band and/or spectrum block edges to meet emission requirements outside its allocated spectrum block.

For bullet 1, it is about the multi-carrier operation for NB-IoT. For the NB-IoT NTN inband operation, the network can guarantee mixed mode operation would not be configured for the NB-IoT NTN UE. That requirement can be guaranteed based on network implementation using the existing signaling. Thus, from the RRC signaling point of view, we think no further stage-3 impact is foreseen. That requirement is currently captured in TS 36.102. It seems a spontaneous logic that we should also capture this in clause 5.5a of 36.300 for scenario clarification and text completeness. The corresponding text proposal is given in Annex A for your reference. NOTE that such clarification shall not be considered as RAN2 impact since there is no procedural text or UE behavior change.

Observation 1: For NB-IoT NTN inband operation in Rel-18, the operational mode of anchor and non-anchor carriers will be the same (i.e. guard-band or standalone) by network implementation based on the current RRC signaling. There is no RAN2 stage-3 impact foreseen.

Proposal 1: RAN2 to capture the operational mode of anchor and non-anchor carriers will be the same for NB-IoT NTN inband operation in clause 5.5a of Rel-18 36.300.

For bullet 2, without a doubt, it can be supported by the existing RRC signaling without any change.

Observation 2: For NB-IoT NTN inband operation in Rel-18, guardband-r13 should be always indicated in operationModeInfo-r13 in MasterInformationBlock-NB. It can be supported by the current specification. There is no RAN2 impact foreseen.

For bullet 3, we think the network can use the existing cell bar mechanism to achieve the RAN4 requirement. Specifically, for the NB-IoT NTN cell whose carrier is located on a given channel number defined for NB-IoT NTN inband operation, the network can set the field cellBarred-NTN to 'barred' as long as the field inband-SamePCI-r13 or inband-DifferentPCI-r13 is included in operationModeInfo-r13 in MasterInformationBlock-NB. With that, the UE cannot access the cells indicating inband-SamePCI-r13 or inband-DifferentPCI-r13. This can also be guaranteed by network implementation by leveraging on the existing NTN cell bar field.

Observation 3: To make sure a UE will not access a cell indicating inband-SamePCI-r13 or inband-DifferentPCI-r13 in operationModeInfo-r13 in MasterInformationBlock-NB, cellBarred-NTN shall be set to 'barred' on that cell. There is no RAN2 impact foreseen as the existing cell bar mechanism can fulfill this goal.

Proposal 2: For NB-IoT NTN inband operation in Rel-18, it is RAN2’s understanding that cellBarred-NTN shall be set to 'barred' if the cell indicating inband-SamePCI-r13 or inband-DifferentPCI-r13 in MasterInformationBlock-NB (No spec change is required).

For bullet 4, it is totally up to network deployment. There is no RAN2 impact foreseen.

Based on the above analysis, we think there is no issue to support NB-IoT NTN inband operation in Rel-18 from RAN2 perspective. No RAN2 stage-3 impact is foreseen.

Proposal 3: For NB-IoT NTN inband operation in Rel-18, RAN2 responds to RAN4 that the RAN4 assumptions are aligned with RAN2 specification and no RAN2 stage-3 impact is foreseen.

3 Conclusion

In this contribution, we have provided our understanding of how to support inband operation for NB-IoT NTN and NR-NTN from the RAN2 point of view in Rel-18. And we make the following observations and proposals,

Observation 1: For NB-IoT NTN inband operation in Rel-18, the operational mode of anchor and non-anchor carriers will be the same (i.e. guard-band or standalone) by network implementation based on the current RRC signaling. There is no RAN2 stage-3 impact foreseen.

Observation 2: For NB-IoT NTN inband operation in Rel-18, guardband-r13 should be always indicated in operationModeInfo-r13 in MasterInformationBlock-NB. It can be supported by the current specification. There is no RAN2 impact foreseen.

Observation 3: To make sure a UE will not access a cell indicating inband-SamePCI-r13 or inband-DifferentPCI-r13 in operationModeInfo-r13 in MasterInformationBlock-NB, cellBarred-NTN shall be set to 'barred' on that cell. There is no RAN2 impact foreseen as the existing cell bar mechanism can fulfill this goal.

Proposal 1: RAN2 to capture the operational mode of anchor and non-anchor carriers will be the same for NB-IoT NTN inband operation in clause 5.5a of Rel-18 36.300.

Proposal 2: For NB-IoT NTN inband operation in Rel-18, it is RAN2’s understanding that cellBarred-NTN shall be set to 'barred' if the cell indicating inband-SamePCI-r13 or inband-DifferentPCI-r13 in MasterInformationBlock-NB (No spec change is required).

Proposal 3: For NB-IoT NTN inband operation in Rel-18, RAN2 responds to RAN4 that the RAN4 assumptions are aligned with RAN2 specification and no RAN2 stage-3 impact is foreseen.

4 Reference

[1] R4-2503035, LS on NR NB-IoT in-band operation, RAN4.

[2] R4-2453030, Way Forward on IoT-NTN in-band, EchoStar.

[3] R4-2503036, CR to 36.102 NB-IoT NTN inband operation with NR NTN, MediaTek Inc..

Annex A: TP for 36.300

5.5a	Multi-carrier operation for NB-IoT

For NB-IoT, multi-carrier operation is supported.

The UE in RRC_CONNECTED can be configured, via UE-specific RRC signaling, to a non-anchor carrier, for all unicast transmissions. The UE in RRC_IDLE, based on broadcast/multicast signaling, can use a non-anchor carrier for SC-PTM reception. The UE in RRC_IDLE can, based on broadcast signaling, use a non-anchor carrier for paging reception. The UE in RRC_IDLE or RRC_CONNECTED, based on broadcast signaling, can use a non-anchor carrier for random access. If the non-anchor carrier is not configured for the UE, all transmissions occur on the anchor carrier. The valid anchor and non-anchor carrier combinations are provided in Table 5.5a-1 for FDD and Table 5.5a-2 for TDD.

Table 5.5a-1: Anchor and non-anchor carrier deployment combinations in FDD

|  | Anchor Carrier |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | In-band | Guard-band | Standalone |
| Non-Anchor Carrier | In-band | Valid (Note 1) | Valid (Note 1) | Valid (Note 2, Note 3) |
|  | Guard-band | Valid (Note 1) | Valid (Note 1, Note 4) | Valid (Note 2, Note 3) |
|  | Standalone | Valid (Note 2, Note 3) | Valid (Note 2, Note 3) | Valid (Note 2, Note 4) |

Table 5.5a-2: Anchor and non-anchor carrier deployment combinations in TDD

|  | Anchor Carrier |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | In-band | Guard-band | Standalone |
| Non-Anchor Carrier | In-band | Valid (Note 1) | Valid (Note 1) | Invalid |
|  | Guard-band | Valid (Note 1) | Valid (Note 1) | Invalid |
|  | Standalone | Invalid | Invalid | Valid (Note 2) |

NOTE 1:	Both carriers associated with the same LTE cell.

NOTE 2:	Total frequency span to not exceed 20MHz and both anchor and non-anchor carriers synchronised.

NOTE 3:	Not applicable to SC-PTM reception.

NOTE 4: For NB-IoT NTN inband operation, operational mode of anchor and non-anchor carriers shall be the same in this release of the specification.