3GPP TSG-RAN WG2 Meeting #129bis                                                           R2-2501774

Wuhan, China, April 7th– 11th , 2025

Agenda item:	8.8.2

Source:	vivo

Title:	Discussion on DL Coverage in NTN

Document for:	Discussion and Decision

- Introduction

In RAN2#129 [1], the issue of DL coverage enhancement was discussed and the following agreements were made.

Agreements: 1. RAN2 assumes it will be possible to have different SSB periodicity among neighbour cells in the same frequency layer 2. RAN2 assumes that in a NR NTN cell, SSB beam sweeping in different spatial directions is possible as in a NR TN cell: the whole cell is covered by the different SSB beams in half-frame 3. RAN2 also assumes that, with the current status of RAN1 discussion, if one cell is defined by multiple “satellite beams”, the satellite beams are all simultaneously active or inactive (“beam hopping” applies equally to all the satellite beams of a given cell) 4. The number of SMTC/gaps a UE needs to consider at any time will not be increased further 5. From RAN2 perspective, we support option a (clustered cells illuminated) and option b (scattered cells illuminated) for further discussion on SMTC. No LS is sent to RAN1. 6. RAN2 observes that if the cells active simultaneously are in clusters, existing SMTC mechanism (a maximum of 4 SMTCs per frequency) may be sufficient (FFS if any solution identified for option b applies to option a as well). 7. RAN2 observes that if the cells active simultaneously are scattered, (for the case of intra-frequency neighbouring cells), the SSBs of surrounding neighbour cells may be transmitted at different times and existing SMTC mechanism may not be enough.

However, it was still unclear whether the SMTC enhancement is needed for option b, i.e., the scattered cells illuminated in both the IDLE state and the CONNECTED state and how to address the issue. In the contribution, we will continue to analyze the issue and give our views.

- Discussion

In RAN2#129, based on the offline discussion report [2], the following candidate solutions were proposed on the issue that the SSBs of surrounding cells may be transmitted at different times and existing SMTC mechanisms may not be enough if the cells active simultaneously are scattered.

1. Configure multiple SMTCs (more than 4) and SMTC selection (e.g. location-based SMTC selection, UE autonomous selection) 2. SSB-specific SMTC 3. UE assistance information reporting 4. Configure one SMTC with small periodicity, in each period (SMTC occasion) UE is indicated which PCI to use

The above solutions do not apply to the same scenario, i.e., IDLE or CONNECTED, thus to be more specific, we will continue to discuss separately for IDLE and CONNECTED state.

For RRC IDLE state:

For IDLE state, there is no need for UE and NW to have the same understanding of the SMTC configurations. That is, UE can decide which SMTC to choose or how to adjust the SMTC configurations based on UE’s location. In general, the above option 1, option 2, and option 4 can work for IDLE state to address the issue that the SSBs of surrounding cells may be transmitted at different times.

As for option 1, the spec impact is quite large, i.e., it requires the NW to configure multiple SMTCs, the reference location and also the location threshold for UEs associated with each SMTC configuration, so that UE can decide which SMTC is used. Further, the UE behaviour is quite complicated since UE needs to evaluate the distance and decide which SMTC(s) is used. As for option 2, this quite depends on the satellite cell/satellite beam/NR beam deployment scenario, only if the NR SSBs are associated with satellite beams, then the solution work. As for option 4, even NW can indicate which period is associated with which PCI so that UE can directly decide which SMTC period is needed, but the spec impact is also inevitable, i.e., how to associate the SMTC period with the corresponding PCIs. Besides, configuring small SMTC periodicity may lead to more UE power consumption, since UE needs to measure each SMTC period. From our perspective, more suitable solution is to extend the number of the SMTCs, i.e. by 6 to cover the surrounding neighbour cells of UE. However, extending the number of SMTCs has RAN4 impacts, so it is proposed to send LS to RAN4 to check whether R19 can make such enhancement.

Proposal 1: If RAN2 decides to extend the number of SMTC, RAN2 send LS to check with RAN4 whether the enhancement can be done in R19.

For RRC CONNECTED state:

As for RRC CONNECTED state, option 3 and option 4 apply. For option 4, the spec impact is inevitable as the analysis provided above. Besides, the associated PCIs for each SMTC period should be UE-specific to ensure UE and NW have the same understanding. Thus, there is no clear benefit for option 3 rather than NW directly configure the specific SMTCs for each UE based on some assistance information provided by UE.

For option 3, UE can provide some assistance information to assist the NW in deciding how to configure the SMTC. According to the spec [3], UE can report its coarse UE location to NW upon measurement reporting based on measurement configuration or provide its coarse UE location to NW via UEInformationResponse upon NW's request. From our perspective, based on coarse location of UE and the deployment of the beam hopping pattern of the neighbour cell surrounding the UE as shown in Figure 1, NW can decide how to configure each UE’s SMTC configurations correspondingly. So there is no need for any other enhancement in RRC CONNECTED to address the issue that the SSBs of surrounding cells may be transmitted at different times.

Figure 1: Active cells are scattered

Observation 1: NW can decide the SMTC configuration for specific UE based on reported UE coarse location and knowledge of the beam-hopping pattern surrounding the UE.

Proposal 2: If the cells active simultaneously are scattered and UE is in CONNECTED state, NW configures the corresponding SMTCs for each UE based on the reported coarse location (No further enhancement is needed).

- Conclusion

In this contribution, we have further discussed the DL coverage aspect. And we make the following observation and proposals:

Observation 1: NW can decide the SMTC configuration for specific UE based on reported UE coarse location and knowledge of the beam-hopping pattern surrounding the UE.

Proposal 1: If RAN2 decides to extend the number of SMTC, RAN2 send LS to check with RAN4 whether the enhancement can be done in R19.

Proposal 2: If the cells active simultaneously are scattered and UE is in CONNECTED state, NW configures the corresponding SMTCs for each UE based on the reported coarse location (No further enhancement is needed).

- Reference

- Draft_RAN2_129_Meeting_Report_v1.

- R2-2501415	Report of [AT129][304][R19 NR NTN] LS to RAN1 on DL coverage enhancements	Huawei discussion	 Rel-19	NR_NTN_Ph3-Core

- TS36.331, Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification.