3GPP TSG-RAN WG2 Meeting #129bis                                                           R2-2501776

Wuhan, China, April 7th– 11th, 2025

Agenda item:	8.8.6

Source:	vivo

Title:	Remaining Issues on LTE to NR NTN Mobility

Document for:	Discussion and Decision

- Introduction

In the LTE TN to NR NTN idle mobility discussion in RAN2#129 [1], whether to support the redirection from LTE TN to NR NTN was discussed. The following agreements were made.

1. RAN2 confirms that redirection from LTE TN to NR NTN is not supported before Rel-19. 2. Redirection from LTE TN to NR NTN is supported in Rel-19 introducing a new capability signaling for LTE TN to NR NTN redirection purpose and also a satellite ID(s) in RedirectedCarrierInfo.

In the contribution, we will analyze the following stage 3 issues and give our view.

- UE behaviour in case the satellite information is not provided;

- UE capability.

- Discussion

## 2.1 UE behaviour in case the satellite information is not provided

According to the endorsed running CR [2] for LTE TN to NR NTN mobility, the satellite ID is not mandatory provided by NW. If the satellite ID is not provided, it is not clear how UE to handle the corresponding measurement.

NTN-RedirectedCarrierInfoNR-r19 ::= SEQUENCE { satAssistanceInfoList-r19 SEQUENCE (SIZE(1..maxSat-r17)) OF SatelliteId-r18 OPTIONAL, -- Need ON redirectedCarrierInfo-r19 CarrierInfoNR-r15 }

Besides, a similar issue is also present for the cell reselection case when there is no satellite ID associated with the frequency info provided within the SIB3(-NB) and SIB5(-NB) [3]. For that case, it is specified that the UE can consider the corresponding frequency for the TN frequency and perform the measurements correspondingly. In general, we think similar handling can also be applied to the case that the satellite ID is not provided within the redirection information for NR NTN.

satelliteAssistanceInfo List of satellite ID(s), used to associate with the satellite assistance information for neighbour cell measurements on this frequency. If the field is not present for a frequency and SystemInformationBlockType33-NB is broadcast, the UE considers the cells on the frequency to be terrestrial cells.

Proposal 1: In case the satellite ID is not present in the redirection information for NR NTN and SIB33 is broadcast, the UE considers the cells on the frequency to be terrestrial cells.

Another issue is that in case the satellite ID is present in the redirection information for NR NTN, but there is no corresponding satellite information for the satellite ID in SIB33, how UE behaves is unclear. The following is for the UE handling for cell reselection cases when there is satellite ID but without corresponding satellite information in SIB31 or SIB33 [4]. Similar handling can also be applied to the case that the satellite ID is present in the redirection information for NR NTN, but there is no corresponding satellite information for the satellite ID in SIB33.

For a UE in idle mode, it is up to UE implementation whether to perform NTN neighbour cell measurements on an NTN cell which is indicated in SIB3/SIB5 but without corresponding satellite information in SIB31 or SIB33.

Proposal 2: In case the satellite ID is present in the redirection information for NR NTN, but there is no corresponding satellite information for the satellite ID in SIB33, it is up to UE implementation whether to perform NTN neighbour cell measurements on the NTN cell.

## 2.2 UE capability

According to the endorsed running CR [3] for LTE TN to NR NTN mobility, a new UE capability was introduced for supporting the redirection from LTE TN to NR NTN.

#### 4.3.34.x ntn-RedirectionNR-r19 This field indicates whether the UE supports the inter-RAT redirection from an E-UTRA terrestrial network cell to an NR NTN cell using satellite assistance information provided by RRCConnectionRelease.

Besides, the legacy UE capability to indicate the supported NR band information includes the NR TN and NR NTN. Supporting the redirection from LTE TN to NR NTN implies that the UE shall support the corresponding NR NTN frequency band. Thus, it is proposed to add clarification in the field description of the newly introduced UE capability that UE supporting the ntn-RedirectionNR-r19 shall also indicate the support of supportedBandListNR-SA-r15.

#### 4.3.34.3 supportedBandListNR-SA-r15 This field indicates whether UE supports standalone NR, as specified in TS 38.331 [35], and includes the supported NR bands as defined in TS 38.101-1 [33], TS 38.101-2 [34], and TS 38.101-5 [xx]. The presence of this field also indicates that the UE can perform both NR SS-RSRP and SS-RSRQ measurement in the included NR band(s) as specified in TS 38.215 [36].

Proposal 3: UE supporting the ntn-RedirectionNR-r19 shall also indicate the support of supportedBandListNR-SA-r15.

In the annex 1, the corresponding TP is attached.

- Conclusion

In this contribution, we have further discussed the redirection issue from LTE TN to NR NTN. And we make the following proposals:

Proposal 1: In case the satellite ID is not present in the redirection information for NR NTN and SIB33 is broadcast, the UE considers the cells on the frequency to be terrestrial cells.

Proposal 2: In case the satellite ID is present in the redirection information for NR NTN, but there is no corresponding satellite information for the satellite ID in SIB33, it is up to UE implementation whether to perform NTN neighbour cell measurements on the NTN cell.

Proposal 3: UE supporting the ntn-RedirectionNR-r19 shall also indicate the support of supportedBandListNR-SA-r15.

- Reference

- Draft_RAN2_129_Meeting_Report_v1.

- R2-2501418	Introduction of LTE TN to NR NTN IDLE mode mobility

- TS36.331, Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification.

- 36.300, Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2.

- R2-2501417	Introduction of LTE TN to NR NTN Mobility UE Capability

Annex 1

## 6.8	Other features

### 6.8.1	System Information Block Type 16

It is optional for UE, including UEs of any ue- Category-NB, to support the reception of SystemInformationBlockType16 as specified in TS 36.331 [5].

### 6.8.2	QCI1 indication in Radio Link Failure Report

It is optional for the UE to include drb-EstablishedWithQCI-1 in RLF-Report as specified in TS 36.331 [5].

### 6.8.3	Enhanced random access power control

It is optional for UE to support enhanced random access power control for FDD as specified in TS 36.321 [4] and TS 36.213 [22], clauses 16.2.1.1.1 and 16.3.1. This feature is only applicable if the UE supports any ue-Category-NB.

### 6.8.4	MO-EDT for Control Plane CIoT EPS Optimization

It is optional for UE to support MO-EDT for Control Plane CIoT EPS optimizations as specified in TS 24.301 [28]. This feature is only applicable if the UE supports ce-ModeA-r13, or for FDD if the UE supports any ue-Category-NB.

### 6.8.5	Void

### 6.8.6	Enhanced PHR

It is optional for UE to support enhanced PHR in MSG3 for FDD, as defined in TS 36.321 [4]. This feature is only applicable if the UE supports any ue-Category-NB.

### 6.8.7	void

### 6.8.8	Resynchronization Signals

It is optional for UE to support resynchronization signals, as defined in TS 36.211 [17]. This feature is only applicable if the UE supports ce-ModeA-r13.

### 6.8.9	Measurement gaps for higher UE velocity

It is optional for UE to support measurement gaps for higher UE velocity, as defined in TS 36.331 [5] and TS 36.133[16]. This feature is only applicable if the UE supports ce-ModeA-r13.

### 6.8.10	MT-EDT for Control Plane CIoT EPS Optimisation

It is optional for UE to support MT-EDT for Control Plane CIoT EPS Optimisation, as defined in TS 24.301 [28]. If the UE supports 'MT-EDT for Control Plane CIoT EPS Optimisation' it shall support 'MO-EDT for Control Plane CIoT EPS Optimisation' as described in clause 6.8.4. This feature is only applicable if the UE supports ce-ModeA-r13, or for FDD if the UE supports any ue-Category-NB.

### 6.8.11	MT-EDT for User Plane CIoT EPS Optimisation

It is optional for UE to support MT-EDT for User Plane CIoT EPS Optimisation, as defined in TS 24.301 [28]. If the UE supports 'MT-EDT for User Plane CIoT EPS Optimisation' it shall support earlyData-UP-r15 as described in clause 4.3.8.7. This feature is only applicable if the UE supports ce-ModeA-r13, or for FDD if the UE supports any ue-Category-NB.

### 6.8.12	Void

### 6.8.13	Reduced MIB/SIB1-BR acquisition time

It is optional for UE to support reduced MIB/SIB1-BR acquisition time requirements as specified in TS 36.133 [16]. This feature is only applicable if the UE supports ce-ModeB-r13.

### 6.8.14	High speed dedicated network features

It is optional for UE to support HSDN cell reselection handling in RRC_IDLE and RRC_INACTIVE (if the UE supports eutra-5GC-r15) as specified in TS 36.304 [14] and TS 36.331 [5].

### 6.8.15	Carrier specific NRSRP thresholds for NPRACH resource selection

It is optional for UE to support carrier specific NRSRP thresholds for NPRACH resource selection as specified in TS 36.321 [4]. This feature is only applicable if the UE supports any ue-Category-NB and multiCarrier-NPRACH-r14 or multiCarrierPagingTDD-r15.

### 6.8.16	Protection against improper reselection to GERAN/UTRAN

It is optional for UE to support protection against improper reselection to GERAN/UTRAN as specified in TS 36.304 [14].

### 6.8.17	Inter-RAT cell reselection of an NR mobile IAB cell

It is optional for UE to support inter-RAT cell reselection priority handling of an NR mobile IAB cell in RRC_IDLE and RRC_INACTIVE (if the UE supports eutra-5GC-r15) as specified in TS 36.304 [14] and TS 36.331 [5].

### 6.8.x	Inter-RAT measurement on an NR NTN cell

It is optional for UE in RRC_IDLE, or in RRC_INACTIVE (if the UE supports eutra-5GC-r15) to support inter-RAT measurement for cell reselection from an E-UTRA terrestrial network cell to an NR NTN cell as specified in TS 36.304 [14] and TS 36.331 [5]. A UE indicating support of ntn-RedirectionNR-r19 shall also indicate support of supportedBandListNR-SA-r15.

END OF CHANGE