3GPP TSG-RAN WG2 Meeting #129bis                                                              R2-2501778

Wuhan, China, April 7th – 11th, 2025

Agenda item:	8.9.3

Source:	vivo

Title:	Discussion on CB-EDT Mechanism

Document for:	Discussion and Decision

1 Introduction

In the last RAN2 meetings, RAN2 has made great progress on the CB-EDT mechanism. The achieved agreements and working assumptions are listed below for reference [1]:

RAN2#129 Agreements: - It is FFS if separate CB-msg3 resources would be needed for CB-msg-3 using OCC or if the same CB-msg3 resources could be used. - RAN2 assumes that one possibility to take power imbalance under control is to define RSRP ranges that need to be respected to transmit CB-msg3 using OCC. - RAN2 assumes that at least the following will be part of the shared resources configuration for CB-msg3 (FFS on other aspects) - Time domain resources for (N)PUSCH occasions: periodicity and start time (e.g., start subframe, start SFN) - Frequency domain resources for (N)PUSCH occasions - repetition number - (N)PDCCH resource - MCS - For CB-msg3 transmission, for eMTC NTN, up to three separate RSRP thresholds (on top of the minimum RSRP threshold and possibly different from the thresholds for PRACH) can be supported for achieving at most 4 CE levels; for NB-IoT NTN, up to two separate RSRP thresholds (on top of the minimum RSRP threshold possibly different from the thresholds for PRACH) can be supported for achieving at most 3 repetition levels. - The UE shall at most have one ongoing CB EDT procedure at any time. - The CB EDT Config has one minimum RSRP threshold (as agreed in RAN2#128) to use CB EDT. - The CB EDT Config has two RSRP thresholds for NB-IoT for the three CE levels. - CB EDT Config has three RSRP thresholds for eMTC for the four CE levels. - As Signalling design Baseline RAN2 assumes the PUR config and the NPRACH config for shared (N)PUSCH config can be used and some of the parameters can be included in a new CB EDT config. - RAN2 consider a new CBEDT-ConfigSIB-NB IE for configuring the CB EDT feature. RAN2#129 Working assumption: - One CB-MSG4 can target multiple UEs simultaneously (FFS on the details). - For CB-MSG3, the Transmission window is configured by the network with a starting point (e.g. H-SFN offset), a window length, and a window periodicity (window length and periodicity could be the same). For k=1 the window length can be equal to 1: same behaviour as today The UE first selects the next DSA transmission window and then randomly select K replicasinside the window. RAN2 assumes that a pointer solution is not needed in Rel-19.

In this contribution, we will further discuss the CB-EDT mechanism used for IoT-NTN. In the following, we will provide our understanding of supporting CB-EDT from scenario and configuration, DSA modeling, and response reception points of view.

2 Discussion

  - 2.1 CB-EDT scenario and configuration

In the previous, there is an FFS that if anything is needed in dedicated signalling for the TA validation parameters, if needed, for the case of 15kHz SCS NB-IoT and eMTC CE mode B. In our understanding, to support CB-EDT transmission in 15kHz SCS NB-IoT and eMTC CE mode B, TA validation based on timer solution is needed for performance robustness (i.e. guaranteeing that the UE transmits the CB-EDT in the same cell where it is released from CONNECTED to IDLE). If so, the corresponding RAN4 requirement is needed for the NTN case. However, currently,  RAN4 is only discussing the TA validation requirement for CG-SDT for RedCap UE in NR-NTN. No NB-IoT specific requirement can be defined in Rel-19. Thus, we think CB-EDT transmission in 15kHz SCS NB-IoT or eMTC CE mode B should be deprioritized considering the performance gain can not be guaranteed.

Proposal 1: RAN2 deprioritizes the CB-EDT in 15kHz SCS NB-IoT or eMTC CE mode B cases.

In the [Post129][307] email discussion [2], how to indicate the SA and DSA mode is proposed. However, it is not so clear to us whether SA can DSA can be configured simultaneously for a given CE level. In our understanding, for resource selection simplicity, we would like to say the SIB should configure only one CB-EDT resource configuration. And that single resource configuration can be SA, or DSA, or OCC2, or DSA with OCC2 (if OCC can be supported). Otherwise, if SIB includes multiple CB-EDT resource configurations, then the UE has to determine which resource should be prioritized (e.g. the UE has to select one resource amongst the resource for SA, the resource for OCC2, and the resource for DSA) or support parallel transmission amongst different resources. It is quite complicated. So, we propose currently SA and DSA cannot be configured simultaneously for a given CE level.

Proposal 2: SA and DSA cannot be configured simultaneously for a given CE level.

Further, no conclusion was made regarding the DSA window. In our understanding, the DSA window is used to group the scattered periodical CB-EDT occasions for DSA occasion selection. The length of it can be indicated in multiples of CB-EDT occasion periodicity, similar to the configuredGrantTimer in NR. Then, the UE can calculate and determine every DSA window similarly to considering the CG resource, illustrated in Figure 1. Moreover, the DSA window is determined at the beginning of the first symbol of the CB-EDT occasion.

configuredGrantTimer                INTEGER (1..64)                                                         OPTIONAL,   -- Need R

Figure 1: DSA window determination

Proposal 3: DSA window length is configured in multiples of CB-EDT occasion periodicity (e.g. 2..32).

Proposal 4: The starting point of the DSA window is located at the beginning of the first symbol of the responding CB-EDT occasion.

  - 2.2 DSA modeling in MAC:

After CB-EDT occasion selection, the UE should do UL-SCH data transfer and the actual transport block transmission via the physical layer. The main preparation steps for the PUSCH transmission are given as follows:

- The MAC entity receives the UL grants;

- The MAC entity delivers the UL grants and the associated HARQ information to the HARQ entity;

- The HARQ entity instructs the multiplexing and assembly entity to generate a MAC PDU;

- The HARQ entity obtains the MAC PDU to transmit from the multiplexing and assembly entity;

- The HARQ entity triggers the corresponding HARQ process to perform a new transmission/retransmission;

- The corresponding HARQ process	instructs the PHY layer to generate a transmission;

- The PHY layer transmits the PUSCH for the obtained MAC PDU with PHY procedure (e.g. power control).

To minimize the potential RAN1 impacts with the introduction of DSA, we think each replica should be modeled as a new transmission. On one hand, since the network side cannot perform soft combining for DSA replicas due to the lack of information (based on the WA that the pointer is not supported in Rel-19) on which occasions the same UE selects, there is no need to model it as a retransmission. On the other hand, if we treat the replica as a retransmission, we have to consult with RAN1 whether the same transmit power can be reused for this retransmission and what RV index should be considered for it, and so on.

Further, by treating the replica transmission as a new transmission, then the MAC should generate a MAC PDU for each replica transmission. As the content in each replica should be the same, a possible and simple modeling is to obtain the MAC PDU from the HARQ buffer for the 2nd replica. At the physical layer, it can treat each replica transmission as the new transmission as the legacy case (e.g. independently doing the power control, resource mapping, etc), as depicted in Figure 2 below. Moreover, we think that HARQ process id 0 and RV0 can be reused, similar to Msg3 transmission.

Figure 2: DSA replica transmission modeling

Proposal 5: MAC considers each replica transmission as a new transmission.

Proposal 6: HARQ entity obtains the MAC PDU from the HARQ process buffer for the secondary replica.

Proposal 7: HARQ process id 0 and RV 0 are used for CB-EDT.

  - 2.3 Response reception:

After the CB-EDT transmission, the UE needs to start a PDCCH monitoring window for response reception. For DSA case, in the [Post129][307] email discussion [2], views are diverging. Some companies thought that PDCCH monitoring should be started after every replica transmission so that the UE can monitor the response as soon as possible and potentially discard the remaining replica transmission for power saving and avoid necessary CB-EDT collision, at least for the eMTC case. Some think that the monitoring window should be started only after the DSA window due to the half-duplex FDD operation of NB-IoT case. For simplicity, we suggest RAN2 adopt a unified design for both eMTC and NB-IoT case. Therefore, we propose,

Proposal 8: For both eMTC and NB-IoT, the UE starts a PDCCH monitoring window at the end of the corresponding DSA window plus UE-eNB RTT.

With one window used for response reception for all the replicas within a DSA window, to save PDCCH overhead, we think multiplexing of multiple responses for different UEs using different replicas is possible and it is efficient, as discussed in the previous meeting. So we suggest to confirm the WA made in the last meeting. Meanwhile, pointer is not needed for this single response solution.

Proposal 9: RAN2 confirms that one CB-MSG4 can target multiple UEs simultaneously (FFS on the details).

Proposal 10: RAN2 confirms that a pointer solution is not needed in Rel-19.

Next, regarding the RNTI issue, it can be configurable at the cost of signaling overhead. To make it simple, similar to PRACH repetition case, the UE can determine the RNTI based on the last occasion within the corresponding DSA transmission. With this, the network and UE can have the same understanding on the RNTI for CB-EDT transmission (scrambling PUSCH) and CB-Msg4 reception.

Proposal 11: The RNTI for CB-EDT transmission and CB-Msg4 reception is calculated based on the last occasion within the corresponding DSA transmission.

Upon reception, if no matched responses are received while all the response windows have expired, the UE should attempt the next CB-EDT transmission, selecting the CB-EDT resources again. In the re-attempt, power ramping should be supported so that the previously failed UE has more priority in the next contention resolution run. Further, as analyzed in our paper RR2-2408018, the PLA performance gain difference between SA and SA-BI is quite obvious. Thus the BI-reattempt (like PRACH) solution should be adopted for collision randomization. Regardless of which access mode (e.g. DSA, SA, OCC2) is adopted, a common backoff mechanism should be supported.

Proposal 12: If no matched responses are received while PDCCH monitoring window has expired, the UE re-attempts CB-EDT transmission.

Proposal 13: Power ramping is supported for the CB-EDT re-attempt.

Proposal 14: Backoff time (like PRACH) can be included in CB-Msg4.

Last but not least, considering the network may respond to a UE with RRC message (transiting the UE into CONNECTED), then RAN2 should consider a new MAC PDU design to allow a MAAC subPDU indicating 12-bit TA command, 16-bit C-RNTI, and 48-bit UE Contention Resolution Identity, HARQ feedback resource, and TPC, potentially together with the RRC message (e.g. RRC Setup message). An example is given below.

Figure 3, new subPDU design for CB-EDT

Proposal 15: RAN2 introduces a new MAC PDU indicating 12-bit TA command, 16-bit C-RNTI, and 48-bit UE Contention Resolution Identity, 7-bit HARQ feedback resource, and 2-bit TPC command.

3 Conclusion

In this contribution, we have provided our understanding of CB-EDT procedure. And we made the following proposals:

CB-EDT scenario and configuration:

Proposal 1: RAN2 deprioritizes the CB-EDT in 15kHz SCS NB-IoT or eMTC CE mode B cases.

Proposal 2: SA and DSA cannot be configured simultaneously for a given CE level.

Proposal 3: DSA window length is configured in multiples of CB-EDT occasion periodicity (e.g. 2..32).

Proposal 4: The starting point of the DSA window is located at the beginning of the first symbol of the responding CB-EDT occasion.

DSA modelling in MAC:

Proposal 5: MAC considers each replica transmission as a new transmission.

Proposal 6: HARQ entity obtains the MAC PDU from the HARQ process buffer for the secondary replica.

Proposal 7: HARQ process id 0 and RV 0 are used for CB-EDT.

Response reception:

Proposal 8: For both eMTC and NB-IoT, the UE starts a PDCCH monitoring window at the end of the corresponding DSA window plus UE-eNB RTT.

Proposal 9: RAN2 confirms that one CB-MSG4 can target multiple UEs simultaneously (FFS on the details).

Proposal 10: RAN2 confirms that a pointer solution is not needed in Rel-19.

Proposal 11: The RNTI for CB-EDT transmission and CB-Msg4 reception is calculated based on the last occasion within the corresponding DSA transmission.

Proposal 12: If no matched responses are received while PDCCH monitoring window has expired, the UE re-attempts CB-EDT transmission.

Proposal 13: Power ramping is supported for the CB-EDT re-attempt.

Proposal 14: Backoff time (like PRACH) can be included in CB-Msg4.

Proposal 15: RAN2 introduces a new MAC PDU indicating 12-bit TA command, 16-bit C-RNTI, and 48-bit UE Contention Resolution Identity, 7-bit HARQ feedback resource, and 2-bit TPC command.

4 Reference

[1] R2-2501702, RAN2#129 Meeting Report, MCC.

[2] R2-2502773, Report of [Post129][307][R19 IoT NTN] CB-msg3/CB-msg4, MediaTek Inc.