3GPP TSG-RAN WG2 Meeting #129bis                                                           R2-2501779

Wuhan, China, April 7th– 11th, 2025

Agenda item:	8.9.4

Source:	vivo

Title:	Further Discussion on PWS Support for NB-IoT

Document for:	Discussion and Decision

- Introduction

In RAN2#128 [1], the following agreements were made by RAN2 on PWS support for NB-IoT.

1. We will clarify in 36.300 section 4.10 that in general a NB-IoT UE may not support all PWS requirements. 2. We will extend the existing ETWS/CMAS notification RRC procedures for eMTC to NB-IoT. FFS if SIB1-NB-NB acquisition is needed 3. Add the following PWS indication in Paging-NB: - etws-Indication; - cmas-Indication.

In the contribution, we will analyze the open issue highlighted in yellow and give our view.

- Discussion

When we reviewed the spec, we found that the normal procedure for PWS acquisition includes the following steps:

Step 1: UE acquires the PWS notification either via direct indication information or via the paging message;

Step 2: UE acquires the SIB1-NB and decides the PWS scheduling information;

Step 3: UE acquires the corresponding PWS message in the corresponding resource.

Considering the urgency of the PWS message, the earlier the UE acquires the PWS message the earlier UE can make the corresponding response.

To enable the UE to accelerate the PWS acquisition, the candidate solution can be:

Option 1: NW provides the PWS scheduling information within SIB1-NB even though there is no PWS notification, and UE can avoid reading the SIB1-NB upon acquiring the PWS message.

According to the spec [2], the PWS information change does affect the value tag of the SIB1-NB. Thus for this option, it is totally up to NW implementation, i.e., NW always includes the PWS scheduling information even if there is no PWS notification. From the UE perspective, it can use the stored SIB1-NB directly upon receiving the PWS notification. One may argue there may be some resource consumption, but considering the urgency of the PWS, such a solution is also beneficial. Besides, option 1 can be totally up to implementation, there is no spec impact.

E-UTRAN may not update systemInfoValueTag upon change of some system information e.g. ETWS information, CMAS information, RLOS indication (i.e., rlos-Enabled), regularly changing parameters like time information (SystemInformationBlockType8, SystemInformationBlockType16, hyperSFN-MSB in SystemInformationBlockType1-NB), EAB and AB parameters, UAC parameters, positioning system information blocks, or satellite assistance information. Similarly, E-UTRAN may not include the systemInfoModification within the Paging message upon change of some system information.

Option 2: NW provides the PWS scheduling information or the PWS message directly within the paging message.

For this option, providing the PWS scheduling information in the paging message has a similar performance as option 1, while providing the PWS message in the paging message may be faster for PWS acquisition by UE. But the spec impact is larger, and it may impact RAN1, i.e.,  the PWS notification must be included in the paging message since the current LTE spec does not support the paging message and the direct indication information is provided at the same time [3]. Moreover, the signalling overhead is larger due to indicating the scheduling information or the PWS message in the paging message.

| #### 6.4.3.3 DCI Format N2 DCI format N2 is used for paging, direct indication, scheduling of one NPDSCH codeword carrying SC-MCCH in one cell, and notifying SC-MCCH change. The following information is transmitted by means of the DCI format N2: - If the format N2 CRC is scrambled by P-RNTI: - Flag for paging/direct indication differentiation – 1 bit, with value 0 for direct indication and value 1 for paging - Else if the format N2 CRC is scrambled by a SC-RNTI: - Information for SC-MCCH change notification – 1 bit as defined in clause 5.8a of [6] - If the format N2 CRC is scrambled by P-RNTI and Flag=0: - Direct Indication information – 8 bits provide direct indication of system information update and other fields, as defined in [6] - Reserved information bits are added until the size is equal to that of format N2 with Flag=1 - If the format N2 CRC is scrambled by P-RNTI and Flag=1, or if the format N2 CRC is scrambled by SC-RNTI: - Resource assignment – 3 bits as defined in clause 16.4.1.3 of [3] - Modulation and coding scheme – 4 bits as defined in clause 16.4.1.5 of [3] - Repetition number – 4 bits as defined in clause 16.4.1.3 of [3] - DCI subframe repetition number – 3 bits as defined in clause 16.6 of [3] |  |
| --- | --- |

In the following, we make a comparison of the above options. And it is clear the option 1 is better.

|  | Pros | Cons |
| --- | --- | --- |
| Option 1 | UE can avoid reading the SIB1-NB to accelerate the PWS acquisition; No spec impacts. | Resource consumption is reserved for PWS. |
| Option 2 | UE can avoid reading the SIB1-NB to accelerate the PWS acquisition. | Spec impacts are required; RAN1 may be involved; Signaling overhead. |

Besides, according to the following description [2], UE can assume the scheduling information in SIB1(-NB) is unchanged when acquiring the SIB31(-NB), that is, NW can provide the corresponding scheduling information in advance. Such a mechanism is similar to option 1. Thus, we think there is no complicated spec impact for option 1.

NOTE 3: When acquiring SIB31(-NB) in RRC_CONNECTED, UE may assume that the scheduling is unchanged.

Observation 1: The current spec already supports the NW to provide the SIB31(-NB) scheduling information to UE before the SIB31(-NB) is scheduled.

Proposal 1: To avoid UE reacquire the SIB1-NB upon acquiring the PWS message, the network provides the SIB10-NB/SIB11-NB/SIB12-NB scheduling information within SIB1-NB.

Proposal 2: When acquiring SIB10-NB/SIB11-NB/SIB12-NB, UE may assume that the scheduling is unchanged.

- Conclusion

In this contribution, we have further discussed the PWS support for NB-IoT. And we make the following observation and proposals:

Observation 1: The current spec already supports the NW to provide the SIB31(-NB) scheduling information to UE before the SIB31(-NB) is scheduled.

Proposal 1: To avoid UE reacquire the SIB1-NB upon acquiring the PWS message, the network provides the SIB10-NB/SIB11-NB/SIB12-NB scheduling information within SIB1-NB.

Proposal 2: When acquiring SIB10-NB/SIB11-NB/SIB12-NB, UE may assume that the scheduling is unchanged.

- Reference

- Draft_RAN2_128_Meeting_Report_v1.

- TS36.331, Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification.

- TS36.212, Evolved Universal Terrestrial Radio Access (E-UTRA); Multiplexing and channel coding.