3GPP TSG-RAN WG2 Meeting #129bis		R2-2501792

Wuhan, China, April 7th – 11th, 2025

Source:	vivo

Title:	Discussion on LCM and spec impact for AIML mobility

Agenda Item:	8.3.5

Document for:	Discussion and Decision

- Introduction

The study of AI for mobility has the following objective regarding LCM and specification impact [1]:

- Potential AI mobility specific enhancement should be based on the Rel19 AI/ML-air interface WID general framework (e.g. LCM, performance monitoring etc) [RAN2] - NOTE: This would only be treated after sufficient progress is made in the Rel-19 AI/ML air interface WID - Potential specification impacts of AI/ML aided mobility [RAN2]

In this contribution, we will discuss the LCM for AIML mobility based on the simulation results and current progress of AI/ML for air interface.

- Discussion

- 2.1 Common aspects of UE-sided model

2.1.1 General LCM Framework

The currently agreed LCM framework for the beam management use case under the AI/ML for NR air interface work item is as below:

Figure 2.1-1: General LCM procedure for beam management

We think that the above general LCM procedure can be the baseline for AI mobility with some minor adaptations. The following figure shows an example of potential LCM procedure for AI mobility:

Figure 2.1-2: Potential LCM procedure for AI mobility

We understand that RAN2 can take the above Figure 2.1-2 as potential LCM procedure for AI mobility into account for further discussion, including the following aspects:

- Data collection for model training

- UE capability

- Applicability reporting

- Inference configuration and reporting

- Performance monitoring and management

- The general LCM framework for beam management can be the baseline for AI mobility. RAN2 takes the above Figure 2.1-2 as potential LCM procedure for AI mobility into account for further discussion, including the following aspects:

- Data collection for model training

- UE capability

- Applicability reporting

- Inference configuration and reporting

- Performance monitoring and management

2.1.2 Data collection for model training

Regarding data collection for UE-sided model training for beam management, RAN2 reached the following agreements:

Agreements Extend the following agreements on data collection configuration in AI/ML based beam management to general UE-side data collection configuration: - Data collection related configuration(s) and associated ID(s)(if needed) can be included in training data collection configuration. - For data collection configuration UE-side model training, the UE can send a request for data collection (e.g. start/stop). FFS whether a suggested data collection configuration/associated IDs (if specified)/parameters can be provided to the network. - The network can provide or release the data collection configuration (at any point in time), with or without UE request. - The following methods for network control of the initiation and configuration for data collection: - The network can decide when to start/stop the data collection and send configuration. - The network can configure whether UE is allowed to initiate request for data collection (e.g. start/stop indication).

In NR, the measurement reference signal can be SSB or CSI-RS. In beam management use case, the two types of reference signals are supported. However, for AI mobility, we understand that at least SSB-based measurement prediction should be supported in Rel-19/20 since the mobility is typically based on SSB. RAN2 can discuss whether CSI-RS is also considered.

- For the reference signal type in AI mobility, SSB is supported as the baseline. RAN2 to discuss whether CSI-RS is also considered.

If CSI-RS-based measurement is supported for AI mobility, similarly to beam management, the UE needs to request the data collection configuration. We understand that the data collection schemes for beam management can be the baseline for model training of AI mobility.

- If CSI-RS is supported as a reference signal for AI mobility, the data collection configuration schemes for beam management can be the baseline for UE-sided model training of AI mobility, e.g., network can configure whether UE is allowed to initiate request for CSI-RS configuration.

For beam management use case, NW-sided additional conditions were discussed in RAN1 for the determination of the applicability of a functionality, which aims to ensure the consistency between model training and inference configuration, especially for spatial domain prediction. RAN2 needs to discuss whether a similar problem is valid and needs to be resolved for AI mobility.

- RAN2 to discuss whether NW-sided additional conditions (e.g., associated ID) are needed to ensure the consistency between training and inference for UE-sided model in AI mobility.

2.1.3 UE capability

Similar to beam management use case, we understand that the capability for AI mobility should be introduced in UE access capability. Based on the AI capability, the gNB can configure the inference configuration or request the applicability reporting. The detailed content can be discussed during the normative phase after all the schemes are clear.

- UE reports supported AI-based mobility features/FGs via UE radio access capability information. The detailed capability information can be discussed during the normative phase.

2.1.4 Applicability reporting

For beam management use case, RAN2 reached the following agreements about applicability reporting:

Agreements - Inference configuration/parameters can be signalled in step 3 and/or Inference configuration can be signalled in step 5 (i.e. option a and option b from RAN1). - The full inference configuration is sent in CSI-ReportConfig. - Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability. - FFS signaling details for option B (e.g. whether it is signaling in CSI-Report Config or otherconfig) - Agreements applicability reporting and management Support the explicit reporting of applicability/inapplicability in initial report and subsequent reporting it reports only applicability it changed. FFS if we report explicit cause If option A is configured in Step 3, for periodic CSI reporting, the UE autonomously activate the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete in step 4 (i.e. without need to wait RRCReconfiguration in Step 5). The provided periodic CSI configuration should be consistent with reported UE capabilities

- Take the agreements on applicability reporting of UE-sided model in AI-based beam management as the baseline, i.e., support both Option A and Option B like scheme.

2.1.5 Inference configuration and reporting

In the existing RRM measurement configuration framework, the network can configure one or more measurement IDs. Each measurement ID is associated with one measurement object and one report configuration. Each measurement object is associated with one frequency and each report configuration is associated with one report type, e.g., periodical, event-triggered. For RRM measurement prediction and measurement event prediction, the potential assumption is following the measurement model in TS38.300. Therefore, it is natural that the existing RRM measurement configuration framework can be the baseline for inference configuration for RRM measurement prediction and measurement event prediction.

- Reuse the existing RRM measurement framework for inference configuration and prediction results report for RRM measurement prediction and measurement event prediction.

- For prediction results report of RRM measurement prediction of UE-side model, both periodic and event-triggered reporting are supported.

In the current AI/ML for mobility study, the following 3 sub-use cases are considered for L3 cell-level RRM measurement prediction:

- Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 cell-level measurement result is generated

- Sub-use case 2: L3 Cell-level measurement result(s) is predicted based on actual L3 cell-level measurement result(s)

- Sub-use case 3: L3 Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s)

Different sub-use case utilizes different types of measurement results as input. However, from the view of the specification, the focus points are the prediction results and how to use the prediction results. One issue is whether to need to configure the input of the model by the network or leave it to UE implementation. We think that the latter is more reasonable.

- The network does not configure the input of inference for the UE-sided model, and the input is up to UE implementation.

As to the output, in the previous meeting, RAN2 agreed the following two goals:

- Goal 1: measurement reduction

- Goal 2: handover performance improvement

In the whole RRM measurement procedure, there are three phases:

- Phase 1: the UE performs the measurement to obtain the measurement results.

- Phase 2: the UE determines whether to trigger the measurement report based on measurement results.

- Phase 3: if triggered, the UE sets the content of the measurement report and sends the measurement report to the network.

Achievement of Goal 1 depends on measurement reduction during Phase 1. For Goal 1, we think that RAN2 should firstly discuss how to perform the prediction (e.g., prediction granularity) to achieve the measurement reduction goal before discussing the inference configuration.

The main purpose of measurement reduction goal is to save the UE power and achieve no/less measurement gap. There are two main steps when the UE performs the measurement of a frequency in Phase 1:

- Step 1: the UE receives the signal of the frequency via RF receiver.

- Step 2: the UE processes the signal to obtain the measurement result of all the cells in the frequency via baseband.

According to the industry materials, the proportion of power consumption in Step 1 is about 80% and the one in Step 2 is about 20%. In NR, the measurement object is configured per frequency for RRM measurement. It means that the UE needs to measure all the cells for a measured frequency. The difference in power consumption between measuring all the cells of a frequency and measuring one cell of the frequency may be quite small since the main power consumption is receiving the signal of the frequency by the RF receiver. Moreover, if there is any one cell of the measured frequency that needs to be measured with gap, the measurement gap needs to be configured.

Observation 1: The most power consumption of UE measurement is the RF receiver, not the baseband processing. If there is any one cell of a specific frequency that needs to be measured, no gain of measurement gap reduction can be achieved and the gain of power saving is quite limited.

Therefore, we think that for frequency domain prediction, RAN2 should consider the per-frequency granularity prediction (i.e., predict all the cells of a frequency) to achieve the goal of measurement reduction. Similarly, for temporal domain prediction, all the cells of the measured frequency should be predicted in the predicted time instances to save the signal receiving of the RF receiver in the predicted time instances to achieve the measurement reduction effect. In the measurement procedure, the RF receiver needs to receive the signals from multiple different directions for different beams. If some directions can be predicted, the power can be saved. There may be multiple cells for the measured frequency in the predicted directions. Therefore, it is also a type of per-frequency granularity prediction.

- To achieve the goal of measurement reduction, the prediction should be per-frequency granularity, i.e., all the cells in a frequency layer should be predicted, either for temporal domain, frequency domain, or spatial domain prediction for measurement reduction.

Goal 2 is related to Phase 2 and Phase 3. To reduce the power consumption of inference, gNB may provide a list of cells that need to be predicted. Therefore, we propose:

- To achieve the goal of HO performance improvement, the prediction can be per-frequency or per-cell granularity.

In addition, how to use the prediction results to achieve the goal of handover performance improvement in Phase 2 and Phase 3 should be discussed. From our understanding, at least the following two approaches of event-triggered RRM measurement prediction reporting should be considered:

- Approach 1: RRM measurement prediction reporting is triggered based on the legacy measurement event in Phase 3. In Approach 1, measurement report triggering is based on the existing scheme, i.e., based on actual measurement results, but the measurement report can carry the predicted results of the triggered cells and serving cell. Based on the predicted results of the triggered cells and serving cell, the gNB can better determine whether to perform the handover or handover to which cell, which can decrease the occurrence of, e.g., ping-pong handover and the handover to wrong cell. For example, if the serving cell becomes better and all the triggered cells become worse, the gNB can determine not to perform the handover; if there are two triggered cells, the gNB can determine to handover to the cell that is becoming better, not the cell that is becoming worse.

- Approach 2: RRM measurement prediction reporting is triggered based on predicted results in Phase 2, i.e., indirect measurement event prediction. In approach 2, the measurement report is triggered based on predicted results in the temporal domain and can be triggered in advance. Based on the measurement report, the gNB can perform the handover procedure (e.g., handover preparation procedure) earlier.

- To achieve the goal of HO performance improvement, the following two approaches for event-triggered prediction reporting can be considered:

      - Approach 1: RRM measurement prediction reporting can be triggered by legacy measurement event based on actual measured results.

      - Approach 2: RRM measurement prediction reporting can be triggered by measurement event prediction.

2.1.6 Monitoring and Management

For beam management use case, RAN2/RAN1 reached the following agreements:

Agreements: 1 For UE-sided model, for the functionality management, the “network decision, network-initiated” AI/ML management is supported as a baseline. The following can be considered further “UE autonomous, decision reported to the network”, “Network decision, UE-initiated” (i.e. proactive approach). 2 “UE-autonomous, UE’s decision is not reported to the network” is not considered for Rel-19

Agreement @R2#117 For BM-Case1 and BM-Case2 with a UE-side AI/ML model: - Support Type 1 performance monitoring, including the following two options: - Option 1 (NW-side performance monitoring): - UE sends a report to NW (for the calculation of performance metric at NW) - Measurement results from resource set for monitoring, e.g., L1-RSRP and/or RS index is supported as the content of the report - FFS on other contents - The report is at least configured/triggered by NW - Note: this may or may not have additional spec impact - Option 2 (UE-assisted performance monitoring): - UE calculates performance metric(s) - FFS how to report and what to report - FFS whether to trigger the report based on event(s) for Option 1 and/or Option 2 - FFS Type 2 performance monitoring

For AI mobility, the above agreements can be applicable.

- Take the agreements on monitoring and management of UE-sided model in AI-based beam management as a baseline:

- The network can make management decisions based on performance monitoring.

- The network can request UE to report measurement results or performance metrics for monitoring.

For beam management, there are three types of CSI reporting: periodic, semi-persistent, and aperiodic. There is the activation/deactivation scheme due to three types of CSI-reporting.

- Agreements applicability reporting and management If option A is configured in Step 3, for periodic CSI reporting, the UE autonomously activate the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete in step 4 (i.e. without need to wait RRCReconfiguration in Step 5). The provided periodic CSI configuration should be consistent with reported UE capabilities Semi-persistent and aperiodic CSI reporting of applicable functionality is activated following legacy CSI framework: - Semi-persistent reporting, activated by MAC CE/DCI - Aperiodic CSI reporting, activated by DCI

However, there is no such issue for AI mobility. We think that no explicit management procedure (e.g., activation/deactivation) is needed for AI mobility.

- No need to introduce explicit management procedures (e.g., activation/deactivation) for AI mobility. I.e., management of the UE-sided model is realized implicitly by the provision of inference configuration.

- 2.2 RRM measurement prediction-specific aspects

2.2.1 UE-sided model

Based on the simulation results, the key parameters that have an impact on the performance of RRM measurement prediction are summarized in [3].

Scenario 2 Proposal 2: The key parameters for scenario 2 for graphical illustration are MRRT and UE speed. Proposal 3: Restrict the number of combinations of the key parameters for capturing the simulation results in a graphic illustration in the TR. Proposal 4: For scenario 2 graphic illustration, the UE speed is set to 30 km/h and 90 km/h; MRRT is set to 50%, 66%, 80% and 90%. Scenario 4 Proposal 5: For scenario 4 graphic illustration, the key parameters are [OW:PW] or PW, and UE speed. Proposal 6: For scenario 4 graphic illustration, the UE speed is set to 60 km/h and 120 km/h. Two options will be considered: - OW:PW ratio is set to 1:1, 2:1 and 1:2. The setting of PW and OW will be considered if needed. - Only consider PW value. Scenario 3 Proposal 7: For scenario 3, use the cluster setting for graphic illustration. Scenario 6 Proposal 8: For Scenario 6, the key parameters for graphic illustration are MRRS and UE speed. If necessary, graphic illustrations including clustering can be added later. Proposal 9: For Scenario 6, rely on the rapporteur to illustrate the findings based on the available simulation results in a best-effort manner. We can later discuss whether to capture those findings in the TR.

Therefore, we propose:

- For RRM measurement prediction of UE-sided model, the network may configure the following parameters in inference configuration:

      - Temporal domain case B prediction: information related to MRRT;

      - Temporal domain case A prediction: information related to PW length;

      - Spatial domain prediction: information related to MRRS.

As to the UE speed, it was concluded that a higher UE speed will result in decreased prediction accuracy. However, the NW can not control the UE speed, while can acquire UE speed based on the UE history information and RRM measurement. The NW can send appropriate inference configurations to UE at certain speeds. Besides, if the prediction accuracy of a high-speed UE exceeds the accuracy requirement, the NW can release the inference configuration based on the performance monitoring result. Similar solution is also applicable for NW-side model.

Therefore, we propose:

- For RRM measurement prediction, UE speed does not have a specification impact from RAN2 perspective.

In addition to the LCM procedure, there are some potential issues for RRM measurement prediction, which may have specification impacts and need to be discussed by RAN2. For example, how to perform the cell derivation or layer-3 filtering based on inference output since the inference output naturally has the problem of prediction accuracy compared with actual measurement results?

- For RRM measurement prediction of UE-sided model, RAN2 to discuss the mechanism of cell-level result derivation/layer-3 filtering based on inference output.

2.2.1 NW-sided model

For NW-sided model, the data collection schemes for beam management are being discussed in RAN2 and have reached some fruitful agreements. The related schemes can be the baseline for data collection of model training of AI mobility. The potential AI mobility-specific enhancement can be discussed in the normative phase.

- For RRM measurement prediction of NW-sided model, the data collection schemes for beam management can be the baseline for model training and inference operation of AI mobility, e.g., the network can configure UE to log multiple instances of RRM measurement results. The potential AI mobility-specific enhancement can be discussed in the normative phase.

For beam management use case, RAN2 concluded that the existing L1 measurement configuration and report framework can be used to configure and report the input data for inference of the NW-sided model.

- For AI/ML based beam management, RAN2 assumes the L1 measurement framework shall be used for configuring the input data of the NW side AI/ML model inference.  FFS if further enhancements are needed

Agreements for NW-sided model for Beam Management

1	For the network-side model, required network side additional condition is left up to the network implementation

2	RAN2 will wait for RAN1 for any required UE side additional conditions.

3	For network-sided model for BM use case, RAN2 confirms that UE inputs for inference at network-sided model will rely on L1 signaling, RAN2 will not further spend time on this aspect.

We think that similarly the existing RRM measurement framework can be reused for configuring the input data for the inference of NW-sided model.

- For RRM measurement prediction of NW-sided model, the existing RRM measurement framework can be reused for configuring the input data for the inference of the NW-sided model.

Furthermore, RAN2 discussed the monitoring and management of NW-sided model for beam management and had the following agreement:

Agreements

1	RAN2 confirms that UE will not be informed about any gNB/LMF-sided model/functionality management decision (e.g., selection, (de)activation, switching, fallback, etc.)

2	RAN2 confirms that UE will not be involved in any gNB/LMF-sided model/functionality management decision making (e.g., selection, (de)activation, switching, fallback, etc.), except being configured to provide the required measurement/data.

3	RAN2 focuses on the data collection procedure from UE to NW (e.g., gNB, LMF, or OAM) for the sake of NW-sided model LCM (including training, inference, management).

4	The gNB is responsible for monitoring its own performance.  RAN2 will work on RAN2 specifications enhancements associated to gNB-side model monitoring, only based on RAN1 inputs, if any

We think that the above conclusions can be also applicable to AI mobility.

- Take the agreements on monitoring and management of NW-sided model in AI-based beam management as the baseline:

      - UE will not be informed about the management decision of NW-sided model.

      - UE is not involved in management decision making of NW-sided model, except being configured to provide the required measurement/data.

      - gNB can perform the monitoring based on the required measurement/data.

- 2.3 Measurement event prediction-specific aspects

For the measurement event prediction, the following observations were made based on simulation results.

For indirect measurement event prediction based on intra-frequency temporal domain case A, the following observations are made: Most of the simulation results show that the F1 score is very good For indirect measurement event prediction based on intra-frequency temporal domain case B, the following observations are made: Very good F1 score can be achieved, which depends on filtering approach or PW length Good F1 score can be achieved with small PW length Higher MRRT value correlates with decreased F1 score F1 score for direct measurement is very good based on the simulation results by assuming 50% probability threshold Editor note 1: Indirect event prediction based on frequency domain prediction will be considered for the specification impact study without explicit simulations

However, the observations are not completed since the simulation discussion is still ongoing. Detailed specification impact analysis should wait for further progress, e.g., whether to support direct event prediction.

In addition to the LCM procedure, there are some potential issues for measurement event prediction, which may have specification impacts and need to be discussed by RAN2. For example,

- How to configure the related parameters?

- How to enhance the event triggering scheme based on prediction result?

- How to enhance the measurement report to carry prediction results?

RAN2 should identify potential issues as much as possible during the study phase and discuss the potential solutions.

- For measurement event prediction, RAN2 to identify and discuss the potential issues and solutions, e.g., the configuration of related parameters, enhanced event triggering scheme based on predicted results, enhanced measurement report content.

- Conclusion

In the contribution, we have the following observations and proposals:

Common aspects of UE-sided model

- The general LCM framework for beam management can be the baseline for AI mobility. RAN2 takes the above Figure 2.1-2 as potential LCM procedure for AI mobility into account for further discussion, including the following aspects:

- Data collection for model training

- UE capability

- Applicability reporting

- Inference configuration and reporting

- Performance monitoring and management

- For the reference signal type in AI mobility, SSB is supported as the baseline. RAN2 to discuss whether CSI-RS is also considered.

- If CSI-RS is supported as a reference signal for AI mobility, the data collection configuration schemes for beam management can be the baseline for UE-sided model training of AI mobility, e.g., network can configure whether UE is allowed to initiate request for CSI-RS configuration.

- RAN2 to discuss whether NW-sided additional conditions (e.g., associated ID) are needed to ensure the consistency between training and inference for UE-sided model in AI mobility.

- UE reports supported AI-based mobility features/FGs via UE radio access capability information. The detailed capability information can be discussed during the normative phase.

- Take the agreements on applicability reporting of UE-sided model in AI-based beam management as the baseline, i.e., support both Option A and Option B like scheme.

- Reuse the existing RRM measurement framework for inference configuration and prediction results report for RRM measurement prediction and measurement event prediction.

- For prediction results report of RRM measurement prediction of UE-side model, both periodic and event-triggered reporting are supported.

- The network does not configure the input of inference for the UE-sided model, and the input is up to UE implementation.

Observation 1: The most power consumption of UE measurement is the RF receiver, not the baseband processing. If there is any one cell of a specific frequency that needs to be measured, no gain of measurement gap reduction can be achieved and the gain of power saving is quite limited.

- To achieve the goal of measurement reduction, the prediction should be per-frequency granularity, i.e., all the cells in a frequency layer should be predicted, either for temporal domain, frequency domain, or spatial domain prediction for measurement reduction.

- To achieve the goal of HO performance improvement, the prediction can be per-frequency or per-cell granularity.

- To achieve the goal of HO performance improvement, the following two approaches for event-triggered prediction reporting can be considered:

- Approach 1: RRM measurement prediction reporting can be triggered by legacy measurement event based on actual measured results.

- Approach 2: RRM measurement prediction reporting can be triggered by measurement event prediction.

- Take the agreements on monitoring and management of UE-sided model in AI-based beam management as a baseline:

- The network can make management decisions based on performance monitoring.

- The network can request UE to report measurement results or performance metrics for monitoring.

- No need to introduce explicit management procedures (e.g., activation/deactivation) for AI mobility. I.e., management of the UE-sided model is realized implicitly by the provision of inference configuration.

RRM measurement prediction-specific aspects

- For RRM measurement prediction of UE-sided model, the network may configure the following parameters in inference configuration:

      - Temporal domain case B prediction: information related to MRRT;

      - Temporal domain case A prediction: information related to PW length;

      - Spatial domain prediction: information related to MRRS.

- For RRM measurement prediction, UE speed does not have a specification impact from RAN2 perspective.

- For RRM measurement prediction of UE-sided model, RAN2 to discuss the mechanism of cell-level result derivation/layer-3 filtering based on inference output.

- For RRM measurement prediction of NW-sided model, the data collection schemes for beam management can be the baseline for model training and inference operation of AI mobility, e.g., the network can configure UE to log multiple instances of RRM measurement results. The potential AI mobility-specific enhancement can be discussed in the normative phase.

- For RRM measurement prediction of NW-sided model, the existing RRM measurement framework can be reused for configuring the input data for the inference of the NW-sided model.

- Take the agreements on monitoring and management of NW-sided model in AI-based beam management as the baseline:

      - UE will not be informed about the management decision of NW-sided model.

      - UE is not involved in management decision making of NW-sided model, except being configured to provide the required measurement/data.

      - gNB can perform the monitoring based on the required measurement/data.

Measurement event prediction-specific aspects

- For measurement event prediction, RAN2 to identify and discuss the potential issues and solutions, e.g., the configuration of related parameters, enhanced event triggering scheme based on predicted results, enhanced measurement report content.

- Reference

- RP-234055		Study on AI (Artificial Intelligence)/ML (Machine Learning) for mobility in NR

- R2-2500011	Reply LS on applicable functionality reporting for beam management UE-sided model

- R2-2501557 	[AT129][022][AI Mob] Simulations (Mediatek)	Mediatek Inc.