3GPP TSG-RAN WG2 Meeting #129bis		R2-2501807

Wuhan, China, 7th – 11th, April 2025

Source:	vivo

Title:		Discussion on LCM for UE-sided model for Beam Management

Agenda Item:	8.1.2.2

Document for:	Discussion and Decision

# Introduction

At the RAN2#129 meeting, RAN2 achieved the following agreements regarding LCM for UE-sided model for beam management use case:

Agreements - Inference configuration/parameters can be signalled in step 3 and/or Inference configuration can be signalled in step 5 (i.e. option a and option b from RAN1). - The full inference configuration is sent in CSI-ReportConfig. - Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability. - FFS signaling details for option B (e.g. whether it is signaling in CSI-Report Config or otherconfig) - Agreements applicability reporting and management - Support the explicit reporting of applicability/inapplicability in initial report and subsequent reporting it reports only applicability it changed. FFS if we report explicit cause - If option A is configured in Step 3, for periodic CSI reporting, the UE autonomously activate the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete in step 4 (i.e. without need to wait RRCReconfiguration in Step 5). - The provided periodic CSI configuration should be consistent with reported UE capabilities - FFS option B - Semi-persistent and aperiodic CSI reporting of applicable functionality is activated following legacy CSI framework: - Semi-persistent reporting, activated by MAC CE/DCI - Aperiodic CSI reporting, activated by DCI

In this contribution, we will further discuss the LCM for UE-sided model for beam management use case based on RAN2 agreements and RAN1’s reply LS[1].

# Discussion

## 2.1 Whether to use “functionality” concept in RAN2

In the RAN1’s reply LS [1]:

In RAN1’s discussion of RAN 2 terminologies on beam management, - The concept/terminology “functionality” of Supported functionalities may refer to UE-capability information/parameters i.e., Rel-19 AI/ML-enabled Features/FGs - The concept/terminology “functionality” of Applicable functionalities may refer to CSI-ReportConfig for inference configuration or a set of inference related parameters - The Activated functionalities may be enabled based on CSI framework. Therefore, the meaning and the granularity of “functionality” for Applicable functionalities, Activated functionalities and Supported functionalities may or may not be the same. … Q1: In Step 2, what is the granularity of functionality? For example, whether it is a use case (e.g. beam management), whether it is a sub-use case (e.g. beam management Case 1), or others? Answer to Q1: In Step 2, RAN1 expects that UE reports its UE-capability information/parameters, i.e., Rel-19 AI/ML-enabled Features/FGs (including components and corresponding value ranges). These UE capability information/parameters will depend on how FGs are defined including the granularity, that will be discussed in RAN1 later in the WI.

RAN1 thinks that the meaning and the granularity of “functionality” for Applicable functionalities, Activated functionalities and Supported functionalities may or may not be the same, and they refer to different things. Moreover, if going through RAN1’s chairman note in the RAN1#119 meeting, it seems that RAN1 is avoiding to use the “functionality” concept. One issue is whether to remain using the “functionality” concept in RAN2 from the RAN2’s perspective. In current TS 38.300/331 running CRs, there is also the following editor’s note:

Editor's Note: FFS how to update the definition, e.g., replace 'functionality'

According to RAN1’s LS [1], we think that RAN2 can keep the “functionality” concept at least for applicability reporting for the convenience/brevity of RAN2 specification description. From the view of RAN2, the “functionality” can be defined as “inference configuration or a set of inference related parameters”.

- RAN2 keeps the “functionality” concept at least for applicability reporting for the convenience/brevity of RAN2 specification description. The “functionality” is defined as “inference configuration or a set of inference related parameters”.

## 2.2 Signalling for inference configuration and applicability reporting

Based on current RAN1 and RAN2 agreements, we think that it can be categorized into two aspects：one is inference configuration and the other is applicability reporting. We will discuss the signaling for these two aspects separately in the following two subsections.

### 2.2.1 Inference configuration

There are the following RAN1’s agreements in [2]:

Agreement - In Step 3, following configurations are provided from NW to UE: - UE is allowed to do UAI reporting via OtherConfig, - The applicability report is based on A) and/or B) - It is up to RAN 2 to design the container - A) one or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in CSI framework as working assumption applied) - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3 - B) One set or multiple sets of inference related parameters for applicability report only (not for inference) - It is up to RAN2 to design the container. - The set of inference related parameters selected from the IEs in/or the IEs referred by CSI-ReportConfig as a starting point, e.g., - the associated ID - Note: this doesn’t imply the associated ID is mandatory - Set A related information - Set B related information - Report content related information - For BM-Case 2, - Time instances related information for measurements - Time instances related information for prediction - In Step 4, UE reports applicability for all the above A) one or more CSI-ReportConfig and/or B) set(s) of inference related parameters - FFS on whether/what other information along with the applicability is needed - If A) is configured in Step 3, - Applicable aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after the applicability reported. - Applicable periodic CSI Report is considered as activated only if the applicability of the corresponding CSI-ReportConfig is reported in RRCReconfigurationComplete. - In Step 5, NW can optionally configure CSI-ReportConfig for inference configuration in RRCReconfiguration, where the associated ID may be configured in CSI framework as working assumption applied. - Note: Step 5 may be optional if UE has already been configured with CSI-ReportConfig in Step 3

RAN1 agreed on two options: Option A and Option B. The inference configuration is configured in Step 3 for Option A and in Step 5 for Option B. For Option A, the UE needs to report applicability for the inference configuration. For Option B, the UE does not need to report applicability for the inference configuration in Step 5. One issue is how the UE knows whether to report applicability upon receiving inference configuration since inference configuration is configured via the RRCReconfiguration message in both options. To address the issue, we think that there needs to be an indicator associated with the configured inference configuration to indicate whether to feedback the applicability report for the inference configuration.

- An indicator associated with configured inference configuration is introduced to indicate whether to feedback the applicability report for the inference configuration.

Inference configuration configured by the network may be non-applicable, especially for Option A. What is UE behavior upon receiving a non-applicable inference configuration? Two potential approaches are as below:

- Alt 1: the UE autonomously releases the non-applicable inference configuration;

- Alt 2: the UE maintains the configuration until the network releases it;

We think that Alt 2 is desirable since it is more in line with RAN2's usual practice.

- If the inference configuration configured by the network is non-applicable, the UE maintains the configuration until the network releases it.

### 2.2.2 Applicability reporting

At the RAN2#127bis meeting, RAN2 agreed “UAI is supported and RRCReconfigurationComplete message can be used to report applicable functionality”. At the RAN2#129 meeting, RAN2 agreed “Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete”. However, it is still FFS on signaling details for option B.

For Option B, we think that “one set or multiple sets of inference related parameters” should be other configuration since it is only used to obtain the applicability reporting. Therefore, the one set or multiple sets of inference related parameters for Option B should be configured in the otherConfig.

- For Option B, the one set or multiple sets of inference related parameters are configured in the otherConfig.

Upon receiving inference related parameters for Option B, the UE needs to feedback the applicability reporting. If Proposal 4 is agreed, similar to other “other configuration”, e.g., overheating assistance config, the UE should feedback the applicability reporting via a UAI message.

- For Option B, the UE feedbacks the applicability reporting via a UAI message upon receiving one set or multiple sets of inference related parameters.

According to RAN1 agreements, the UE may be configured with multiple CSI-ReportConfig for Option A or multiple sets of inference related parameters for Option B. One issue is how to configure the triggering of applicability reporting, e.g., whether it is necessary to configure the triggering per CSI-ReportConfig or per set of inference related parameters? We think that if the network wants to know the applicability status change, it should be for all the configured CSI-ReportConfig or sets of inference related parameters, not just for part of them. Therefore, it is sufficient to configure the triggering per UE.

- The allowance of applicability reporting for applicability status change of inference configuration is configured per UE.

In the latest RAN2 meeting, RAN2 agreed “Support the explicit reporting of applicability/inapplicability in initial report and subsequent reporting it reports only applicability it changed. FFS if we report explicit cause”. We think that the explicit cause is unnecessary because the cause cannot significantly help the network for subsequent handling even if the cause is reported. Moreover, the cause may expose the implementation inside the UE.

- Not support the explicit cause for non-applicable functionality in the applicability reporting.

# Conclusion

In the contribution, we have the following observations and proposals:

Whether to use “functionality” concept in RAN2

- RAN2 keeps the “functionality” concept at least for applicability reporting for the convenience/brevity of RAN2 specification description. The “functionality” is defined as “inference configuration or a set of inference related parameters”.

Inference configuration

- An indicator associated with configured inference configuration is introduced to indicate whether to feedback the applicability report for the inference configuration.

- If the inference configuration configured by the network is non-applicable, the UE maintains the configuration until the network releases it.

Applicability reporting

- For Option B, the one set or multiple sets of inference related parameters are configured in the otherConfig.

- For Option B, the UE feedbacks the applicability reporting via a UAI message upon receiving one set or multiple sets of inference related parameters.

- The allowance of applicability reporting for applicability status change of inference configuration is configured per UE.

- Not support the explicit cause for non-applicable functionality in the applicability reporting.

# Reference

- R1-2410898 Reply LS on applicable functionality reporting for beam management UE-sided model, RAN1

- Draft Report of 3GPP TSG RAN WG1 #119 v0.3.0