3GPP TSG-RAN WG2 Meeting#129bis       	R2-2401809

Wuhan, China, 7th – 11th, April 2025

Source:	vivo

Title:	Discussion on NW side data collection

Agenda Item:	8.1.3

Document for:	Discussion and Decision

# Introduction

At the RAN2#129, RAN2#128 and RAN2#127bis meeting, on NW side data collection, RAN2 made the following agreements[1, 2, 3]:

All agreements for NW side data collection

- Support the use of L3 measurement event triggered (i.e. L3 serving cell measurements becoming worse/better than a threshold for TTT) to determine whether the UE performs logging or not.  L1 measurement event triggered will not be supported.    FFS what to log

- Low power bit indication is supported

- Data availability indication is supported.  FFS when this would be triggered

- As baseline, the UEInformationResponse contains one or more logged measurement entries in chronological order (i.e. starting from the oldest measurement entries stored in the UE memory), and an availability indication if there are further data available for transmission. Same principles as for logged MDT.

- UE retains logged data during handover (HO).  FFS if there is scenarios where the UE needs to release the data and how does the UE know and if control from network is needed

- UE indicates availability of logged data during handover (i.e., within the RRCReconfigurationComplete message) (if data is retained in the UE).

- FFS how to handle idle/inactive and RLF cases and whether we have a unified.

Agreements on NW side data collection - Focus on the following three radio condition event based logging - L3 serving cell measurement based (e.g. X1/X2 similar to A1/A2) - Beam based events (e.g. beam becomes top-1 beam and number of measurements is less than configured value) - L1 beam level measurement - Measurements on aperiodic CSI resources are not reported for NW sided data collection. - Data collection is controlled by the network. The UE will not autonomously stop when low power state is detected. - The UE reports to the network when the power state is low. We will not specify how the UE determines low power state. The network should de-configure the data collection (this can be captured in stage 2). - The UE reports to the network when buffer is or may become full. FFS when it reports (before and/or after). - The UE can report the reason for triggering of indication for the status (e.g. low power state, low memory). FFS how this is signalled and if the reporting can be part of availability indication.

and

Agreements on NW side data collection

- Periodic logging is supported for training data collection procedure in R19

- Event-triggered data logging will be supported.  At least radio condition based event triggered logging will be supported.  FFS the details of radio condition based event.  FFS if other events are supported.

- Periodic reporting of logged data is not supported.

- On-demand reporting of the logged measurements will be specified

- UEInformationRequest/UEInformationResponse is used for on-demand reporting of AI/ML training data collection.   FFS of details of the message

- The UE can indicates the availability of logged data to the network to assist network to trigger UEInformationRequest.  FFS trigger/definition of availability indication.   and FFS how data availability indication is sent to the network.

- Low priority SRB will be used.    FFS new SRB or use of SRB4

- For data collection for both NW-sided/UE sided BM model training, at least L1-RSRPs and/or beam-IDs needs to be collected by UE.  FFS if other data needs to be collected based on RAN1 progress.

In this contribution, we will further discuss some remaining issues on NW-side data collection for model training for beam management.

# Discussion

## Data logging

In addition, RAN1 discussed and reached the following WA regarding associated ID.

Working Assumption Regarding the associated ID for Rel-19, the UE assumes that NW-side additional conditions with the same associated ID are consistent at least within a cell - FFS: whether/how UE assumption can be applicable for multiple cells (including the feasibility study)

The above WA is related to the generalization of the model for beam management. Specifically, when the associated ID of a specific cell is the same during the training and inference phases, the UE assumes the NW-side additional conditions for UE-sided model are fulfilled.

Correspondingly, for NW-sided model, only the data collected by the UE under specific cell(s) with the specific associated ID(s) can be used for model training. In other words, when the UE moves to a serving cell that is not associated with an associated ID, it should stop data collection.

- Introduce an area (e.g., cell ID) configuration to restrict the area in which the UE performs training data logging. And each cell within the area should be associated with an associated ID.

R1 has discussed data collection for training for NW-sided model [4], the discussion was summarized as follows:

Send LS to RAN 2, to inform RAN 2 that from RAN 1 perspective, for content for data collection for training for NW-sided model via higher layer signaling, for BM-Case1, the following three types of contents for each instance can be considered supported: - Type 1: All L1-RSRPs of measured based the resources (for Set A/B) configured to UE - Type 2: All L1-RSRPs measured based on resources (for Set B) configured to UE, and beam information of K beams based on some resources (for Set A) configured to UE - Type 3: All L1-RSRPs measured based on resources (for Set B) configured to UE, and beam information and L1-RSRPs of K2 beams based on some resources (for Set A) configured to UE - Wherein the K beams are the beams with the largest K L1-RSRPs based on the resources (for Set A) in one instance - K is configured by NW.

For Type 1, L1-RSRPs for both Set A and B are collected and can be considered model traning. Compared to Type2and Type3, Type 1 provides more information as it includes both Set A and Set B related. For Type 2 and Type 3, Type 3 may provide usefull information for model training.

- As baseline RAN2 can assume the following mesurements are usefull for NW-side model training:

- Type 1: All L1-RSRPs of measured based the resources (for Set A/B) configured to UE;

- Type 3: All L1-RSRPs measured based on resources (for Set B) configured to UE, and beam information and L1-RSRPs of K beams based on some resources (for Set A) configured to UE.

- For NW side model, LS to RAN1 to feedback umderstanding on the following RAN2 following assumptions:

- Introduction of area (e.g., cell ID) configuration to restrict the area in which the UE performs training data logging. And each cell within the area should be associated with an associated ID;

- Consider the following type of data content:

  - Type 1: All L1-RSRPs of measured based the resources (for Set A/B) configured to UE;

  - Type 3: All L1-RSRPs measured based on resources (for Set B) configured to UE, and beam information and L1-RSRPs of K beams based on some resources (for Set A) configured to UE.

## Data collection configuration

For UE-side model, the network can configure whether UE is allowed to initiate a request for training data collection. Data collection-related configuration (e.g., measurement resources configuration) and associated ID can be included in the training data collection configuration.

It was argued that it may happen that UE cannot perform data collection based on received configuration. The motivation behind this assumption is not clear. From our understanding, the network would configure UE to perform data collection based on UE capability, i.e., UE is not expected to be configure with configurations that are not aligned with UE capability. If some errors occur in RRC configuration that UE cannot comply with, this can be handled by current RRC specification. No further data collection specific discussion is necessary. In such the UE does not need to indicate anything to network.

FFS whether an indication from UE to network is needed when UE cannot perform data collection based on received configuration.

- There is no data collection specific issue of UE not being able to perform data collection based on received configuration and there is no need of any indication from UE to network.

## Availability indication of logged data

At the last RAN2 meeting, it was agreed that the UE can indicate the availability of logged data to the network to assist network to trigger UEInformationRequest message. The trigger/definition of availability indication is FFS. From our understanding, at least the following cases should be considered:

- When logging data, based on NW configuration, UE AS buffer may be full, and UE can indicate it to network via UAI. In this case, UE does not need to indicate how much data it has in its buffer; the indication can just be a single bit to NW. Based on this indication the network then can request to report the logged data.

- During the SI phase, it was agreed that the data collection for model training should focus on the RRC_CONNECTED state (for both data generation and reporting). Furthermore, to avoid further discussion on the RRC status transition, we think the UE should release the training data collection configuration and logged training data upon transitions from RRC_CONNECTED state to other states. Similar to to when UE transitions to RRC IDLE/INACATIVE, when UE experiences RLF, UE may go to RRC release. From that point UE behavior is the same as UE directly transioning to RRC IDLE. Additionally, when UE experience RLF, UE may restablesh to another gNB far from the one where RLF happens. Thus, logged data may not be useful for the target gNB.

RAN2 has also agreed that the UE reports to the network when buffer is or may become full.

Based on the above analysis, we propose:

- A single bit indication would be sufficient for UE to indicate to NW that UE has available logged data. No additional information, e.g., logged data size, is needed.

- There is no need for threshold-based buffer status report, UE should send the availability indication opon the buffer becomes full.

- UE should release the training data collection configuration and logged training data upon transitions from RRC_CONNECTED to RRC_INACTIVE/IDLE.

- When UE experience RLF, to guarantee unified behavior to the case of UE transitioning  from RRC_CONNECTED to RRC_INACTIVE, UE releases the training data collection configuration and logged training data

## UE behavior upon availability indication due to low power state

As for the availability indication of logged data due to low power state, RAN2 has come to the following agreements.

- Data collection is controlled by the network. The UE will not autonomously stop when low power state is detected. - The UE reports to the network when the power state is low. We will not specify how the UE determines low power state. The network should de-configure the data collection (this can be captured in stage 2).

It is clearly agreed that, after indicating the logged data is available due to the low power state, UE is expected to receive the de-configuration of data collection but keeps logging data. However, in case UE does not receive any reaction for NW addressing UE’s availability indication or, if NW provides UE with some configuration that UE cannot apply, UE may unnecessarily waste its power on logging data.

- UE keeps power consumption down if NW cannot stop UE from logging data after indication on UE low power state, and has no enough power for other services with higher priority.

Considering that UE may not be de-configured the data collection with low power, a NW-controlled timer can be introduced for UE to release data collection. Specifically, when configuring the availability indication due to low power state in otherconfig via RRCReconfiguration, NW can also configure UE with a time period as well, which can be used to control when UE is able to release the data collection configuration after reporting the data availibilty triggered by the low power state. That is, UE should start a timer upon detecting its low power state and reporting the data availibilty. UE would stop the timer when receiving the de-configuration of data collection. At the timer expiry, UE can stop the data collection procedure directly. In this solution, UE is able to save some power from logging data following NW control.

- The network can configure UE with a timer, which is started when UE indicates the logged data availability due to low power state; and is stopped when UE receives de-configuration of data collection. At the timer expiry, UE can stop logging the data.

## SRB for logged data reporting

RAN2 has agreed to use SRB to report the logged data to network. But RAN2 did not conclude on the SRB type, i.e., whether to introduce a new SRB or reuse SRB4. For the motivation to introduce a new SRB, some argue to have a low priority SRB. We do not see the necessity to introduce a new SRB given that SRB4 is already a low priority SRB for QoE reporting. Thus, it’s straightforward to reuse SRB4 for logged training data reporting.

- SRB4 is reused for logged training data reporting.

# Conclusion

In this contribution, some remaining issues on NW-side data collection for model training for beam management and we have the following proposals:

Data logging

- Introduce an area (e.g., cell ID) configuration to restrict the area in which the UE performs training data logging. And each cell within the area should be associated with an associated ID.

- As baseline RAN2 can assume the following mesurements are usefull for NW-side model training:

- Type 1: All L1-RSRPs of measured based the resources (for Set A/B) configured to UE;

- Type 3: All L1-RSRPs measured based on resources (for Set B) configured to UE, and beam information and L1-RSRPs of K beams based on some resources (for Set A) configured to UE.

- For NW side model, LS to RAN1 to feedback umderstanding on the following RAN2 following assumptions:

- Introduction of area (e.g., cell ID) configuration to restrict the area in which the UE performs training data logging. And each cell within the area should be associated with an associated ID;

- Consider the following type of data content:

  - Type 1: All L1-RSRPs of measured based the resources (for Set A/B) configured to UE;

  - Type 3: All L1-RSRPs measured based on resources (for Set B) configured to UE, and beam information and L1-RSRPs of K beams based on some resources (for Set A) configured to UE.

Data collection configuration

- There is no data collection specific issue of UE not being able to perform data collection based on received configuration and there is no need of any indication from UE to network.

Availability of logged data

- A single bit indication would be sufficient for UE to indicate to NW that UE has available logged data. No additional information, e.g., logged data size, is needed.

- There is no need for threshold-based buffer status report, UE should send the availability indication opon the buffer becomes full.

- UE should release the training data collection configuration and logged training data upon transitions from RRC_CONNECTED to RRC_INACTIVE/IDLE.

- When UE experience RLF, to guarantee unified behavior to the case of UE transitioning  from RRC_CONNECTED to RRC_INACTIVE, UE releases the training data collection configuration and logged training data.

Low power indication

- UE can keep power consumption down if NW does not stop UE from logging data after indication on UE low power state, and has no enough power for other services with higher priority.

- The network can configure UE with a timer, which is started when UE indicating low power state and/or logged data availability of the logged data; and the timer is stopped upon UE receiving de-configuration of data collection. At the timer expiry, UE can stop logging the data.

SRB for data reporting

- SRB4  is reused for logged training data reporting.

- Reference

- RAN2#129 Chairman notes

- RAN2#128 Chairman notes

- RAN2#127bis Chairman notes

- R1-2410736, FL summary #3 for AI/ML in beam management