3GPP TSG-RAN WG2 Meeting #129bis	R2-2501835

Wuhan, China, Apr. 7th – 11th , 2025

Agenda Item:	8.6.2

Source:	HONOR

Title: 	Further discussion on inter-CU LTM

Document for:	Discussion and Decision

1	Introduction

In Rel-19 Mobility enhancements WI, the following objective was achieved:

- Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured. - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: - As secondary priority, support the case where CU is acting as SN and MN is unchanged. - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released. - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling. - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support

In RAN2#129, the following agreements were made:

- Using the term “LTM” to represent “intra-CU LTM” and “inter-CU LTM” in MAC specification and introduce separate term for “conditional intra-CU LTM” in the specification(s). - Same as Rel-18 intra-CU LTM, Rel-19 inter-CU LTM could coexist with CovE, (e)RedCap, while couldn’t coexist with NRU. FFS on MIMO 2TA and NES. - Same as Rel-18 intra-CU LTM, CFRA resource configuration could be included in LTM cell switch command MAC CE for Rel-19 inter-CU LTM. - For inter-CU MCG LTM, when Rel-19 ID of candidate cell is different from serving cell, the UE performs PDCP re-establishment for SRB1/SRB2. - For inter-CU MCG LTM, when the Rel-19 ID of candidate cell is the same with serving cell, the UE performs PDCP SDU discard for SRB1/SRB2. - How to support shared RACH resources for inter-CU LTM is left to RAN3

In this contribution, we discuss the remaining issues in inter-CU LTM with non-DC and DC scenarios.

2. Discussion

During UL synchronization with LTM candidate cell(s), after performing TA acquisition based on CFRA, the source-DU obtains TA via the interaction between CU-CU and CU-DU nodes. In detail, the candidate DU notifies the associated CU of the obtained TA. The candidate CU forwards the TA to the source CU. Finally, the source DU obtains the TA from the source CU. Therefore, it is natural for source DU to manage TA validity. If not, while the TA is about to be used, the source DU has to obtain TA from source CU, which will have negative impacts on cell switch latency.

Upon cell switch, if the TA associated with candidate cell can continue to be maintained by NW side, the signalling overhead might be reduced, since it might avoid re-obtaining TA in the following period of time. To achieve this purpose, the source CU forwards TA information to the target CU, then, the target DU obtains the TA from the target CU.

Proposal 1: After cell switch, the TA associated with candidate cell(s) can continue to be kept in the following serving DU.

In the previous meeting, RAN3 agreed the following scenarios to support LTM with NR-DC:

- SN initiated inter-CU SCG LTM without MCG changes (high priority)

- Inter-CU MCG LTM without SN release

- Inter-CU MCG LTM with SN release

- Inter-CU MCG LTM with SN addition

In subsequent inter-CU MCG LTM, if candidate MNs with SCG released and with SCG kept are configured simultaneously, the issue that SCG has been released prior to MCG LTM with SCG kept, should be taken into consideration. For instance, the UE switches to a target MN1 with SCG released, then the UE switches to a target MN2 with SCG kept.

Regarding the identified issue, there are probably two main solutions. One solution is to configure a consistent SCG handling between MNs, which will have small impacts on UE. However, this is based on the restriction on the NW configuration. Alternatively, it is allowed to configure candidate MNs with SCG released and with SCG kept. If SCG has been released and the target MN has configured with SCG kept indication, an SCG addition procedure needs to be triggered before the LTM cell switch command. After completing SCG addition procedure, the UE is able to apply SCG configuration and perform the MN LTM with SCG kept.

If SCG addition is adopted to solve this issue, another issue such as when and how to trigger SCG addition, should be taken into consideration. There are two options as follows:

Option1: SCG addition will always be executed before cell switch

For instance, before cell switch, if SCG is released, no matter SCG is configured with SCG released or SCG kept in the target MN, a SCG addition procedure will be triggered by serving MN.

Option2: SCG addition will be executed before cell switch, when it is indicated with SCG kept in the target MN

In this option, if the serving MN find the SCG is already released, in order to apply SCG configuration after cell switch, SCG addition should be executed before cell switch. However, this requires the serving MN to perceive the indication information of the target MN, which is currently not supported, since the indication is configured in a container and the serving MN doesn't even decode it. One feasible approach is that target MN needs to explicitly provide the SCG handling indication to serving MN.

Compared with the two options, option 1 might result in unnecessary SCG addition action, which will have a negative impact on signalling overhead over the air interface. Therefore, option 2 is preferred.

Proposal 2: Before cell switch, if SCG is released, and it is indicated with SCG kept in the target MN, a SCG addition procedure will be triggered.

In the mentioned SCG addition procedure, the serving MN is responsible for SCG addition. After completing SCG addition, the UE can apply SCG after switching to the target MN. However, if the difference between the current added SCG configuration and the SCG configuration in the target MN happens, the issue how to apply SCG configuration should be taken into further discussion.

Proposal 3: In SCG addition procedure, if the current added SCG configuration is different from that in the target MN, the issue how to apply SCG configuration should be taken into further discussion.

InRAN2#129, the following reference configuration related conclusion were achieved:

- The reference configuration related to inter-SN LTM is included within the new ltm-ConfigSCG field. - UE does not combine reference configurations from ltm-Config and ltm-ConfigSCG.

In LTM preparation stage, the MN can configure the reference configuration including both MCG part and SCG part. However, the issue how to indicate whether the reference configuration is a complete configuration or not was identified, and the FFS whether only one indication is needed to indicate LTM configuration as complete configuration was left.

In RAN3, it was agreed that the candidate gNB indicates whether a LTM candidate configuration is a complete candidate configuration in the Handover Request Acknowledge and LTM configuration update acknowledge message. That is, the candidate gNB can decide and indicate whether the candidate SCG/MCG configuration is delta or complete. In order to make it flexible to indicate SCG part and MCG part separetely, an addtional indication should be introduced for the SCG part.

Proposal 4: Introduce an additional indication to indicate LTM configuration as complete or delta configuration for the SCG part.

3 Conclusions

In this contribution, we discussed the issues related to inter-CU LTM. Based on the discussion, the following observations and proposals are concluded:

Proposal 1: After cell switch, the TA associated with candidate cell(s) can continue to be kept in the following serving DU.

Proposal 2: Before cell switch, if SCG is released, and it is indicated with SCG kept in the target MN, a SCG addition procedure will be triggered.

Proposal 3: In SCG addition procedure, if the current added SCG configuration is different from that in the target MN, the issue how to apply SCG configuration should be taken into further discussion.

Proposal 4: Introduce an additional indication to indicate LTM configuration as complete or delta configuration for the SCG part.

4 References