3GPP TSG-RAN WG2 Meeting #129bis                                                             R2-2501850

Wuhan, China, April. 7th – 11th, 2025

Source:	CATT

Title:	Discussion on other aspects for SBFD

Agenda Item:	8.11.3

Document for:	Discussion

1. Introduction

The objectives of Sub-band full duplex (SBFD) which are related with RAN2 except random access can be found as below:

- For subband non-overlapping full duplex (SBFD) operation at gNB side within a TDD carrier:

  - Specify semi-static indication of time location of SBFD subbands to UEs in RRC_CONNECTED mode [RAN1, RAN2]

  - Specify semi-static indication of frequency domain location of SBFD subbands to UEs in RRC_CONNECTED mode [RAN1, RAN2]

  - Specify UE transmission, reception and measurement behavior and procedures in SBFD symbols and/or non-SBFD symbols for SBFD aware UE [RAN1, RAN2]

    - Transmission and reception behaviours on SBFD subbands configured in DL and/or flexible symbol indicated by TDD-UL-DL-ConfigCommon

    - Enhancement on resource allocation in frequency domain in SBFD symbols, including

    - Enhancements on physical channels/signals and procedure across SBFD symbols and non-SBFD symbols in different slots, where each transmission/reception within a slot has either all SBFD or all non-SBFD symbols,

    - Configurations for SRS, PUCCH and PUSCH on SBFD symbols and non-SBFD symbols, e.g., resources, frequency hopping parameters, UL power control parameters and/or beam/spatial relation

    - Collision handling between DL reception in DL subband(s) and UL transmission in UL subband in a SBFD symbol

  - Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4]

    - UL resource muting for PUSCH[RAN1, RAN2, RAN4]

    - L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework,[RAN1, RAN2, RAN4]

We are going to discuss the RAN2 impacts on CLI resource configuration, CLI report configuration and UE capability based on RAN1 progress in this contribution.

2. Discussion

2.1 CLI resource configuration

According to the agreement at RAN2#129 meeting, a new SP CLI measurement resource set activation/deactivation MAC CE is required.

Agreement on other aspects for SBFD

A new SP CLI measurement resource set activation/deactivation MAC CE is introduced to activate/deactivate the SP CLI measurement resource.

According to RAN1 agreement, some RAN1 parameters are included in the MAC CE as below:

Agreement

For aperiodic L1 CLI-RSSI/CLI-SRS-RSRP reporting on PUSCH based on a set of semi-persistent CLI measurement resources, the gNB may activate and deactivate the configured semi-persistent CLI measurement resource sets by sending a new SP CLI Measurement Resource Set Activation/Deactivation MAC CE

- TCI state IDi field is included in the new MAC CE, which is used as typeD QCL source for the CLI measurement resource within the Semi Persistent CLI measurement resource set. Either all TCI state ID fields are present or none.

- If the TCI state ID field(s) are absent, the UE QCL assumptions are the default rules agreed in RAN1#118bis

The new MAC CE for SP CLI Measurement Resource Set Activation/Deactivation can be designed to include at least below fields:

-	A/D: This field indicates whether to activate or deactivate indicated SP CLI measurement resource set(s). The field is set to 1 to indicate activation, otherwise it indicates deactivation;

-	Serving Cell ID: This field indicates the identity of the Serving Cell for which the MAC CE applies. The length of the field is 5 bits;

-	BWP ID: This field indicates a DL BWP for which the MAC CE applies as the codepoint of the DCI bandwidth part indicator field as specified in TS 38.212 [9]. The length of the BWP ID field is 2 bits;

-	SP L1 CLI-RSSI resource set ID and/or SP L1 CLI-SRS-RSRP resource set ID

-	TCI State IDi

-	R: Reserved bit, set to 0.

So we proposed:

Proposal 1: The new MAC CE for SP CLI Measurement Resource Set Activation/Deactivation may consist of the following fields:

-	A/D: This field indicates whether to activate or deactivate indicated SP CLI measurement resource set(s). The field is set to 1 to indicate activation, otherwise it indicates deactivation;

-	Serving Cell ID: This field indicates the identity of the Serving Cell for which the MAC CE applies. The length of the field is 5 bits;

-	BWP ID: This field indicates a DL BWP for which the MAC CE applies as the codepoint of the DCI bandwidth part indicator field as specified in TS 38.212 [9]. The length of the BWP ID field is 2 bits;

-	SP L1 CLI-RSSI resource set ID and/or SP L1 CLI-SRS-RSRP resource set ID: These fields specify the corresponding measurement resource sets.

-	TCI State IDi: This field contains TCI-StateId

-	R: Reserved bit, set to 0.

There are two options to include the field ‘SP L1 CLI-RSSI resource set ID and/or SP L1 CLI-SRS-RSRP resource set ID’ because of and/or:

- Option 1: Only one type of resources (either L1 CLI-RSSI or L1 CLI-SRS-RSRP) can be included in one MAC CE.

Below please find an example of option 1 for the MAC CE:

The Flag field indicates CLI-RSSI or CLI-SRS-RSRP included in this MAC CE.

A/D

Serving Cell ID

BWP ID

R

flag

SP L1 CLI-RSSI resource set ID or

SP L1 CLI-SRS-RSRP resource set ID

R

TCI state ID0

R

TCI state IDN

Oct 1

Oct 2

Oct 3

Oct N+3

...

- Option 2:  Both of the two types of SP L1 CLI measurement resource (both L1 CLI-RSSI and L1 CLI-SRS-RSRP)  can be included in one MAC CE.

Below please find an example of option 2 for the MAC CE:

The F1 field indicates if there is L1 CLI-SRS-RSRP resource set Id in this MAC CE, and F2 field indicates if there is L1 CLI-RSSI resource set Id in this MAC CE.

A/D

Serving Cell ID

BWP ID

F1

F2

SP L1 CLI-RSSI resource set ID or

SP L1 CLI-SRS-RSRP resource set ID

R

R

SP L1 CLI-SRS-RSRP resource set ID

R

TCI state ID0

R

TCI state IDN

Oct 1

Oct 2

Oct 3

Oct 4

Oct N+4

...

R

TCI state ID0

R

TCI state IDM

Oct N+5

Oct N+M+5

...

These options have their own values and applicable scenarios:

| Feature‌ | Option 1‌ | Option 2‌ |
| --- | --- | --- |
| ‌Complexity‌ | Low (single indicator field) | High (nested conditional fields) |
| ‌Flexibility‌ | Only supports single resource type activation | Supports dynamic combined activation (e.g., concurrent resource types) |
| ‌MAC CE Size‌ | Fixed length | Variable length (depends on Indicator) ‌ |
| ‌Applicable Scenarios‌ | Infrequent resource type switching | Frequent switching or concurrent activation requirements |

So we proposed to evaluate the two options and make a discussion based on the analysis above.

Proposal 2: RAN2 to discuss which option to support for the new SP CLI MAC CE:

-	Option 1: Only one type of resource (L1 CLI-RSSI or L1 CLI-SRS-RSRP) can be included in one MAC CE,

-	Option 2:  Both types of SP L1 CLI measurement resources (L1 CLI-RSSI and L1 CLI-SRS-RSRP) can be included in one MAC CE.

RAN1 has discussed and is finalizing the RRC parameters on the L1 based UE-to-UE CLI measurement mechanism and reach the agreement as below:

Agreement

In CSI-ReportConfig, reuse resourcesForChannelMeasurement providing the ID of a CSI-ResourceConfig that contains configuration of measurement resources for L1-SRS-RSRP and/or L1-CLI-RSSI.

RAN2 has reached the agreement at #127bis below

Agreements on other aspects for SBFD

For L1 based UE-to-UE CLI measurement mechanism, the configuration of periodic, semi-persistent or aperiodic UE-to-UE CLI measurement resource (set) should be introduced based on R1 agreement.

So we proposed:

Proposal 3: Legacy CSI-MeasConfig is reused for the configurations of L1 CLI-RSSI and L1 SRS-RSRP measurement. Detail parameters are FFS based on RAN1 RRC Parameter list.

2.2 CLI report configuration

RAN1 has discussed the CLI report and reach the agreement as below:

Agreement

     Periodic L1 CLI-RSSI reporting for L1 based UE-to-UE CLI measurement and reporting is supported

     Semi-persistent L1 CLI-RSSI reporting for L1 based UE-to-UE CLI measurement and reporting is NOT supported

     Periodic/Semi-persistent L1 SRS-RSRP reporting for L1 based UE-to-UE CLI measurement and reporting is NOT supported

Below please find the summary of the L1 CLI report based on the RAN1 progress:

|  | Periodic report | Semi-persistent report | Aperiodic report |
| --- | --- | --- | --- |
| L1 CLI-RSSI | Support | NOT support | Support |
| L1 SRS-RSRP | NOT support | NOT support | Support |

Proposal 4: The legacy CSI-ReportConfig is reused for L1 CLI-RSSI Periodic report configuration, L1 CLI-RSSI Aperiodic report and L1 SRS-RSRP Aperiodic report. Detail parameters are FFS based on RAN1 RRC Parameter list.

Proposal 5: The legacy Aperiodic CSI Trigger State Subselection MAC CE is reused for the aperiodic report.

2.3 UE capability

The term of ‘SBFD aware UE’ is used by RAN2 and RAN1 widely. RAN2 doesn't discuss this terminology yet. Since RAN1 is discussing the capacities, it seems that SBFD aware UE will be defined by UE capability. Below please find the agreement in RAN1 on the related capability.

Agreement:

Introduce following basic FG for SBFD, and FFS on some potential components (that may be separate FGs).

| X-1 | Basic feature for SBFD-aware UE TX/RX/measurement behaviour and procedures | 1. Support of cell-common configuration of time and frequency location of SBFD subbands 2. Support of UL transmission in one UL subband and DL reception in one[/two] DL subband in SBFD symbols 3. Support of link direction determination based on configured/scheduled transmissions/receptions and collision handling FFS: other component (which may be separate FG) |  | YES | n/a | Basic feature for SBFD-aware UE TX/RX/ measurement behaviour and procedures is not supported | [Per Band] | TDD only | n/a | n/a | [Note: Only UL transmissions are allowed within UL subband and only DL receptions (except for CLI measurement) are allowed within DL subband(s)] [Note: The maximum number of UL subbands for SBFD operation in an SBFD symbol within a TDD carrier is one.] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement:

Add the following component in basic FG for SBFD.

- “Support of UL transmissions or DL receptions across SBFD symbols and non-SBFD symbols in different slots, where the transmissions or receptions are restricted to SBFD symbols or non-SBFD symbols (Configuration 1)”

The capabilities of supporting SBFD random access on single RACH configuration (Option 1) and the other based on two RACH configurations (Option 2)is also defined by RAN1 as well.

Agreement:

Introduce following FGs for SBFD random access, one based on single RACH configuration (Option 1) and the other based on two RACH configurations (Option 2), and FFS on whether UE supporting SBFD random access needs to support at least Option 1 or Option 2.

- Alt.1: UE supporting SBFD random access needs to support at least Option 1

- Alt.2: UE supporting SBFD random access needs to support at least Option 2

- Alt.3: UE supporting SBFD random access supports Option 1 or Option 2 (i.e., no default option)

| X-3 | SBFD random access operation based on one single RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode | Support of SBFD random access operation based on one single RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode (RACH configuration Option 1) FFS: Other component(s) which may be separate FG | X-1 | YES | n/a | SBFD random access operation based on one single RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode is not supported | [Per Band] | TDD only | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X-4 | SBFD random access operation based on two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode | Support of SBFD random access operation based on two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode (RACH configuration Option 2) FFS: Other component(s) which may be separate FG | [X-1, X-3] | YES | n/a | SBFD random access operation based on two separate RACH configurations in RRC_IDLE/INACTIVE/CONNECTED mode is not supported | [Per Band] | TDD only | n/a | n/a |  | Optional with capability signalling |

So we proposed:

Proposal 6: RAN2 won’t discuss the SBFD capabilities until the LS from RAN1 is sent to RAN2.

3. Conclusion

Based on the analysis in section 2, the proposals are summarized as following:

CLI resource configuration

Proposal 1: The new MAC CE for SP CLI Measurement Resource Set Activation/Deactivation may consist of the following fields:

-	A/D: This field indicates whether to activate or deactivate indicated SP CLI measurement resource set(s). The field is set to 1 to indicate activation, otherwise it indicates deactivation;

-	Serving Cell ID: This field indicates the identity of the Serving Cell for which the MAC CE applies. The length of the field is 5 bits;

-	BWP ID: This field indicates a DL BWP for which the MAC CE applies as the codepoint of the DCI bandwidth part indicator field as specified in TS 38.212 [9]. The length of the BWP ID field is 2 bits;

-	SP L1 CLI-RSSI resource set ID and/or SP L1 CLI-SRS-RSRP resource set ID: These fields specify the corresponding measurement resource sets.

-	TCI State IDi: This field contains TCI-StateId

-	R: Reserved bit, set to 0.

Proposal 2: RAN2 to discuss which option to support for the new SP CLI MAC CE:

-	Option 1: Only one type of resource (L1 CLI-RSSI or L1 CLI-SRS-RSRP) can be included in one MAC CE,

-	Option 2:  Both types of SP L1 CLI measurement resources (L1 CLI-RSSI and L1 CLI-SRS-RSRP) can be included in one MAC CE.

Proposal 3: Legacy CSI-MeasConfig is reused for the configurations of L1 CLI-RSSI and L1 SRS-RSRP measurement. Detail parameters are FFS based on RAN1 RRC Parameter list.

CLI report configuration

Proposal 4: The legacy CSI-ReportConfig is reused for L1 CLI-RSSI Periodic report configuration, L1 CLI-RSSI Aperiodic report and L1 SRS-RSRP Aperiodic report. Detail parameters are FFS based on RAN1 RRC Parameter list.

Proposal 5: The legacy Aperiodic CSI Trigger State Subselection MAC CE is reused for the aperiodic report.

UE capability

Proposal 6: RAN2 won’t discuss the SBFD capabilities until the LS from RAN1 is sent to RAN2.

4. References

- RP-241614 Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)  Huawei

- 3GPP TR 38.858 Study on Evolution of NR Duplex Operation (Release 18) V2.0.0 (2023-12)

- RAN1 Chair Notes WG1 #120

- RAN2 Chair Notes WG2 #129