3GPP TSG-RAN WG2 Meeting #129bis	R2-2501862

Wuhan, China, April 7th –11th,2025

Agenda item:       8.3.2.1

Title:	RRM measurement prediction specification impact discussion

Source:	NEC

Document for:     Discussion and Decision

# 1. Introduction

In the previous RAN2 meeting, there were discussion on Simulation assumptions and evaluation methodology for RRM measurement prediction. The agreements about simulation assumptions for RRM measurement prediction were achieved [1][2][3] with the following agreed evaluation scenarios to start the relevant simulations.

| Case number | Prioritization | Evaluation scenario combination | target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

In last RAN2#127bis [4], RAN2#128 [5] and RAN2#129 [6], the following agreements/observations on intra-frequency temporal domain prediction (Case 2/4) and inter-frequency prediction (Case 3) were achieved as below.

RAN2#127bis:

Agreements on intra-frequency

- For intra-frequency temporal domain, higher UE speeds result in larger prediction errors

- Initially, increasing the OW length can enhance prediction accuracy in the temporal domain case A, especially when the OW is relatively short. However, once the OW exceeds a certain threshold, further increases do not yield significant benefits. Conversely, for PW, longer durations correlate with decreased prediction accuracy.  RAN2 will not define the actual threshold and fast fading assumption.

- Majority of companies observe that among sub cases 1, 2, and 3, at least with shorter prediction window sub case 2 demonstrates the highest prediction accuracy

- Companies can provide multiple real time RSRP value(s) and/or average RSRP value over the entire window and should indicate in their simulation results what they have used.  The companies should at least provide the results of only one value it should be the last value at the end of the PW.   We will add two columns in the spreadsheet to capture the last value and the average value.

Agreements on inter-frequency

- Companies need to report whether earlier predicted results are also used as inputs for future RRM prediction.

- Companies should report with their simulation the correlation coefficient

- Higher-to-lower and lower-to-higher frequency prediction is comparable

- For co-located scenario,  the UE speed in the inter-frequency case has minor impact on prediction accuracy

RAN2#128:

Agreements/Observations to be captured in TR

- The prediction accuracy for intra-frequency temporal domain case B reduces as MRRT increases.

- For temporal domain case B, with the same MRRT, different skipping pattern can provide different prediction performance.  Companies may report the adopted skipping pattern when providing simulation results.   Companies are not required to run new simulations but can clarify in the spreadsheet.    We can capture in the TR how the skipping patter affects the performance.

- For the inter-frequency prediction, the evaluation results show that the higher the correlation coefficient is between two frequency layers, the higher the prediction accuracy.   FFS on observations on low correlations.

- For inter-frequency, cluster as input (i.e. measurements from different cells as inputs )can improve the prediction accuracy than single cell as input.    For temporal domain, the gains are unclear.   RAN2 will focus on frequency domain for cluster based approach.

- In cluster approach the model takes measurements from more than one cell as inputs.

Agreements/observations

- Companies can compare results with non-AI approaches.   Temporal domain, sample and hold, and frequency domain, pathloss offset.   Companies can consider other simple models (e.g. ARIMA).

- For FR1 intra-frequency temporal domain case B, when PW is short, the performance between AI and sample-and-hold is not significant. However, when PW becomes larger, AI outperforms sample-and-hold.

- In frequency domain prediction cases (2GHz and 4GHz), cell-based approaches achieve limited gain compared to pathloss offset without the help of neighbor cell measurement. The cluster-based shows better performance compared to pathloss offset.

- For intra-frequency temporal domain case A, AI can provide gains (in terms of L3 cell RSRP difference) compared to sample and hold. The gain improves with UE speed.

RAN2#129:

Agreements/Observations

1	For Case B temporal domain prediction, RAN2 to capture that AI can provide gain (i.e., lower L3 cell-level RSRP difference) compared to Non-AI (i.e., Sample and Hold) and the gain increases as MRRT increases.

2	Regarding temporal domain prediction case B, the gain that AI can provide is more obvious in high speed scenario when compared to sample and hold.

3	Regarding the temporal domain prediction case A, the gain that AI can provide increases with the increase of prediction window length (up to a certain window length) when compared to sample and hold.

4     Average L3 cell RSRP difference and last predicted point L3 cell RSRP     difference of measurement results within PW is captured in TR

In this contribution, we discuss the possible specification impact for RRM measurement prediction.

# 2. Discussion

[7] highlights studying and evaluating the potential benefits and gains of AI/ML aided mobility for network triggered L3-based handover, considering AI/ML based RRM measurement and measurement event prediction. This includes both intra and inter-frequency cell-level prediction. Both UE sided and network sided models were agreed to be explored in this study item. The first study goal decided is to reduce measurement efforts in temporal, spatial or frequency domain by using predicted measurements. The second study goal is to improve the handover performance (e.g., Ping-pong HO, HOF/RLF, short time of stay, Handover interruption).

As can be observed from the agreements done so far in the RAN2 meetings, it can be concluded that cell level intra-frequency scenarios for temporal domain Case A and Case B have been evaluated for RRM measurement prediction use case. The AI/ML model for RRM measurement prediction is found to outperform non-AI based solutions under certain conditions ( for example , UE speed, MRRT and the prediction window length etc). This indicates the possibility of using AI/ML based model for RRM prediction for achieving handover gain and reducing measurement overhead. In this context, we would explore the possible scenarios for implementing the AI/ML model RRM measurement prediction and the interaction between UE and gNB to successfully leverage the advantages of the deployed model.

In case of a UE sided model, the results of the AI/ML model needs to be reported to network for handover decision. However, in case of a network sided model, UE reports the measurement results to the network and the AI/ML model at network decides the handover. The input to the AI/ML model can be the current RSRP, RSRQ, SINR, CQI and the output of the AI/ML model is cell level RRM measurement prediction results based on cell level results or beam level results.

If we specifically consider an UE sided model, where the AI/ML for RRM measurement prediction is run at the UE, the results of the prediction is sent to the network for handover decision. However, the model characteristics, and the input for the AI/ML model has to be configured by the network. If UE uses a static AI/ML model for performing RRM measurement prediction, then the model may not adapt well to the dynamic channel and network conditions (for example, speed, SINR fluctuations). Thus, we propose an adaptive AI/ML model selection for UE side RRM prediction. This can be enabled through UE reporting the model accuracy and confidence scores to the network along with the prediction results. For example, the average L3 cell RSRP difference and last predicted point L3 cell RSRP difference of measurement results within PW (agreed in RAN2#129) can be reported to the network to take a decision for model change adapting to the network conditions. The model change instruction can be sent through a RRC message to the UE with the updated characteristics of the AI/ML model. This will help improve the handover decision by the network.

Proposal1: In case of UE sided model for Intra-frequency temporal domain Case A and Case B and inter-frequency prediction. The accuracy of prediction is shared to the network through the measurement report and the network executes a model switch command to change the AI/ML model for the prediction if the accuracy is below a defined threshold.

When we consider the objective of reducing the measurement overhead, a corresponding signaling framework needs to be defined. We propose having an event triggered measurement reporting framework for reducing the overhead. We define an event based RRM reporting where measurement reports are triggered only when the predicted values deviate significantly from the past trends or fall below a confidence threshold.

Proposal2: Intra-frequency temporal domain Case A and Case B and inter-frequency prediction model for RRM measurement prediction, an event triggered measurement reporting framework for reducing measurement overhead should not compromise the handover performance.

# 2. Conclusion

Proposal1: In case of UE sided model for Intra-frequency temporal domain Case A and Case B and inter-frequency prediction. The accuracy of prediction is shared to the network through the measurement report and the network executes a model switch command to change the AI/ML model for the prediction if the accuracy is below a defined threshold.

Proposal2: Intra-frequency temporal domain Case A and Case B and inter-frequency prediction model for RRM measurement prediction, an event triggered measurement reporting framework for reducing measurement overhead should not compromise the handover performance.

# Reference

- R2_125bis_ChairNotes_24-04-19_final, Apr 2024

- R2_126_ChairNotes_24-05-24_final, May 2024

- R2_127_ChairNotes_24-08-23_17-00_eom, Aug 2024

- R2_127bis_ChairNotes_24-10-18_15-45_eom_clean, Oct 2024

- R2_128_ChairNotes_24-11-22_17-00_eom, Nov 2024

- R2_129_ChairNotes_25-02-20_19-00_eom, Feb 2025

- RP-240082, Revised SID on AIML for mobility in NR, OPPO.