3GPP TSG-RAN WG2 Meeting #129bis	R2-2501881

# Wuhan, China, 7 – 11 April 2025

Agenda item:		8.7.5

Source:	Xiaomi

Title:	RLC AM retransmission enhancements

Document for:		Discussion and Decision

Introduction

In RAN2#129 meeting, following was agreed regarding RLC retransmission enhancements [1]:

Autonomous retransmissions and polling enhancements - Autonomous retransmission and/or polling should be triggered when the remaining time of an RLC SDU falls below a specified threshold. FFS if remaining time is determined based on discardTimer at PDCP or new timer at RLC - Only a single autonomous retransmission will be triggered per RLC SDU. - There is no dynamic activation/deactivation of the autonomous retransmission mechanism. - We have separate thresholds for autonomous reTx and for polling Unnecessary retransmissions avoidance - When the TX RLC entity receives a discard indication of the SDU from PDCP, the TX RLC entity considers the SDU as an outdated SDU. The TX RLC entity does not perform any transmission and retransmission of such SDU/SDU segment. - A new RLC timer at the TX is not introduced to determine outdated RLC SDUs. - The new RLC timer at the RX is per RLC entity - The duration of the new RLC timer is not lower than that of t-reassembly - Proposals 4 and 6 from R2-2500380 and P3 and 4 from R2-2500401 will be discussed together with RLC CR review

In this contribution, we discus RLC AM retransmission enhancements.

Discussion

Avoid unnecessary RLC retransmissions

In email discussion “[POST129][512][XR] RLC running CR (vivo)”, rapporteur proposed to capture that statue report is triggered upon the expiry of RLC timer (t-RxDiscard). Some discussion was triggered on this aspect, which is finally left as FFS in RLC running CR [2], as below:

[FFS -	Detection of obsolescence of an AMD PDU:

-	The receiving side of an AM RLC entity shall trigger a STATUS report when t-RxDiscard expires.]

Before technical discussion, it is helpful to review previous RAN2 agreements.

RAN2#127bis meeting - RAN2 confirm the previous baseline assumption: the RLC receiving window always advances to any given RLC SN before the transmitting window does. - Rx informs Tx side about the abandoned SDUs, as a baseline we assume existing SR can be reused unless issues are identified. RAN2#128 meeting: - The abandoned RLC SDUs determined by a new RLC timer are positively acknowledged in the STATUS report. RAN2#129 meeting - The duration of the new RLC timer is not lower than that of t-reassembly.

The concerns on sending status report after expiry of t-RxDiscard is that this can be handled by other status report triggering conditions, e.g. expiry of t-Reassembly and polling, or based on gNB implementation to advance Tx window. However there are issues with these approaches.

In case of expiry of t-Reassembly, state variables RX_Highest_Status and RX_Next_Status_Trigger are updated, but the lower edge of RLC receiver window (RX_Next) is not updated, as from TS 38.322 clause 5.2.3.2.4, as copied below:

When t-Reassembly expires, the receiving side of an AM RLC entity shall:

-	update RX_Highest_Status to the SN of the first RLC SDU with SN >= RX_Next_Status_Trigger for which not all bytes have been received;

-	if RX_Next_Highest> RX_Highest_Status +1: or

-	if RX_Next_Highest = RX_Highest_Status + 1 and there is at least one missing byte segment of the SDU associated with SN = RX_Highest_Status before the last byte of all received segments of this SDU:

-	start t-Reassembly;

-	set RX_Next_Status_Trigger to RX_Next_Highest.

Therefore when status report is generated, RX_Next is still indicated as NACK. After the expiry of t-RxDiscard, state variable RX_Next is updated, and this needs to be informed to the transmitter so that the transmitter can consider the old RX_Next as correctly received and advance Tx window if needed. In summary, status report due to expiry of t-Reassembly cannot cover the status report due to expiry of t-RxDiscard.

Regarding polling, it should be noted that blind polling might cause unnecessary overhead of sending status report when there is no change of receiver side status. As for gNB side advancing Tx window, it is not in line with RAN2 agreement that “the RLC receiving window always advances to any given RLC SN before the transmitting window does”. Both polling and advancing Tx window by gNB rely on gNB implementation to guess the appropriate timing, which increases gNB complexity.

Sending RLC status report upon expiry of RLC Rx timer (t-RxDiscard) is a simple way (minimal stage-3 change as can be seen from running RLC CR) to avoid Tx window stall, without additional increase of gNB complexity.

Proposal 1: Upon expiry of RLC Rx timer (t-RxDiscard), RLC status report is triggered.

Timely RLC retransmission

In RAN2#129 meeting, following was agreed “Autonomous retransmission and/or polling should be triggered when the remaining time of an RLC SDU falls below a specified threshold. FFS if remaining time is determined based on discardTimer at PDCP or new timer at RLC”.

The key difference between discardTimer at PDCP or new timer at RLC is when the timer is started. PDCP discardTimer is started upon reception of PDCP SDU from upper layer, while RLC timer can be started upon receiving RLC SDU (i.e. PDCP PDU) from PDCP layer. The timing that UE submitting PDCP PDU to RLC layer is related to UE implementation, taking into account of pre-processing as well as minimizing PDCP reordering delay at receiving PDCP entity (e.g. in case a PDCP SDU is discarded). TS 38.323 has a note that in case of split bearer, UE needs to minimize the amount of PDCP PDUs submitted to RLC before receiving UL grant, as below:

NOTE 2:	If the transmitting PDCP entity is associated with two RLC entities, or with one or more RLC entities and either an SRAP entity or the N3C, the UE should minimize the amount of PDCP PDUs submitted to lower layers before receiving request from lower layers and minimize the PDCP SN gap between PDCP PDUs submitted to two associated RLC entities, or to the one or more RLC entities and either the SRAP entity or the N3C, to minimize PDCP reordering delay in the receiving PDCP entity.

In summary, there can be large time difference between the start of discardTimer at PDCP and the start of RLC timer, while the start of RLC timer is UE implementation dependent. Considering that autonomous retransmission and polling are targeted for delay critical SDUs, discardTimer at PDCP is more aligned with SDU QoS requirement (e.g. packet delay budget) than the timer at RLC layer. This is similar to DSR, where discardTimer at PDCP is use to determine the remaining time. It is therefore proposed to reuse the discardTimer at PDCP to determine the remaining time for autonomous retransmission and enhanced polling.

Proposal 2: For autonomous retransmission and enhanced polling, remaining time is determined based on discardTimer at PDCP.

Polling enhancements

Since the main point of polling is to inform the receiver to provide status report, the condition of having delay critical SDU alone is not sufficient for polling. For example, if a delay-critical SDU has not been transmitted yet, it is not useful to include a poll bit. Considering that HARQ retransmission is in general helpful to improve spectrum efficiency, it is helpful to only trigger polling when the time duration of submitting a delay critical RLC SDU to MAC layer exceeds a configured threshold. An example is shown in Figure 1 below. Polling is triggered when both conditions are satisfied: 1) A RLC SDU is delay critical (as the remaining time until expiry of discardTimer is less than remaininingTimeThreshold); and 2) time duration of submitting the RLC SDU to MAC layer (Δt) exceeds a configured threshold.

Figure 1: Polling enhancement considering transmission status

Proposal 3: The Tx RLC entity includes a poll when the time duration of submitting a delay critical RLC SDU (whose remaining time falls below a threshold) to MAC layer exceeds a configured threshold.

Autonomous retransmission

One Editor’s Note in RLC running CR is “It is still open how Autonomous Retransmission coexists with ARQ procedures, i.e. whether/how to increment the RETX_COUNT for Autonomous Retransmission.

RETX_COUNT maintenance is related to RLC RLF. Related specification text in TS 38.322 is as follows:

When an RLC SDU or an RLC SDU segment is considered for retransmission, the transmitting side of the AM RLC entity shall:

-	if the RLC SDU or RLC SDU segment is considered for retransmission for the first time:

-	set the RETX_COUNT associated with the RLC SDU to zero.

-	else, if it (the RLC SDU or the RLC SDU segment that is considered for retransmission) is not pending for retransmission already and the RETX_COUNT associated with the RLC SDU has not been incremented due to another negative acknowledgment in the same STATUS PDU:

-	increment the RETX_COUNT.

-	if RETX_COUNT = maxRetxThreshold:

-	indicate to upper layers that max retransmission has been reached.

If RETX_COUNT reaches maxRetxThreshold, RLF is declared. The motivation of autonomous retransmission is that there is not sufficient time for RLC retransmission based on status report for XR traffic with short packet delay budget (PDB). In RAN2#129 meeting, it was agreed that “Only a single autonomous retransmission will be triggered per RLC SDU”. Therefore legacy behavior for RETX_COUNT is applicable, and special handling for autonomous retransmission is not needed.

Proposal 4: Existing RLC procedure for RETX_COUNT is applicable for autonomous retransmission. No specification change is needed.

In current RLC AM, retransmissions are prioritized over initial transmissions, as from TS 38.322 clause 5.2.3.1.1, as copied below:

The transmitting side of an AM RLC entity shall prioritize transmission of AMD PDUs containing previously transmitted RLC SDUs or RLC SDU segments over transmission of AMD PDUs containing not previously transmitted RLC SDUs or RLC SDU segments.

Autonomous retransmissions are based on conditions like timer / counter, instead of actual RLC status report. Therefore prioritizing autonomous retransmission over initial transmission might impact system performance (e.g. capacity, packet latency). It is suggested to differentiate autonomous retransmission when considering prioritization in RLC transmitter side.

Proposal 5: Autonomous retransmission should be differentiated from normal RLC retransmission base on RLC status report when considering prioritization in RLC transmitter side.

Conclusion

In this contribution, we discuss RLC AM retransmission enhancements, and propose the following:

Proposal 1: Upon expiry of RLC Rx timer (t-RxDiscard), RLC status report is triggered.

Proposal 2: For autonomous retransmission and enhanced polling, remaining time is determined based on discardTimer at PDCP.

Proposal 3: The Tx RLC entity includes a poll when the time duration of submitting a delay critical RLC SDU (whose remaining time falls below a threshold) to MAC layer exceeds a configured threshold.

Proposal 4: Existing RLC procedure for RETX_COUNT is applicable for autonomous retransmission. No specification change is needed.

Proposal 5: Autonomous retransmission should be differentiated from normal RLC retransmission base on RLC status report when considering prioritization in RLC transmitter side.

- References

[1] R2-2501335, Session chair (Huawei), “Report from session on R18 MBS, R18 QoE and R19 XR”

[2] R2-2502162, vivo, “Running RLC CR for XR”