3GPP TSG-RAN WG2 Meeting #129bis	R2-2501882

# Wuhan, China, 7 – 11 April 2025

Agenda item:		8.7.6

Source:	Xiaomi

Title:	XR rate control

Document for:		Discussion and Decision

Introduction

In RAN2#129 meeting, following was agreed on XR rate control [1]:

Agreements on XR rate control - RAN2 assumes for XR rate control, the gNB receives QoS flow information from the CN, specifying which QoS flows are subject to uplink rate control (i.e., Option 2). Send an LS to RAN3 and SA2. We may revisit UAI option based on SA2/RAN3 reply - Specify a new table for XR rate control. FFS distribution (exponential, linear), codepoints etc. - We will try to design a table first and check whether it is possible to meet the required range/granularity. Afterwards, we can check whether multipliers are needed - Working assumption: - Support rate query MAC CE with the target to use same design that we will agree for rate indication MAC CE. - The rate query MAC CE is configurable by the network, i.e. the network may turn it off completely (same as legacy). - Companies to check with their SA4 colleagues whether there are any issues with this

In RAN#106 meeting, downlink congestion indication (in addition of the uplink) was discussed, and slide#3 of RP-243305 [3] is endorsed, with related content as below:

RAN2 to continue working on the uplink solution and to liaise with SA2/SA4 if there are issues if the same approach would be extended for the DL as well, assuming the solution extension for DL would not cause issues in RAN2 side (not more work than what would be needed to prevent use of the solution for downlink direction). RAN3 to be kept informed of the progress.

In this contribution, we discus XR rate control.

Discussion

Rate indication

In RAN2#128 meeting, RAN2 agreed that “Rate indication from gNB to the UE on a per QoS flow level is supported. FFS the details, e.g. if: 1) flows are indicated by MAC CE or 2) by RRC while MAC CE is per DRB.”

Regarding how per QoS flow level is signalled, there are mainly two alternatives:

- Option a: Indicate QoS flow IDs directly in MAC CE

- Option b: Indicate QoS flows IDs to be rate controlled in RRC signalling, and indicate the DRB ID / LCID in MAC CE.

In Option a, signalling QFI alone is not sufficient since QFI is only unique with a PDU session. In NR SDAP header, PDU Session ID is not signalled since one DRB can only contain data from one PDU Session, as from ASN.1 below. However in MAC CE, indication of PDU Session ID / DRB ID / LCID is needed to uniquely identify a QoS flow. Considering QFI has 6 bits, the signalling overhead to indicate QoS flow is 14 / 12 / 12 bits when using PDU Session ID / DRB ID / LCID, respectively. For comparison, in Option b, only 6 bits are needed to signal DRB ID / LCID.

SDAP-Config ::=                     SEQUENCE {

pdu-Session                         PDU-SessionID,

sdap-HeaderDL                       ENUMERATED {present, absent},

sdap-HeaderUL                       ENUMERATED {present, absent},

defaultDRB                          BOOLEAN,

mappedQoS-FlowsToAdd                SEQUENCE (SIZE (1..maxNrofQFIs)) OF QFI OPTIONAL, -- Need N

mappedQoS-FlowsToRelease            SEQUENCE (SIZE (1..maxNrofQFIs)) OF QFI OPTIONAL, -- Need N

...

}

PDU-SessionID ::=   INTEGER (0..255)

The other difference between Option a and b is mainly related to the rate control of multiple QoS flows within a DRB. In Option a, the rate of each QoS flow is controlled explicitly by gNB. In Option b, if gNB configures in RRC signalling that multiple QoS flows within a DRB are rate controlled, then after receiving the MAC CE of rate control, UE implementation can decide the rate distribution among different QoS flows. From previous discussion, companies have different views on which way is preferred. Given that Option b has clear benefit of less overhead, it is proposed to agree Option b.

Table 1: Comparison of rate indication options

|  | Option a | Option b |
| --- | --- | --- |
| Signalling overhead to indicate QoS flow | 14 / 12 / 12 bits when using PDU Session ID / DRB ID / LCID + QFI | 6 bits when using DRB ID / LCID |
| Rate control of multiple QoS flows within a DRB | Rate of each QoS flow is controlled explicitly by gNB | UE implementation can decide the rate distribution among different QoS flows. |

Proposal 1: QoS flows to be rate controlled are configured in RRC signalling, and DRB ID / LCID is signalled in MAC CE.

DL rate control

In RAN plenary, following was concluded on DL rate control:

RAN2 to continue working on the uplink solution and to liaise with SA2/SA4 if there are issues if the same approach would be extended for the DL as well, assuming the solution extension for DL would not cause issues in RAN2 side (not more work than what would be needed to prevent use of the solution for downlink direction). RAN3 to be kept informed of the progress.

RAN2 discussion is focused on the bit rate table design. The rate control MAC CE can be naturally extended to DL similarly to legacy Recommended Bit Rate MAC, where an Uplink/Downlink (UL/DL) field is used, as below:

TS 38.321 Figure 6.1.3.20-1: Recommended bit rate MAC CE

-	Uplink/Downlink (UL/DL): This field indicates whether the recommended bit rate or the recommended bit rate query applies to uplink or downlink. The length of the field is 1 bit. The UL/DL field set to 0 indicates downlink. The UL/DL field set to 1 indicates uplink;

As the main purpose of DL rate control is to adjust the DL data rate at the communication peer based on the adjustment of UL data rate, the range / granularity of uplink bit rate recommended by SA4 is applicable for DL rate control, i.e. there is no need to consider the specific requirements of bit rate of DL XR traffic.

It is therefore proposed that RAN2 to confirm that the Rel-19 XR rate control signalling is applicable for both UL data and DL data, and inform SA2/SA4/RAN3 about the agreement.

Proposal 2: RAN2 confirm that the Rel-19 XR rate control signalling is applicable for both UL data and DL data, and inform SA2/SA4/RAN3 about the agreement.

Range / granularity of bit rate

In RAN2#129 meeting, following was agreed “Specify a new table for XR rate control. FFS distribution (exponential, linear), codepoints etc.” In SA4 reply LS S4-242182 [2] (with key content copied below), the uplink bit rate range of XR applications is from 100 kbps to 40 Mbps, and preferred granularity is 1%.

The range varies from 100kbps to 40Mbps. The numbers are based on the SA4 traffic characteristics study for XR services documented in 3GPP TR 26.926. Specifically, for XR split rendering and cloud gaming, a typical uplink data rate is 200 kbps. For AR conversational, a typical uplink data rate is 20Mbps.

The granularity varies among existing data rate adaptation algorithms, and it can be as small as 1% for some algorithms (e.g., transport-wide congestion control as documented in 3GPP TR 26.822), yet also coarser for other algorithms (e.g., video codecs that can adapt to a given target bit rate constraint). From an application perspective, finer granularity, e.g., 1%, is generally preferred, if possible, but coarser granularity may also be sufficient, e.g., 10%.

With linear distribution and 1 kbps granularity (i.e. 1% of 100 kbps), the number of code points needed are (40 Mbps – 100 kbps) / 1 kbps + 1 = 39901. The granularity becomes too detailed for large data rate, e.g. 0.0025% at 40 Mbps.

For exponential distribution, assuming same formula used for BSR as below is used:

Bk=Bmin×1+pk, with p=Bmax/Bmin1/N-1-1

The number of code points N can be calculated below:

N=1+lnBmax/Bminln(1+p)

We can derive that N = 604, with p = 0.01, Bmax = 40000, Bmin = 100.

It is obvious that linear distribution requires much larger number of code points compared with exponential distribution (39901 vs. 604). The extremely-fine granularity of linear distribution at higher data rate might not be useful. It is therefore proposed to use exponential distribution for XR rate control.

Proposal 3: Exponential distribution is used to encode the bit rate for XR rate control.

If exponential distribution is used, the granularity (p in equation above) is 0.587% and 1.18% for 10 bits (N=1024) and 9 bits (N=512) bit rate signalling.

Observation 1: Assuming bit rate range of 100 kbps to 40 Mbps, 9 and 10 bits of bit rate signalling have the granularity of 1.18% and 0.587%, respectively.

Granularity 1.18% is close to SA4’s preference, therefore it is sufficient to use 9 bits to encode the bit rate. The bit rate table has the minimum and maximum value of 100 kbps and 40 Mbps, as indicated by SA4.

Proposal 4: Bit rate table is designed with following parameters: 9 bit (512 entries), 100 kbps as minimum bit rate, 40 Mbps as maximum bit rate.

MAC CE content

From above discussion, 9 bit table is needed to satisfy SA4 requirements. Bit Rate field in legacy Recommended Bit Rate MAC CE has 6 bits, which is not sufficient. In addition, the index to bit rate mapping is also different from legacy Recommended Bit Rate MAC CE. It is therefore proposed to introduce a new Rate Control MAC CE.

Proposal 5: A new MAC CE is introduced for Rel-19 rate control.

One open issue is whether bit rate multiplier as in legacy Recommended Bit Rate MAC CE should be used. For a simple analysis of whether multiplier is useful or not, we assume n bits are used to encode bit rate when multiplier is not used, and n – 1 bits are used to encode bit rate when multiplier is used, and 1 additional bit is used for multiplier. A comparison is shown in Figure 1 below. With 1 bit multiplier, there are two tables: the base table (Table L in the figure), and the table derived from multiplier (Table H in the figure). Assume Table L is exactly the same as the lower half of the bit rate table when multiplier is not used. Depending on the choice of multipliers, there are three cases when multiplier is used. In case A, Table H is the same as upper half of the bit rate table when multiplier is not used. In case B, there is some gap between Table H and Table L. In case C, there is some overlapping between Table L and Table H, which means that code points are not used efficiently. In summary, what can be achieved by a multiplier solution is the design without multiplier. With improper selection of multipliers, the overall bit rate table (considering both Table L and Table H as in the example figure) has either some coverage holes or overlapping coverage of the bit rates. Considering that multiplier increases additional complexity without clear benefits, it is proposed to not consider multiplier.

Figure 1: Analysis of multiplier

Proposal 6: Bit rate multiplier is not used.

From Proposal 1 in above discussion, DRB ID / LCID should be included in MAC CE to identify QoS flows for rate. From Proposal 2, UL/DL field is also needed. An example MAC CE for rate control is shown in Figure 2 below.

Figure 2: Example MAC CE for Rate Control

Proposal 7: New MAC CE introduced for rate control includes the following fields: Bit Rate, DRB ID or LCID, UL/DL.

In email discussion “[POST129][509][XR] MAC running CR (Qualcomm)”, there is one editor’s note regarding physical-layer bit rate: “FFS whether this bit rate is physical-layer bit rate (as in the legacy recommended bit rate procedure) or some other type of bit rate.”

Our understanding is that the “physical-layer bit rate” is the bit rate of MAC PDU (Transport Block), not the bit rate after encoding, which is dynamic for each TB. The other interpretation of bit rate is the bit rate of SDAP SDUs / PDCP SDUs, and is denoted as application layer bit rate in the following discussion. The main difference between physical and application layer bit rate is then the layer 2 overhead: PDCP header, MAC-I, RLC header, MAC subheader.

We can estimate the ratio of layer 2 overhead relative to one MAC PDU for XR. In SA4 reply LS S4-242182 [2], uplink bit rate range is 100 kbps to 40 Mbps, based on TR 26.926. For XR split rendering and cloud gaming, 30 and 60 fps are assumed in TR 26.926. To estimate maximum rate of layer 2 overhead, we need to assume the smallest MAC PDU size, which corresponds to 100 kbps and 60 fps. The smallest layer 2 SDU size is 100*103 / 60 / 8 = 208 bytes, and the largest layer 2 SDU size is 40*106 / 60 / 8 = 83333 bytes (we can assume 9000 byte PDCP SDU size limitation). Assuming 3 byte PDCP header (corresponding to 18 bit PDCN SN, which is overkill for 60 fps traffic), 4 byte MAC-I, 3 byte RLC header (corresponding to 18 bit RLC SN), 2 or 3 byte MAC subheader (1 byte L field if MAC SDU is 256 bytes or less; 2 byte L field otherwise), there are 12 or 13 bytess of layer 2 header. Layer 2 overhead ratio is 5.45% and 0.144% for 208 and 9000 byte PDCP SDUs, respectively.

From above analysis, we can see that the definition of bit rate (physical layer vs. application layer) matters for low data rate, but not for high data rate.

Observation 2: Difference between physical layer bit rate and application layer bit rate can be over 5% for low data rate (e.g. 100 kbps).

What matters to gNB is mainly the physical layer bit rate, which is closely related to the radio resource for scheduling. It is therefore proposed to confirm that bit rate is physical layer bit rate (as in legacy Recommended Bit Rate MAC CE).

Proposal 8: RAN2 to confirm that bit rate is physical layer bit rate (as in legacy Recommended Bit Rate MAC CE).

Split bearer in DC

In DC scenario, split bearer can be used for XR services to boost the throughput. If both MN and SN send MAC CEs for rate control of the same split bearer or same QoS flow(s) mapped to split bearer, some discussion might be needed on UE behavior regarding which data rate to be applied for the split bearer or DRB as indicated in the MAC CEs, e.g. whether UE follows latest MAC CE from MN or SN, or UE should consider the MAC CEs jointly.

Proposal 9: RAN2 to discuss whether split bearer should be considered for rate control.

Conclusion

In this contribution, we discuss XR rate control, and have the following observations:

Observation 1: Assuming bit rate range of 100 kbps to 40 Mbps, 9 and 10 bits of bit rate signalling have the granularity of 1.18% and 0.587%, respectively.

Observation 2: Difference between physical layer bit rate and application layer bit rate can be over 5% for low data rate (e.g. 100 kbps).

We propose the following:

Proposal 1: QoS flows to be rate controlled are configured in RRC signalling, and DRB ID / LCID is signalled in MAC CE.

Proposal 2: RAN2 confirm that the Rel-19 XR rate control signalling is applicable for both UL data and DL data, and inform SA2/SA4/RAN3 about the agreement.

Proposal 3: Exponential distribution is used to encode the bit rate for XR rate control.

Proposal 4: Bit rate table is designed with following parameters: 9 bit (512 entries), 100 kbps as minimum bit rate, 40 Mbps as maximum bit rate.

Proposal 5: A new MAC CE is introduced for Rel-19 rate control.

Proposal 6: Bit rate multiplier is not used.

Proposal 7: New MAC CE introduced for rate control includes the following fields: Bit Rate, DRB ID or LCID, UL/DL.

Proposal 8: RAN2 to confirm that bit rate is physical layer bit rate (as in legacy Recommended Bit Rate MAC CE).

Proposal 9: RAN2 to discuss whether split bearer should be considered for rate control.

- References

[1] R2-2501335, Session chair (Huawei), “Report from session on R18 MBS, R18 QoE and R19 XR”

[2] S4-242182, SA4 LS, “Reply to LS on appropriate range and granularity of bit rate adaptation for XR applications”

[3] RP-243305, Nokia (moderator), “Release 19 XR enhancements - Way forward on WID scope revision”