3GPP TSG-RAN WG2 Meeting #129bis	R2-2501883

# Wuhan, China, 7 – 11 April 2025

Agenda item:		8.11.3

Source:	Xiaomi

Title:	Other aspects of SBFD

Document for:		Discussion and Decision

Introduction

In RAN2#129 meeting, following was agreed regarding other aspects of SBFD [1]:

A new SP CLI measurement resource set activation/deactivation MAC CE is introduced to activate/deactivate the SP CLI measurement resource.

In this contribution, we discuss RAN2 aspects of SBFD other than RACH.

Discussion

gNB-to-gNB CLI measurement

RAN3 indicates the following in LS R3-250888:

RAN3 has agreed that SBFD time and frequency configuration info should be exchanged between neighbour gNBs in Served Cell Information NR IE, and also between gNB-DU and gNB-CU in case of split architecture. RAN3 also reached the common understanding that RAN3 needs RAN2 to provide the concrete IE design on SBFD time and frequency configuration information, which should be based on parameters provided by RAN1.

With the agreement and common understanding reached in RAN3, RAN3 would like to ask RAN2 to work on the IE design of SBFD time and frequency configuration information similar to the TDD-UL-DL-ConfigurationCommon IE which is signalled over Xn and F1.

In email discussion “[Post129][217][SBFD] List of open issues of RRC impact”, there was following proposed solution: For SBFD gNB-to-gNB CLI measurement, to include the set of one or more periodic NZP CSI-RS resources configuration into inter-node message MeasurementTimingConfiguration.

It is helpful to review RAN3 agreements before discussing further RAN2 actions based on RAN3 LS. The above RAN3 LS is the outcome of RAN3 offline discussion, with the summary in R3-250803, whose relevant content is copied below:

Issue 1: Exchange of SBFD configuration

Send an LS to RAN2 and cc RAN1, asking to work on the IE design on SBFD time and frequency configuration

gNB-DU provides the SBFD time and frequency configuration to gNB-CU in Served Cell Information IE, and

gNB-CU provides the SBFD time and frequency configuration info received over Xn interface from gNB-CU to gNB-DU, in Served Cell Information IE in Served Cell Information IE in gNB-CU Configuration Update message.

Issue 2: Exchange of measurement configuration

For exchange of SSB info over Xn, nothing to be done, already supported in RAN3 spec

WA: For exchange of NZP CSI-RS over Xn, to put a new IE in Served Cell Information NR IE with reference to RAN2 definition

Related RAN3 Chair notes are also copied below:

Exchange of SBFD configuration:

gNB-DU provides the SBFD time and frequency configuration to gNB-CU in Served Cell Information IE, and gNB-CU provides the SBFD time and frequency configuration info received over Xn interface from gNB-CU to gNB-DU, in Served Cell Information IE in Served Cell Information IE in gNB-CU Configuration Update message.

The SBFD time and frequency configuration is encoded as container defined by RAN2.

Exchange of measurement configuration:

For exchange of SSB info over Xn, nothing to be done, already supported in RAN3 spec.

For exchange of NZP CSI-RS over Xn, to add a new IE in Served Cell Information NR IE with reference to RAN2 definition?

ZTE: Why NZP CSI-RS to be designed as explicit information rather than container?

It can be seen that during RAN3 discussion, SBFD time / frequency configuration and measurement configuration (including NZP CSI-RS configuration) are separate issues. RAN3 LS is only about SBFD configuration, and RAN3 is still discussing measurement configuration (blue sentence above). Therefore RAN2 can wait for RAN3 discussion regarding NZP CSI-RS configuration.

Proposal 1: RAN2 wait for RAN3’s decision regarding signaling of measurement resource configuration (including NZP CSI-RS resource configuration) for gNB-to-gNB CLI measurement.

For signalling of SBFD time and frequency configuration, RAN3 LS indicates the following “RAN3 would like to ask RAN2 to work on the IE design of SBFD time and frequency configuration information similar to the TDD-UL-DL-ConfigurationCommon IE which is signalled over Xn and F1.” TDD configuration in Xn and F1 signalling is indicated as a container referring to SIB1, e.g. in TS 38.423 clause 9.2.2.11 Served Cell Information NR, the TDD configuration IE is included as follows:

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| … |  |  |  |  |  |  |
| CHOICE NR-Mode-Info | M |  |  |  | – |  |
| … |  |  |  |  |  |  |
| >TDD |  |  |  |  |  |  |
| >>TDD Info |  | 1 |  |  | – |  |
| … |  |  |  |  |  |  |
| >>>TDD UL-DL Configuration Common NR | O |  | OCTET STRING | Includes the tdd-UL-DL-ConfigurationCommon contained in the SIB1 message as defined in TS 38.331 [10] | YES | ignore |

Therefore as long as RAN2 defines SBFD time and frequency configuration in RRC, RAN3 can simply refer to it in Xn and F1 specifications. RAN2 should follow RAN1 RRC parameter list to define SBFD time and frequency configuration. RAN1 RRC parameter list R1-2501645 [2] only includes the time domain information of SBFD configuration as from previous agreements:

For configuration of SBFD symbols within a TDD-UL-DL pattern period, the following parameters are supported

-	A starting slot index

-	A starting symbol index within the starting slot

-	An ending slot index

-	An ending symbol index within the ending slot

Frequency domain information of SBFD configuration is not yet included in RAN1 RRC parameter list, although RAN1 has already made agreements regarding frequency resource indication in RAN1#119 meeting, as below:

Confirm the following working assumption made in RAN1#118.

Working Assumption

For cell-specific configuration of frequency locations of SBFD UL subband, for each SCS configuration in SCS-SpecificCarrierList for UL, starting RB and bandwidth of SBFD UL subband are indicated by a RIV-based indication as defined in 38.214 setting NBWPsize=275.

For cell-specific configuration of frequency locations of SBFD DL subband(s), for each SCS configuration in SCS-SpecificCarrierList for DL, starting RB and bandwidth of each SBFD DL subband are indicated by a RIV-based indication as defined in 38.214 setting NBWPsize=275.

- One or two SBFD DL subbands can be configured

Although RAN2 may design RRC signalling for frequency domain indication based on RAN1 agreement, there is no need to rush for that. RAN2 can wait for RAN1’s update on RRC parameter list before finalizing the RRC signalling for SBFD time and frequency configuration.

Proposal 2: RAN2 wait for RAN1’s update on RRC parameter list before finalizing the RRC signalling for SBFD time and frequency configuration.

SP CLI measurement resource set activation/deactivation MAC CE

- In RAN2#129 meeting, following was agreed “A new SP CLI measurement resource set activation/deactivation MAC CE is introduced to activate/deactivate the SP CLI measurement resource.” In email discussion “[Post129][218][SBFD] Running CR for TS 38.321 (Samsung)”, procedure for the MAC CE was captured, with MAC CE format FFS.

The newly introduced SP CLI measurement resource set activation/deactivation MAC CE is based on the following RAN1#119 meeting agreement:

For aperiodic L1 CLI-RSSI/CLI-SRS-RSRP reporting on PUSCH based on a set of semi-persistent CLI measurement resources, the gNB may activate and deactivate the configured semi-persistent CLI measurement resource sets by sending a new SP CLI Measurement Resource Set Activation/Deactivation MAC CE

- TCI state IDi field is included in the new MAC CE, which is used as typeD QCL source for the CLI measurement resource within the Semi Persistent CLI measurement resource set. Either all TCI state ID fields are present or none.

- If the TCI state ID field(s) are absent, the UE QCL assumptions are the default rules agreed in RAN1#118bis

In RAN1 RRC parameter list R1-2501645 [2], SP CLI measurement resource set related fields are copied as below:

| Sub-feature group | Parameter name in the spec | New or existing? | Description | Value range | Per (UE, cell, TRP, …) | UE-specific or Cell-specific | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
| L1 UE-to-UE CLI measurement and reporting | cli-MeasurementResourceSetList | New | List of references to SRS-RSRP measurement resources or CLI-RSSI measurement resources used for L1 SRS-RSRP measurement and reporting or L1 CLI-RSSI measurement and reporting. | srs-RSRP-MeasurementResourceSetList or cli-RSSI-MeasurementResourceSetList | Per DL BWP In CSI-ResourceConfig | UE-specific | Agreement Extend CSI-ResourceConfig to include two CLI measurement resource set lists for SRS-RSRP and CLI-RSSI measurements based on Rel-16 SRS-ResourceConfigCLI and rssi-ResourceConfigCLI defined in MeasObjectCLI for L3 based SRS-RSRP and CLI-RSSI measurement. - The resourceType for the two new CLI measurement resource set lists can be set to periodic, semi-persistent or aperiodic - The number of periodic/semi-persistent CLI measurement resource sets is restricted to 1 in a CSI-ResourceConfig as in current specifications. - Note: No new usage needs to be defined for SRS resource sets |
| L1 UE-to-UE CLI measurement and reporting | cli-RSSI-MeasurementResourceSetList | New | List of references to CLI-RSSI measurement resources for L1 CLI-RSSI measurement and reporting in a CLI measurement resource set. Contains up to maxNrofCLI-RSSI-MeasurementResourceSetsPerConfig resource sets if resourceType is 'aperiodic' and 1 otherwise. FFS: maxNrofCLI-RSSI-MeasurementResourceSetsPerConfig. | A length-N list of IDs of CLI-RSSI measurement resourceSet. N is FFS. | Per DL BWP In cli-MeasurementResourceSetList | UE-specific | Agreement Extend CSI-ResourceConfig to include two CLI measurement resource set lists for SRS-RSRP and CLI-RSSI measurements based on Rel-16 SRS-ResourceConfigCLI and rssi-ResourceConfigCLI defined in MeasObjectCLI for L3 based SRS-RSRP and CLI-RSSI measurement. - The resourceType for the two new CLI measurement resource set lists can be set to periodic, semi-persistent or aperiodic - The number of periodic/semi-persistent CLI measurement resource sets is restricted to 1 in a CSI-ResourceConfig as in current specifications. - Note: No new usage needs to be defined for SRS resource sets |
| L1 UE-to-UE CLI measurement and reporting | srs-RSRP-MeasurementResourceSetList | New | List of references to SRS-RSRP measurement resources used for L1 SRS-RSRP measurement and reporting in a CLI measurement resource set. Contains up to maxNrofSRS-RSRP-MeasurementResourceSetsPerConfig resource sets if resourceType is 'aperiodic' and 1 otherwise. FFS: maxNrofSRS-RSRP-MeasurementResourceSetsPerConfig. | A length-N list of IDs of SRS-RSRP measurement resource set. N is FFS. | Per DL BWP In cli-MeasurementResourceSetList | UE-specific | Agreement Extend CSI-ResourceConfig to include two CLI measurement resource set lists for SRS-RSRP and CLI-RSSI measurements based on Rel-16 SRS-ResourceConfigCLI and rssi-ResourceConfigCLI defined in MeasObjectCLI for L3 based SRS-RSRP and CLI-RSSI measurement. - The resourceType for the two new CLI measurement resource set lists can be set to periodic, semi-persistent or aperiodic - The number of periodic/semi-persistent CLI measurement resource sets is restricted to 1 in a CSI-ResourceConfig as in current specifications. - Note: No new usage needs to be defined for SRS resource sets |

It can be seen that RAN1 agreed to extend CSI-ResourceConfig to include two CLI measurement resource set lists for SRS-RSRP and CLI-RSSI measurements. For the MAC CE to activate SP CLI measurement resource set, it needs to include CLI measurement resource set ID (for CLI-RSSI or SRS-RSRP measurement). Similar to SP CSI-RS/CSI-IM Resource Set Activation/Deactivation MAC CE, following fields should also be indicated: A/D, Serving Cell ID, BWP ID, TCI State IDs.

Proposal 3: SP CLI measurement resource set activation/deactivation MAC CE includes following fields: A/D, Serving Cell ID, BWP ID, CLI measurement resource set ID (for CLI-RSSI or SRS-RSRP measurement), TCI State IDs.

Interference issue

In Rel-19 SBFD, gNB can receive in UL and transmit in DL simultaneously. An example is shown in Figure 1 below. In Case 1, the DL reception may cause interference to the PUSCH reception in network side. In Case 2, neighbor cell’s DL transmission may also cause interference to the PUSCH reception in network side, i.e., CLI. Although the interference issue above could be mitigated to certain extent by implementation as well as CLI handling techniques, the PUSCH transmitted in SBFD symbols still suffers more interference than PUSCH transmitted in non-SBFD symbols. RAN2 needs to discuss whether such PUSCH quality difference between SBFD and non-SBFD symbols should be addressed or not.

Figure 1: Interference issue of PUSCH in SBFD symbols

Note that quality difference between SBFD and non-SBFD symbols for RACH was already identified by RAN1, which agreed the following in RAN1#118bis meeting:

For RACH configuration Option 1, support separate power control parameters for PRACH transmission in SBFD symbols and non-SBFD symbols

One potential enhancement to consider PUSCH quality difference between SBFD and non-SBFD symbols is to make LCP aware whether PUSCH resource contains SBFD symbols, e.g. during the step of selection of logical channel.

Proposal 4: RAN2 to discuss whether PUSCH quality difference between SBFD and non-SBFD symbols should be addressed or not.

Conclusion

In this contribution, we discuss RAN2 aspects of SBFD other than RACH, and propose the following:

Proposal 1: RAN2 wait for RAN3’s decision regarding signaling of measurement resource configuration (including NZP CSI-RS resource configuration) for gNB-to-gNB CLI measurement.

Proposal 2: RAN2 wait for RAN1’s update on RRC parameter list before finalizing the RRC signalling for SBFD time and frequency configuration.

Proposal 3: SP CLI measurement resource set activation/deactivation MAC CE includes following fields: A/D, Serving Cell ID, BWP ID, CLI measurement resource set ID (for CLI-RSSI or SRS-RSRP measurement), TCI State IDs.

Proposal 4: RAN2 to discuss whether PUSCH quality difference between SBFD and non-SBFD symbols should be addressed or not.

- References

[1] R2-2501332, RAN2 Vice Chairman (CATT), "Report from session on Rel-18 MIMO, Rel-19 MIMO, LPWUS, SBFD, NR Others"

[2] R1-2501645, Moderator (Ericsson), "Consolidated Rel-19 higher layers parameters list Post RAN1#120"