3GPP TSG-RAN WG2 Meeting #129bis	R2-2501887

Wuhan, China, Apr. 7th – 11th, 2025

Agenda Item:	8.6.2

Source: 	OPPO

Title:  	Discussion on open issues for inter-CU LTM

Document for:	Discussion and Decision

Introduction

In this contribution, we would like to present our considerations on the open issues related to inter-CU LTM.

Discussion

Security handling

In last SA3 meeting, progress was made on security handling for inter-CU LTM in non-DC cases. In their LS reply [1], it is indicated that the NCC can be transmitted to the UE in plaintext via MAC CE message.

1. Overall Description: SA3 thanks RAN2 for the LS on the security handling for inter-CU LTM in non-DC cases. Considering the RAN2’s request on avoiding RRC configuration between cell switches for subsequent inter-CU LTM in solution selection, SA3 reached a compromise that the NCC can be transmitted to the UE in plaintext via MAC CE message, even though the unprotected MAC leads to increased attack scenarios and probability of handover failure. In addition, SA3 will not consider the protection of MAC CE messages in this release, but may consider studying MAC CE protection in the future. The details on key distribution between the source gNB and the target gNB are left to RAN3 to decide and inform SA3. RAN3 should consider the scenario where the serving gNB utilizes the unused NH (if any is received from a path switch procedure at execution phase) to derive the KNG-RAN* for LTM handover.

RAN2 can simply follow SA3’s decision on the method to provide NCC, i.e., in non-DC cases, the NCC value to be used at inter-CU LTM execution is included in the LTM cell switch command MAC CE.

RAN2 confirms that in case of non-DC the NCC value to be used at inter-CU LTM execution is included in LTM cell switch command MAC CE.

Besides non-DC case, we also support inter-CU MCG LTM with DC. In this case, the UE is required to perform refresh of security key at both MN and SN side. For MN key update, we understand the same way as in non-DC case can be followed, i.e., based on the NCC value included in LTM cell switch command MAC CE.

For inter-CU MCG LTM with DC, the NCC value to be used for MN key update is included in LTM cell switch command MAC CE, i.e., follow the same way as in non-DC case.

For SN security key update in inter-CU MN LTM with DC, it was agreed to apply R15 RRC reconfiguration with sync procedure, i.e., the sk-counter configured in LTM candidate cell configuration is applied for SKgNB derivation.

Agreements-RAN2#127 16. Upon execution of inter-CU MN LTM with DC, the UE is required to perform refresh of security key, re-establishment of RLC and PDCP, and MAC reset at both MN and SN side (i.e. Rel-15 principle is applied). 17. For the SN key update in inter-CU MN LTM with DC, the UE applies legacy R15 RRC reconfiguration with sync procedure.

In case of consecutive inter-CU MCG LTM (i.e., each LTM execution is inter-CU MCG LTM), one sk-counter is enough to derive a unique SKgNB due to the change of KgNB. While according to previous agreements, a mixture of inter-CU LTM and intra-CU LTM can be configured to a UE and MN security key update is not required for each LTM attempt. With KgNB unchanged, the same SKgNB can be derived when the UE accesses to the same LTM candidate cell after several times of intra-CU LTM as the case shown in the figure below.

To be specific, in the first intra-CU LTM attempt, the UE moves from cell 1 to cell2 and derives the SKgNB#1 based on KgNB#1 and the sk-counter configured in cell2 RRC configuration. Then the UE leaves cell2 and accesses to cell3 in the second intra-CU LTM attempt. In the third LTM attempt, the UE switches back to cell2 and the same SKgNB#1 as in the first LTM attempt will be generated due to the use of the same KgNB #1 and sk-counter.

For above scenario, there is COUNT reuse issue since the same SKgNB will be generated while the PDCP state variables (i.e., SRB and UM DRB) are initialized during the LTM execution, thus it may not be enough to rely on one sk-counter for subsequent LTM procedures. To solve the problem, we may have the following solutions.

- Solution1: Up to NW to update the sk-counter after each intra-CU MCG LTM with DC.

- Solution2: RAN2 works on solutions to prevent the reuse of the same SKgNB. Examples include preconfiguring a list of sk-counters or including the sk-counter in the LTM cell switch command.

RAN2 discuss how to avoid SkgNB reuse issue in case that the UE accesses to the same LTM candidate cell after several times of intra-CU LTM.

  - Solution1: Up to NW to update the sk-counter after each intra-CU MCG LTM with DC.

  - Solution2: For MCG LTM with SCG configuration, preconfigure a list of sk-counter or include the sk-counter in LTM cell switch command.

The above security handling for inter-CU MCG LTM with DC has not been confirmed by SA3 and there are also RAN3 impacts such as KSN generation, thus it is suggested to send an LS to SA3 and RAN3 for confirmation and awareness.

Sends an LS to SA3 and RAN3 to inform RAN2 agreements on security handling for inter-CU MCG LTM with DC.

Fast failure recovery

Fast failure recovery based on preconfigured LTM candidate cell configurations was discussed in previous meetings and the agreements were made as follows.

Agreements-RAN2#127bis 1. The Rel18 handling on failure is reused in R19 if the UE selects an intra-CU LTM candidate cell after intra-CU LTM failure; for other cases, e.g. inter-CU LTM failure, the failure handling is FFS (related to SA3’s inputs).

As agreed, intra-CU LTM candidate cells can be selected for fast recovery after intra-CU LTM failure. For inter-CU LTM failure, the discussion was postponed due to its dependency on how to provide the NCC for security key update.

According to SA3 inputs, NCC value to be used for security key update during inter-CU LTM execution is provided in LTM cell switch command MAC CE. There is no doubt that the provided NCC can be used for the security key derivation for the target cell while it is unclear whether it is also applicable for other inter-CU LTM candidate cells. Depending on how the source gNB distributes the security key among the candidate cells, we may have the following alternatives when considering the failure handling for inter-CU LTM failure.

- Alt1: The NCC value provided in LTM cell switch command MAC CE can be only used for the target cell. In this alternative, the UE is not available of the security update information for other inter-CU LTM candidate cells, thus only intra-CU LTM candidate cells (i.e., intra-CU is determined by comparing with the source cell) can be selected for failure recovery by reusing the security context of the source cell.

- Alt2: The NCC value provided in LTM cell switch command MAC CE can be used for the target cell and partial inter-CU LTM candidate cells, e.g., LTM candidate cells within the same CU as the target cell. In this alternative, the source gNB needs to generate the security keys for inter-CU candidate cells other than the target cell, making them feasible for performing failure recovery in addition to intra-CU LTM candidate cells.

- Alt3: The NCC value provided in the LTM cell switch command MAC CE can be used for the target cell and all the other inter-CU LTM candidate cells. This requires the source gNB to prepare the security keys for all inter-CU LTM candidate cells, which may increase Xn signalling overhead but facilitates an extension of the candidate cells that can be selected for failure recovery.

Alt1 can be supported as a baseline since recovering to an intra-CU LTM candidate cell requires minimal effort due to the availability of security context. Furthermore, to fully utilize the preconfigured LTM candidate cell configurations, it is beneficial to also consider inter-CU LTM candidate cells for fast failure recovery, i.e., Alt2 and Alt3. Compared to Alt3, Alt2 is more signalling efficiency since the source gNB just need to interact with the target gNB and this procedure is anyway needed for triggering LTM to the target cell. Based on this understanding, it is proposed that the inter-CU LTM candidate cells within the same CU as the target cell can be also applied for fast recovery and further discuss whether to support other inter-CU LTM candidate cells.

For inter-CU LTM failure, at least intra-CU LTM candidate cell configurations (in relative to the source cell) can be selected for fast recovery.

For inter-CU LTM failure, inter-CU LTM candidate cells belonging to the same CU as the target cell can be selected for fast recovery. RAN2 further discuss whether to support other inter-CU LTM candidate cells for fast recovery.

If P6 is agreed, send an LS to RAN3 to inform RAN2 agreement.

In R18, the LTM candidate cell configuration can be also applied for fast failure recovery after RLF. We think the case should also be supported in R19. In this case, since there is no LTM cell switch command received, only intra-CU LTM candidate cells can be selected for recovery.

LTM failure recovery after RLF is supported in R19 and only intra-CU LTM candidate cell configurations can be selected for recovery.

Coexistence

One FFS left from last RAN2 meeting regards to coexistence issue.

2. Same as Rel-18 intra-CU LTM, Rel-19 inter-CU LTM could coexist with CovE, (e)RedCap, while couldn’t coexist with NRU. FFS on MIMO 2TA and NES.

In R18, LTM support the coexistence with R18 MIMO 2TA. For inter-CULTM, we see no issue to support the same coexistence, i.e., for the target cell configured with MIMO 2TA, the TAG of the TA is determined by the TAG ID pointer associated with the indicated the TCI state.

Support the coexistence of R18 MIMO two TA and inter-CU LTM.

R19 MIMO further extend 2TA to UL-only TRP scenario, which require a dynamic adjustment on PL_offset for supporting UL transmission on UL-only TRP. Based on RAN1 discussion, the PL_offset is obtained by SRS measurement in NW side, which is not supported for LTM candidate. And whether to support coexistence of inter-CU LTM and R19 MIMO 2 TA is also studied in MIMO session. We can wait for their conclusion and align views between Mobility session and MIMO session later.

FFS on the coexistence of R19 MIMO two TA and inter-CU LTM.

For coexistence with NES, the agreement made for R18 LTM is as follows

Agreements Understanding is that an LTM target is not a serving cell and hence it will not apply CELL DTX/DRX until the switch is completed. Nothing is optimized or specified for RACHless

We understand the same principle can be followed for R19 LTM, i.e., Cell DTX/DRX configuration of the target cell has no impact on LTM, and nothing needs to be specified.

Support the coexistence of Cell DTX/DRX and inter-CU LTM and there is no spec impact.

Conclusion

Based on the discussion in section2, we have following observation proposals:

Proposal 1	RAN2 confirms that in case of non-DC the NCC value to be used at inter-CU LTM execution is included in LTM cell switch command MAC CE.

Proposal 2	For inter-CU MCG LTM with DC, the NCC value to be used for MN key update is included in LTM cell switch command MAC CE, i.e., follow the same way as in non-DC case.

Proposal 3	RAN2 discuss how to avoid SkgNB reuse issue in case that the UE accesses to the same LTM candidate cell after several times of intra-CU LTM.

o	Solution1: Up to NW to update the sk-counter after each intra-CU MCG LTM with DC.

o	Solution2: For MCG LTM with SCG configuration, preconfigure a list of sk-counter or include the sk-counter in LTM cell switch command.

Proposal 4	Sends an LS to SA3 and RAN3 to inform RAN2 agreements on security handling for inter-CU MCG LTM with DC.

Proposal 5	For inter-CU LTM failure, at least intra-CU LTM candidate cell configurations (in relative to the source cell) can be selected for fast recovery.

Proposal 6	For inter-CU LTM failure, inter-CU LTM candidate cells belonging to the same CU as the target cell can be selected for fast recovery. RAN2 further discuss whether to support other inter-CU LTM candidate cells for fast recovery.

Proposal 7	If P6 is agreed, send an LS to RAN3 to inform RAN2 agreement.

Proposal 8	LTM failure recovery after RLF is supported in R19 and only intra-CU LTM candidate cell configurations can be selected for recovery.

Proposal 9	Support the coexistence of R18 MIMO two TA and inter-CU LTM.

Proposal 10	FFS on the coexistence of R19 MIMO two TA and inter-CU LTM.

Proposal 11	Support the coexistence of Cell DTX/DRX and inter-CU LTM and there is no spec impact.

- 4. Reference

[1] S3-251124		Reply LS on security handling for inter-CU LTM in non-DC cases