3GPP TSG-RAN WG2 Meeting #129bis                              	  R2-2501888

Wuhan, China, Apr. 7th – 11th, 2025

Agenda Item:	8.6.3

Source: 	OPPO

Title:  	Discussion on event-triggered L1 measurement reporting

Document for:	Discussion and Decision

Introduction

In previous meeting, RAN2 has discussed the issues related to event-triggered L1 measurement reporting and the following agreements were reached:

Agreements on L1 event-triggered MR:

- Confirm on the previous agreement that "For measurement resource configuration, R18 LTM CSI resource configuration is reused".

- Continue to carry the bits for SSB resource set when CSI-RS resource set is configured. FFS whether CSI-RS can be configured without SSB (up to R1 to decide).

- Do not create a new resource configuration just for the event triggered measurements.

- Confirm on the previous agreement "For measurement reporting configuration, R18 LTM-CSI-ReportConfig is reused".

- Ask RAN1 what should be the maximum number of beam measurement results that can be reported in event-triggered measurement report.

- Reuse the existing RSRP values and RSRP offset values and their interpretations in TS 38.133 for event-triggered measurement report. Can revisit if RAN4 identifies any issue.

- TimeToTrigger and reportInterval of L3 measurement report can be reused. FFS on additional value(s) of reportInterval.

- Cell specific offset is not supported for LTM L1 event evaluation.

- The trigger quantity for LTM L1 event triggered measurement is the same as the report quantity.

- Explicit indication in MR MAC CE to inform which beam(s) meets the event condition with TTT. FFS on the detailed signalling.

- Explicit indication in MR MAC CE to inform the leaving event condition is met. FFS whether it is per beam.

- R18 differential RSRP encoding is used as baseline.

- LTM MR MAC CE only include one triggered measurement event and the LTM MR MAC CE can be multiplexed in any available UL grant.

- Select the beam not satisfying the event condition, based on the measured quality (i.e. according to the best beam(s))

- Truncated MR MAC CE includes the triggered beam information:

- One report config ID (event config ID)

- At least one triggered beam id (SSBRI or CRI) + L1 RSRP

- Common format for normal MR MAC CE and truncated MR MAC CE is used. Details are handled by MAC running CR rapporteur.

- For truncated LTM MR MAC CE, an LCID separate from that for non-truncated LTM MR MAC CE is used.

- S-measureConfig is applicable to in the L1 event-triggered MR. No additional change is expected on RRC and MAC.

In this contribution, we discuss the remaining open issues on event-triggered L1 measurement reporting.

Discussion

SP CSI-RS activation/deactivation MAC CE

LS in R1-2501500 shares the latest RAN1 progress on semi-persistent CSI-RS resource for candidate cell L1-RSRP measurement, and it has agreed to use MAC CE to activate/deactivate the semi-persistent CSI-RS resource.

In legacy, there is a similar MAC CE on SP ZP CSI-RS Resource Set Activation/Deactivation as shown below. We understand the MAC CE format for candidate CSI-RS resource activate/deactivate can follow similar way.

The candidate SP CSI-RS activation/deactivation MAC CE should contain the following fields:

- Activation/deactivation indication

- Candidate SP CSI-RS resource (set) ID

As we can see, there is no additional field introduced by comparing with the legacy SP ZP CSI-RS Resource Set Activation/Deactivation MAC CE. And there are 4 R bits can be used for LTM scenario differentiation. Thus, we would like to reuse legacy SP ZP CSI-RS Resource Set Activation/Deactivation MAC CE format and use R bits to indicate whether the MAC CE applies to LTM or not.

The candidate SP CSI-RS activation/deactivation MAC CE contains activation/deactivation indication and candidate SP CSI-RS resource (set) ID.

RAN2 further study whether to reuse legacy SP ZP CSI-RS Resource Set Activation/Deactivation MAC CE format, i.e. use R bits to indicate whether the MAC CE applies to LTM or not.

Coexistence of event triggered reporting and mTRP@serving

In last RAN1 meeting, there is a conclusion on that it is up to RAN2 to decide whether the coexistence of LTM event triggered reporting and mTRP operation at serving cells is supported or not.

Conclusion

The following is up to RAN2: Coexistence of LTM event triggered reporting and mTRP operation at serving cells

In general, we understand supporting LTM event triggered L1 measurement reporting should not cause any restrictions to serving cell configuration. If the coexistence is not supported, it means NW should disable the mTRP related configuration when providing an event triggered L1 measurement reporting configuration for LTM candidates. And LTM support subsequent cell switch execution, the event configuration is provided to UE during LTM preparation period. That is, the system may need to turn off the mTRP operation for a long period as UE may keep LTM config among multiple cell coverages.

Coexistence of LTM event triggered reporting and mTRP operation at serving cells is supported.

Supporting the serving cell to configure mTRP requires minimal additional specification effort, which is how to define the beam of the serving cell since there are two indicated TCI states in this case.

In previous meeting, RAN2 agreed that the current beam (i.e. a beam corresponding to the indicated TCI state) is used for event evaluation in L1 measurement reporting for serving cell. For mTRP case, there are serval options to define the current beam:

- Option1: current beam corresponding to one of the indicated TCI states, FFS on which one.

- Option2: current beam corresponding to either of the indicated TCI states.

- Option3: current beam corresponding to both indicated beams.

RAN2 further study how define the current beam of serving cell for LTM L1 event evaluation if mTRP is configured.

(Truncated)MR MAC CE

In last meeting, there is an offline discussion on whether to include explicit indication in MR MAC CE to inform NW which beam(s) meets the event condition. And the consensus is to introduce such explicit indication as assistance info, while the granularity of the indication is still FFS.

Explicit indication in MR MAC CE to inform which beam(s) meets the event condition with TTT. FFS on the detailed signalling.

Explicit indication in MR MAC CE to inform the leaving event condition is met. FFS whether it is per beam.

There are two ways on the table for the trigger status indication:

- Option1: per beam indication

Per beam indication require UE to include the indication on whether the beam has met the event condition with TTT, which occupying more bits in MR MAC CE. With per beam indication, NW can clearly know which beam is more robust.

- Option2: per MAC CE indication

Per MAC CE indication could save the signalling overheads of the MR MAC CE, and NW can know which event the MAC CE is triggered for. Additionally, whether the MR MAC CE is triggered by entering or leaving condition can also be known with such indication. While the drawback of per MAC CE indication is that NW is not able to know which beam measurement result is more stable. Since there may be a case that the triggered beam is not the best beam due to L1 measurement result fluctuation. If NW select the best beam instead of triggered beam for triggering early sync or LTM cell switch execution, unnecessary failure may be caused.

We understand the triggered status is important to guarantee a proper decision by NW for beam indication of early sync procedure and cell switch execution. Thus, per beam indication is included to inform whether the entering/leaving event condition is met or not.

For MR MAC CE, per beam indication is included to inform whether the entering/leaving event condition is met or not.

Another issue is about the MR MAC CE format, i.e. whether to put triggered beam measurement at the beginning of the MR MAC CE. As we agreed, differential RSRP is used to indicate the L1-RSRP measurement result for MR MAC CE, which implies the first beam in the MR MAC CE is the one with the highest measurement quantity.

While another agreement on truncated MR MAC CE has also been agreed. That is, the truncated MR MAC CE include at least one triggered beam measurement.

Truncated MR MAC CE includes the triggered beam information:

  - One report config ID (event config ID)

  - At least one triggered beam id (SSBRI or CRI) + L1 RSRP

Then, it is questionable whether the first is the triggered beam or best beam if only one beam information can be included in the truncated MR MAC CE.

- Option1: The first beam is triggered beam with absolute RSRP, the second beam is the best beam (absolute RSRP), and remaining beams can use differential RSRP based on the absolute RSRP of best beam.

- Option 2: The first beam is best beam with absolute RSRP, and remaining beams (including triggered beam) can use differential RSRP based on the absolute RSRP of best beam. With this option, UE can only use truncated MR MAC CE if UL grant can accommodate first Nth beams and triggered beam.

We understand both options can work, while option1 is more aligned with previous agreement. With option1, UE can report the triggered beam measurement to NW even the UL grant can only accommodate one beam measurement information.

For MR MAC CE, the first beam is the triggered beam, and the second beam is the best beam.

For triggered beam and best beam, absolute RSRP is used.

SR for MR MAC CE

In earlier meeting, it has agreed that NW can configure a dedicated SR configuration for MR MAC CE transmission. And the legacy SR procedure for resource allocation is the baseline to send the event-triggered L1 measurements MAC CE.

There are still open issues related to SR handling which need to be further studied. The first one is SR cancelation condition. If the SR is triggered for L1 event-triggered measurement report, and if a MAC PDU is transmitted and this PDU includes L1 measurement report MAC CE, the triggered SR should be cancelled accordingly.

The SR triggered for L1 event-triggered measurement report is cancelled if a MAC PDU is transmitted and this PDU includes L1 MR MAC CE or truncated MR MAC CE.

Another issue is about SR overlapping. In current spec, there are some note to clarify the SR/PUCCH resource handling for the cases that PUCCH resources are overlapped among the triggered SRs.

- Case 1: non-BFR SR overlapping

Except for SR for SCell beam failure recovery, the selection of which valid PUCCH resource for SR to signal SR on when the MAC entity has more than one overlapping valid PUCCH resource for the SR transmission occasion is left to UE implementation.

- Case 2: BFR SR& non-BFR SR

When the MAC entity has pending SR for SCell beam failure recovery and the MAC entity has one or more PUCCH resources (other than PUCCH resources of pending SR for beam failure recovery of a BFD-RS set) overlapping with PUCCH resource for SCell beam failure recovery for the SR transmission occasion, the MAC entity considers only the PUCCH resource for SCell beam failure recovery as valid. When the MAC entity has pending SR for beam failure recovery of a BFD-RS set of Serving Cell and the MAC entity has one or more PUCCH resources (other than PUCCH resources of pending SR for beam failure recovery) overlapping with PUCCH resource for beam failure recovery of that BFD-RS set for the SR transmission occasion, the MAC entity considers only the PUCCH resource for beam failure recovery of that BFD-RS set as valid.

- Case 3 Scell BFR SR & BFD-RS set BFR SR

When the MAC entity has PUCCH resource for pending SR for SCell beam failure recovery overlapping with PUCCH resource for pending SR for beam failure recovery of a BFD-RS set for the SR transmission occasion, it is up to UE implementation to select PUCCH resource for SCell beam failure recovery or PUCCH resource for beam failure recovery of a BFD-RS set.

For L1 event triggered measurement report, there may be similar issue. When the MAC entity has PUCCH resource for pending SR for L1 event triggered MR and the MAC entity has one or more PUCCH resources overlapping with PUCCH resource for “other triggered SR”, which PUCCH resource is considered as valid.

In general, there are three ways:

- Option 1: left to UE implementation.

- Option2: MAC entity considers only the PUCCH resource for L1 event triggered MR MAC CE as valid.

- Option3: if “other triggered SR” is for BFR, the MAC entity considers only the PUCCH resource for beam failure recovery as valid.

When the MAC entity has PUCCH resource for pending SR for L1 event triggered MR and the MAC entity has one or more PUCCH resources overlapping with PUCCH resource for “other triggered SR”, UE selects PUCCH resource based on:

- Option 1: left to UE implementation.

- Option2: MAC entity considers the PUCCH resource for L1 event triggered MR MAC CE as valid.

- Option3: if “other triggered SR” is for BFR, the MAC entity considers the PUCCH resource for beam failure recovery as valid.

Conclusion

Based on the discussion above, we have the following proposals:

Proposal 1	The candidate SP CSI-RS activation/deactivation MAC CE contains activation/deactivation indication and candidate SP CSI-RS resource (set) ID.

Proposal 2	RAN2 further study whether to reuse legacy SP ZP CSI-RS Resource Set Activation/Deactivation MAC CE format, i.e. use R bits to indicate whether the MAC CE applies to LTM or not.

Proposal 3	Coexistence of LTM event triggered reporting and mTRP operation at serving cells is supported.

Proposal 4	RAN2 further study how define the current beam of serving cell for LTM L1 event evaluation if mTRP is configured.

Proposal 5	For MR MAC CE, per beam indication is included to inform whether the entering/leaving event condition is met or not.

Proposal 6	For MR MAC CE, the first beam is the triggered beam, and the second beam is the best beam.

Proposal 7	For triggered beam and best beam, absolute RSRP is used.

Proposal 8	The SR triggered for L1 event-triggered measurement report is cancelled if a MAC PDU is transmitted and this PDU includes L1 MR MAC CE or truncated MR MAC CE.

Proposal 9	When the MAC entity has PUCCH resource for pending SR for L1 event triggered MR and the MAC entity has one or more PUCCH resources overlapping with PUCCH resource for “other triggered SR”, UE selects PUCCH resource based on:

-	Option 1: left to UE implementation.

-	Option2: MAC entity considers the PUCCH resource for L1 event triggered MR MAC CE as valid.

-	Option3: if “other triggered SR” is for BFR, the MAC entity considers the PUCCH resource for beam failure recovery as valid.

Reference

[1] RP-234036 New WID-R19 Mobility Enhancements-v5.docx