3GPP TSG-RAN WG2 Meeting #129bis							       R2-2501892

Wuhan, China, Apr. 7th – 11th, 2025

Agenda Item:	8.13.4

Source: 	NEC

Title:	Considerations on Service Continuity of Multi-hop Relay

Document for:	Discussion/Decision

- Introduction

The Rel-19 WID for sidelink relay includes following objectives on service continuity [1].

- Specify the following intra-gNB service continuity scenarios for multi-hop U2N relay based on Rel-17/18 procedures (for remote UE): First Priority: - Intra-gNB multi-hop indirect to direct path switching using existing framework - Intra-gNB multi-hop indirect to single-hop indirect path switching using existing framework Second Priority in order of importance: - Intra-gNB direct to multi-hop indirect path switching - Intra-gNB single-hop indirect to multi-hop indirect path switching The scenarios C and D are limited to path switching to a target indirect path consisting of the last relay UE in “direct” RRC Connected mode and all the other intermediate relay(s) in “indirect” RRC Connected mode to the same cell.

This contribution discusses the procedures of the first priorities scenarios (i.e., Scenario A and Scenario B) and also the mobility of relay UE, such as the intermediate Relay UE.

- Discussion

  - Basic Procedures for Scenario A

Given the source path is an indirect path and the target path is a direct path, reconfigurationWithSync without sl-PathSwitchConfig and the related processes in TS38.331 [2] can be used for Scenario A. The main difference from the processes of Rel-17 I2D procedure is the additional relay UE (i.e., the first relay UE). Then, an added step is introduced to release the first relay UE on the source indirect path.

Using reconfigurationWithSync without sl-PathSwitchConfig for path switching of Scenario A and an added step to release the first relay UE of the source indirect path.

  - Basic Procedures for Scenario B

Given the source path is a multi-hop indirect path and the target path is a single-hop indirect path, reconfigurationWithSync including sl-PathSwitchConfig and the related processes in TS38.331 [2] can be used for scenario B. Similar to Scenario A, an added step to release the first relay UE on the source indirect path is needed.

Using reconfigurationWithSync including sl-PathSwitchConfig for path switching of Scenario B and an added step to release the first relay UE of the source indirect path.

  - Mobility of the Intermediate Relay UE

In RAN2#128 [3] meeting, there is an agreement on control plan procedures shown as below:

In one approach (“approach 2”) of U2N relays, Intermediate Relay UEs (other than the Last Relay UE) can be in any RRC state when the U2N remote UE is in RRC_CONNECTED.

According to the agreement, the intermediate Relay UE may stay in RRC_INACTIVED or RRC_IDLE when it is serving the Remote UE. Thus the intermediate Relay UE can trigger cell reselection or relay UE reselection, but the serving cell of the Remote UE may not be aware of it. Such reselection may lead to the service interruption. To solve this problem, the intermediate relay UE can send notification message to the Remote UE. As a result, the Remote UE can perform RRC connection reestablishment procedure accordingly.

The intermediate Relay UE staying in RRC_INACTIVED or RRC_IDLE may trigger cell reselection or relay reselection, which may lead to the service interruption of Remote UE.

Upon cell reselection or relay UE reselection, the intermediate relay UE can send notification message to the Remote UE. As a result, the Remote UE can perform RRC connection reestablishment procedure accordingly.

  - Role type change

It is possible that during the multi-hop U2N relay service undergoing, the U2N last relay UE’s Uu quality becomes worse that it cannot act as the U2N last relay UE anymore. In this case, traditionally according to Rel-18 behaviour, the U2N last relay UE will send notification message towards U2N remote UE, to trigger the upper layer to release the current PC5 RRC connection. However, when it comes to Rel-19, one alternative way is that the U2N last relay UE can change its role towards U2N intermediate UE.

In details, since its PC5 connection with the U2N remote UE is still stable, the U2N remote UE does not necessarily need to release the PC5 connection with the U2N last relay UE. Instead, the U2N last relay UE would be configured to change its role towards U2N intermediate UE, and it would further perform U2N last relay UE selection/reselection, so that to construct a multi-hop U2N relay link.

On the other hand, it is also possible that the U2N intermediate relay UE would be configured to change its role towards U2N last relay UE, so that the multi-hop U2N relay link would be switched to single-hop U2N relay link.

RAN2 should discuss whether to allow the U2N last relay UE and/or U2N intermediate relay UE to perform role type change while maintain the PC5-RRC connection with U2N remote UE for service continuity.

- Conclusion

Based on the discussion in this document, we have the following observations and proposals:

Observation 1:	The intermediate Relay UE staying in RRC_INACTIVED or RRC_IDLE may trigger cell reselection or relay reselection, which may lead to the service interruption of Remote UE.

Proposal 1:	Using reconfigurationWithSync without sl-PathSwitchConfig for path switching of Scenario A and an added step to release the first relay UE of the source indirect path.

Proposal 2:	Using reconfigurationWithSync including sl-PathSwitchConfig for path switching of Scenario B and an added step to release the first relay UE of the source indirect path.

Proposal 3:	Upon cell reselection or relay UE reselection, the intermediate relay UE can send notification message to the Remote UE. As a result, the Remote UE can perform RRC connection reestablishment procedure accordingly.

Proposal 4:	RAN2 should discuss whether to allow the U2N last relay UE and/or U2N intermediate relay UE to perform role type change while maintain the PC5-RRC connection with U2N remote UE for service continuity.

- Reference

- RP-250188, Rel-19 WID of NR Sidelink Relay.

- TS38.331-i40.

- Chairman Notes, RAN2 #128 meeting.