3GPP TSG-RAN WG2 Meeting #129b	 R2-2501893

Wuhan, China, April 7-11, 2025

Agenda Item:	8.4.2

Source: 	NEC

Title:  	Discussion on LP-WUS in RRC_IDLE INACTIVE

Document for:	Discussion and Decision

- 1 Introduction

In last RAN2#129 meeting, the following agreements are made:

On separate band issue

RAN2 understands that UE can report which band(s) is supported by LR to NW.

RAN2 understands that any potential overload issues could be addressed by current mechanism in spec.

Send LS to RAN1 and RAN4 to inform the agreements.

On sub-grouping

For UE_ID based subgrouping, similar formula defined for PEI subgrouping is reused for LP-WUS subgrouping, i.e.,

SubgroupID = (floor (UE_ID/(N*Ns*Np)) mod subgroupsNumForUEID) + (subgroupsNumPerPO – subgroupsNumForUEID), where

-	UE_ID is related to 5G-S-TMSI,

-	N is the number of total paging frames in one DRX cycle,

-	Ns is the number of the PO for a PF,

-	Np is the number of subgroupNumForUEID for PEI, if configured and UE supports PEI; otherwise, Np is 1,

-	subgroupsNumForUEID and subgroupsNumPerPO are the subgroup number for UE_ID based subgrouping for LP-WUS and the total subgroup number for LP-WUS, respectively.

On SA2 LS (R2-2500050/S2-2412876)

RAN2 sends reply LS to SA2/RAN3/CT1, and CC RAN1 and RAN4 to confirm the “LP-WUS” terminology.

Regarding the SA2 raised issue on UE Radio Capability for Paging Information, R2 understand that there is no issue for NW after Release 17 (in which case the LP-WUS UE-ID based subgrouping UE capability is included in the UE-RadioPagingInfo container). Whether there is issue for the other cases (for the features mentioned by SA2 LS R2-2500050) can be further discussed in the main session.

In this contribution, we further discuss remaing issue mainly focusing on LP-WUS sbugroup.

- 2 Discussion

- 2.1 LP-WUS subgroup

In the last RAN2#129 meeting, the following LP-WUS UE-ID based subgroup is agreed:

SubgroupID = (floor (UE_ID/(N*Ns*Np)) mod subgroupsNumForUEID) + (subgroupsNumPerPO – subgroupsNumForUEID), where

-	UE_ID is related to 5G-S-TMSI,

-	N is the number of total paging frames in one DRX cycle,

-	Ns is the number of the PO for a PF,

-	Np is the number of subgroupsNumForUEID for PEI, if configured and UE supports PEI; otherwise, Np is 1,

-	subgroupsNumForUEID and subgroupsNumPerPO are the subgroup number for UE_ID based subgrouping for LP-WUS and the total subgroup number for LP-WUS, respectively.

It can be seen that a new parameter Np is introduced, it stands for UE-ID based subgroup number for PEI configured by NW. By this way, when the UE chooses to monitor both LP-WUS and PEI, further paging false alarm can be achieved by separating the UEs belonging to same one PEI subgroup into different subgroup in LP-WUS. Based on this agreement, we further discuss some remaining issue.

First issue is the definition of Np, currently, whether the UE needs to consider this Np when calculating subgroup ID depends on whether NW configure PEI and the UE supports PEI. It is true for NW configuration part, however for latter part, i.e., the UE supports PEI, we think it is not necessary. There could be PEI supporting UE and non-PEI supporting UE and both type of UE may use LP-WUS, which results in different LP-WUS monitoring UE uses different LP-WUS calculation, i.e., some are using Np while some are not. This makes subgroup formula a little bit messed up. In our understanding, a unified calculation is more promising, even though for those UEs that are not supporting PEI, they still can calculate subgroup ID with Np, there is no harm or impact on further reducing paging false alarm (i.e., anyway the LP-WUS monitoring UE can be assigned to one subgroup), instead, LP-WUS UE can be assigned to subgroup ID in a more organized way no matter whether the UE is monitoring PEI or not.

Proposal-1: for UE monitoring LP-WUS, no matter whether the UE is monitoring PEI or not, the parameter Np in LP-WUS subgroup should always be applied.

Proposal-1a: change the Np definition as the following:

Np is the number of subgroupsNumForUEID for PEI, if configured and UE supports PEI; otherwise, Np is 1,

Second issue is that we still need to complete the design of UE-ID, i.e., UE-ID: 5G-S-TMSI mod X, FFS value X.

One purpose of using UE-ID is to make sure that all UEs with different UE-ID can be evenly separated into different PF/PO/subgroup. To fully calculate UE-ID, the value of UE-ID should not be less than [N*Ns*Np* subgroupsNumForUEID], currently the max N is 256 without eDRX, Ns is 4 and Np is 8. As for subgroupsNumForUEID for LP-WUS, even though the RAN1 modified it to 31, we still consider it as 32 due to the fact that one common group still exists. Therefore the max UE-ID without eDRX case should be [256*4*8*32] - 1, i.e., 262144 - 1, and X should be 262144. Furthermore, if eDRX is supported, max N is extended to 1024 due to extended paging cycle T, therefore X should be 1048576.

Based on this understanding RAN2 is also suggested to discuss whether eDRX can be supported with LP-WUS simultaneously.

Proposal-2: for LP-WUS subgroup, UE-ID should be 5G-S-TMSI mod X, where X is 262144 if eDRX is not applied.

Proposal-2a: for LP-WUS subgroup, UE-ID should be 5G-S-TMSI mode X, where X is 1048576 if eDRX is applied.

Proposal-3: RAN2 is suggested to discuss whether eDRX will be supported or not with LP-WUS simultaneously.

If our proposals above are agreed, the LP-WUS subgroup formula should be modified like the following

SubgroupID = (floor (UE_ID/(N*Ns*Np)) mod subgroupsNumForUEID) + (subgroupsNumPerPO – subgroupsNumForUEID), where

-	UE_ID is 5G-S-TMSI mod X, where X is 262144 if eDRX is not applied, FFS eDRX case

-	N is the number of total paging frames in one DRX cycle,

-	Ns is the number of the PO for a PF,

-	Np is the number of subgroupsNumForUEID for PEI, if configured and UE supports PEI; otherwise, Np is 1,

-	subgroupsNumForUEID and subgroupsNumPerPO are the subgroup number for UE_ID based subgrouping for LP-WUS and the total subgroup number for LP-WUS, respectively.

One more issue is that RAN3 is currently discussing how to design UE identity index value which stands for UE-ID in RAN2 spec, i.e., whether a new extended type of UE-ID should be introduced or not. And based on RAN3 last meeting, they agree that RAN3 are waiting for RAN2 LP-WUS subgroup formula design to make the final decision on UE identity index value, for example, RAN3 need to consider what is Np, what is the value of X. Therefore, we should send an LS to RAN3 as they expected.

Proposal-4: send an LS to inform RAN3 of the detailed LP-WUS subgroup formula (e.g., the definition of Np, the new value X) to facilitate the design on UE identity index value.

- 2.2 Configuration aspect

However, it seems like majority think UE-specific LP-WUS configuration is not needed, then we are fine to not continue proposing this.

While for enabler/disabler of LP-WUS monitoring, it means the NW can indicate LP-WUS capable UE whether needs to apply LP-WUS feature or not when the UE is related to RRC_IDLE/INACTIVE. If without this enable/disable indication, normally all LP-WUS supporting UEs need to apply LP-WUS feature (i.e., monitoring LP-WUS when entry condition is fulfilled) as long as the NW is broadcasting LP-WUS configuration in system information, this is not ideal and there are following reasons to support dedicated enable/disable indication:

1. Paging false alarm

When large amount of UEs are monitoring LP-WUS, once one UE is paged, unnecessary LP-WUS reception to other UEs is increased. Therefore if NW can control part of UEs (instead of all LP-WUS supporting UEs) monitor LP-WUS and other part of UEs do not monitor LP-WUS, it can provide flexibility and avoid potential serious paging false alarm issue.

2. Latency requirement

For LP-WUS monitoring UE, the paging delay is increased compared with normal paging monitoring UE when the paging is initiated after LP-WUS monitoring occasion and before paging monitoring occasion. If NW want one specific LP-WUS capable UE not to enter LP-WUS monitoring mode (e.g., considering latency), the NW can send this dedicated enable/disable indication to this UE, which provide flexibility and avoid latency issue.

3. Power consumption

The purpose of introducing LP-WUS is for power saving, but if the transition between MR and LR is frequent for one specific UE, the power saving could be overkilled. For example, if the paging probability is relatively high, the UE better not to monitor LP-WUS even though this UE supports LP-WUS feature, otherwise, every time when this UE is monitoring LP-WUS, it will wake up its MR soon, which cannot be accepted. Therefore, NW can, based on implementation (e.g., information from paging probability), to determine whether such kind of UEs need to enter LP-WUS monitoring mode or not.

Some companies could be concerning about mobility issue, in our understanding, the UE who receives dedicated enable/disable indication in RRCRelease can still monitor LP-WUS in different cell when the UE reselects to neighbour cell, i.e., supporting mobility case. As long as the LP-WUS is enabled by RRCRelease, the UE follows the baseline behaviour, i.e., the UE is supporting LP-WUS and NW is providing LP-WUS configuration in system information, then the UE with enable indication could just use LP-WUS among cells. From RAN2 point view, there is no impact, the only thing needs to do is RAN3 need to consider that for CN paging, NG-RAN node need to inform CN of UE LP-WUS state via Ng interface, e.g., NG-AP UE Context Release Complete message, so that CN can properly initiate NGAP paging message with/without LP-WUS. For RAN paging, the NG-RAN send Xn paging message with/without LP-WUS capability, for example, if LP-WUS for one UE is disabled, the anchor gNB will send Xn paging message without LP-WUS capability to neighbour gNB. Since this is related to RAN3 discussion, we may inform RAN3 of this decision if agreed.

Based on the above reasons, we propose:

Proposal-5: support dedicated enable/disable indication in RRCRelease to indicate whether the UE needs to apply LP-WUS feature or not during RRC_IDLE/INACTIVE.

Proposal-5a: the enable/disable indication is applicable to all cells which is supporting LP-WUS feature (i.e., mobility is supported) and whether the LP-WUS capable UE uses LP-WUS or not is based on enable/disable indication and LP-WUS configuration in system information of camping cell.

- 3 Conclusions

Proposal-1: for UE monitoring LP-WUS, no matter whether the UE is monitoring PEI or not, the parameter Np in LP-WUS subgroup should always be applied.

Proposal-1a: change the Np definition as the following:

Np is the number of subgroupsNumForUEID for PEI, if configured and UE supports PEI; otherwise, Np is 1,

Proposal-2: for LP-WUS subgroup, UE-ID should be 5G-S-TMSI mod X, where X is 262144 if eDRX is not applied.

Proposal-2a: for LP-WUS subgroup, UE-ID should be 5G-S-TMSI mode X, where X is 1048576 if eDRX is applied.

Proposal-3: RAN2 is suggested to discuss whether eDRX will be supported or not with LP-WUS simultaneously.

Proposal-4: send an LS to inform RAN3 of the detailed LP-WUS subgroup formula (e.g., the definition of Np, the new value X) to facilitate the design on UE identity index value.

Proposal-5: support dedicated enable/disable indication in RRCRelease to indicate whether the UE needs to apply LP-WUS feature or not during RRC_IDLE/INACTIVE.

Proposal-5a: the enable/disable indication is applicable to all cells which is supporting LP-WUS feature (i.e., mobility is supported) and whether the LP-WUS capable UE uses LP-WUS or not is based on enable/disable indication and LP-WUS configuration in system information of camping cell.

- 4 References