3GPP TSG-RAN WG2 Meeting #129b	 R2-2501894

Wuhan, China, April 7-11, 2025

Agenda Item:	8.4.3

Source: 	NEC

Title:  	Discussion on LP-WUS RRM

Document for:	Discussion and Decision

- 1 Introduction

In last RAN2#129 meeting, the following agreements are made:

The entry condition for MR serving cell RRM relaxation can include both MR and LR measurements.

If LR threshold is configured, the entry condition is when both MR and LR measurement are above the configured thresholds.

In this contribution we further discuss remaining issue.

- 2 Discussion

- 2.1 MR measurement relaxation

The last RAN2#129 meeting, RAN2 agreed that The entry condition for MR serving cell RRM relaxation can include both MR and LR measurements. Furthermore, we also need to discuss whether separate exit condition is needed, and whether it is the same for serving cell and neighbour cell.

Whether separate exit condition is needed

Before the entry condition of MR serving cell relaxation is fulfilled, the UE should perform normal/legacy MR serving cell measurement. However after entry condition is fulfilled, the UE cannot get MR serving cell measurement results in time as the UE has started to perform relaxed MR serving cell measurement. In this regard, MR measurement results may not be accurate to evaluate exit condition. We should consider LR measurement results instead. Note that for entry condition, MR measurement is mandatory and LR measurement is optional based on RAN2 agreement, which is not appropriate. Therefore, we suggest to have a separate exit condition for MR serving cell relaxation, that is to say, when entry condition is fulfilled, the UE performs LR serving cell measurement to evaluate whether exit condition of MR serving cell relaxation is fulfilled or not.

Proposal-1: introduce separate entry/exit condition, i.e., the LP-WUS UE performs relaxed MR serving cell measurement when entry condition is fulfilled and stops performing relaxed MR serving cell measurement when exit condition is fulfilled.

Entry condition: based on MR measurement results and optional LR measurement results;

Exit condition: based on LR measurement results.

Whether it is the same for serving cell and neighbour cell

No matter it is serving cell relaxation criterion or neighbour cell relaxation criterion, same principle is used, i.e., whether the signal quality of the UE is greater than a threshold, there is no too much difference. Also the most important thing is that we need to care about the frequency of MR wake-up since this is related to power saving gain. If either serving cell or neighbour cell is relaxed, but the other is not, MR still need to wake up more frequent compared with the case when both are relaxed. Both measurement results are used for cell reselection, there is no reason to for the one to perform normal measurement while the other to perform relaxed measurement. Furthermore, introducing many criterions could make spec complicated(e.g., one criterion for serving cell relaxation, one for neighbour cell relaxation, one for serving cell offloading…), therefore one MR relaxation criterion for both serving cell and neighbour cell is enough.

Proposal-2: same criterion should be applied to both MR serving cell and neighbour cell measurement relaxation.

- 2.2 MR measurement offloading

Background:

Case #1 have been agreed in RAN4

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #1 Fully offloading case | Off | Off: FFS the condition and the details | ON |

In this section we discuss the relationship between MR serving cell and MR neighbour cell in fully offloading case, based on RAN4 understanding, it is assumed that both should be performed simultaneously, but one detail is that case#1 identified by RAN4 does not preclude an intermediate state (i.e., serving cell MR measurement is OFF, FFS the condition of neighbour cell).

The reason why the UE needs to perform neighbour cell measurement is to ensure the mobility, i.e., cell re-selection in RRC_IDLE/INACTIVE. Our understanding is that once the UE has fulfilled serving cell measurement offloading criterion (i.e., case#1, MR serving cell measurement is offloaded to LR), it means the signal strength/quality is already good enough, in this scenario, there will be no mobility issue. As MR serving cell measurement has been stopped, let along neighbour cell MR measurement. Therefore neighbour cell MR measurement should not be performed (i.e., FFS in case#1 can be removed).

Proposal-3: once the criteria for MR serving cell measurement offloading is met, then both MR serving cell and neighbour cell measurements should be stopped.

- 3 Conclusions

Proposal-1: introduce separate entry/exit condition, i.e., the LP-WUS UE performs relaxed MR serving cell measurement when entry condition is fulfilled and stops performing relaxed MR serving cell measurement when exit condition is fulfilled.

Entry condition: based on MR measurement results and optional LR measurement results;

Exit condition: based on LR measurement results.

Proposal-2: same criterion should be applied to both MR serving cell and neighbour cell measurement relaxation.

Proposal-3: once the criteria for MR serving cell measurement offloading is met, then both MR serving cell and neighbour cell measurements should be stopped.

- 4 References