3GPP TSG-RAN WG2 Meeting #129bis           	R2-2501897

Wuhan, China, Apr. 7th – 11th, 2025

Source:	CATT

Title:	Discussion on inter-CU LTM

Agenda Item:    8.6.2

Document for:  Discussion and Decision

Introduction

In this contribution, we further discuss the remaining issues on inter-CU LTM [1]. The contribution is organized as the following. In section 2, we discuss the following aspects,

- Security update related issues

- Further clarification on intra-CU MCG LTM with SCG configuration

- On the indication of complete candidate configuration

- The co-existence of inter-CU LTM with MIMO two-TA/NES

Our proposals are summarized in Section 3.

Discussion

Security update related issues

MAC CE design for security update

Regarding the security key update for inter-CU MCG LTM, according to SA3 reply [2],

Considering the RAN2’s request on avoiding RRC configuration between cell switches for subsequent inter-CU LTM in solution selection, SA3 reached a compromise that the NCC can be transmitted to the UE in plaintext via MAC CE message, even though the unprotected MAC leads to increased attack scenarios and probability of handover failure.

The NCC value is included in the LTM cell switch command in plaintext. Since the co-existence of intra-CU MCG LTM and inter-CU MCG LTM is supported in R19, the LTM cell switch command MAC CE can be optionally include the NCC for inter-CU MCG LTM.

Proposal 1: The NCC value can be optionally included in the LTM cell switch command MAC CE.

When to include the NCC in the LTM cell switch command MAC CE

If the NW triggers UE to perform LTM to an inter-CU target cell, i.e., the R19 set ID of the target cell is different from the one of source cell, it is straightforward that the NCC should be included in the LTM cell switch command MAC CE to trigger the security key update.

Proposal 2: The NW ensures that NCC is included in the LTM cell switch command MAC CE if the R19 set ID is different between the target cell and source cell. Conversely, if the R19 set ID is same for both cells, the NCC will not be included.

Fast recovery for inter-CU MCG LTM

In the current RRC running CR with regards to the inter-CU MCG LTM, it is still open whether to support the fast recovery for inter-CU MCG LTM.

Editor’s Note: Whether fast RLF recovery (attemptLTM-Switch) is supported for inter-CU LTM is still FFS.

As per our Proposal 1, UE cannot acquire the NCC if UE selects an inter-CU candidate cell while T311 is running. So fast recovery by inter-CU candidate cell (including re-configuration with sync failure and RLF failure) cannot be supported in R19.

Proposal 3: For inter-CU MCG LTM, fast recovery is not supported if UE selects the candidate cell with a R19 ID different from the source cell.

Further clarification on intra-CU MCG LTM with SCG configuration

In R18, it doesn’t support intra-CU MCG LTM with SCG configuration. In R19 it supports inter-CU MCG LTM with SCG configuration and it also supports the coexistence of intra-CU MCG LTM and inter-CU MCG LTM. Another issue that needs further clarification is whether to support the intra-CU MCG LTM candidate configuration including the SCG configuration in R19. From our perspective, due to the support of the co-existence between intra-CU MCG LTM and inter-CU MCG LTM, the intra-CU MCG LTM configuration should be provided as inter-CU MCG LTM due to the order of cell switch is not predictable.

Furthermore, in last meeting, it was agreed that,

The existing field mrdc-ReleaseAndAdd is always included when SCG configuration is included during an MCG LTM execution. Can be revisited if there is any issue.

That means upon the LTM execution, the SCG will be firstly released and then added. So even UE performs LTM cell switch to an intra-CU LTM candidate cell, we can follow the similar inter-CU principle that the field mrdc-ReleaseAndAdd is always included when SCG configuration is included during an intra-CU MCG LTM execution. Therefore, the intra-CU LTM candidate configuration with SCG configuration can be supported without further spec impact.

Proposal 4: The SCG configuration can be included in an intra-CU MCG LTM candidate configuration, similar as inter-CU MCG LTM candidate cell, with no further spec impact.

On the indication of complete candidate configuration

In the current RRC running CR for inter-CU LTM, it still remains the following FFS,

Editor’s Note: FFS whether only one indication is needed to indicate LTM configuration as complete configuration.

The discussion on this issue is due to the fact that for inter-CU MCG/SCG LTM, the reference configuration can include both the MCG part configuration and the SCG part configuration.

Let’s take the inter-CU SCG LTM for example. Upon candidate cell preparation phase, the target SN may indicate to the MN that an inter-CU SCG candidate cell provides a complete candidate SCG configuration. But for the same candidate cell, the MCG candidate configuration may be not a complete one. So companies concern whether to introduce a separate complete configuration indication for SCG part in this case. From our view, the final LTM candidate configuration for inter-CU SCG LTM is generated by the MN, so the MN knows whether the whole LTM candidate configuration is complete or not. Therefore we can just up to (source) MN to determine whether to include the complete field in the LTM candidate configuration. Similar views can also be applied for inter-CU MCG LTM.

Observation 1: For inter-CU MCG/SCG LTM, source MN should be able to determine whether the candidate configuration is complete or not.

Proposal 5: Only one indication is needed to indicate LTM configuration as complete configuration (i.e., same as R18 LTM). No additional indication is needed.

The co-existence of inter-CU LTM with MIMO two-TA/NES

On the co-existence issues, it remains the following FFS.

Same as Rel-18 intra-CU LTM, Rel-19 inter-CU LTM could coexist with CovE, (e)RedCap, while couldn’t coexist with NRU. FFS on MIMO 2TA and NES.

The co-existence of inter-CU LTM with MIMO two-TA

In R18, the LTM candidate cells can be configured with two-TA. When performing early RACH, the F1 will send the TAG information associated with the TA value. Upon LTM execution, it’s up to source DU implementation to ensure that the TCI state in the LTM command MAC CE is associated with the same TAG for the early TA.

When considering R19 inter-CU LTM, to support the candidate cell with two-TA, the only impact is RAN3 needs to exchange the TAG information associated with the early TA over Xn. No further RAN2 impact is founded.

Proposal 6: R19 inter-CU LTM can be coexisted with MIMO 2TA without additional spec impact.

The co-existence of inter-CU LTM with NES

During the candidate cell preparation phase, the candidate cell can accept or reject the requirement from the source cell. And it also supports the candidate cell to trigger the cancel of the procedure. Hence if one candidate cell is in NES mode, it can reject or cancel the LTM procedure. If the serving cell is in NES mode, the serving cell can trigger legacy handover procedure too. So the current RAN3 procedure can support even if it is a NES candidate cell.

Proposal 7: R19 inter-CU LTM can be coexisted with NES without additional spec impact.

Conclusion

Based on the previous analysis in Section 2, our observations and proposals are summarized as follows:

Observation 1: For inter-CU MCG/SCG LTM, source MN should be able to determine whether the candidate configuration is complete or not.

Security update related issues

Proposal 1: The NCC value can be optionally included in the LTM cell switch command MAC CE.

Proposal 2: The NW ensures that NCC is included in the LTM cell switch command MAC CE if the R19 set ID is different between the target cell and source cell. Conversely, if the R19 set ID is same for both cells, the NCC will not be included.

Proposal 3: For inter-CU MCG LTM, fast recovery is not supported if UE selects the candidate cell with a R19 ID different from the source cell.

Further clarification on inter-CU MCG LTM

Proposal 4: The SCG configuration can be included in an intra-CU MCG LTM candidate configuration, similar as inter-CU MCG LTM candidate cell, with no further spec impact.

On the indication of complete candidate configuration

Proposal 5: Only one indication is needed to indicate LTM configuration as complete configuration (i.e., same as R18 LTM). No additional indication is needed.

The co-existence of inter-CU LTM with MIMO two-TA/NES

Proposal 6: R19 inter-CU LTM can be coexisted with MIMO 2TA without additional spec impact.

Proposal 7: R19 inter-CU LTM can be coexisted with NES without additional spec impact.

Reference

[1] Draft_RAN2_129_Meeting_Report_v3

[2] S3-251124 Reply LS on security handling for inter-CU LTM in non-DC cases