3GPP TSG-RAN WG2 Meeting #129bis										R2-2501953

Wuhan, China, 7th - 11th April 2025

Agenda item:		8.19

Title: 	Discussion on UE capabilities to be included in UE Radio Paging Information

Source: 			ZTE Corporation, Sanechips

Document for: 	Discussion and Decision

- Introduction

At RAN2#129, there has been discussion on the SA2 [1] raised issue for UE Radio Capability for Paging Information:

----------------------Discussion and agreements at RAN2#129----------------------

R2-2501089	Procedure and configuration of LP-WUS for IDLE and INACTIVE mode	ZTE Corporation, Sanechips	discussion	Rel-19	NR_LPWUS-Core

- Regarding the SA2 raised issue on UE Radio Capability for Paging Information, R2 understand that there is no issue for NW after Release 17 (in which case the LP-WUS UE-ID based subgrouping UE capability is included in the UE-RadioPagingInfo container). Whether there is issue for the other cases (for the features mentioned by SA2 LS R2-2500050) can be further discussed in the main session.

----------------------Discussion and agreements at RAN2#129----------------------

The capability for LP-WUS UE-ID based subgrouping UE capability would be included in UE-RadioPagingInfo container thus would be no issue.

In this paper, we share some understanding on other features to be included in the UE Radio Capability for Paging Information.

- Discussion

In the SA2 LS, they shared the following concerns:

-----------------------SA2 LS content-----------------------

Initial Registration may be performed on a pre-Rel 19 gNB (or a later gNB that does not support LP-WUS) that analyses the UE’s UE Radio Access Capabilities to create the UE Radio Capability for Paging Information and send it to the AMF, see clause 5.4.4.3 of TS23.501. It will be important that LP-WUS related aspects of UE Radio Capability for Paging Information are not lost when Initial Registration is performed on a pre-Rel-19 gNB. If the LP-WUS aspects are lost, then there can be paging problems, when for example, paging is performed by a gNB that supports LP-WUS.

SA2 invites RAN2 and RAN3 to investigate this and provides solution if any issue is identified.

SA2 was not sure whether this issue also impacts RedCap, eRedCap and other features that are signalled within the UE Radio Capability for Paging Information. SA2 invites RAN2 and RAN3 to investigate this.

-----------------------SA2 LS content-----------------------

Apart from LP-WUS, which can be addressed by having the UE-ID based subgrouping capability in UE-RadioPagingInfo container, the following capabilities are introduced after R15 thus it is possible that there would be gNB not supporting the corresponding features:

Per our understanding, when gNB upgrade to a certain release, it would be able to decode the all the capabilities in the extended field.

For example, a R17 gNB would be able to decode all the following extension in R17 no matter it support the corresponding feature or not. And it would not be able to decode the R18 extension if it has not been upgraded to R18.

With the above understanding, we have the following proposal:

Proposal 1: When gNB is upgraded to a certain release, it is able to decode the UE capabilities in the extended field for this release and include the corresponding UE capabilities in UE Radio Paging Information.

For those capabilities not decoded and included in UE Radio Paging Information due to the release supported at gNB side, when UE Radio Paging Information is transferred upload to and download from the 5GC, and between gNBs, the target node cannot differentiate whether it is not supported by UE or not decoded by the gNB who acquired the capabilities. And one candidate solution is that the target gNB re-acquire UE capabilities when the UE Radio Paging Information is downloaded from CN or from another gNB and it should be left to NW implementation to re-acquire when needed.

Proposal 2: For those capabilities not decoded and included in UE Radio Paging Information due to the release supported at source gNB side, it is up to the target gNB to decide whether to re-acquire UE capabilities after the UE Radio Paging Information is downloaded from CN or transferred from the source gNB. No spec impact is needed.

Proposal 3: Reply to SA2 with RAN3 and CT1 in CC as in the Annex with the above understanding shared.

- Conclusion and proposals

Based on the analysis in previous sections, the following observations and proposals are given:

Proposal 1: When gNB is upgraded to a certain release, it is able to decode the UE capabilities in the extended field for this release and include the corresponding UE capabilities in UE Radio Paging Information.

Proposal 2: For those capabilities not decoded and included in UE Radio Paging Information due to the release supported at source gNB side, it is up to the target gNB to decide whether to re-acquire UE capabilities after the UE Radio Paging Information is downloaded from CN or transferred from the source gNB. No spec impact is needed.

Proposal 3: Reply to SA2 with RAN3 and CT1 in CC as in the Annex with the above understanding shared.

- Reference

- R2-2500050_S2-2412876 LS Reply on LP-WUS subgrouping

- Annex – draft Reply LS to SA2

3GPP TSG-RAN WG2 Meeting #129bis	R2-25xxxxx

Wuhan, China, 7th - 11th April 2025

Title:	Reply LS on Reply on LP-WUS subgrouping

Response to:	S2-2412876 LS Reply on LP-WUS subgrouping

Release:	Release 19

Work Item:	NR_LPWUS-Core

Source:	RAN2

To:	SA2

Cc:	RAN3, CT1

Contact Person:

#### Name:	Yuan Gao

E-mail Address:	gao.yuan66@zte.com.cn

- Overall Description:

Apart from LP-WUS UE-ID based sub-grouping capability to be included in UE-RadioPagingInfo container, for other features that are signalled within the UE Radio Capability for Paging Information, RAN2 understand:

- When gNB is upgraded to a certain release, it is able to decode the UE capabilities in the extended field for this release and include the corresponding UE capabilities in UE Radio Paging Information.

- For those capabilities not decoded and included in UE Radio Paging Information due to the release supported at the source gNB side, it is up to the target gNB to decide whether to re-acquire UE capabilities after the UE Radio Paging Information is downloaded from CN or transferred from the source gNB. No spec impact is needed.

2. Actions:

To RAN4:

RAN2 respectfully asks SA2, RAN3 and CT1 to take the above understanding into consideration.

3. Date of Next TSG-RAN2 Meetings:

RAN WG2 Meeting #130	May 19 – May 23, 2025	Malta, MT

RAN WG2 Meeting #131	August 25 – August 29, 2025	India (TBC), IN