3GPP TSG-RAN WG2 Meeting #129-bis	R2-2501970

Wuhan, China, 7th – 11th April, 2025

Agenda Item:	8.17

Source: 	Huawei, HiSilicon

Title: 	TDD operation for NB-IoT NTN

Document for:	Discussion and Decision

- Introduction

In RAN#105, a new work item was approved to introduce NB-IoT TDD mode exclusively for 1616-1626.5 MHz Mobile Satellite Services MSS band [1]. During the RAN1#119 meeting, some agreements were made for the TDD pattern and the corresponding frame structure [2], as listed below.

Agreement： Regarding operation within the same band as the TDD frame structure of legacy system in the 1.6GHz MSS band (shown below), RAN1 work in Rel-19 considers the following design constraints: - At the satellite, all downlink NB-IoT channels/signals in a cell can only use one of the downlink slots in the TDD frame structure (DL1, DL2, DL3 or DL4) across 90ms periods. - The same downlink slot is used in all the 90ms periods. - At the satellite, all uplink NB-IoT channels in a cell can only use one of the uplink slots in the TDD frame structure (UL1, UL2, UL3 or UL4) across 90ms periods. - The same uplink slot is used in all the 90ms periods. - The one uplink slot and one downlink slot in the TDD frame structure have the same index (DL1 & UL1, DL2 & UL2, DL3 & UL3, or DL4 & UL4). - NOTE: this does not imply that the only configuration(s) to be specified are according to these constraints.

The WID was revised in RAN#106 [3] to reflect RAN1 progress and narrow down the scope of the solutions.

In RAN 2#128 meeting [4], the following agreements were reached for RAN2 impacts of this new WID.

Agreements: 1. RAN2 will continue studying paging aspects based, on RAN1 progress 2. RAN2 assumes Kmac has to be extended (or a new parameter with higher range introduced) to address the case where the number of hops exceeds a certain limit. We continue the discussion in the next meeting to investigate if there are any other implications and in case of any decisions we send an LS to other groups if needed. 3. RAN2 confirms that idle mode eDRX is supported in IoT-NTN TDD network. 4. RAN2 thinks that a change of H-SFN duration (Option 1-1) and/or H-SFN total number (option 2-2) will impact RAN2 and SA2 specification regarding the support of idle mode eDRX in IoT-NTN TDD network and the impact should be evaluated. 5. RAN2 assumes that legacy coverage enhancement techniques (i.e. transmission with repetitions) are supported in IoT-NTN TDD system. 6. RAN2 can continue the discussion also on RAR window 7. Legacy barring bit will be used (FFS is cellBarred or cellBarred-NTN)

In this paper, we discuss the TDD operation for NB-IoT NTN.

- Discussion

  - Impact on Access Control

In the WID [3], it was determined that the NB-IoT TDD mode for NTN will operate exclusively in 1616-1626.5 MHz MSS band, where TN deployment is not expected.

- 4.1 Objective of SI or Core part WI or Testing part WI The work item aims to specify enhancements for NB-IoT NTN to enable NTN operation with a NB-IoT TDD mode leveraging commonalities with half-duplex NB-IoT FDD NTN, by defining a new NB-IoT TDD mode for NTN based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures for the NB-IoT operation in the targeted unpaired MSS allocated band (TN deployment is not expected in this band). The feature is not intended to be applicable to existing 3GPP bands. The work objectives assume the following: - LEO @600 km and @1200 km orbit respectively, with set-1 satellite parameters as reference scenarios (See 3GPP TR 36.763) - Target the 1616-1626.5 MHz MSS allocated band - Standalone deployment with anchor and non-anchor carriers (i.e. operating in carrier(s) used only for NB-IoT) - Operate with Earth fixed Tracking area, with either Earth fixed cells or Earth moving cells for NGSO - The new NB-IoT NTN TDD mode allows configuring the usage of radio resources in the targeted MSS allocated band with a periodic subset of the UL and DL subframes in N radio frames. The periodic pattern should consist of non-overlapping set of usable contiguous UL subframes (U) and set of usable contiguous DL subframes (D), and guard periods, which is periodic every N radio frames, with N=9 for the target MSS allocated band.

Due to this frequency band being newly allocated and not  part of  the existing operating frequency bands, the pre-R19 UEs cannot  access network in this frequency band. So there is no need to enhance the Access Control mechanism to prevent the legacy UEs from camping on.

Observation 1: Pre-R19 UEs cannot access the network in 1616-1626.5 MHz MSS allocated band so there is no need to enhance the Access Control mechanism for legacy UEs.

For R19 UEs which support NB-IoT NTN TDD mode, they will search for the 1616-1626.5 MHz MSS band if needed. Alternatively, the R19 UEs can choose to access the TN network. If the R19 UE detects a frequency within this band before detecting other frequencies during cell search, it may try to camp on  the NTN eNB using NB-IoT TDD mode. In this case, it is up to NW configuration whether to allow the UEs to access using the legacy access control indication cellBarred and there is no need to broadcast cellBarred-NTN in an NB-IoT TDD NTN cell.

Proposal 1: The IE cellBarred in the SIB1-NB message can be reused to control whether a UE supporting NB-IoT NTN TDD mode is allowed to access an NB-IoT TDD NTN cell.

  - Impact on SPS

For the NPRACH transmission, to get better RACH performance, new NPRACH periodicities should be introduced to align with the TDD structure otherwise many ROs will map to invalid NB-IoT subframes due to the mismatch between NRAPCH periodicities and the new TDD pattern. For the NPRACH preamble, the preamble format 0 and 1 used for FDD mode can be reused.

To reduce the overlap of NPRACH occasions with non-U IoT NTN TDD subframes, The following agreements were agreed in RAN1#120.

Agreement For NPRACH, further consider the two options until RAN1#120bis: - Option 1: New NPRACH periodicities (multiple of 90ms) are introduced for NB-IoT NTN TDD. - Option 2: NPRACH transmissions are postponed in non-U NB-IoT subframes until the next U NB-IoT subframe(s) - FFS: The unit of postponement (e.g. PRU)

Similarly, SPS periodicity for NB-IoT is not multiples of 90ms, which causes the mismatch between SPS periodicities and the new TDD pattern. Considering RAN1 agreements that only one UL slot is usable within a 90ms period, it may happen that there is no uplink subframe on the UL SPS occasion.

To resolve the case that there is no available UL subframe on an UL SPS transmission occasion, we propose to follow similar principle with NPRACH.

Proposal 2a: To reduce the overlap of UL SPS with non-U NB-IoT subframes, new SPS periodicities should be introduced to align with the TDD structure.

Proposal 2b: If UL SPS transmission can’t be finished within a TDD pattern, the remaining repetitions can be postponed to the NB-IoT UL subframes of the following TDD patterns.

  - Impact on Random Access

Upon completion of the cell search procedure, the UE achieves DL synchronization with the cell. After camping on the cell, the UE must then obtain UL synchronization and be allocated UL resources before it can initiate UL transmissions. UL synchronization is accomplished through a Random Access procedure, as shown in Figure 1.

Figure 1. Random Access procedure initialization

The implementation of the Random Access procedure initialization depends on the frame structure. Alterations of the frame structure can disrupt the predefined UL and DL message transmission timings in TDD. For instance, from the UE’s perspective, Msg1/Msg3, which are supposed to be transmitted in a UL slot, may not be sent if the current slot is a DL slot. Similarly, Msg2/Msg4, which are intended to be received in a DL slot, may not be received if the current slot is a UL slot.

Hence, it is necessary to further analyse the timings of the messages exchanged during the Random Access procedure to determine whether the configuration of Random Access should be adjusted to adapt to the new frame structure.

- In Msg1, the UE sends an RA preamble. The UE needs to obtain the resource configuration for sending the RA preamble based on the PRACH configuration from the SIB1 message, which should be determined by RAN1.

- In Msg 2, the eNB sends an RA response. The RA Response window starts at the subframe that contains the end of the last preamble repetition, plus 3 subframes, plus UE-eNB RTT and has length defined by  ra-ResponseWindowSize for the corresponding coverage enhancement level.

- In Msg 3 the UE sends RRCSetupRequest.

- In Msg 4 the eNB sends a RRCSetup message to the UE.

UE receives MSG2 in RAR window and MSG4 in Contention Resolution window. The sizes of these two windows are configured by NW.

ra-ResponseWindowSize Duration of the RA response window in TS 36.321 [6]. Value in PDCCH periods. Value pp2 corresponds to 2 PDDCH periods, pp3 corresponds to 3 PDCCH periods and so on. For FDD: The value considered by the UE is: ra-ResponseWindowSize = Min (signaled value x PDCCH period, 10.24s). For TDD: The value considered by the UE is: ra-ResponseWindowSize = Min (signaled value x PDCCH period, 20.48s).

mac-ContentionResolutionTimer Timer for contention resolution in TS 36.321 [6]. Value in PDCCH periods. Value pp1 corresponds to 1 PDCCH period, pp2 corresponds to 2 PDCCH periods and so on. mac-ContentionResolutionTimer-r15 is only applicable for EDT. UE performing EDT shall use mac-ContentionResolutionTimer-r15, if present. For FDD: The value considered by the UE is: mac-ContentionResolutionTimer = Min (signaled value x PDCCH period, 10.24s). For TDD: The value considered by the UE is: mac-ContentionResolutionTimer = Min (signaled value x PDCCH period, 20.48s).

Considering the link budget in NB IoT NTN, normally the PDCCH period is long because of the large repetitions to compensate the link budget. And proper NW implementation can guarantee that the Downlink-to-Uplink Guard Period is shorter than the duration of the RAR window and the CR timer. In this case, there is no issue reusing the current design of RAR window and the CR timer. And UE can receive the corresponding response in the next DL slot even when the DL slot is not available in the current cycle

Based on the above reason, we propose

Proposal 3: The legacy design of RAR window and the CR timer can be reused in NB-IoT NTN TDD system with appropriate values configured.

In addition, the absolute time of RAR window should not exceed 10.24s for FDD, and is extended to 20.48s for TDD due to reduced number of DL subframes. As the length of RAR window is exceeding a H-SFN for TDD, the following RA-RNTI formula for TDD operation should be used according to clause 5.1.4 of TS36.321.

RA-RNTI = 1 + floor(SFN_id/4) + 256*(H-SFN mod 2)

Proposal 4: The RAR window configuration and RA-RNTI calculation for NB-IoT TDD in TN should be reused for IoT NTN TDD.

Additionally, for PUR mechanism, UE monitors the response in the PUR response window. The size of the PUR response window is configured by NW.

pur-PeriodicityAndOffset Indicates the periodicity for the PUR occasions and time offset until the first PUR occasion.

pur-ResponseWindowTimer Duration of the PUR response window in TS 36.321 [6]. Value in PDCCH periods. Value pp2 corresponds to 2 PDCCH periods, pp3 corresponds to 3 PDCCH periods, and so on. The value considered by the UE is: pur-ResponseWindowTimer = Min (signaled value x PDCCH period, 10.24s).

PUR periodicity for NB-IoT is not multiples of 90ms, facing the similar situation as RAR window. The PUR response window is configured by pur-ResponseWindowTimer in the unit of PDCCH periods, i.e. {pp2, pp3, pp4, pp5, pp6, pp7, pp8, pp10}.

Proposal 5: The PUR response window configuration should be adapted to the 90ms IoT NTN TDD frame structure.

  - Impacts on SI

In RAN1#120, the impacts on the system information other than SIB1-NB were left to RAN2 according to the following conclusion:

Conclusion It is RAN1’s understanding that the issue of scheduling and transmitting SI messages in the presence of non-D subframes will be solved mainly (or completely) by RAN2 specifications. RAN1 may discuss potential impact on RAN1 specifications, if any.

In our understanding, SI messages (other than SIB1-NB which is separately discussed in RAN1) can only be transmitted on NB-IoT DL subframes, so that no SI messages need to be dropped due to the mapping on the non-DL subframes. This is beneficial for the system performance.

Proposal 6: NPDSCH for SI messages other than SIB1-NB should be mapped on NB-IoT DL subframes.

In legacy NB-IoT system, the SI-window of n th SI message configured in the SchedulingInfoList-NB-r13 of SIB1-NB starts at the subframe#0 in the radio frame satisfies the following equation.

(H-SFN * 1024 + SFN) mod T = FLOOR(x/10) + Offset

T is the periodicity corresponding to the SI message configured by si-Periodicity-r13 with the value range of {64, 128, 256, 512, 1024, 2048, 4096} radio frames. x=n-1∙w is related to the order of SI-message n and the SI-window length w configured by si-WindowLength-r13 with the value range of {160, 320, 480, 640, 960, 1280, 1600}ms. In the SI window, the TB of a SI-message is mapped to 2 or 8 consecutive valid NB-IoT DL subframes (provided by downlinkBitmap-r13) in the 1st radio frame of every {2, 4, 8, 16} radio frames (provided by si-RepetitionPattern) from the start of SI window. If the number of valid NB-IoT DL subframe is not enough, the TB can be extended the following radio frames not exceeding the SI-window length.

It is observed that the starting radio frame of each repetition are derived based on physical radio frame index. When the si-RepetitionPattern is configured with values other than “every16thRF”, the valid NB-IoT DL subframes of consecutive NPDSCH repetitions in a SI window may overlap due to small portion of valid NB-IoT DL subframes per 90ms. In such case, UE should skip the subsequent reception of SI NPDSCH repetitions.

Proposal 7: For the SI reception within an SI-window, UE should prioritize the previous SI reception in case there is collision between the previous SI repetitions and the subsequent SI reception.

  - Impact on Round-Trip Delay

For the cross-linked satellite networks, the offset Kmac is used to delay the application of a DL configuration indicated by a MAC CE received on NPDSCH/PDSCH, and to determine the UE-eNB RTT. It may be provided by the NW when DL and UL frame timing are not aligned at eNB. The Kmac is also used in the random access procedure, to determine the start time of random access response window after a random access preamble transmission.

The Service link RTT, Feeder link RTT, the RP, the Common TA, Kmac and TTA are illustrated in Figure 2.

Figure 2 Illustration of timing relationship[8]

In RAN1#120, RAN2 assumes Kmac has to be extended to address the case where the number of hops exceeds a certain limit and leaves an FFS on whether there are any other implications and whether an LS is needed.

In our opinion, the impact on the  Kmac value depends on whether the NTN system uses the regenerative satellite or the transparent satellite. So it is better to be discuss separately:

- For a regenerative satellite, the eNB is located on the satellite, and the TA RP is also on the satellite, so there seems no need to increase the Kmac.

- For a transparent satellite, the TA RP may not be located on the satellite, as shown in Figure 2. In this case, if the inter-satellite link involves multiple hops (i.e., multiple satellites), the air interface link latency becomes longer.

Figure 3 Round trip path in cross-linked satellite network [7]

As this depends on the scenario, we may need first to clarify the scenario. And another discussion point is whether inter-satellite link is assumed to be applicable to IoT NTN TDD system. Furthermore, this seems not a specific issue to IoT NTN TDD system but common to all NTN systems, so we may need to confirm this.

Proposal 8: RAN2 to further discuss the following points before extending Kmac:

- Whether this Kmac extension only applies to transparent architecture supporting inter-satellite link

- Whether this issue applies to all NTN system

In addition, since the parameter is introduced by RAN1. We need to inform RAN1 on any RAN2 decision.

Proposal 9: Send an LS to RAN1 on whether there is need to extend the Kmac.

- Conclusion

The observations and proposals made in this contribution are summarized below:

Impact on Access Control

Observation 1: Pre-R19 UEs cannot access the network in 1616-1626.5 MHz MSS allocated band so there is no need to enhance the Access Control mechanism for legacy UEs.

Proposal 1: The IE cellBarred in the SIB1-NB message can be reused to control whether a UE supporting NB-IoT NTN TDD mode is allowed to access an NB-IoT TDD NTN cell.

Impact on SPS

Proposal 2a: To reduce the overlap of UL SPS with non-U NB-IoT subframes, new SPS periodicities should be introduced to align with the TDD structure.

Proposal 2b: If UL SPS transmission can’t be finished within a TDD pattern, the remaining repetitions can be postponed to the NB-IoT UL subframes of the following TDD patterns.

Impact on Random Access

Proposal 3: The legacy design of RAR window and the CR timer can be reused in NB-IoT NTN TDD system with appropriate values configured.

Proposal 4: The RAR window configuration and RA-RNTI calculation for NB-IoT TDD in TN should be reused for IoT NTN TDD.

Proposal 5: The PUR response window configuration should be adapted to the 90ms IoT NTN TDD frame structure.

Impacts on SI

Proposal 6: NPDSCH for SI messages other than SIB1-NB should be mapped on NB-IoT DL subframes.

Proposal 7: For the SI reception within an SI-window, UE should prioritize the previous SI reception in case there is collision between the previous SI repetitions and the subsequent SI reception.

Impact on Round-Trip Delay

Proposal 9: RAN2 to further discuss the following points before extending Kmac:

- Whether this Kmac extension only applies to transparent architecture supporting inter-satellite link

- Whether this issue applies to all NTN system

Proposal 10: Send an LS to RAN1 on whether there is need to extend the Kmac.

- References

RP-242415, New WID on introduction of IoT-NTN TDD mode

Chair notes RAN1#119 eom1

RP‑243293, Revised WID on introduction of IoT-NTN TDD mode

R2-2501333, Report from Break-out session on NR-NTN and IoT-NTN

R1-2500086, Discussion on IoT-NTN TDD mode

Chair notes RAN1#120 eom0

R2-2500390, Discussion on IoT-NTN TDD mode

3GPP TS 36.300, E-UTRA and E-UTRAN; Overall description; Stage 2