3GPP TSG-RAN WG2 Meeting #129bis	                           R2-2501983

Wuhan, China, April 7th – April 11th, 2025

Source: 	Huawei, HiSilicon, Novamint, Sateliot, Thales, Inmarsat, Viasat, Turkcell

Title: 	Remaining issues on PWS support for NB-IoT

Agenda Item:	8.9.4

Document for:	Discussion and decision

# Introduction

The WID [1] supports PWS for NB-IOT with the following description:

- Support broadcast of PWS messages for NB-IoT re-using the LTE mechanisms [RAN2] Note: The solutions will be developed for NTN but can be applicable to TN (if possible). Specific NB-IoT TN optimizations will not be considered as part of this WI. Note: Support for indication of intended service area for ETWS will be considered based on NR NTN discussions/outcome.

In the RAN2#128 meeting, the following agreements were achieved:

Agreements:

1.	We will clarify in 36.300 section 4.10 that in general a NB-IoT UE may not support all PWS requirements.

2.	We will extend the existing ETWS/CMAS notification RRC procedures for eMTC to NB-IoT. FFS if SIB1-NB acquisition is needed

3.	Add the following PWS indication in Paging-NB:

- etws-Indication;

- cmas-Indication.

In the RAN2#129 meeting, the following agreements were achieved:

Agreements:

1.	Introduce geographic information in ETWS messages for geo-fencing in IoT NTN cells

2.	Introduce the Warning Area Coordinates IE for ETWS to describe an emergency area in system information.

3.	Introduce the Warning Area Coordinates IE to existing ETWS SIBs, i.e., SIB10-NB/11-NB, without introducing the need for segmentation of SIB10-NB

4.	We don’t introduce a new SIB for carrying geographical information for emergency areas

In this contribution, we provide our consideration on how to ensure PWS message delivery for UEs currently in RRC_CONNECTED.

# Discussion

Previously, RAN2 sent an LS (in R2-2411194) to RAN1 asking whether it is feasible to let UEs receive PWS without leaving RRC_CONNECTED due to the concern that this may require additional UE capability for monitoring a common search space in addition to the UE specific search space.

RAN1 discussed this issue and replied (in R1-2501613) that developing a RAN1 solution for this may require addition TUs from RAN Plenary. Then in the RAN Plenary, no TU was allocated and a NOTE was added to the revised WID (in RP-250472):

Note: PWS notifications in paging, direct indication, and system information in RRC connected mode is not supported.

Based on this, we understand UEs will not monitor an additional search space for PWS reception. So based on the current mechanism, the NW can release UEs to IDLE for PWS reception. However, we see two issues with this approach:

Issue 1: Without knowing the reason of the release, a UE will immediately initiate RRC Connection establishment if it has UL data to send. This will defeat the purpose of letting UE receive PWS in IDLE.

Issue 2: Even if it happens that the UE doesn’t have any UL data, it has to wait for paging occasion (and paging repetitions) to determine that there is a PWS message coming after being released. This delay before reading PWS SIBs, leads to additional latency, as pictured in figure 1.

Figure 1 Latency of acquiring a PWS message

In our understanding, these issues can be easily resolved by informing the UE that the RRC release is for PWS reception in RRC_IDLE. In this case, after being released to RRC_IDLE, the UE will immediately read the PWS SIB and will not initiate RRC establishment until PWS reception is complete. This solution requires minimal spec changes in RAN2 and has no impact in RAN1. Additionally, it aligns with the NOTE in the WID, as it doesn’t require PWS notifications in paging, direct information or system information for RRC connected.

Proposal: To deliver PWS messages to UEs currently in RRC_CONNECTED, the network may release the UEs to RRC_IDLE and instruct the UEs to read the PWS SIB(s) immediately after transitioning to RRC_IDLE.

# Conclusion

This contribution discussed the support of PWS for NB-IoT. Proposals are summarized as follows:

Proposal: To deliver PWS messages to UEs currently in RRC_CONNECTED, the network may release the UEs to RRC_IDLE and instruct the UEs to read the PWS SIB(s) immediately after transitioning to RRC_IDLE.

# References

RP-243278, Revised WID: Non-Terrestrial Networks (NTN) for IoT Phase 3.

3GPP TS 22.268, Public Warning System (PWS) requirements