3GPP TSG-RAN WG2 Meeting #129bis                               R2-2501986

Wuhan, China, 07th – 11th April 2025

Agenda Item:	8.12.3

Source:	Ofinno

Title:	Enhancements for UE-initiated/event-driven beam management

Document for:	Discussion and Decision

# Introduction

The revised WID [1] of NR MIMO evolution Phase 5 was agreed in RAN#105 meeting. The objectives related to UE-initiated/event-driven beam management are listed below:

- Specify enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management

  - UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching

  - UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting

In this contribution, we present our views on the remaining issues of UE-initiated/event-driven beam management.

# Remaining issues on the first PUCCH

In RAN1#119, the following agreement has been reached regarding the container of first PUCCH.

Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding first PUCCH channel configuration, Alt-2 is supported for both mode-A and mode-B: the first PUCCH channel is a new UCI type - Alt-2 (new UCI type): Introduce RRC parameter, e.g., firstPUCCHResourceConfig- UEIBR, for the periodic PUCCH resource configuration. - It is RAN1’s understanding that the RRC parameter is NOT associated with SchedulingRequestId. - 1-bit to PUCCH resource is encoded by reusing the encoding mechanism of positive/negative SR. - The dedicated RRC parameter at least comprises the following: - periodicityAndOffset - PUCCH-ResourceID - Above applies at least for the single CC case. - Reuse multiplexing/dropping rule(s) of SR as baseline - FFS: overlapping with SR/LRR or PUSCH Note: Further details on first PUCCH retransmission (if supported) for mode A and mode B will be continue to be separately discussed in RAN1.

For the first PUCCH transmission in UE-initiated/event-driven beam reporting, it has been agreed in RAN1 #119 to introduce a new UCI type for the periodic PUCCH resource configuration. The new UCI type is different from SR and is not linked to SchedulingRequestId.

In the legacy systems, the active time of a DRX operation includes the time when an SR is sent on PUCCH (see the text copied below from 38.321 Section 5.7). This is to monitor PDCCH for reception of a DCI providing an uplink grant.

## 5.7 Discontinuous Reception (DRX) When DRX is configured, the Active Time for Serving Cells in a DRX group includes the time while: …… - a Scheduling Request is sent on PUCCH and is pending (as described in clause 5.4.4 or 5.22.1.5). If this Serving Cell is part of a non-terrestrial network, the Active Time is started after the Scheduling Request transmission that is performed when the SR_COUNTER is 0 for all the SR configurations with pending SR(s) plus the UE-gNB RTT; or

In Mode A of UE-initiated CSI reporting, the UE transmits a first PUCCH requesting uplink resources to transmit UE-initiated beam report. After the first PUCCH transmission, the UE receives a DCI providing an uplink grant to transmit UE-initiated beam report in PUSCH. Finally, the UE transmits the UE-initiated beam report in the PUSCH. If DRX is configured, to be able to receive the DCI after transmitting the first PUCCH transmission, the UE should monitor PDCCH. To achieve this, the active time of a DRX operation should include the time when a new UCI for UE-initiated beam reporting is sent on PUCCH in Mode A of UE-initiated CSI reporting.

As Mode B of UE-initiated CSI reporting uses Type 1 configured grant PUSCH to transmit UE-initiated beam report (thus no DCI is required to transmit beam report), there is no need to monitor PDCCH after the first PUCCH transmission. So, there is no impact on the active time of a DRX operation in Mode B of UE-initiated CSI reporting.

- In Mode A of UE-initiated CSI reporting, the active time of a DRX operation includes the time when a new UCI for UE-initiated beam reporting is sent on first PUCCH.

# Remaining issues on the second PUSCH

In RAN2#AH1807, the following agreement has been reached regarding CSI reports outside of DRX active time.

RAN2 #AH1807 (Agreement) - confirm that periodic CSI and semi-persistent CSI on PUCCH is only reported when in active time. - confirm the RAN1 agreement that semi-persistent CSI on PUSCH is only reported when in active time and update the MAC specification accordingly. - confirm that aperiodic CSI reporting on PUSCH can be sent outside of active time if the gNB decides to request so.

According to RAN2#AH1807 agreement, a UE does not transmit periodic CSI on PUCCH and semi-persistent CSI on PUSCH/PUCCH outside of DRX active time, but transmits aperiodic CSI outside of DRX active time (see the text copied below from 38.321 Section 5.7).

## 5.7 Discontinuous Reception (DRX) … Regardless of whether the MAC entity is monitoring PDCCH or not on the Serving Cells in a DRX group, the MAC entity transmits HARQ feedback, aperiodic CSI on PUSCH, and aperiodic SRS defined in TS 38.214 [7] on the Serving Cells in the DRX group when such is expected. … When DRX is configured, the MAC entity shall: … 1> if DCP monitoring is configured for the active DL BWP as specified in TS 38.213 [6], clause 10.3; and 1> if the current symbol n occurs within drx-onDurationTimer duration; and 1> if drx-onDurationTimer associated with the current DRX cycle is not started as specified in this clause: 2> if the MAC entity would not be in Active Time considering grants/assignments/DRX Command MAC CE/Long DRX Command MAC CE received and Scheduling Request sent until 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in this clause; and 2> if allowCSI-SRS-Tx-MulticastDRX-Active is not configured, or if cfr-ConfigMulticast is not configured for any of the active BWP(s) of the Serving Cell(s), or if all multicast DRXes would not be in Active Time considering multicast assignments/DRX Command MAC CE for MBS multicast received until 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in Clause 5.7b and all multicast sessions are configured with multicast DRX: 3> not transmit periodic SRS and semi-persistent SRS defined in TS 38.214 [7]; 3> not report semi-persistent CSI configured on PUSCH; 3> not report semi-persistent CSI on PUCCH; 3> if ps-TransmitPeriodicL1-RSRP is not configured with value true: 4> not report periodic CSI that is L1-RSRP on PUCCH. 3> if ps-TransmitOtherPeriodicCSI is not configured with value true: 4> not report periodic CSI that is not L1-RSRP on PUCCH. 1> else: 2> in current symbol n, if a DRX group would not be in Active Time considering grants/assignments scheduled on Serving Cell(s) in this DRX group and DRX Command MAC CE/Long DRX Command MAC CE received and Scheduling Request sent until 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in this clause; and 2> if allowCSI-SRS-Tx-MulticastDRX-Active is not configured, or if cfr-ConfigMulticast is not configured for any of the active BWP(s) of the Serving Cell(s), or, in current symbol n, if all multicast DRXes corresponding to the DRX group would not be in Active Time considering multicast assignments/DRX Command MAC CE for MBS multicast received until 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in Clause 5.7b and all multicast sessions corresponding to the DRX group are configured with multicast DRX: 3> not transmit periodic SRS and semi-persistent SRS defined in TS 38.214 [7] in this DRX group; 3> not report CSI on PUCCH and semi-persistent CSI configured on PUSCH in this DRX group.

- Outside of DRX active time, a UE only transmits aperiodic CSI report, and does not transmit periodic CSI report or semi-persistent CSI report on PUCCH/PUSCH.

Since the UE-initiated CSI report is different from legacy CSI reports (e.g., periodic, semi-persistent, aperiodic CSI), whether to transmit UE-initiated CSI report outside of DRX active time needs to be studied.

- To discuss whether UE transmits UE-initiated CSI report outside of DRX active time.

In RAN2#109-e, the following agreement has been reached regarding CSI reports for a dormant cell.

RAN2#109e (Agreement) - Performing periodic or semi-persistent CSI measurements on dormancy SCell with the corresponding report transmitted on other cell (i.e., sPCell or non-dormancy SCell) is supported. Aperiodic CSI reporting (no matter it is triggered via self-carrier scheduling or cross-carrier scheduling, no matter it is transmitted on dormant SCell or on other non-dormant SCell) is not supported.

According to RAN2#109-e agreement, a UE transmits both periodic CSI report and semi-persistent CSI report for a dormant BWP. However, the UE does not transmit an aperiodic CSI report for the dormant BWP (see the text copied below from 38.321 Section 5.15.1).

## 5.15 Bandwidth Part (BWP) operation ### 5.15.1 Downlink and Uplink … For each activated Serving Cell configured with a BWP, the MAC entity shall: … 1> if a BWP is activated and the active DL BWP for the Serving Cell is dormant BWP: 2> stop the bwp-InactivityTimer of this Serving Cell, if running. 2> not monitor the PDCCH on the BWP; 2> not monitor the PDCCH for the BWP; 2> not receive DL-SCH on the BWP; 2> not report CSI on the BWP, report CSI except aperiodic CSI for the BWP; 2> not transmit SRS on the BWP; 2> not transmit on UL-SCH on the BWP; 2> not transmit on RACH on the BWP; 2> not transmit PUCCH on the BWP; 2> clear any configured downlink assignment and any configured uplink grant Type 2 associated with the SCell respectively; 2> suspend any configured uplink grant Type 1 associated with the SCell; 2> if configured, perform beam failure detection and beam failure recovery for the SCell if beam failure is detected;

- For a dormant BWP, a UE transmits both periodic CSI report and semi-persistent CSI report but does not transmit aperiodic CSI report.

Since the UE-initiated CSI report is different from legacy CSI reports (e.g., periodic, semi-persistent, aperiodic CSI), whether to transmit UE-initiated CSI report for a dormant BWP needs to be discussed.

- To discuss whether UE transmits UE-initiated CSI report for a dormant BWP.

In the legacy skipping mechanism with CSI reporting, if an aperiodic CSI report to be transmitted on PUSCH, the UL grant cannot be skipped—even when no data is available in the MAC layer. The reason is that CSI multiplexing at the PHY requires the generation of a MAC PDU (see the text copied below from 38.321 Section 5.4.3.1.3).

### 5.4.3 Multiplexing and assembly ##### 5.4.3.1.3 Allocation of resources … The MAC entity shall: 1> if the MAC entity is configured with enhancedSkipUplinkTxDynamic with value true and the grant indicated to the HARQ entity was addressed to a C-RNTI, or if the MAC entity is configured with enhancedSkipUplinkTxConfigured with value true and the grant indicated to the HARQ entity is a configured uplink grant: 2> if there is no UCI to be multiplexed on this PUSCH transmission as specified in TS 38.213 [6]; and 2> if there is no aperiodic CSI requested for this PUSCH transmission as specified in TS 38.212 [9]; and 2> if the MAC PDU includes zero MAC SDUs; and 2> if the MAC PDU includes only the periodic BSR and there is no data available for any LCG, or the MAC PDU includes only the padding BSR: 3> not generate a MAC PDU for the HARQ entity. 1> else if the MAC entity is configured with skipUplinkTxDynamic with value true and the grant indicated to the HARQ entity was addressed to a C-RNTI, or the grant indicated to the HARQ entity is a configured uplink grant: 2> if there is no aperiodic CSI requested for this PUSCH transmission as specified in TS 38.212 [9]; and 2> if the MAC PDU includes zero MAC SDUs; and 2> if the MAC PDU includes only the periodic BSR and there is no data available for any LCG, or the MAC PDU includes only the padding BSR: 3> not generate a MAC PDU for the HARQ entity.

Since the UE-initiated CSI report is different from legacy CSI reports (e.g., periodic, semi-persistent, aperiodic CSI), whether the UL grant is skipped or not for UE-initiated CSI report needs to be studied.

- To discuss whether UL grant is skipped when there is no UE-initiated CSI report to be transmitted.

# RRC configurations

In RAN1#118, the following agreement has been reached regarding the CC to report the beam report in Mode-B. In particular, the UE-initiated beam report of a CSI report configuration is transmitted on the CC where the CSI report configuration is configured.

Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Event-2, for at least Mode-B, the beam report should be carried in the second UL channel in the CC where the corresponding CSI report configuration is configured. - Above applies to both cross-CC and same-CC beam report. - Note: Above is applied to the case that second UL channel is PUSCH. - FFS: Whether the first and second channels can be from the same/different CC.

In the running CR, the parameter resourceForSecondChannelOfModeB-r19 comprises servCellIndex parameter indicating the CC where the configured grant configuration for the second PUSCH carrying the beam report is located. However, the servCellIndex parameter is not needed, as per RAN1#118 agreement, the CC where the configured grant configuration is located is already implicitly determined based on the CC where the CSI report configuration is configured. The same discussion also holds for the ul-BWP-Id-r19.

- Remove servCellIndex-r19 and ul-BWP-Id-r19 from resourceForSecondChannelOfModeB-r19.

# Conclusion

In this contribution, we make the following proposals.

- In Mode A of UE-initiated CSI reporting, the active time of a DRX operation includes the time when a new UCI for UE-initiated beam reporting is sent on first PUCCH.

- Outside of DRX active time, a UE only transmits aperiodic CSI report, and does not transmit periodic CSI report or semi-persistent CSI report on PUCCH/PUSCH.

- To discuss whether UE transmits UE-initiated CSI report outside of DRX active time.

- For a dormant BWP, a UE transmits both periodic CSI report and semi-persistent CSI report but does not transmit aperiodic CSI report.

- To discuss whether UE transmits UE-initiated CSI report for a dormant BWP.

- To discuss whether UL grant is skipped when there is no UE-initiated CSI report to be transmitted.

- Remove servCellIndex-r19 and ul-BWP-Id-r19 from resourceForSecondChannelOfModeB-r19.

# References

- RP-242394, WID revision: NR MIMO Phase 5, Samsung, RAN#105, Sept. 2024