3GPP TSG-RAN WG2 #129bis			R2-2501987

Wuhan, China, April. 7th – 11st, 2025

Agenda item:	8.2.3

Title:	Ambient-IoT Random Access

Source:	Ofinno

Document for:	Discussion and Decision

# Introduction

In Rel-19 a study item on Ambient IoT (A-IoT) was concluded with the results captured in [1]. At RAN #106 a new work item was approved [2]. The RAN2 scop in the WID is:

- RAN2 scope:

  - Specify the necessary functions and procedures for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission:

    - A-IoT Paging, including subsequent paging for the same service. Support the options that a paging message contains one identifier, and that a paging message contains no identifier. Temporary identifier is not supported, unless required by SA WGs.

Note: RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes.

    - A-IoT Random access, including re-access for failure handling. Contention-based and contention-free cases are supported. For the contention-based random access, only Solution 1 (3-step only) is included (unless RAN2 decides to use Solution 3 (unified solution) by RAN2#129).

    - A-IoT data transmission, including data (re-)transmission for failure handling. Segmentation is supported at least in D2R.

    - Only MAC layer is included

An email discussion [POST129][036][AIoT] discusses options for the AS ID to be used for CFRA and CBRA. This contribution provides our views on other topics also related to the A-IoT random access: transaction ID associated to a A-IoT service, resource information to be used by Msg1, monitoring operation of the re-transmitted Msg2 and Msg1/2 association.

# Discussion

## Transaction ID (associated to an A-IoT Service)

In RAN2, the following agreements were made in relation to the A-IoT transactions/services:

- Agreements on paging (RAN2 #128) - An ID in paging message is beneficial to avoid duplicated response for same service request. FFS whether ID is generated by the reader or CN. FFS on the ID size. - 2 Capture the scenario that different readers may page the same device for same service request. We will discuss this in WI phase (if in scope of WI) and consider progress from all the WG. - Agreements (RAN2 #129) - Parallel service requests by the same reader is not supported. - The device is expected to only perform one procedure at a time. FFS device behaviour if multiple requests are received in parallel (if needed). - The “transaction ID” can be generated by reader based on CN corelation ID. FFS how reader will generate “transaction ID”. FFS the size of transaction ID - 1 bit solution is excluded. FFS the size. Aim to have a reasonable size. - RAN2 acknowledges that multi-reader scenario may exist but we will not specify something specific for this purpose. We can rely on transaction ID and implementation to handle it. - Agreements (RAN2 #129) - For Rel-19, only 3-step CBRA is supported for A-IoT - We will specify both CBRA and CFRA. - Re-use the subsequent paging message to trigger re-access. There is no need to differentiate msg1 resource for initial access vs re-access.

The reader can perform inventory to one or multiple A-IoT device(s) that are associated with the same A-IoT service (e.g., A-IoT inventory service or A-IoT command service) by sending one or more paging message(s). In our understanding, it is up to the reader’s implementation to decide how many paging messages the reader will send to perform inventory of all the A-IoT devices associated to one specific A-IoT service. Reader’s decision may vary depending on multiple factors for example, depending on the availability of the radio resource used for paging and random access, as well as, a success or failure rate of inventorying the A-IoT devices, etc.

The paging message could be followed by one or more subsequent R2D messages (e.g., subsequent paging messages) associated with the same A-IoT service/transaction. It may be unclear how this association (i.e., messages that belong to the same A-IoT service/transaction) could be distinguished. For this, the transaction ID in the paging message could be used to identify each specific A-IoT procedure. Moreover, this transaction ID could also be helpful for scenarios in which different readers send paging messages to the same UE (or group of UEs) associated to different A-IoT services. Summarizing, the paging message and one or more subsequent R2D messages (e.g., subsequent paging messages) that are associated with the same A-IoT service should include the same transaction ID.

- The paging message and its one or more subsequent R2D messages (e.g., subsequent paging messages) that are associated with the same A-IoT service (or transaction) should include the same transaction ID.

## Resource Information for Msg1

In RAN1 #120 and RAN2 #129, the following agreements were made in relation to Msg1 transmission and resources:

Agreement (RAN1 #120) Support FDMA with Y≥1 D2R transmissions for Msg.1 from multiple devices in response to a R2D transmission triggering random access. - The maximum value of Y >1 is determined in AI 9.4.2, based on feasible values of small frequency shift to be decided in AI 9.4.2. - Signalling details should be discussed and determined in AI 9.4.4. - Agreements (RAN2 #129) - The A-IoT paging message can include a number of msg1 resources - From RAN2 perspective, after initial paging message, the R2D transmission which determines the Msg1 resource(s), can be achieved by one of the below two ways, unless RAN1 concludes to use L1 signaling later: - Way-1: introducing new R2D message other than the paging message, e.g., QueryRep-like; or - Way-2: reusing the same paging message, using field(s) to indicate it is only to determine the Msg1 resource(s) and omitting the paging identifier (device ID/group ID) field - The service type of A-IoT (e.g., inventory only, inventory + command) is not included in paging message. - Agreements (RAN2 #129) - For Rel-19, only 3-step CBRA is supported for A-IoT - We will specify both CBRA and CFRA. - Re-use the subsequent paging message to trigger re-access. There is no need to differentiate msg1 resource for initial access vs re-access.

The slotted-ALOHA is the baseline for A-IoT random access procedure [1]. In the A-IoT random access, radio resources available for A-IoT Msg1 are divided into one or more time-frequency resources, and an A-IoT device randomly selects one of the time-frequency resources to transmit an A-IoT Msg1. A time-frequency resource for A-IoT device(s) to perform access (e.g., transmitting the A-IoT Msg1 by the device) is defined as an access occasion (i.e., A-IoT Msg1 resources), and a set of access occasion(s) for different A-IoT device(s) is scheduled via an R2D message [1]. For example, the R2D message includes a paging message and/or a separate R2D trigger message for the A-IoT random access procedure.

When the R2D message triggers A-IoT devices to respond during the A-IoT random access procedure, an A-IoT device selects (e.g., randomly) one access occasion from the set of access occasion(s) scheduled via the R2D message. By allocating multiple access occasions, the reader can distribute A-IoT devices across these occasions to reduce the likelihood of collisions. The total number of access occasions is a key parameter for A-IoT devices to properly participate in slotted-ALOHA based A-IoT random access procedure. To be able to perform the selection, the A-IoT device must know the total number of access occasions scheduled by the R2D message.

There are implicit or explicit ways to indicate the total number of access occasions (i.e., A-IoT Msg1 resources). For example, the total number of access occasions is determined by Q = X×Y, where X and Y indicate the total number of TDMed access occasions (e.g., A-IoT Msg1 time resources) and the total number of FDMed access occasions (e.g., A-IoT Msg1 frequency resources), respectively. The R2D message (e.g., A-IoT paging message) can include one or more fields indicating at least one value of Q, X, or Y. Any value of Q, X, or Y can be also predefined.

In RAN2#129, RAN2 agreed that RAN2 re-uses the subsequent paging message to trigger re-access and there is no need to differentiate Msg1 resource for initial access vs re-access. Thus, the subsequent A-IoT paging message to trigger re-access can also include one or more fields indicating at least one value of Qre-access, Xre-access, or Yre-access, where Qre-access, Xre-access, and Yre-access indicate total number of access occasions (i.e., A-IoT Msg1 resources) for re-access, the total number of TDMed access occasions (e.g., A-IoT Msg1 time resources) for re-access, and the total number of FDMed access occasions (e.g., A-IoT Msg1 frequency resources) for re-access, respectively. This subsequent A-IoT paging message may adjust/update (Q, X, or Y) to (Qre-access, Xre-access, or Yre-access) based on a number of detected collisions, a number of identified/unidentified A-IoT devices, etc.

- For the R2D message (for initial and/or subsequent A-IoT paging message) triggering a random access, to discuss if the agreed A-IoT Msg1 resource information is defined in terms of option (a) the total number of access occasions, option (b) the total number of TDMed access occasions, and/or option (c) the total number of FDMed access occasions .

## Monitoring A-IoT Msg2 (considering re-transmission)

In RAN1 #119 and RAN2 #128, the following agreements were made:

Agreement (RAN1#119) For Msg2 transmission in response to multiple Msg1 transmissions, which is initiated by a R2D transmission triggering random access, RAN1 identifies at least the following options to determine the time duration for Msg2 monitoring - Option 1: Predefined - Option 2: Indicated in the R2D transmission triggering random access - Note: Option 1 and Option 2 may not be mutually exclusive. Agreements (RAN2 #128) - For 2 step and 3step CBRA, capture the unified solution in the TR and capture advantage and disadvantage of the solutions. This requires that devices would support both forms. - FFS recommendation on whether we have 3step only or we have unified solution for 2step and 3step CBRA. - Capture in TR as one option for unification for CBRA and CFRA. For this option random ID is always included in msg1. Capture disadvantage of reader doesn’t know whether the intended device is responding on the given resources. - RAN2 assumes, A-IoT reader can send a Msg2 transmission to a specific device for the random ID, if it doesn’t receive msg3 from that ID - If msg2 contains multiple devices, re-transmitted Msg2 will only include the random IDs of failed devices (if supported). - Capture the following option in TR: The devices can be triggered by R2D message to re-access before subsequent paging retransmissions

The reader can re-transmit the A-IoT Msg2 to specific device(s) in case of failure reception of the A-IoT Msg3. There are two cases when the reader doesn’t receive the A-IoT Msg3 and re-transmits the A-IoT Msg2.

- When the A-IoT device transmits the A-IoT Msg3, but the reader fails to receive the A-IoT Msg3.

- When the A-IoT device fails to receive the A-IoT Msg2 and doesn’t transmit the A-IoT Msg3.

The reader may not distinguish those two cases and transmits the re-transmitted A-IoT Msg2 either way (i.e., in both two cases). According to the RAN1 agreements during SI phase, RAN1 may consider a time duration (e.g., random access response window in NR) for A-IoT Msg2 monitoring. If the time duration for A-IoT Msg2 monitoring is considered by RAN1, RAN2 needs to discuss how the A-IoT device monitors for the re-transmitted A-IoT Msg2. Otherwise, the A-IoT device may not monitor the R2D channel and miss the re-transmitted A-IoT Msg2.

- If the time duration for A-IoT Msg2 monitoring is agreed by RAN1, RAN2 needs to also discuss how the A-IoT device monitors the re-transmitted A-IoT Msg2 (in addition of the initial A-IoT Msg2).

## Msg1/2 association

RAN1 have made the following agreements:

Agreement (RAN1 #118b) RAN1 studies the starting time and time duration for Msg2 monitoring for Msg2 reception. Agreement (RAN1 #119) For Msg2 transmission in response to multiple Msg1 transmissions, which is initiated by a R2D transmission triggering random access, RAN1 identifies the following for the starting time for Msg2 monitoring - For Option 1, where a PRDCH for Msg2 transmission corresponds to an A-IoT Msg1 received from one device, - Option a: the starting time for Msg2 monitoring is separate for each Msg1 resource. - Option b: the starting time for Msg2 monitoring is common for multiple Msg1 resources. - For Option 2, where a PRDCH for Msg2 transmission corresponds to multiple A-IoT Msg1 received from different devices, the starting time for Msg2 monitoring is common for multiple Msg1 resources. Agreement (RAN1 #119) For Msg2 transmission in response to multiple Msg1 transmissions, which is initiated by a R2D transmission triggering random access, RAN1 identifies at least the following options to determine the time duration for Msg2 monitoring - Option 1: Predefined - Option 2: Indicated in the R2D transmission triggering random access Note: Option 1 and Option 2 may not be mutually exclusive. Agreement (RAN1#120) Support following two options for Msg2 transmission in response to multiple Msg1 transmissions, which are initiated by a R2D transmission triggering random access. - Option 1: A PRDCH for Msg2 transmission corresponds to a A-IoT Msg1 received from one device - Option 2: A PRDCH for Msg2 transmission corresponds to multiple A-IoT Msg1 received from different devices - FFS the maximum number of Msg1 responses that can be carried within a PRDCH for Msg2.

A-IoT Msg1 includes at least one 16-bit random ID generated by the A-IoT device. When the A-IoT device identifies the start of its own access occasion, it transmits the random ID via the A-IoT Msg1 to the reader [1]. The reader responds to A-IoT Msg1 with the successfully received random ID in A-IoT Msg2. If the A-IoT device receives the A-IoT Msg2 including a random ID, which is the same as the previously transmitted one in A-IoT Msg1, the A-IoT device considers the contention resolution as successful. The A-IoT Msg2 also contains parameters related to A-IoT Msg3 transmission.

RAN2 should consider the impact of the progress of RAN1 related to the agreements above. For example, an A-IoT device receives an A-IoT Msg2 (including a response to an A-IoT Msg1 transmission) which occurs after one or more other A-IoT Msg1 transmissions by other A-IoT devices. This case can occur when multiple access occasions (TDMed and/or FDMed) for multiple A-IoT Msg1 transmissions are allocated before the A-IoT device receives the A-IoT Msg2. In this case, echoing the random ID of the A-IoT device in A-IoT Msg2 is not enough to achieve the contention resolution since another A-IoT device could transmit the same random ID via a different access occasion before the A-IoT device receives the A-IoT Msg2. Furthermore, the number of A-IoT Msg2s (e.g., random IDs) included/multiplexed in the A-IoT Msg2 transmission may not be the same as (e.g., be less than) the total number of access occasions for A-IoT Msg1 transmissions, e.g., due to number of access occasions being larger than the number of paged A-IoT devices, due to A-IoT Msg1 transmission failure, or due to contention resolution failure, etc. As a result, if multiple A-IoT Msg2s (e.g., random IDs) are included/multiplexed in the A-IoT Msg2 transmission, an A-IoT devices cannot determine which A-IoT Msg2 among the multiple A-IoT Msg2s is corresponding to its Msg1 transmission.

Similar issue can happen for the case of re-transmitted A-IoT Msg2. According to RAN2 agreement during SI phase, re-transmitted A-IoT Msg2 can only include the random IDs of failed devices. If the re-transmitted A-IoT Msg2 includes multiple random IDs, echoing the random ID of the A-IoT device in A-IoT Msg2 is also not enough to achieve the contention resolution, regardless of whether multiple access occasions are TDMed or FDMed.

An association or mapping between an access occasion for A-IoT Msg1 and a corresponding A-IoT Msg2 can resolve this issue. We can consider two options as below:

- Option 1: the A-IoT Msg2 transmission includes a bitmap indicating the association or mapping between an access occasion for A-IoT Msg1 and a corresponding A-IoT Msg2. For example, each bit of the bitmap is associated with a respective time-frequency resource of A-IoT Msg1. If a value of a bit field in the bitmap is set to a first value (e.g., a value ‘0’), an A-IoT Msg2 (e.g., random ID) corresponding to an A-IoT Msg1 transmitted via an access occasion associated with the bit field is not present in the A-IoT Msg2 transmission; Otherwise (e.g., the value of the bit field is set to a second value, e.g., a value ‘1’), the A-IoT Msg2 (e.g., random ID) corresponding to the A-IoT Msg1 is present in the A-IoT Msg2 transmission. Thus, the A-IoT device can identify, based on the value of the bit field, whether the A-IoT Msg2 (e.g., random ID) corresponding to its A-IoT Msg1 is present or not and, if present, which A-IoT Msg2 among multiple A-IoT Msg2s in the A-IoT Msg2 transmission is for its A-IoT Msg1.

- Option 2: the A-IoT Msg2 transmission includes an indication (e.g., index) of time-frequency resource of an access occasion for the A-IoT Msg1. For example, an A-IoT Msg2 corresponding to the A-IoT Msg1 transmitted via a time-frequency resource includes an indication of the time-frequency resource (e.g., time-frequency resource index), a time resource indication of the time-frequency resource (e.g., time resource index), or a frequency resource indication of the time-frequency resource (e.g., frequency resource index). Indexing the time-frequency resources may be predefined. Based on the indication, the A-IoT device can identify which A-IoT Msg2 among multiple A-IoT Msg2s in the A-IoT Msg2 transmission is for its A-IoT Msg1. We can also consider the A-IoT Msg2 transmission multiplexing A-IoT Msg2s corresponding to A-IoT Msg1 transmissions via the same time resource with different frequency resources. In this case, a time resource indication (e.g., time resource index) is common for all A-IoT Msg2s and can be present one time in the A-IoT Msg2 transmission.

Thus, we have the following proposal:

- To discuss the association or mapping between an access occasion for A-IoT Msg1 and the corresponding A-IoT Msg2. For this, to discuss if A-IoT Msg2 includes option (1) bitmap indication, or option (2) an indication (or index) time-frequency resource of an access occasion for the A-IoT Msg1.

# Conclusion

Proposal 1. The paging message and its one or more subsequent R2D messages (e.g., subsequent paging messages) that are associated with the same A-IoT service should include the same transaction ID.

Proposal 2. For the R2D message (for initial and/or subsequent A-IoT paging message) triggering a subsequent random access, to discuss if the agreed A-IoT Msg1 resource information is defined in terms of option (a) the total number of access occasions, option (b) the total number of TDMed access occasions, and/or option (c) the total number of FDMed access occasions.

Proposal 3. If the time duration for A-IoT Msg2 monitoring is agreed by RAN1, RAN2 needs to also discuss how the A-IoT device monitors the re-transmitted A-IoT Msg2 (in addition of the initial A-IoT Msg2).

Proposal 4 To discuss the association or mapping between an access occasion for A-IoT Msg1 and the corresponding A-IoT Msg2. For this, to discuss if A-IoT Msg2 includes option (1) bitmap indication, or option (2) an indication (or index) time-frequency resource of an access occasion for the A-IoT Msg1.

# References

[1] 3GPP TR 38.769 V2.0.1.

[2] RP-243326, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”, RAN#106.