3GPP TSG RAN WG2 Meeting #129-bis	R2-2501989

Wuhan, China, February 17 - 21, 2025

Title:	Discussion on IoT-NTN TDD mode

Source: 	Iridium, CCL

Document for:	Discussion and Decision

Agenda Item:	8.17

# Introduction

The current NB-IoT NTN network can only operate in FDD mode. While the NB-IoT specifications include a TDD mode, this mode requires changes in direction coordinated around the timing of individual sub-frames (1 msec).   An NTN system has significant propagation time between the UE and SV that makes the TDD mode in its current form not viable for NTN systems.

It is beneficial for the NB-IoT NTN ecosystem to support the TDD mode of operation, allowing carriers with unpaired spectrum to provide NB-IoT NTN services. To enable this, the IoT-NTN TDD mode work item was approved in RAN#105 [3] and later revised in RAN#106 [1] following the conclusion of the RAN1 study phase. The revised WID outlines the following objectives for RAN2:

- Specify a new NB-IoT TDD NTN mode based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures, including: - Definition, configuration (if needed) and signaling (if needed) of the periodic pattern, necessary adaptation (if needed) and associated UE procedures [RAN1, RAN2] - Support a pattern with a period of 9 radio frames for the target MSS allocated band, where D=U=8 with a fixed guard period. RAN1 to consider whether there is a need for a mechanism to achieve an adjustable guard period for the purpose of allowing deployment with the TDD frame structure of the legacy system operating in the target MSS allocated band. Other necessary impacts on higher layers [RAN2]

The RAN1 study phase agreements can be found in the WI status report [2] and RAN1#120 agreements can be found in the feature lead summary [4].

In RAN2#129 following agreements were made [5]:

- RAN2 invites the WI rapporteur to submit a TP for Stage2 to RAN1 so that they can endorse it and send it back to RAN2 for final approval - RAN2 will continue studying paging aspects based, on RAN1 progress - RAN2 confirms that idle mode eDRX is supported in IoT-NTN TDD network. - RAN2 assumes Kmac has to be extended (or a new parameter with higher range introduced) to address the case where the number of hops exceeds a certain limit. We continue the discussion in the next meeting to investigate if there are any other implications and in case of any decisions we send an LS to other groups if needed. - RAN2 thinks that a change of H-SFN duration (Option 1-1) and/or H-SFN total number (option 2-2) will impact RAN2 and SA2 specification regarding the support of idle mode eDRX in IoT-NTN TDD network and the impact should be evaluated. - RAN2 assumes that legacy coverage enhancement techniques (i.e. transmission with repetitions) are supported in IoT-NTN TDD system. - RAN2 can continue the discussion also on RAR window - Legacy barring bit will be used (FFS is cellBarred or cellBarred-NTN)

For NB-IoT frame structure type-1 to be utilized on the same frequency in both uplink and downlink, separate spectrum time is allocated for each, separated by a time gap. The period during which uplink operations are allowed is referred to here as the active uplink with duration of U subframes, and the period during which downlink operations are allowed is referred to as the active downlink with duration of D subframes in Figure 1. In RAN#106 it was agreed that the periodicity of the active uplink and downlink times are fixed for NGSO satellite system operating in L-band (1616 – 1626.5 MHz) at 90ms with N=9. There is no RF transmission or reception during inactive times, resulting in discontinuous downlink transmissions. It is also agreed that duration of uplink and downlink active time i.e. D=U=8ms are fixed for MSS spectrum spanning 1616.0 to 1626.5 MHz.

Figure 1 IoT-NTN TDD mode with HD-FDD frame structure

In RAN1#120, it is agreed that any subframes within inactive downlink and uplink times are not considered valid NB-IoT downlink or uplink subframes, respectively.

The “non-D NB-IoT subframes” are subframes not contained within the set of D=8 usable consecutive downlink subframes in the IoT-NTN TDD structure. Similarly, the “non-U NB-IoT subframes” are subframes not contained within the set of U=8 usable consecutive uplink subframes in the IoT-NTN TDD frame structure.

The system parameters to consider in the normative phase are the following:

| Parameters | Values/Comments |
| --- | --- |
| Constellation | LEO at 600 and 1200 km |
| NTN Reference Scenario | Set-1 (3GPP TR 36.763) |
| Targeted MSS band | 1616-1626.5 MHz |
| NB-IoT mode | Standalone with anchor and non-anchor carriers |
| Cell type | Earth-fixed and Earth-moving cells |
| Baseline System for Structure and Procedures | NB-IoT NTN FDD |
| NTN-TDD pattern | Periodic pattern of N=9 radio frame constituted of U=8 non-overlapping set of usable contiguous UL subframe and D=8 set of usable contiguous DL subframe separated by fixed guard period as per Figure 1. |

In this contribution, we present the definition of IoT-NTN TDD mode as discussed in RAN1, and the necessary adaptions and associated UE procedures.

# Discussion

## EUTRA Overall Description

With the introduction of IoT-NTN TDD mode, 36.300 needs to be updated to include IoT-NTN TDD mode definitions and the associated changes as per the proposals below.

Proposal 1: A description of IoT-NTN TDD mode shall be added in 36.300 [§5] as follows:

Downlink and uplink transmissions are organized into radio frames with 10 ms duration. Three radio frame structures are supported:

- Type 1, applicable to FDD and IoT-NTN TDD;

- Type 2, applicable to TDD;

…

For IoT-NTN TDD mode, Frame Structure Type-1 is used where uplink and downlink transmissions are separated in the time domain and constitute of set of D non-overlapping usable contiguous DL subframes and set of U usable contiguous UL subframes separated by fixed guard period (GP) as per Figure 1. This pattern is repeated every N radio frames. The values of N, D, U, [TBD for GP and start subframe for D] are fixed per IoT-NTN TDD band specified in [TBD]

Proposal 2: Anchor carrier definitions in 36.300 §3 needs updating:

Anchor carrier: in NB-IoT, a carrier where the UE assumes that NPSS/NSSS/NPBCH/SIB-NB for FDD and IoT-NTN TDD or NPSS/NSSS/NPBCH for TDD are transmitted.

Proposal 3: In 36.300 §23.21 should be updated to reflect that NTN is applicable to both FDD and IoT-NTN TDD system:

In this release of the specification, NTN is only applicable to FDD and IoT-NTN TDD system.

Note: As per RAN2 VC instructions after the RAN2 #129 [5], the rapporteur of IOT NTN TDD WI has submitted the above three proposals for consideration into RAN1 120-bis, and these three proposals brought up here again are simply for completeness.

## Scheduling

In this section, we present our views on various scheduling aspects covering MIB-NB, SIB1-NB, SI, NPDCCH, NPDSCH, NPUSCH and NPRACH with IoT-NTN TDD frame pattern.

#### SIB1-NB

In NB-IoT FDD mode, SIB1 uses a fixed schedule with a periodicity of 2560ms. It is transmitted on subframe 4 of every other frame and 8 subframes make up one transport block. The starting frame for the first transmission of the SIB1 is derived from the cell PCID.

The transport block size and the repetition rate of SIB1 is configurable, ranging from 208 to 680 bits, and 4, 8 or 16 block repetitions within a 2560ms period.

The IoT-NTN TDD frame pattern limits the number of SIB1 subframes a UE can see within a SIB1 periodicity window of 2560ms. Table 1 shows the maximum number of SIB1 subframes that are visible and can be soft combined within a SIB1 periodicity window of 2560ms.

| SIB1 Repetitions | Maximum number of SIB1 subframes |
| --- | --- |
| 4 | 3 |
| 8 | 7 |
| 16 | 15 |

Table 1 Maximum number of SIB1 subframes visible within SIB1 period of 2560ms in IoT-NTN TDD mode with N=9 and D=8

The link-level simulation results for SIB1-NB decoding performance were provided in our contribution paper [4], which was discussed at RAN1#120. The observations and proposals are captured here.

Observation 1: When operating in IoT-NTN TDD mode, the SIB1 repetition of 16 radio frames works even with the maximum allowed transport block size of 680 bits with adequate link budget margin of 3.91 dB and 4.51 dB for LEO-600 and LEO-1200 respectively.

Observation 2: When operating in IoT-NTN TDD mode, SIB1 repetition pattern should be configured as 16 radio frames to allow every alternate active downlink opportunity to carry SIB1 subframes for optimal SIB1 decoding performance and acquisition time.

Observation 3: When operating in IoT-NTN TDD mode, with SIB1 repetition pattern of every 16 RFs, maximum delay in receiving SIB1 is 2520ms.

Proposal 4: When operating in IoT-NTN TDD mode, there is no need to update the SIB1 scheduling mechanism in the existing specifications, except for disabling transmissions during downlink inactive time.

#### System Information (SI) Scheduling

In RAN1#120 it is concluded that, SI scheduling aspect is decided by RAN2.

As per RAN1#120: Conclusion It is RAN1’s understanding that the issue of scheduling and transmitting SI messages in the presence of non-D subframes will be solved mainly (or completely) by RAN2 specifications. RAN1 may discuss potential impact on RAN1 specifications, if any. Agreement The non-D NB-IoT subframes are not considered by the UE as “NB-IoT DL subframes” from the specifications - NPDCCH / NPDSCH (not containing SI) are mapped as per current specifications.

The SI messages are transmitted within periodically occurring time domain windows called SI-windows using scheduling information provided in SIB1. Each SI message is associated with a SI-window and the SI-windows of different SI messages do not overlap.

Within the SI-window, the corresponding SI message can be transmitted several times over 2 or 8 consecutive NB-IoT downlink subframes depending on TBS. Any TBS higher than 120 bits uses 8 consecutive NB-IoT downlink subframes.

Every SI window starts at subframe#0 which gets postponed to subframe#1 as subframe#0 is reserved for NPBCH.

With IoT-NTN TDD mode frame pattern, it is possible that the SI window start time falls in the inactive downlink time. This can be resolved by postponing the SI window to the next active downlink opportunity.

Observation 4: With the IoT-NTN TDD mode frame pattern, it is possible for the SI window start time to fall within the inactive downlink period.

Proposal 5: In IoT-NTN TDD mode, if start of the SI window falls within the downlink inactive time, it shall be postponed to the next downlink active time.

Observation 5: With IoT-NTN TDD mode frame pattern, it is not possible to transmit all 8 subframes of SI transport blocks within a single downlink active time opportunity when SI TBS is greater than 120 bits.

Proposal 6: In IoT-NTN TDD mode, subframes from a single repetition of the SI transmission shall be allowed to be postponed to the next downlink active time opportunity.

##### Overlap Within SI Window

SI transmission postponement to the next active downlink opportunity could result in overlap with the next repetitions within the SI window. This can be resolved by truncating SI transport block transmission if it overlaps with the next SI repetition. As illustrated in Figure 2, transmission of first repetition gets truncated at SFN 8 where start of second repetition is scheduled. Thus, next downlink active time carries subframes from the second repetition instead of carrying the postponed last three subframes of the first repetition.

Figure 2 Truncation of SI repetition with repetition pattern every 8th radio frame

In case of overlap, truncating the overlapped part of the first transmission is preferred over allowing the first transmission and disabling the initial overlapped part of the second transmission. This approach simplifies UE implementation, as it can schedule receiving the next SI message from the current time without needing to check past SI transmissions that may have been postponed.

Observation 6: With the IoT-NTN TDD mode frame pattern, postponement of the transmission of SI transport block to the next downlink active time can result in overlap with the transmission of the next SI repetition when N=9 and SI repetition pattern is configured as lower than every 16th radio frame.

Observation 7: With the IoT-NTN TDD mode frame pattern, postponement of the transmission of SI transport block to the next downlink active time results in no overlap when N=9 and the SI repetition pattern is configured as every 16th radio frame.

Proposal 7: In IoT-NTN TDD mode, SI transmissions shall be truncated if, due to the proposed postponement, they overlap with the transmission of the next SI repetition within the SI window.

##### SI Window Overflow

The SI transmission postponement to the next downlink active time could also result in SI transmission in the subframes outside of the configured SI window. This could be resolved by proposing changes in (36.331, 18.1.0) to truncate any SI transmissions outside of the SI transmission window as shown in Figure 3

Figure 3 SI window overflow shall not be allowed

Observation 8: With IoT-NTN TDD mode frame pattern, the postponement of transmission of SI transport block to the next downlink active time can result in SI transmission in the subframes outside of the configured SI window.

Proposal 8: In IoT-NTN TDD mode, SI transmission shall be truncated if, due to the proposed SI transmission postponement, it overflows the configured SI window length.

Proposal 9: In IoT-NTN TDD mode, by making modifications to the SI scheduling mechanism, all SI messages can be delivered to the UE. This can be achieved by updating 36.331 §5.2.3a as follows:

1> if the UE is a NB-IoT UE:

2>	receive and accumulate SI message transmissions on DL-SCH from the start of the SI-window and continue until the end of the SI-window whose absolute length in time is given by si-WindowLength, starting from the radio frames as provided in si-RepetitionPattern and in subframes as provided in downlinkBitmap, or until successful decoding of the accumulated SI message transmissions excluding the subframes used for transmission of NPSS, NSSS, MasterInformationBlock-NB/ MasterInformationBlock-TDD-NB, and SystemInformationBlockType1-NB, and in the IoT-NTN TDD mode excluding subframes outside of active downlink time. If there are not enough subframes for one SI message transmission in the radio frames as provided in si-RepetitionPattern, the UE shall continue to receive the SI message transmission in the radio frames following the radio frame indicated in si-RepetitionPattern;

In the IoT-NTN TDD mode, if postponement of SI subframes due to downlink inactive time results in overlap with the subframes of the next SI repetition, or falls outside SI window, SI transmission should be truncated to the last non-overlapping subframe or last subframe within the SI window.

#### NPDCCH Scheduling

RAN-1 has not yet decided on whether a new NPDCCH configuration parameter is required for IoT-NTN TDD mode but in RAN1#120 it is agreed that NPDCCH and NPDSCH are scheduled only in active downlink subframes by means of postponement to next valid NB-IoT downlink subframe.

As per RAN1#120: Agreement The non-D NB-IoT subframes are not considered by the UE as “NB-IoT DL subframes” from the specifications - NPDCCH / NPDSCH (not containing SI) are mapped as per current specifications.

In IoT-NTN TDD mode, with the existing set of configuration parameters, it is possible that NPDCCH search spaces can fall within the downlink inactive times. However, with the above RAN1#120 agreement, these search spaces shall be postponed to the next downlink active time. Based on our understanding, the existing set of NPDCCH configuration parameters can work for IoT-NTN TDD mode, but RAN1 has not yet decided whether new NPDCCH configuration parameters need to be introduced which can have RRC ASN.1 impact.

Observation 9: RAN2 should wait for RAN1 decision on NPDCCH scheduling configuration for IoT-NTN TDD mode.

## UE Procedures

#### Paging

In RAN1#120 following agreements are made which impacts paging procedure:

As per RAN1#120: Agreement The non-D NB-IoT subframes are not considered by the UE as “NB-IoT DL subframes” from the specifications - NPDCCH / NPDSCH (not containing SI) are mapped as per current specifications. Conclusion For determining the offset between the 90ms TDD structure and the 10240ms H-SFN: the UE may derive the 90ms TDD structure based on observations of downlink signals. No specification impact.

In RAN1#120 it is agreed that there is no need to change SFN and H-SFN duration so RAN2 can analyze impact on the paging procedure in IoT-NTN TDD mode.

UE paging is possible only in certain frames and subframes. The frames where paging is allowed are called Paging Frames (PF), and the subframes within these paging frames are referred to as Paging Occasions (PO). The NPDCCH with DCI format N2 is carried in the PO. Each UE is assigned to one of these paging frames based on its IMSI value, so it only needs to monitor one paging occasion per DRX cycle. The subframes that can be used for paging occasions are restricted by the number of POs per frame, and the only allowed subframes are subframe #0, subframe #4, subframe #5, and subframe #9 (36.304 §7.2).

PF is calculated as frames satisfying: SFN mod T= (T div N)*(UE_ID mod N), where:

T = DRX cycle of the UE

UE_ID=IMSI mod 4096

N=min(T, nB)

In IoT-NTN TDD mode, the available paging capacity is reduced because only 8 subframes every 90ms are available to carry paging occasions and their corresponding paging messages. In this mode, the PF and associated PO calculated for a UE based on its IMSI can fall into a downlink inactive time. When a PO falls within a downlink inactive time, it is postponed to the next downlink active time, as illustrated in Figure 4. Consequently, all the paging opportunities within the 90ms period are compressed into the 8ms downlink active time.

Figure 4 Illustration of paging opportunities postponement

When paging opportunities for UEs fall within downlink inactive time and overlap with those in downlink active time, the network scheduler can decide which UEs to page. This is because all UEs will be listening to the paging during the same downlink active time based on the rule of postponement. This behavior is like the existing network behavior where the network prioritizes paging among multiple UEs mapped to the same paging opportunities.

A paging message is transmitted in consecutive NB-IoT downlink subframes.  In RAN1#120 it is agreed how NPDCCH and NPDSCH scheduling can work in IoT-NTN TDD mode with minimal impact on existing specifications by expanding the definition of NB-IoT downlink subframes to exclude those falling into downlink inactive time. This agreement also results in postponement of any paging opportunities that fall into downlink inactive time to the next downlink active time.

In the case of paging, the network is unaware of the coverage class of a UE. Therefore, NPDCCH/NPDSCH is transmitted with many repetitions to ensure that UEs in poor coverage areas can receive the paging message. According to the link-level simulation results submitted to RAN1#120 for NPDCCH and NPDSCH, four repetitions should be sufficient to achieve a 99% success rate in receiver decoding for the LEO-600 and LEO-1200 reference scenarios.

In IoT-NTN TDD mode, paging in eDRX can operate according to the existing NB-IoT specifications. A UE can calculate the Paging Hyper Frame (PH) and Paging Transmit Window (PTW) following the established rules. The PTW duration, specified in seconds, can accommodate multiple downlink active time opportunities. Any paging opportunity within a PTW that falls during downlink inactive time will be postponed to the next downlink active time.

Observation 10: After RAN1#120 agreement on expanding the definition of “NB-IoT DL subframes” to exclude all inactive downlink subframes, changes to the paging procedure for IoT-NTN TDD in 36.304 is not required.

Observation 11: In IoT-NTN TDD mode, eDRX can work without any changes. Within PTW, when calculated paging opportunity of a UE falls in downlink inactive time, it shall get postponed to the next downlink active time.

Proposal 10: In IoT-NTN TDD mode, when calculated paging opportunity of a UE falls in downlink inactive time, it shall get postponed to the next downlink active time.

#### Random Access Procedure

In RAN1#120 following agreements are made related to NPRACH scheduling:

As per RAN1#120: Agreement For NPRACH, further consider the two options until RAN1#120bis: - Option 1: New NPRACH periodicities (multiple of 90ms) are introduced for NB-IoT NTN TDD. - Option 2: NPRACH transmissions are postponed in non-U NB-IoT subframes until the next U NB-IoT subframe(s) - FFS: The unit of postponement (e.g. PRU)

In FDD mode, NPRACH opportunities can be configured with periodicities of 40ms, 80ms, 160ms, 240ms, 320ms, 640ms, 1280ms, and 2560ms, none of which are multiples of 90ms. The current NPRACH configuration parameters do not allow for full alignment with IoT-NTN TDD mode uplink active time occurrences. RAN1 has yet to finalize the NPRACH scheduling mechanism for IoT-NTN TDD mode. Depending on the outcome, it may be necessary to update the NPRACH configuration parameters in ASN.1.

Observation 12: Depending on the RAN1 decision regarding the NPRACH scheduling mechanism for IoT-NTN TDD mode, it may be necessary to update the RRC ASN.1 parameters.

##### RA-RNTI Calculation

During the RACH procedure, after transmission of the Random-Access Preamble, the UE starts monitoring NPDCCH common search space to check for the RAR scrambled with the RA-RNTI. For NB-IoT UEs, the RA-RNTI associated with the PRACH in which the Random-Access Preamble is transmitted, is computed as:

RA-RNTI=1+floor SFNid4+256*carrier_id

where SFN_id is the index of the first radio frame of the specified PRACH and carrier_id is the index of the UL carrier.

In IoT-NTN TDD mode, subject to agreement in RAN1, if NPRACH opportunities that fall within inactive uplink times (non-U subframes) are postponed to the next active uplink times (U subframes), the calculation of RA-RNTI becomes ambiguous. The eNodeB calculates RA-RNTI based on the SFN of the first radio frame in which the Random-Access Preamble is received. Therefore, the UE should calculate RA-RNTI based on the SFN of the first radio frame in which the Random-Access Preamble is transmitted after the postponement, not the actual occurrence of the NPRACH opportunity.

Proposal 11: In IoT-NTN TDD mode, The RA-RNTI should be calculated based on the SFN of the first radio frame in which the Random-Access Preamble is transmitted after the postponement, not the actual occurrence of the NPRACH opportunity.

##### RAR Window

The RAR window size and Contention Resolution timer are specified in terms of NPDCCH periods, with maximum values of 10 and 64 NPDCCH periods, respectively. The maximum value of RAR window size and Contention Resolution timer is capped at 10.24s.  In NTN, start of the RAR window is delayed considering Kmac value as illustrated in Figure 5. The RACH procedure can operate with the existing values of these parameters. However, a smaller NPDCCH period value will result in either no downlink active opportunities or very few within the maximum allowed RAR window size of 10 NPDCCH periods. For instance, with the minimum NPDCCH period value of 4ms, the maximum RAR window size is only 40ms.

Figure 5 RAR window start time and size

In IoT-NTN TDD mode, with D=8 and N=9, an NPDCCH period less than 64ms is not practical, as most of the search spaces become unusable due to NPDCCH postponements around NB-IoT downlink subframes. Network operators are expected to configure practical values for the NPDCCH period, using a value of 64ms or higher. In the ongoing RAN1#120bis meeting, we have proposed either providing network configuration guidance or make it explicit in the specification that, for the IoT-NTN TDD mode, the minimum allowed value of the NPDCCH period is 64ms instead of the current minimum allowed value of 4ms.

Observation 13: In IoT-NTN TDD mode, when the minimum NPDCCH period is configured as 64, the existing set of RAR window configuration parameters is sufficient.

#### MAC Timers

There are various protocol timers in the MAC which are specified in multiples of NPDCCH period (pp). These timers are copied here for ease of reference:

logicalChannelSR-ProhibitTimer-r13 = ENUMERATED {pp2, pp8, pp32, pp128, pp512, pp1024, pp2048, spare}

PeriodicBSR-Timer-NB-r13 = ENUMERATED {pp2, pp4, pp8, pp16, pp64, pp128, infinity, spare}

RetxBSR-Timer-NB-r13 =ENUMERATED {pp4, pp16, pp64, pp128, pp256, pp512, infinity, spare}

pur-ResponseWindowTimer-r16 ENUMERATED {pp1, pp2, pp3, pp4, pp8, pp16, pp32, pp64}

mac-ContentionResolutionTimer-r13 ENUMERATED {pp1, pp2, pp3, pp4, pp8, pp16, pp32, pp64}

C-DRX related timers:

onDurationTimer-r13 ENUMERATED {pp1, pp2, pp3, pp4, pp8, pp16, pp32, spare}

drx-InactivityTimer-r13 ENUMERATED {pp0, pp1, pp2, pp3, pp4, pp8, pp16, pp32}

drx-RetransmissionTimer-r13 ENUMERATED {pp0, pp1, pp2, pp4, pp6, pp8, pp16, pp24, pp33,}

drx-ULRetransmissionTimer-r13 ENUMERATED {pp0, pp1, pp2, pp4, pp6, pp8, pp16, pp24, pp33, pp40, pp64, pp80, pp96, pp112, pp128, pp160, pp320}

These timers are not counted in terms of postponed NPDCCH search spaces around NB-IoT downlink subframes. If the NPDCCH period is lower than 64, not all the allowed values for these timers are practical and can have a negative impact on the protocol performance.  For instance, with the minimum NPDCCH period value of 4ms, timer values lower than pp16 are not functional in IoT-NTN TDD mode.

Network operators are expected to configure practical values for the NPDCCH period, using a value of 64ms or higher. For the upcoming RAN1#120-bis meeting, we have proposed either to provide network configuration guidance or to make it explicit in the specification that, for the IoT-NTN TDD mode, the minimum allowed value of the NPDCCH period is 64ms instead of the current minimum allowed value of 4ms.

Proposal 12: RAN2 should send LS to RAN1 to ensure that in IoT-NTN TDD mode, the minimum allowed value of the NPDCCH period is 64ms instead of the current minimum allowed value of 4ms when D=8 and N=9.

#### Cell Access and Barring

A UE must acquire SIB1 to determine if a cell is barred for access in either a terrestrial or non-terrestrial network, based on the IE cellBarred-r13 and cellBarred-NTN-r17, respectively. The IoT-NTN TDD mode and the associated EUTRA RF band will be introduced in Release 19, meaning any pre-Release 19 UE will not be able to detect the IoT-NTN TDD cell. There is no overlap in the downlink between the new IoT-NTN TDD band (1616 – 1626.5 MHz) and the existing EUTRA bands.

For Rel-19 and later UEs, if a UE does not support IoT-NTN TDD mode, it shall not be able to detect the NPSS or read the MIB and SIB1 of the IoT-NTN TDD cell.

For Rel-19 and later UEs supporting IoT-NTN TDD mode, they shall be able to detect an IoT-NTN TDD cell. The existing SIB1 fields, cellBarred-r13 and cellBarred-NTN-r17, should be sufficient to control access to the IoT-NTN TDD cell.

Proposal 13: In IoT-NTN TDD mode, existing cell barring mechanism using the IE cellBarred-r13 and cellBarred-NTN-r17 in SIB1 is sufficient to control access to the IoT-NTN TDD cell.

## Round Trip Delay

Figure 7 Illustration of timing relationship [10]	Figure 8 Round trip path in cross-linked satellite network

In the Figure 7:

- Kmac = Offset or delay configured to equal the roundtrip time between RP (Reference Point) and eNB. The value is defined as a 9-bit integer representing 1-512ms of delay.

- Reference Point (RP) is the uplink time synchronization point where DL and UL frames are aligned

Figure 8 depicts a round-trip communication path between a gateway and a UE in a cross-linked satellite network.

Most of Iridium network traffic is transmitted using 4 or fewer satellites before it is terminated at a gateway. This traffic is easily transmitted with the 512ms of allowable latency permitted by the 3GPP NTN standard. A small percentage of Iridium’s network traffic is transmitted over larger number of satellites before it is terminated at a gateway. While the Iridium network is designed to allow these long-range, inter-satellite routes, the latency inherent in such routing pushes our limits above the 512ms of latency permitted by the maximum value of Kmac.  Another case where this latency might need an increase (in a satellite network in general) would be for gateway diversity/redundancy of the routes to ensure reliable packet delivery; this means sometimes packets need to traverse a non-optimal route.

We believe that other satellite service providers whose constellation supports cross-linking could come across the same issue. Cross-linking is beneficial not only for reliability but also to alleviate congestion, or to save costs. For these reasons Iridium suggests that the Kmac be updated to permit latency of up to 1024ms.

We can see two ways of allowing for larger Kmac; One would be extending the Kmac range by 1 bit, from 512 to 1024 ms. Another way would be creating a new optional parameter Kmac_extension, that would be additional to the existing Kmac and the two together would allow for larger delay.

Observation 14: For the cross-linked satellite networks, there may be a need to increase Kmac to allow for longer round-trip delay.

Proposal 14: Increase Kmac, effectively either by increasing defined value from 9 to 10 bits, or by defining another optional parameter Kmac_extension, and the two together would allow for larger delay.

In NB-IoT NTN specifications, the use of Kmac is specified for scenarios where the UE to eNodeB round trip time makes it impractical for a UE to receive downlink responses to uplink transmissions. Kmac is used in the following cases:

- During the RACH procedure, to delay the start of the RAR timer window after preamble transmission.

- During the RACH procedure, to delay the start of the contention resolution window.

- To increase the HARQ round trip timer value.

- During the PUR procedure, to delay the start of the PUR response window after uplink transmission using PUR.

- During the PUR procedure, to delay the time at which the UE should start monitoring the NPDCCH.

- During SR without HARQ transmission, to delay the monitoring of the NPDCCH search space.

Therefore, increasing the Kmac value should only affect procedures that depend on the UE-eNB round trip delay. A higher Kmac value can help in UE power saving by allowing the UE to avoid monitoring downlink channels.

Proposal 15: Increasing the Kmac value from 512 to 1024ms should not have an impact on any other NB-IoT procedures.

# Conclusion

In this contribution, we discussed the impact of IoT-NTN TDD mode on associated RAN2 procedures. Since some of the scheduling aspects are still under discussion in RAN1, it is premature to assess the impact on RAN2, including any required ASN.1 changes. However, after the RAN1#120 agreements, RAN2 can decide on SI scheduling and paging. Our observations and proposals are listed below:

Observation 1: When operating in IoT-NTN TDD mode, the SIB1 repetition of 16 radio frames works even with the maximum allowed transport block size of 680 bits with adequate link budget margin of 3.91 dB and 4.51 dB for LEO-600 and LEO-1200 respectively.

Observation 2: When operating in IoT-NTN TDD mode, SIB1 repetition pattern should be configured as 16 radio frames to allow every alternate active downlink opportunity to carry SIB1 subframes for optimal SIB1 decoding performance and acquisition time.

Observation 3: When operating in IoT-NTN TDD mode, with SIB1 repetition pattern of every 16 RFs, maximum delay in receiving SIB1 is 2520ms.

Observation 4: With the IoT-NTN TDD mode frame pattern, it is possible for the SI window start time to fall within the inactive downlink period.

Observation 5: With IoT-NTN TDD mode frame pattern, it is not possible to transmit all 8 subframes of SI transport blocks within a single downlink active time opportunity when SI TBS is greater than 120 bits.

Observation 6: With the IoT-NTN TDD mode frame pattern, postponement of the transmission of SI transport block to the next downlink active time can result in overlap with the transmission of the next SI repetition when N=9 and SI repetition pattern is configured as lower than every 16th radio frame.

Observation 7: With the IoT-NTN TDD mode frame pattern, postponement of the transmission of SI transport block to the next downlink active time results in no overlap when N=9 and the SI repetition pattern is configured as every 16th radio frame.

Observation 8: With IoT-NTN TDD mode frame pattern, the postponement of transmission of SI transport block to the next downlink active time can result in SI transmission in the subframes outside of the configured SI window.

Observation 9: RAN2 should wait for RAN1 decision on NPDCCH scheduling configuration for IoT-NTN TDD mode.

Observation 10: After RAN1#120 agreement on expanding the definition of “NB-IoT DL subframes” to exclude all inactive downlink subframes, changes to the paging procedure for IoT-NTN TDD in 36.304 is not required.

Observation 11: In IoT-NTN TDD mode, eDRX can work without any changes. Within PTW, when calculated paging opportunity of a UE falls in downlink inactive time, it shall get postponed to the next downlink active time.

Observation 12: Depending on the RAN1 decision regarding the NPRACH scheduling mechanism for IoT-NTN TDD mode, it may be necessary to update the RRC ASN.1 parameters.

Observation 13: In IoT-NTN TDD mode, when the minimum NPDCCH period is configured as 64, the existing set of RAR window configuration parameters is sufficient.

Observation 14: For the cross-linked satellite networks, there may be a need to increase Kmac to allow for longer round-trip delay.

Proposal 1: A description of IoT-NTN TDD mode shall be added in 36.300 [§5] as follows:

Downlink and uplink transmissions are organized into radio frames with 10 ms duration. Three radio frame structures are supported:

- Type 1, applicable to FDD and IoT-NTN TDD;

- Type 2, applicable to TDD;

…

For IoT-NTN TDD mode, Frame Structure Type-1 is used where uplink and downlink transmissions are separated in the time domain and constitute of set of D non-overlapping usable contiguous DL subframes and set of U usable contiguous UL subframes separated by fixed guard period (GP) as per Figure 1. This pattern is repeated every N radio frames. The values of N, D, U, [TBD for GP and start subframe for D] are fixed per IoT-NTN TDD band specified in [TBD]

Proposal 2: Anchor carrier definitions in 36.300 §3 needs updating:

Anchor carrier: in NB-IoT, a carrier where the UE assumes that NPSS/NSSS/NPBCH/SIB-NB for FDD and IoT-NTN TDD or NPSS/NSSS/NPBCH for TDD are transmitted.

Proposal 3: In 36.300 §23.21 should be updated to reflect that NTN is applicable to both FDD and IoT-NTN TDD system:

In this release of the specification, NTN is only applicable to FDD and IoT-NTN TDD system.

Note: As per RAN2 VC instructions after the RAN2 #129 [5], the rapporteur of IOT NTN TDD WI has submitted the above three proposals for consideration into the upcoming RAN1 120-bis, and these proposals brought up here again are simply for completeness.

Proposal 4: When operating in IoT-NTN TDD mode, there is no need to update the SIB1 scheduling mechanism in the existing specifications, except for disabling transmissions during downlink inactive time.

Proposal 5: In IoT-NTN TDD mode, if start of the SI window falls within the downlink inactive time, it shall be postponed to the next downlink active time.

Proposal 6: In IoT-NTN TDD mode, subframes from a single repetition of the SI transmission shall be allowed to be postponed to the next downlink active time opportunity.

Proposal 7: In IoT-NTN TDD mode, SI transmissions shall be truncated if, due to the proposed postponement, they overlap with the transmission of the next SI repetition within the SI window.

Proposal 8: In IoT-NTN TDD mode, SI transmission shall be truncated if, due to the proposed SI transmission postponement, it overflows the configured SI window length.

Proposal 9: In IoT-NTN TDD mode, by making modifications to the SI scheduling mechanism, all SI messages can be delivered to the UE. This can be achieved by updating 36.331 §5.2.3a as follows:

1> if the UE is a NB-IoT UE:

2>	receive and accumulate SI message transmissions on DL-SCH from the start of the SI-window and continue until the end of the SI-window whose absolute length in time is given by si-WindowLength, starting from the radio frames as provided in si-RepetitionPattern and in subframes as provided in downlinkBitmap, or until successful decoding of the accumulated SI message transmissions excluding the subframes used for transmission of NPSS, NSSS, MasterInformationBlock-NB/ MasterInformationBlock-TDD-NB, and SystemInformationBlockType1-NB, and in the IoT-NTN TDD mode excluding subframes outside of active downlink time. If there are not enough subframes for one SI message transmission in the radio frames as provided in si-RepetitionPattern, the UE shall continue to receive the SI message transmission in the radio frames following the radio frame indicated in si-RepetitionPattern;

In the IoT-NTN TDD mode, if postponement of SI subframes due to downlink inactive time results in overlap with the subframes of the next SI repetition, or falls outside SI window, SI transmission should be truncated to the last non-overlapping subframe or last subframe within the SI window.

Proposal 10: In IoT-NTN TDD mode, when calculated paging opportunity of a UE falls in downlink inactive time, it shall get postponed to the next downlink active time.

Proposal 11: In IoT-NTN TDD mode, The RA-RNTI should be calculated based on the SFN of the first radio frame in which the Random-Access Preamble is transmitted after the postponement, not the actual occurrence of the NPRACH opportunity.

Proposal 12: RAN2 should send LS to RAN1 to ensure that in IoT-NTN TDD mode, the minimum allowed value of the NPDCCH period is 64ms instead of the current minimum allowed value of 4ms when D=8 and N=9.

Proposal 13: In IoT-NTN TDD mode, existing cell barring mechanism using the IE cellBarred-r13 and cellBarred-NTN-r17 in SIB1 is sufficient to control access to the IoT-NTN TDD cell.

Proposal 14: Increase Kmac, effectively either by increasing defined value from 9 to 10 bits, or by defining another optional parameter Kmac_extension, and the two together would allow for larger delay.

Proposal 15: Increasing the Kmac value from 512 to 1024ms should not have an impact on any other NB-IoT procedures.

# References

RP-243293, Revised WID on introduction of IoT-NTN TDD mode, RAN#106, 2024-12, Madrid, ES.

RP-242652, Status Report - WID on introduction of IoT-NTN TDD mode, Iridium, RAN#106, 2024-12, Madrid, ES.

RP-242415, “New WID on introduction of IoT-NTN TDD mode”, RAN #105, Melbourne, Australia, 2024-09

RP-2501614, “Feature lead summary #3 on IoT-NTN TDD mode”, 3GPP TSG RAN WG1 Meeting #120, Athens, Greece, February 17-21, 2025.

R2-2501333 “Report from Break-out session on NR-NTN and IoT-NTN”, 3GPP TSG RAN WG2 Meeting #129, Athens, Greece, February 17-21, 2025.