3GPP TSG-RAN WG2 #129bis	R2-2501990

Wuhan, China, 7th – 11nd April 2025

Agenda item:	   8.3.5

Source:		Lenovo

Title:                             LCM and Spec Impact for AI/ML Mobility

Document for:		Discussion and Decision

# Introduction

As instructed by the SID, the AI for mobility SI will study the RRM measurement prediction and measurement event prediction. In this paper, we discuss essential LCM aspects for AI/ML mobility, considering these use cases and the respective scenarios of spatial and temporal domain predictions.

# Discussion

In R19 AI/ML for NR air interface work item, the fundamental framework of AI/ML functionality has been discussed considering the following modules for the beam management and positioning use cases:

- Life Cycle Management (LCM)

  - LCM for NW-sided model

  - LCM for UE-sided model

- Data collection

  - Data collection for NW-sided model

  - Data collection for UE-sided model

The general aspects of LCM have focused on the procedures to enable AI/ML functionality and eventually provide an inference outcome for an intended use case or a sub-use case.  The procedures that were addressed include network configuration for AI/ML-related functionalities, determination of applicability of functionality, signaling mechanisms for applicability report, inference reporting from the UE, etc. The agreed AI/ML framework should be extendable to other use cases, such as AI/ML for mobility: RRM measurement prediction and RRM event prediction.

## 2.1. RRM Measurement Prediction

The high-level categorization of RRM measurement prediction is done in two ways: prediction in the spatial domain and prediction in the temporal domain. At the same time, pure temporal prediction is studied as the baseline for measurement predictions.

The AI for mobility SID suggests studying both UE-sided and NW-sided models for RRM measurement prediction. Whether RRM measurement prediction is conducted by the UE or the network, it is, in either case beneficial in determining the mobility-related decision and configuration by the network. For example, it can be used by the network for configuring the beams, in making handover decisions, or configuring thresholds for mobility-related measurements.

Proposal 1: Both UE-sided and NW-sided RRM measurement predictions can be equally beneficial to improve mobility performance.

The two possible approaches that were studied in this SI are beam-level and cell-level predictions. Beam-level measurement predictions can be used for the beam selection in case of mobility. During RACH initial access in a mobility scenario, the UE may select the “best beam” in the target cell based on the prediction. Thus, measurement prediction can help to reduce the number of measurements performed by the UE. Additionally, cell-level measurement prediction, such as temporal domain prediction, enables the selection of a “best target cell” for a potential handover. Thus, improving the performance of handover by reducing the probability of a handover failure.

Beam level measurements are generated directly from the L1 measurements, while cell level measurements are derived from the L1 measurements using certain rules and filtering. For beam and cell level measurements, intra and inter-cell spatial domain measurement predictions have also been considered in this study. As cell-level measurements are the focus of this study, they can be either generated or predicted based on the beam-level or cell-level inputs. RAN2 agreement lists three possible cases for cell-level RRM measurement prediction. It is important to discuss how LCM aspects apply to these different types of measurement prediction cases for NW-sided and UE-sided models.

### 2.1.1  NW-sided RRM Measurement Prediction

RRM measurement predictions performed by the NW-sided model can be implemented with a least specification impact other than data collection procedures. As agreed in AI/ML for air interface WI, UE does not need to be informed of and involved in any gNB-sided functionality management decision (e.g., selection, de(activation), switching, fallback, etc), except being configured to provide the required measurement/data. Thus, the network can configure the UE for RRM measurements and reporting required for inputs (e.g., L1-RSPR, L3-RSRP) to the NW-sided models. As per the progress of LCM for the NW-sided model for beam management (BM) in AI/ML for air interface, RAN2 can assume that the following agreements can be directly extended to the RRM measurement prediction use case.

Observation: The following agreements can be reviewed and extended to RRM measurement prediction:

LCM and data collection for NW-sided model

- RAN2 focuses on the data collection procedure from UE to NW (e.g., gNB, LMF, or OAM) for the sake of NW-sided model LCM (including training, inference, management).

- RAN2 to consider an RRC configuration to configure radio measurements and the related reporting to enable data collection for NW-side training

- For AI/ML based beam management, RAN2 assumes the L1 measurement framework shall be used for configuring the input data of the NW side AI/ML model inference.  FFS if further enhancements are needed

For the BM use case, as the input and predictions are mainly dependent on the L1 measurements, the following agreement indicates the use of L1 signaling. However, for the case of RRM measurement prediction, since cell level measurement is the final inference outcome and the input for inference depends on which case of RRM measurement prediction is selected, along with L1 signaling, L3 signaling is also required for the input.

For network-sided model for BM use case, RAN2 confirms that UE inputs for inference at network-sided model will rely on L1 signaling, RAN2 will not further spend time on this aspect.

Proposal 1: For the network-sided model for RRM measurement use case, RAN2 confirms that L1/L3 signaling can be used for UE inputs for inference.

The input from UE for inference can include any legacy L1/L3 beam measurement results or legacy L3 cell measurement results. At the same time, we believe the L1 beam prediction result by a UE supporting the AI for air interface beam prediction use case could also be considered as possible input for further L3 beam/cell level result prediction at NW.

Proposal 2: The input from UE could be either legacy L1/L3 beam/cell measurements or L1 beam prediction results (if UE is capable of L1 beam prediction in the AI for air interface).

As described in the SID, the AIML model used for RRM measurement prediction can also be deployed at NW. compared to using a UE-sided model, using a NW-sided model for RRM measurement prediction may have the following advantages:

- NW has more powerful computing capability

- NW can easily collect enough training data from UEs under its coverage via legacy L1/L3 measurement reports. The complexity of data collection to train a UE-sided model can be avoided.

- NW can start the NW-sided AIML inference without additional prediction related configuration towards UE.

- For the sake of model performance monitoring, NW node can easily configure the UE to do necessary L1/L3 measurements and report, or even request the neighbour NW node to provide the necessary UE measurements after a handover execution.

From RAN2 point of view, how NW requests input to support RRM measurement prediction using NW-sided model would be the focus. As a starting point assumption, NW can request input from UE in one-shot (aperiodic), periodic, or event-triggered manner.

Proposal 3: NW requests input from UE in a periodic, one-shot (aperiodic), or event-triggered manner for RRM measurement prediction for NW-sided model. FFS details on events.

### 2.1.2  UE-side RRM Measurement Prediction

The goal of temporal domain and spatial domain prediction can be derived from the final prediction output. RAN2 is suggested to discuss if temporal domain and spatial domain RRM measurement prediction can be applied at the same time at the UE, as it can affect how UE determines and reports applicability.

Proposal 4: RAN2 should discuss the feasibility of the coexistence of temporal domain and spatial domain RRM measurement predictions.

Option 4A: Not mutually exclusive: Temporal domain and spatial domain prediction (periodic) configuration can be active simultaneously.

Option 4B: Mutually exclusive: Temporal domain and spatial domain prediction (periodic) configuration cannot be active simultaneously.

The different cases of temporal and spatial domain prediction for inter/intra-cell scenarios have also been studied. The assumption is that a configuration from the network is always required, which is used for or in the process of measurement inference. However, the measurement configuration may be different for each case and depends on whether the cell-level measurements are predicted based on predicted beam-level results, cell-level results, or beam-level results of the same cell or a different cell. In some cases, it may be a configuration of a set of beams, as agreed in the R19 AI/ML air interface for BM. Based on the progress of LCM in the R19 AI/ML air interface, the legacy measurement configuration framework can be used as a baseline, which can be extended further considering different cases.

Observation: The network configuration can differ for the different cases of spatial and temporal domain RRM measurement prediction.

Proposal 5: Legacy RRC measurement configuration and reporting framework is used for data collection and inference as baseline. RAN2 will discuss further extension for different cases of RRM measurement prediction.

In legacy, s-measureconfig is configured to UE, which controls when UE to performs measurement for a non-serving cell. This is intended for power savings since a UE located at the cell centre will not be handed over to a neighbour cell. RAN2 agreed to support intra-frequency, intra and inter-cell spatial domain measurement predictions for beam and cell level measurements. Similarly, when UE stays at the centre of one cell, UE is not expected to predict Inter-cell measurement results. One option is that legacy s-measureconfig can be used to control when the UE should perform measurement prediction for non-serving cells. Alternatively, one separate threshold can be configured for the UE to determine when the measurement predictions should be performed for the non-serving cell. For example, if the quality of the serving cell is less than the configured threshold, the measurement prediction for non-serving cells can be triggered.

Proposal 6: UE may perform measurement prediction for non-serving cells if the channel quality of the serving cell is worse than a configured threshold. FFS if the threshold is from legacy s-measureconfig IE or a separate threshold.

## 2.1. Measurement Event Prediction

The events A1-A5 which are utilized to evaluate the serving and neighbour cells located at intra/inter frequency, are designed for mobility purpose. When one of the events is met, UE is triggered to report the measurement results. Once the network receives the measurement results, the network will decide whether to prepare handover and which neighbour cell can be selected as target cell. Event A3 was agreed as a baseline since it is most relevant for mobility.

In current specifications, the entering condition and leaving condition are specified for each event. When the entry condition of an event is met during TTT, the event is determined as satisfied. This implies UE enters the cell edge and consecutively, the measurement report is triggered. When the leaving condition of an event is met during TTT, it means UE returns to the cell centre from cell edge. UE is triggered to report the measurement result if reportOnLeave is configured. Similarly, the network can configure UE to predict whether the entering condition or the leaving condition is fulfilled. The prediction of the leaving condition could be helpful for the network to decide a suitable time instance for handover execution.

Proposal 7: Network can configure UE to predict whether the entering condition or the leaving condition is met.

UE receives a configuration of event prediction. Meanwhile, prediction window may be provided together with the configuration. If prediction window is provided, UE is expected to predict whether an event occurs within the prediction window.

Proposal 8: The prediction window may be configured to UE. If a prediction window is provided, UE is expected to predict whether an event occurs within the prediction window.

Similar to the case of RRM measurement prediction, network configuration (s-measureconfig) controls when UE performs measurement for non-serving cell. It is also beneficial to consider when UE performs event prediction depending on UE position in the cell, cell centre or cell edge. For this purpose, a condition can be configured to UE by the serving cell. For example, when the channel quality of the serving cell, e.g., PCell is not good enough, UE starts measurement event prediction.

Proposal 9: UE may perform measurement event prediction based on a trigger condition, e.g., the channel quality of serving cell becomes bad.

# 3	Conclusion

Based on the discussion above, we request RAN2 to discuss and agree on the following proposals:

RRM measurement prediction

Observation: Both UE-sided and NW-sided RRM measurement predictions can be equally beneficial to improve mobility performance.

NW-sided RRM measurement prediction

Observation: The following agreements can be reviewed and extended to RRM measurement prediction:

LCM and data collection for NW-sided model

- RAN2 focuses on the data collection procedure from UE to NW (e.g., gNB, LMF, or OAM) for the sake of NW-sided model LCM (including training, inference, management).

- RAN2 to consider an RRC configuration to configure radio measurements and the related reporting to enable data collection for NW-side training

- For AI/ML based beam management, RAN2 assumes the L1 measurement framework shall be used for configuring the input data of the NW side AI/ML model inference.  FFS if further enhancements are needed

Proposal 1: For the network-sided model for RRM measurement use case, RAN2 confirms that L1/L3 signaling can be used for UE inputs for inference.

Proposal 2: The input from UE could be either legacy L1/L3 beam/cell measurements or L1 beam prediction results (if UE is capable of L1 beam prediction in the AI for air interface).

Proposal 3: NW requests input from UE in a periodic, one-shot (aperiodic), or event-triggered manner for RRM measurement prediction for NW-sided model. FFS details on events.

UE-sided RRM measurement prediction

Proposal 4: RAN2 should discuss the feasibility of the coexistence of temporal domain and spatial domain RRM measurement predictions.

Option 4A: Not mutually exclusive: Temporal domain and spatial domain prediction (periodic) configuration can be active simultaneously.

Option 4B: Mutually exclusive: Temporal domain and spatial domain prediction (periodic) configuration cannot be active simultaneously.

Proposal 5: Legacy RRC measurement configuration and reporting framework is used for data collection and inference as baseline. RAN2 will discuss further extension for different cases of RRM measurement prediction.

Proposal 6: UE may perform measurement prediction for non-serving cells if the channel quality of the serving cell is worse than a configured threshold. FFS if the threshold is from legacy s-measureconfig IE or a separate threshold.

Measurement event prediction

Proposal 7: Network can configure UE to predict whether the entering condition or the leaving condition is met.

Proposal 8: The prediction window may be configured to UE. If a prediction window is provided, UE is expected to predict whether an event occurs within the prediction window.

Proposal 9: UE may perform measurement event prediction based on a trigger condition, e.g., the channel quality of serving cell becomes bad.

# Reference

[1] RP-234055, Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR, RAN#102.

[2] TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18)