3GPP TSG-RAN2#129bis 	R2-2501992

Wuhan, China, 7th Apr – 11th Apr, 2025

Agenda item:		8.4.4 (NR_LPWUS-Core)

Source:	LG Electronics Inc.

Title: 	Remainng issues on LP-WUS in RRC_CONNECTED

Document for:	Discussion and Decision

1.	Introduction

In the last meeting, RAN2 made some agreements for LP-WUS in RRC_CONNECTED as follows.

For Option 1-1, UE monitors LP-WUS outside C-DRX active time at least when long DRX cycle is used. FFS whether short DRX cycle is used. RAN2 confirm the (Long) DRX command MAC CE can be used with option 1-1 to stop drx-onDurationTimer and drx-InactivityTimer. RAN2 confirm the (Long) DRX command MAC CE can be used with option 1-2 to stop the new timer and drx-InactivityTimer.

In post e-mail discussion [1], RAN2 made MAC running CR based on RAN2 agreements, but some discussion points are remained.

In this contribution, we further discuss remaining issues on LP-WUS in RRC_CONNECTED.

2.	Discussion

In [1], RAN2 made MAC running CR for LP-WUS related UE behaviour. However, whether LP-WUS is applicable to short DRX cycle is still remained as FFS.

In Rel-16, there were similar discussion whether DCP is applicable to short DRX cycle or not. In RAN2#107bis, RAN2 sent LS [2] to ask RAN1 if there are technical feasibility concerns to support WUS for short DRX. In RAN2#109, in [3], RAN1 replied that RAN1 cannot reach consensus because some companies thought that there is no impact on RAN1 specification and the other companies raised issues which impacts on RAN1 specification, e.g. SearchSpace, PS_offset and UE procedure with dynamic triggering of Short DRX.

Finally, RAN1 concluded that RAN1’s working assumption still stands, which DCP is not applicable for the short DRX cycle if both long and short DRX cycles are configured for the UE

[R2-2000017] RAN1 would like to thank RAN2 for the LS on WUS for short DRX in R1-1911500 (R2-1914060). RAN1 had discussed the technical feasibility and benefit of WUS for short DRX. The following opinions were expressed (not necessary exhaustive) - 1st opinion: - No RAN1 spec impacts were identified - 2nd opinion: Potential RAN1 spec impacts - Whether SearchSpace could be used for short and long DRX - Whether one PS_offset could be configured for short and long DRX - Whether UE procedures with dynamic triggering of short DRX RAN1 can not reach consensus on whether or not there is benefit, technical feasibility concern, or RAN1 spec impact of introducing WUS for short DRX. RAN1’s working assumption as communicated in LS R1-1911475 that WUS is not applicable for the short DRX cycle If both long and short DRX cycles are configured for the UE still stands. [R1-1911475] RAN1 had discussed the PDCCH-based power saving signal/channel triggering UE adaptation to DRX operation for UE wakeup or not at the next DRX ON with respect to the configuration of long and short DRX cycles in RAN1#98bis with the following agreements, - The PDCCH-based power saving signal/channel can be configured for long DRX for a UE - (Working assumption) If there is also short DRX cycle configured for the UE, the PDCCH-based power saving signal/channel is not applicable for the short DRX cycle

In our view, the situation is same for Rel-19 LP-WUS as Rel-16 DCP. In order to use LP-WUS with Short DRX cycle, RAN1 needs further discussion, e.g. mechanism or configuration for LP-WUS periodicity/offset considering both Short DRX cycle and Long DRX cycle.

In addition, we think that the use of LP-WUS with Short DRX cycle makes UE behaviour complicated, while power saving gain is small.

Therefore, LP-WUS should not be applicable to Short DRX cycle, i.e., LP-WUS is used with Long DRX cycle only.

Proposal 1. LP-WUS should not be applicable to Short DRX cycle, i.e., LP-WUS is used with Long DRX cycle only.

Another remaining issue is whether to introduce LP-WUS per DRX group. In the last meeting, some companies proposed LP-WUS per DRX group. However, we think LP-WUS per DRX group may have RAN1 impact.

Currently, DCP per group is not supported. In Rel-16 TEI, RAN2 asked RAN1 about the impact of the secondary DRX group in [4], and RAN1 replied that RAN1 impact of secondary DRX related to the UE’s behavior of detecting DCP and the respective procedures in [5]. Thus, RAN2 agreed that both the secondary DRX group and DCP are not configured simultaneously.

drx-ConfigSecondaryGroup Used to configure DRX related parameters for the second DRX group as specified in TS 38.321 [3]. The network does not configure secondary DRX group with DCP simultaneously nor secondary DRX group with a dormant BWP simultaneously.

Thus, it would be better to ask RAN1 whether LP-WUS and the secondary DRX group can be configured simultaneously, different from DCP. If RAN1 replies yes, RAN2 continues further discussion, e.g. whether the new timer can have different value for FR1 and FR2.

Proposal 2. RAN2 asks RAN1 whether LP-WUS and the secondary DRX group can be configured simultaneously, different from DCP.

3.	Conclusion

In this document, we discuss issues on LP-WUS in RRC_CONNECTED, and make proposals as follows.

Proposal 1. LP-WUS should not be applicable to Short DRX cycle, i.e., LP-WUS is used with Long DRX cycle only.

Proposal 2. RAN2 asks RAN1 whether LP-WUS and the secondary DRX group can be configured simultaneously, different from DCP.

4.	Reference

- [Post129][210][LPWUS] Running CR for TS38.321 (Apple)

- R2-1914060	LS to RAN1 on WUS for short DRX cycle	vivo

- R2-2000017	LS reply to RAN2 on WUS for short DRX cycle	CATT

- R2-1916597	LS on secondary DRX group	Ericsson

- R2-2004325	LS response on secondary DRX group Ericsson