3GPP TSG-RAN WG2 Meeting #129bis	R2-2501995

Wuhan, China, Apr. 7th – 11st, 2025

| Agenda Item: | 8.6.2 (NR_Mob_Ph4-Core) |
| --- | --- |
| Source: | LG Electronics Inc. |
| Title: | Remaining issues of inter-CU LTM |
| Document for: | Discussion and Decision |

1.	Introduction

For inter-CU LTM, this paper discusses security update handling and LTM-based recovery.

2.	Discussion

2.1.	Security update for inter-CU LTM

Security key change handling based on SA3 conclusion

During the last meeting, the SA3 group reached the following conclusions regarding the security handling to support subsequent cell switching for inter-CU LTM. This information will be addressed as an LS response at the upcoming RAN2#129bis meeting [1]:

SA3 discussed the issue and concluded that the NCC value can be transmitted in plaintext through MAC CE message. Due to there is no protection to MAC CE messages, NCC values may be tampered with, resulting in asynchronous NCC values between UE and gNB. Therefore, RAN2 needs to consider the mechanism of re-establishing the RRC connection between the UE and the gNB when the NCC values are out of sync. In addition, SA3 will not consider the protection of MAC CE messages in this release.

According to the SA3 discussion results, for inter-CU LTM, the source cell provides the NCC value to the UE during the transmission of the LTM cell switch command for security update. RAN2 has provided all workable options for security update methods, and based on these options, SA3 decided not to consider the protection of MAC CE messages in Release-19. Therefore, we think that RAN2 can follow the SA3 conclusion without considering additional critical issues.

Proposal 1: For inter-CU LTM, the NCC value can be included in the LTM CSC MAC CE.

Additionally, RAN2 should consider potential issues by unprotected NCC value, as raised by SA3.

The first case is that an attacker configures a fake cell to deliver a different NCC value to the UE, forcing the UE to perform an unintended LTM. This issue fundamentally arises because the cell switching command is not protected, making it impossible for the UE to detect the issue until the UE accesses the target cell. Consequently, if a security problem arises due to a wrong NCC value contained in the LTM CSC, the UE has no choice but to declare a security failure, resulting in the existing RRC Re-establishment procedure.

The second case is that an attacker maliciously intercepts the NCC value intended for a specific UE. If the attacker discovers the security pattern of the NCC value being used by UEs in the current cell, they can connect to the cell as a fake UE and continuously intercept data information. To prevent this issue, the security update for inter-CU LTM should consider both the horizontal key derivation (i.e., source gNB key based security update) and vertical key derivation (i.e., NCC value based security update) methods, similar to how it is handled during other existing MN mobilities to make security tracking by attackers more challenging.

When considering the existing MN mobility procedure rather than LTM, if the NCC value is provided in the mobility command, the UE performs a security update using vertical key derivation. If the NCC value is not present, the UE performs a security update using horizontal key derivation based on the source gNB key. We think that inter-CU LTM should also provide the same level of security as existing MN mobility to make it difficult for attackers to track security information.

Proposal 2: For inter-CU LTM, RAN2 should consider security updates based on horizontal key derivation (i.e., source gNB key based security update) to meet the same level of security in existing MN mobility.

To support horizontal key derivation, a similar way is necessary to include information in the LTM CSC that allows the UE to determine the security update method upon reception. Considering the principle of security information included in the mobility command of the current RRC procedure, if the LTM CSC contains the NCC value, the UE performs the security update using vertical key derivation (i.e., NCC value based security update). If the LTM CSC does not include the NCC value, the UE performs the security update using horizontal key derivation. Therefore, to ensure the same level of security as existing MN mobility and to prevent security vulnerabilities, we propose the following proposal:

Proposal 3: For inter-CU LTM, the UE performs a security update based on the horizontal key derivation (i.e., source gNB key based security update) if the LTM CSC does not include the NCC value.

Redundant PDCP reestablishment during subsequent LTM cell switches in mixture of inter-CU and intra-CU LTM with DC

In RAN2#127 meeting, RAN2 agreed the following:

For the SN key update in inter-CU MN LTM with DC, the UE applies legacy R15 RRC reconfiguration with sync procedure.

This means that the legacy SN security update procedure is re-used for the case where CU is acting as MN and SN is unchanged. Note that the legacy SN security update procedure is always performed if sk-Counter is included in the RRCReconfiguration message.

Specifically in legacy, the network sends to the UE the MN RRCReconfiguration message including the new MN security configuration and the single counter value (i.e., sk-Counter) for updating the SN security if the MN security update is needed due to, e.g, MN handover while the SN connection is maintained before/after MN handover. To this end, the SN RRCReconfiguration message including sk-Counter is embedded in the MN RRCReconfiguration message, i.e., sk-Counter is provided within nrSCG within MRDC-SecondaryCellGroupConfig included into MN RRCReconfiguration message. Upon receiving the RRCReconfiguration message, the UE updates the MN security based on the new MN security configuration (i.e., new KgNB). Subsequently, the UE updates the SN security based on the new KgNB and sk-Counter (i.e. new S- KgNB). To sum up, the SN security key update is based only on the provision of sk-Counter in the case of MN handover with keeping the existing SN connection.

For LTM cell switch, one or more candidate cell configurations are provided to the UE beforehand. Upon receiving an LTM cell switch command (CSC) MAC CE from the network, the UE performs the LTM cell switch toward the candidate cell indicated by the LTM CSC MAC CE, i.e., the UE first applies the stored candidate cell configuration and subsequently the UE accesses the candidate cell based on the stored candidate cell configuration and the LTM CSC MAC CE. In order to update the SN security with legacy Rel-15 RRC reconfiguration with sync procedure in the case of inter-CU MCG LTM while the SN connection is maintained, the pre-configured candidate cell configuration of MCG LTM target cell should include the single counter value, i.e., sk-Counter, for updating SN security.

In RAN2#125bis meeting, RAN2 agreed the following:

Rel-19 inter-CU LTM also supports mixture of subsequent inter-CU LTM and subsequent intra-CU LTM after an inter-CU or intra-CU LTM switch.

Based on the above agreement, the following scenario (i.e., Figure 1) may occur.

Figure 1. Subsequent intra-CU MCG LTM cell switches after inter-CU MCG LTM cell switch in case where the SN connection via a specific cell is maintained.

Figure 1 describes a scenario that subsequent intra-CU MCG LTM cell switches are performed after inter-CU MCG LTM cell switch in case where the SN connection via a specific cell is maintained.

Scene#0: The UE is configured with DC, where Cell#0 belonging to CU#0 is PCell and Cell#5 belonging to CU#5 is PSCell. The MN security key and the SN security key are KgNB#0 and S-KgNB#0, respectively. Cell#1 and Cell#2 belonging to CU#1 are configured with candidate cells for MCG LTM cell switch with keeping the SN connection via the current PSCell (i.e., Cell#5).

Scene#1: Inter-CU MCG LTM cell switch with keeping the existing SN connection is performed, i.e., PCell change from Cell#0 to Cell#1. Due to PCell change, the MN security is updated, i.e., the MN key is updated from KgNB#0 to KgNB#1. Subsequently, the SN security is updated based on KgNB#1 and sk-Counter included in the candidate cell configuration for Cell#1, where sk-Counter value is 1. Consequently, the SN security is updated to S-KgNB#1.

Scene#2: Intra-CU MCG LTM cell switch with keeping the existing SN connection is performed, i.e., PCell change from Cell#1 to Cell#2. Since CU for MCG (i.e., MN) is not changed due to PCell change, the MN security update is not performed, i.e., the MN key is maintained as KgNB#1. On the other hand, for the SN key, the SN security update is performed because the candidate cell configuration for Cell#2 includes sk-Counter. Consequently, the SN key is updated to S-KgNB#2 based on KgNB#1 and sk-Counter value 2. Since the MN security is not updated, the SN security update is redundant and unnecessary, which can result in data interruption at SCG due to the random access procedure for SN security update.

Based on the legacy Rel-15 RRC reconfiguration with sync procedure, whether to perform the SN security update during MN RRC reconfiguration (e.g., MN handover) does not depend on whether the MN security update is performed but rather on whether sk-Counter is provided as shown below.

5.3.5.3 Reception of an RRCReconfiguration by the UE […] 1> if the RRCReconfiguration includes the sk-Counter: 2> perform security key update procedure as specified in 5.3.5.7; […]

This means that the legacy Rel-15 RRC reconfiguration with sync procedure cannot prevent redundant SN security update during subsequent intra-CU MCG LTM cell switches. Hence, as described in scene#2, a redundant SN security update may be performed during subsequent intra-CU MCG cell switches.

reestablishPDCP Indicates that PDCP should be re-established. Network sets this to true whenever the security key used for this radio bearer changes. Key change could for example be due to termination point change for the bearer, reconfiguration with sync, resuming an RRC connection, or the first reconfiguration after reestablishment. It is also applicable for LTE procedures when NR PDCP is configured. Network doesn't include this field for DRB if the bearer is configured as DAPS bearer, if the RadioBearerConfig IE is part of an RRCReconfiguration message within the LTM-Config IE, or if the RadioBearerConfig IE is part of an RRCReconfiguration message associated with subsequent CPAC within the ConditionalReconfiguration IE

Based on the field description of reestablishPDCP shown above, PDCP reestablishment is followed by security key update. This implies that redundant SN security update during subsequent intra-CU MCG LTM cell switches results in redundant PDCP reestablishment at the PDCP entities associated with SN. Consequently, the UE undergoes data interruption due to redundant PDCP reestablishment.

Observation 1. Redundant SN security update during subsequent intra-CU MCG LTM cell switches can result in redundant PDCP reestablishment at the PDCP entities associated with SN and consequently data interruption can occur.

Scene#3: Intra-CU MCG LTM cell switch with keeping the existing SN connection is performed, i.e., PCell change from Cell#2 to Cell#1. Since CU for MCG (i.e., MN) is not changed due to PCell change, the MN security update is not performed, i.e., the MN key is maintained as KgNB#1. On the other hand, for the SN key, the SN security update is performed because the candidate cell configuration for Cell#1 includes sk-Counter. Consequently, the SN key is updated to S-KgNB#1 based on KgNB#1 and sk-Counter value 1. Here, the SN key that were used in the past (i.e. S-KgNB#1) is re-used, which violates the security requirement that the security key that were used in the past should not be re-used. This violation can incur a severe security related problem, e.g., leakage of the SN key may occur due to repetitive intra-CU MCG LTM cell switches with keeping the existing SN connection.

Observation 2. During subsequent intra-CU MCG LTM cell switches, the SN key that were used in the past may be re-used, which violates the security requirement.

It seems that the problem incurred by the observation 2 is a consequence of the problem incurred by the observation 1. This implies that a violation of security requirement due to duplicated SN security key use during subsequent intra-CU MCG LTM cell switches can be prevented by blocking redundant SN security update during subsequent intra-CU MCG LTM cell switches. To this end, the network should ensure that SN security update at intra-CU LTM will not happen during subsequent LTM cell switches in mixture of intra-CU LTM and inter-CU LTM. For example, the network reconfigures sk-Counter values for inter-CU MCG LTM with unchanged SN after an inter-CU LTM cell switch without SN change.

Proposal 4. The network ensures that SN security update at intra-CU LTM will not happen during subsequent LTM cell switches in mixture of intra-CU LTM and inter-CU LTM with DC (i.e., SN is not changed).

2.2.	LTM based recovery for inter-CU LTM

RAN2 has not reached a conclusion on whether LTM-based recovery can be performed using inter-CU LTM candidate cells while SA3 is discussing the security update method for inter-CU LTM [2]:

In RAN2#127bis - The Rel18 handling on failure is reused in R19 if the UE selects an intra-CU LTM candidate cell after intra-CU LTM failure; for other cases, e.g. inter-CU LTM failure, the failure handling is FFS (related to SA3’s inputs).

Since SA3 has concluded on the security update method for inter-CU LTM by including the NCC value in the LTM CSC, we think that RAN2 should discuss the possibility of performing LTM-based recovery using inter-CU LTM candidate cells.

We think that inter-CU LTM candidate cells should be used for LTM-based recovery to minimize data interruption and achieve advanced mobility robustness. If inter-CU LTM candidate cells are not used for LTM-based recovery in Release-19, the recovery performance will degrade compared to Release-18 in terms of mobility robustness. This is because configuring inter-CU LTM candidate cells for inter-CU LTM will inevitably reduce the number of intra-CU LTM candidate cells that can be configured for the UE due to the limitation on the maximum number of candidate cells.
In inter-CU LTM configurations, the number of intra-CU LTM candidate cells is typically small, and inter-CU LTM attempts are more frequent. As a result, if inter-CU LTM fails, the UE is more likely to perform RRC Re-establishment without any recovery, leading to more frequent data interruptions.

Proposal 5. Support inter-CU LTM candidate cell based failure recovery.

The security update method concluded by SA3 is to include security information in the LTM CSC. Thus, when performing LTM-based recovery using inter-CU LTM candidate cells, the source cell can provide security information for the inter-CU LTM candidate cells to the UE in the same manner, and we think that no remaining concerns exist.

It is not appropriate to reuse the security key attempted during LTM failure, as reusing security key does not meet security requirements. Therefore, to perform LTM-based recovery using inter-CU LTM candidate cells, the LTM CSC should additionally provide the NCC value for LTM-based recovery using inter-CU LTM candidate cells.

Furthermore, as stated in Section 2.1, LTM-based recovery also should ensure the same level of security as existing MN Mobility. Thus, even when the NCC value for LTM-based recovery is not available, the source gNB key-based security update should be considered to maintain the same level of security while performing recovery. Accordingly, we think that if security information for LTM-based recovery is not provided to the UE, the UE should be able to perform the LTM-based recovery using source gNB key-based security update.

Considering the above, we propose the following proposals:

Proposal 6: For inter-CU LTM failure, the UE can perform LTM-based recovery using vertical key derivation for inter-CU LTM candidate cell if the NCC value for LTM-based recovery is provided in LTM CSC.

Proposal 7: For inter-CU LTM failure, the UE can perform LTM-based recovery using horizontal key derivation for inter-CU LTM candidate cell if the NCC value for LTM-based recovery is not provided in LTM CSC.

As shown below in Rel-18, the UE does not remove the LTM configuration after successful recovery because LTM can still work after LTM-based recovery.

5.3.7.3 Actions following cell selection while T311 is running […] 1> if the cell selection is triggered by detecting radio link failure of the MCG or re-configuration with sync failure of the MCG for an LTM cell switch procedure triggered upon the indication by lower layers as specified in clause 5.3.5.18.6; and 1> if attemptLTM-Switch is configured; and 1> if the selected cell is one of the LTM candidate cells in the LTM-Candidate IE within ltm-Config associated with the MCG: 2> perform the LTM cell switch procedure for the selected LTM candidate cell according to the actions specified in 5.3.5.18.6; NOTE 2: In case both attemptCondReconfig and attemptLTM-Switch are configured, it is left to the UE implementation which procedure to execute. […]

Similar to Rel-18 LTM-based recovery, the UE keeps the LTM configuration after successful LTM-based recovery.

Proposal 8. UE keeps the LTM configuration as result of the LTM recovery based on inter-CU candidate cell.

3.	Conclusion

In this paper, we have made the following observations and proposals:

Security update for inter-CU LTM

Proposal 1: For inter-CU LTM, the NCC value can be included in the LTM CSC MAC CE.

Proposal 2: For inter-CU LTM, RAN2 should consider security updates based on horizontal key derivation (i.e., source gNB key based security update) to meet the same level of security in existing MN mobility.

Proposal 3: For inter-CU LTM, the UE performs a security update based on the horizontal key derivation (i.e., source gNB key based security update) if the LTM CSC does not include the NCC value.

Observation 1. Redundant SN security update during subsequent intra-CU MCG LTM cell switches can result in redundant PDCP reestablishment at the PDCP entities associated with SN and consequently data interruption can occur.

Observation 2. During subsequent intra-CU MCG LTM cell switches, the SN key that were used in the past may be re-used, which violates the security requirement.

Proposal 4. The network ensures that SN security update at intra-CU LTM will not happen during subsequent LTM cell switches in mixture of intra-CU LTM and inter-CU LTM with DC (i.e., SN is not changed).

LTM based recovery for inter-CU LTM

Proposal 5. Support inter-CU LTM candidate cell based failure recovery.

Proposal 6: For inter-CU LTM failure, the UE can perform LTM-based recovery using vertical key derivation for inter-CU LTM candidate cell if the NCC value for LTM-based recovery is provided in LTM CSC.

Proposal 7: For inter-CU LTM failure, the UE can perform LTM-based recovery using horizontal key derivation for inter-CU LTM candidate cell if the NCC value for LTM-based recovery is not provided in LTM CSC.

Proposal 8. UE keeps the LTM configuration as result of the LTM recovery.

4.	References

[1] S3-250732/ R2-2411136 Reply LS on security handling for inter-CU LTM in non-DC cases, CATT

[2] R2-2409211 Report from session on V2X/SL, R19 NES and MOB,	Vice Chairman (Samsung)