3GPP TSG-RAN WG2 Meeting #129bis	R2-2501996

Wuhan, China, Apr. 7th – 11st, 2025

| Agenda Item: | 8.6.4 (NR_Mob_Ph4-Core) |
| --- | --- |
| Source: | LG Electronics Inc. |
| Title: | Remaining issues of CLTM |
| Document for: | Discussion and Decision |

1.	Introduction

This paper handles details of early TA handling, RACH-less CLTM execution procedure, and coexsitence related issue. Specifically, candidate TA maintenance during LTM cell switch is discussed for details of early TA handling. For RACH-less CLTM execution procedure, we discuss the following remaining issue, i.e., FFS whether there is no CG resource associated with the triggered beam for RACH-less CLTM cell switch. For coexistence, it is discussed FFS whether Rel-19 CLTM can coexist with (e)RedCap and CovEnh.

2.	Discussion

2.1.	Details of early TA handling

In the previous meeting RAN2#129, the followings are agreed [1]:

CLTM TAT is started/re-started when UE receives TA value for the candidate cell sent by the MAC CE from current serving cell.

If the CLTM TAT associated with one candidate cell is running, UE considers the TA for the candidate cell is valid; if CLTM TAT associated with one candidate cell expires, UE considers the TA for the candidate cell is invalid.

If UE has valid TA value at the time CLTM execution, UE performs RACH-less CLTM, otherwise UE performs RACH base CLTM.

For the detailed handling of early TA for the target cell during RACH-less CLTM cell switch execution, the following options are currently on the table [1]:

CLTM TAT for target cell is not stopped upon CLTM cell switch execution to the target cell. Rapporteur suggests to discuss how/whether to start the timeAlignmentTimer associated with PTAG upon CLTM cell switch execution to the target cell during CB session: - Option 1 (revert the offline agreement): start the timeAlignmentTimer associated with PTAG as in Rel-18; - Option 2 (keep the offline agreement): discuss the options below in the MAC running CR o Option 2.1: clarify the running TAT to act as the timeAlignmentTimer associated with PTAG by a note in the MAC running CR; o Option 2.2: define that UE can perform UL tx when the TAT for the target cell is running. Besides, we need to add a note that, if SCell and SpCell are in the different TAG, SCell may not work when this TAT for target cell is running. o Option 2.3: the UE stops the CLTM timer and starts the PTAG using the remaining time of the CLTM timer.

In legacy principle, a TA timer is restarted only when a new TA value is indicated by the network, which means that the TA is only controlled by network indication. In our view, the Option 1 (i.e., start the timeAlignmentTimer associated with PTAG as in Rel-18) does not follow the legacy principle. Further, Option 1 seems to have no benefits because the validity of candidate TA is not visible to the target cell regardless of which option is selected between Option 1 and Option 2. Therefore, Option 2 is preferable in our view.

Proposal 1. CLTM TA timer for target cell is not stopped upon CLTM cell switch execution to the target cell.

Basically, the target cell does not know whether the candidate TA at UE is still valid when the UE arrives at the target cell via RACH-less CLTM cell switch execution. This is because the candidate cell cannot identify the UE during RA-based early TA acquisition procedure. To be specific, during the RA-based early TA acquisition procedure, the candidate cell calculates the TA value based on RA preamble reception regardless of which UE sent the RA preamble and then the candidate cell forwards the calculated TA value to the source cell via network interfaces (e.g., Xn and/or F1AP). The source cell sends the LTM Candidate TA Command MAC CE including the received TA value to the UE. In other words, the candidate TA is maintained based on the LTM Candidate TA Command MAC CE sent by the source cell. This implies that the validity of candidate TA is not visible to the target cell even though it is important information to the target cell.

Observation 1. During RACH-less CLTM cell switch execution, the validity of candidate TA associated with the target cell is not visible to the target cell even though it is important information to the target cell.

As described in Observation 1, the validity of candidate TA associated with the target cell is not visible to the target cell. This can result in misalignment of uplink synchronization between the target cell and the UE. Due to the misalignment, the UE may undergo large interruption. For example, consider the case that the UE performs the RACH-less CLTM cell switch toward the target cell, where the TA timer associated with the target cell is about to expire. In this case, the UE needs to initiate the RA procedure to acquire UL grant if the TA has not beed updated after the CLTM cell switch and if uplink data occurs after TA timer expiry.

To resolve the problem given above, the network always sends a new TA command to the UE after RACH-less CLTM cell switch completion. That is, the network ensures that the TA is always updated after RACH-less CLTM cell switch completion. However, TA update after every RACH-less CLTM completion may reduce the network flexibility. Alternatively, the UE reports the remaining TA timer value to the target cell during RACH-less CLTM cell switch execution. For example, the UE transmits the RRCReconfigurationComplete including the remaining TA timer value to the target cell. Based on the remaining TA timer, the target cell may send a new TA value to the UE if the remaining timer value is small.

Proposal 2. To resolve the misalignment of UL synchronization between the UE and the network, RAN2 selects one from the following options:

- Option 1. The network ensures that the TA is always updated during/after RACH-less CLTM cell switch completion.

- Option 2. The UE reports the remaining TA timer value to the target cell during RACH-less CLTM cell switch execution.

In the last meeting, RAN2 had not reached a conclusion about the following open issue [1]:

FFS on whether the received CLTM TAs of other candidate cells is released or still valid at UE side upon CLTM execution.

As described above, the candidate TAs are maintained based on the LTM Candidate TA Command MAC CE sent by the source cell. This implies that the validity timers for CLTM candidates are maintained at both the source and the UE. If the TAs of other CLTM candidate cells are kept at UE side upon/after CLTM execution, the source cell may need to transfer the validity information of the TAs of other CLTM candidate cells to the target cell. However, in this case, a remaining TA timer for a candidate cell can be distorted at the network side due to network signaling delay. This can result in the misalignment of the validity information of the TAs of other CLTM candidate cells between the target cell and the UE. The network signaling for transferring the validity information of the TAs of other CLTM candidate cells from the source to the target may become complicated to calibrate the distortion. For instance, the source cell needs to include a reliable time information (e.g., GPS based time) into the transfer message and then the target cell calibrates the remaining timer values of other CLTM candidate cells based on the transfer message. Hence, we argue that for simplicity the TAs of other CLTM candidate cells are released at UE side upon CLTM cell switch execution.

Proposal 3. The UE releases the TAs of other CLTM candidate cells upon CLTM cell switch execution.

2.2.	RACH-less CLTM execution procedure

In the last RAN2 meeting, the followings are agreed for RACH-less CLTM [1]:

When the C-LTM execution condition is satisfied with a candidate beam (any beam from the candidate RS set) / cell, UE performs RACH-less C-LTM (with that triggered beam for L1 based C-LTM case) if UE has a valid TA for the associated candidate cell. Otherwise, UE performs RACH-based C-LTM.

For L1 based C-LTM (assuming single beam fulfils C-LTM execution condition), beam selection for RACH-less LTM with CG is based on the beam meets C-LTM execution condition.

For L1 based C-LTM events, the C-LTM execution is triggered when at least one beam fulfills the C-LTM event condition. When multiple candidate beams satisfy the C-LTM condition, it is up to UE implementation to select a beam and perform C-LTM.

Do not support DG-based first UL transmission for RACH-less conditional LTM.

Based on the above agreements, if the UE has a valid TA value for the candidate cell, the UE performs RACH-less CLTM cell switch toward the candidate cell when an execution condition for the candidate cell is met. This means that whether to perform RACH during CLTM cell switch execution depends only on the UE has a valid TA for the associated candidate cell. In the case of L1 event based RACH-less CLTM cell switch execution, UE performs RACH-less CLTM toward the candidate cell with the triggered beam (i.e., the candidate beam of a satisfied CLTM execution condition). That is, the UE autonomously selects the target beam based on evaluation of CLTM executions.

CG is configured per beam (i.e., SSB) for RACH-less LTM cell switch based on the RRC specification. Since the UE autonomously selects the target beam based on evaluation of CLTM executions, a CLTM candidate cell should configure CGs for all candidate beams associated with CLTM execution conditions if the UE is allowed to access the candidate cell without RA procedure (i.e., RACH-less CLTM). In the case where CG is not configured for the triggered beam in CLTM execution, there is no proper UE action for RACH-less cell switch because DG-based approach is not supported for RACH-less CLTM.

To address the problem described above, the network should configure CGs for all candidate beams associated with candidate cells that allow for UE to perform RACH-less CLTM. But, it seems like too much burden on the network. Further, reserving many CGs for CLTM cell switch may lead to inefficient radio resource utilization. Alternatively, the UE performs RACH-based CLTM cell switch if CG is not configured for the triggered beam.

Proposal 4. For RACH-less CLTM cell switch, RAN2 selects one from the following options:

- Option 1. The network ensures that a CG is configured for all candidate beams associated with candidate cells that allow for UE to perform RACH-less CLTM.

- Option 2. The UE performs RACH-based CLTM cell switch if CG is not configured for the triggered beam.

2.3.	Coexistence

In the last meeting, RAN2 agreed that Rel-19 inter-CU LTM could coexist with CovEnh and (e)RedCap [1]. For CLTM, the coexistence with (e)RedCap and CovEnh is still FFS. In our view, there is no blocking issue to support the coexistence with (e)RedCap and CovEnh for CLTM. Therefore, same as Rel-18 LTM and Rel-19 inter-CU LTM, Rel-19 CLTM could coexist with CovEnh and (e)RedCap.

Proposal 5. Support coexistence between Rel-19 CLTM and (e)RedCap.

Proposal 6. Support coexistence between Rel-19 CLTM and CovEnh.

3.	Conclusion

In this paper, we have made the following observations and proposals:

Details of early TA handling

Proposal 1. CLTM TA timer for target cell is not stopped upon CLTM cell switch execution to the target cell.

Observation 1. During RACH-less CLTM cell switch execution, the validity of candidate TA associated with the target cell is not visible to the target cell even though it is important information to the target cell.

Proposal 2. To resolve the misalignment of UL synchronization between the UE and the network, RAN2 selects one from the following options:

- Option 1. The network ensures that the TA is always updated during/after RACH-less CLTM cell switch completion.

- Option 2. The UE reports the remaining TA timer value to the target cell during RACH-less CLTM cell switch execution.

Proposal 3. The UE releases the TAs of other CLTM candidate cells upon CLTM cell switch execution.

RACH-less CLTM execution procedure

Proposal 4. For RACH-less CLTM cell switch, RAN2 selects one from the following options:

- Option 1. The network ensures that a CG is configured for all candidate beams associated with candidate cells that allow for UE to perform RACH-less CLTM.

- Option 2. The UE performs RACH-based CLTM cell switch if CG is not configured for the triggered beam.

Coexistence

Proposal 5. Support coexistence between Rel-19 CLTM and (e)RedCap.

Proposal 6. Support coexistence between Rel-19 CLTM and CovEnh.

4.	References

[1] R2-2501331, Report from session on V2X/SL, R19 NES and MOB, Vice Chairman (Samsung), 3GPP TSG RAN WG2#129, Athens, Greece, 17th - 21st February 2025