3GPP TSG RAN2 Meeting #129bis                                                              R2-2501997

7th Apr. – 11th Apr. 2025, Wuhan, China

Agenda item:	8.4.2

Source:	Samsung

Title:	Procedure and Configuration of LP-WUS in RRC Idle /
Inactive Mode

Document for:	Discussion & Decision

Introduction

Energy conservation has emerged as an important issue in recent mobile communication network environments. Accordingly, RAN2 starts with the new Work Item (WI), Low-power wake-up signal and receiver for NR (LP-WUS/WUR) [1]. And during the previous meeting #129, with respect to procedure and configuration of LP-WUS in RRC idle/inactive mode agenda, RAN2 agreed as follows:

On separate band issue RAN2 understands that UE can report which band(s) is supported by LR to NW. RAN2 understands that any potential overload issues could be addressed by current mechanism in spec. Send LS to RAN1 and RAN4 to inform the agreements. On sub-grouping For UE_ID based subgrouping, similar formula defined for PEI subgrouping is reused for LP-WUS subgrouping, i.e., SubgroupID = (floor (UE_ID/(N*Ns*Np)) mod subgroupsNumForUEID) + (subgroupsNumPerPO – subgroupsNumForUEID), where - UE_ID is related to 5G-S-TMSI, - N is the number of total paging frames in one DRX cycle, - Ns is the number of the PO for a PF, - Np is the number of subgroupNumForUEID for PEI, if configured and UE supports PEI; otherwise, Np is 1, - subgroupsNumForUEID and subgroupsNumPerPO are the subgroup number for UE_ID based subgrouping for LP-WUS and the total subgroup number for LP-WUS, respectively. On SA2 LS (R2-2500050/S2-2412876) RAN2 sends reply LS to SA2/RAN3/CT1, and CC RAN1 and RAN4 to confirm the “LP-WUS” terminology. Regarding the SA2 raised issue on UE Radio Capability for Paging Information, R2 understand that there is no issue for NW after Release 17 (in which case the LP-WUS UE-ID based subgrouping UE capability is included in the UE-RadioPagingInfo container). Whether there is issue for the other cases (for the features mentioned by SA2 LS R2-2500050) can be further discussed in the main session.

In this contribution, we would like to discuss the detailed procedures and configurations to support LP-WUS in RRC IDLE/INACTIVE mode.

Discussion

Based on the LP-WUS/WUR study conducted during the Study Item (SI) phase in Release 18, it has been proposed that LP-WUS/WUR should be implemented as a power saving mechanism specifically designed for RRC_IDLE and RRC_INACTIVE mode devices. Furthermore, the concept of an "ultra-deep sleep state" has emerged as a potential power saving strategy for both RRC_IDLE and RRC_INACTIVE modes. By leveraging LP-WUS/WUR, a user equipment's (UE) Main Receiver (MR) can effectively transit into an ultra-deep sleep state, thereby achieving significant power savings. While in this ultra-deep sleep state, the UE can use its LP-WUS/WUR functionality to continuously monitor network signaling activities, waking up the MR when necessary.

- 2.1 Entry/Exit condition

According to the previous RAN2 discussion, now we have the entry/exit condition(s) of LP-WUS monitoring configured in SIB (e.g., SIB1).

Since with the agreed entry condition and with the NR measurement already ruled out from the exit condition, now the remained discussion is with respect to exit condition of LP-WUS monitoring, which is the serving cell measurement result based on LR below a configured threshold.

So far the RAN2 agreements related with entry/exit condition of LP-WUS monitoring when UE is in RRC Idle/ Inactive mode are as follows:

Baseline for entry condition definition: If the serving cell quality, e.g. RSRP, RSRQ from MR, is above threshold(s) (if configured), UE may start to monitor LP-WUS, if UE monitors LP-WUS, it may stop monitoring the legacy PO. FFS if any measurement from LR is needed. Baseline for exit condition definition: If the serving cell measurement result based on LR is below a threshold (if configured), UE monitors PO as in legacy and it may stop monitoring the LP-WUS. Separate entry/exit thresholds can be configured for OFDM-based and OOK-based WUR if a cell supports both types of LRs. Signalling details are FFS. Working assumption (can revisit if R1/R4 reached different conclusions): If the entry/exit conditions are configured, besides MR-based thresholds, LP-WUS monitoring entry condition can also include LR-based thresholds. The metrics for serving cell quality measured by MR/LR for entry condition includes (LP-)RSRP and optional (LP-)RSRQ. The metrics for serving cell quality measured by LR for exit condition includes (LP-)RSRP and optional (LP-)RSRQ. If NW configure thresholds for both MR and LR measurements, then the entry condition is met when all the measured results are above the configured threshold(s). The LPWUS monitoring exit condition does not include MR measurements.

Summarizing the above agreements, we have the below agreed conditions:

Observation 1. Conditions of LP-WUS monitoring when UE is in RRC Idle/ Inactive mode

Entry condition(s)

- If the serving cell quality from MR (or, from both MR and LR) is above threshold(s)

- The metrics for serving cell quality includes (LP-)RSRP and optional (LP-)RSRQ

- Separate entry thresholds can be configured for OFDM-based and OOK-based WUR if a cell supports both types of LRs.

Exit condition(s)

- If the serving cell quality from LR is below a threshold

- The metrics for serving cell quality includes LP-RSRP and optional LP-RSRQ

- Separate exit thresholds can be configured for OFDM-based and OOK-based WUR if a cell supports both types of LRs.

Now the remaining debate revolves around incorporating extra entry/exit criteria, is specifically "low-mobility." Given the scenarios where LP-WUS monitoring occurs while UEs operate in RRC idle/inactive modes, rapid movement within a cell might cause inaccurate behavior due to reliance solely on measurement-based thresholds.

For instance, consider high-speed moving devices like those on vehicles or trains. These UEs aren't ideal candidates for LP-WUS monitoring because they'll quickly transition back to regular MR paging due to their high-speed. Consequently, frequent toggling between these states leads to ping-pong and even increased energy usage of the device.

To address this issue, we propose to integrate a "low mobility" criterion as an optional feature that networks can configure according to specific requirements. This addition aims to enhance efficiency and reduce unnecessary power consumption during LP-WUS operations.

Proposal 1: The entry/exit conditions of LP-WUS monitoring may optionally include low mobility criterion based on serving cell quality from MR (FFS LR)

- 2.2 LP-WUS configuration

Previously RAN2 agreed that the LP-WUS configuration of a cell can be provided in SIB, especially for RRC Idle/ Inactive UEs. And there was another FFS whether the additional dedicated configuration, e.g., RRC release message, is required or not.

Considering the LP-WUS configuration is cell-specific and broadcasted, there is no solid reason to have additional RRC signalling. It would be the redundant configuration. Hence, we propose to limit the LP-WUS configuration for Idle/ Inactive mode with SIB only.

Proposal 2: The LP-WUS configuration for Idle/Inactive mode is not provided in dedicated RRC signalling.

- 2.3 MR Wakeup delay handling

Upon receiving the LPWUS in LO, UE needs time to wakeup MR to monitor paging. This wakeup delay depends on UE capability and is reported by UE to network. In order to take care of this wakeup delay following has been agreed:

- Network may configure one or two values for lo-Offset. lo-Offset if the offset between PF and LO to be monitored by UE.

- If single value is configured for lo-Offset and if the gap between the LO and the UE’s PO is >= the wake-up delay:

  - the UE monitors the LO associated with the offset

- else

  - UE does not monitor LO and follows the legacy paging monitoring procedure.

- If more than one values are configured for lo-Offset and if the gap between the LO associated with the largest offset and the UE’s PO is >= the wake-up delay:

  - the UE monitors the LO associated with the smallest offset value that has a gap between the LO and the PO >= the wake-up delay

- else

  - UE does not monitor LO and follows the legacy paging monitoring procedure.

In case PEI is configured in the cell and UE supports PEI, UE may monitor PEI-O after receiving the LPWUS. MR needs to wake up to receive PEI. SO, wake up delay needs to be considered to wake up MR for monitoring PEI.

If the gap between the monitored LO and the UE’s PEI-O is >= the wake-up delay, UE monitors PEI-O after receiving LPWUS. Otherwise, UE does not monitor PEI-O after receiving LPWUS.

Proposal 3: In case PEI is configured in the cell and UE supports PEI:

If the gap between the monitored LO and the UE’s PEI-O is >= the wake-up delay, UE monitors PEI-O after receiving LPWUS. Otherwise, UE does not monitor PEI-O after receiving LPWUS.

- 3. Conclusion

RAN2 is requested to discuss and agree to the following proposals:

Observation 1. Conditions of LP-WUS monitoring when UE is in RRC Idle/ Inactive mode

Entry condition(s)

- If the serving cell quality from MR (or, from both MR and LR) is above threshold(s)

- The metrics for serving cell quality includes (LP-)RSRP and optional (LP-)RSRQ

- Separate entry thresholds can be configured for OFDM-based and OOK-based WUR if a cell supports both types of LRs.

Exit condition(s)

- If the serving cell quality from LR is below a threshold

- The metrics for serving cell quality includes LP-RSRP and optional LP-RSRQ

- Separate exit thresholds can be configured for OFDM-based and OOK-based WUR if a cell supports both types of LRs.

Proposal 1: The entry/exit conditions of LP-WUS monitoring may optionally include low mobility criterion based on serving cell quality from MR (FFS LR)

Proposal 2: The LP-WUS configuration for Idle/Inactive mode is not provided in dedicated RRC signalling.

Proposal 3: In case PEI is configured in the cell and UE supports PEI:

If the gap between the monitored LO and the UE’s PEI-O is >= the wake-up delay, UE monitors PEI-O after receiving LPWUS. Otherwise, UE does not monitor PEI-O after receiving LPWUS.

- References

[1] RP-234056 Low-power wake-up signal and receiver for NR (LP-WUS/WUR), CMCC