3GPP TSG RAN2 Meeting #129bis                                                              R2-2501998

7th Apr. – 11th Apr. 2025, Wuhan, China

Agenda item:	8.4.3

Source:	Samsung

Title:	RRM measurement relaxation and offloading in RRC Idle /
Inactive Mode

Document for:	Discussion & Decision

1 Introduction

Energy conservation has emerged as an important issue in recent mobile communication network environments. Accordingly, RAN2 starts with the new Work Item (WI), Low-power wake-up signal and receiver for NR (LP-WUS/WUR) [1].

And during the previous meeting #127bis~129, with respect to RRM measurement relaxation and offloading agenda, RAN2 agreed as follows:

#127bis - Working assumption For neighbor cell measurement relaxation for UEs capable of LP-WUS, do not define additional MR-based criterion over the R16 criteria. RAN2 assume ‘UE not at cell edge’ is reused, FFS on ‘UE with low mobility’. FFS (if needed) on enhancements based on R16 criteria (e.g., based on the LR measurements) for the case when MR serving cell measurement results are not available. #128 The entry condition for serving cell RRM relaxation is at least ‘if serving cell quality measured by MR is higher than relaxation threshold, e.g. RSRP and/or RSRQ’. FFS if LR measurement is needed. FFS on exit condition for serving cell RRM relaxation, e.g., whether a separate exit condition other than ‘not fulfilling the entry condition’ is needed, or whether exit condition include MR and/or LR-based measurements FFS if the entry condition for serving cell RRM measurement relaxation is the same as neighbour cell RRM measurement relaxation. #129 The entry condition for MR serving cell RRM relaxation can include both MR and LR measurements. If LR threshold is configured, the entry condition is when both MR and LR measurement are above the configured thresholds.

In this contribution, we would like to discuss the detailed RRM measurement relaxation and offloading methods to support LP-WUS in RRC IDLE/INACTIVE mode.

2 Discussion

A UE in RRC_IDLE or RRC_INACTIVE state needs to perform mobility measurements as defined in TS 38.304 to camp on the optimal cell. Consequently, for the majority of UEs, the serving cell measurements require the most frequent execution, thus having the greatest influence on UE energy consumption.

Using the MR for serving cell measurements has a significant impact on UE energy consumption and reduces the LP-WUS/WUR gain according to the observations from the study evaluation results. In order to achieve the UE power saving gain by LP-WUS/WUR, the RRM measurement on serving cell and neighboring cell via MR can be relaxed or may be stopped when UE is using LP-WUS and MR is in ultra-deep sleep. However, it should be noted that these RRM relaxations may affect the mobility performance in RRC_IDLE or RRC_INACTIVE states.

RRM relaxation is a key feature to reduce NR UE power consumption, implemented through less frequent measurement of serving and neighbor cell qualities, such as Reference Signal Received Power (RSRP), Reference Signal Received Quality (RSRQ), and Signal-to-Interference-and-Noise Rate (SINR). RRM relaxation can be initiated through RRM relaxation triggers, and in Release 16, two types of triggers were specified: low mobility criteria and not at cell edge criteria. The subjective term 'low mobility' is utilized to define the mobility level of a UE with the preconfigured specific values like RSRPSearchDeltaP and TSearchDeltaP.  The subjective term 'not at cell edge' is utilized to define the channel quality of a UE with the preconfigured threshold, RSRPSearchThresholdP. When a condition (or both conditions) are satisfied, the terminal may alleviate RRM measurement by performing RRM once every multiple of a pre-configured DRX cycle, or even stop RRM measurement.

  - Serving cell RRM measurement relaxation and measurement offloading

As previously agreed upon by RAN2, NR supports MR serving cell RRM measurement relaxation and/or measurement offloading under conditions where the UE resides near the cell center, assuming that RRM measurement on LR is feasible and supported. Subsequently, RAN2 defined the ‘Serving cell measurement offloading’ and ‘Serving cell RRM measurement relaxation’ as follows:

Serving cell measurement offloading

: The serving cell measurement of an LP-WUS capable UE is fully offloaded to LR and no further MR serving cell measurements are mandated

Serving cell RRM measurement relaxation

: serving cell RRM measurement of an LP-WUS capable UE is relaxed with larger periodicity

If a UE supports the serving cell measurement offloading or RRM measurement relaxation, the serving cell quality could be measured using LP-WUR via LR-SS.

Considering the use cases of serving cell measurement offloading and relaxation, the triggering criteria for the entry/exit condition itself does not need to differ from the current criteria of NR RRM measurement relaxation. And RAN2 made consensus on the entry/exit conditions as follows:

Observation 1. Conditions of serving cell measurement offloading when UE is in RRC connected mode

Entry condition(s)

- If the serving cell quality from MR (or, from both MR and LR) is above threshold(s)

- The metrics for serving cell quality includes (LP-)RSRP and optional (LP-)RSRQ

Exit condition(s)

- If the serving cell quality from LR is below a threshold

- The metrics for serving cell quality includes LP-RSRP and optional LP-RSRQ

Observation 2. Conditions of MR serving cell RRM measurement relaxation when UE is in RRC connected mode

Entry condition(s)

- If the serving cell quality from MR (FFS LR) is above threshold(s)

- The metrics for serving cell quality includes RSRP and optional RSRQ

Exit condition(s)

- FFS on exit condition for serving cell RRM relaxation

Currently the ongoing topic of discussion is about exit condition of MR serving cell RRM measurement relaxation when UE is in RRC connected mode. Specifically, debate centers around whether the ‘not fulfilling the entry condition’ suffices, or whether exit condition include MR and/or LR-based measurements.

Given that explicit exit criteria for MR neighbour RRM measurement relaxation, analogous criteria could conceivably apply here too, by ‘not fulfilling the entry condition’. Nevertheless, considering regular LP-SS measurements conducted on the serving cell, LR-derived exit thresholds seems more reasonable.

Proposal 1: Similar to serving cell offloading, the exit conditions of MR serving cell RRM measurement relaxation are:

    - If the serving cell quality from LR is below a threshold

    - FFS low mobility criterion

Another extra entry/exit criteria, specifically "low-mobility" can also be considered. Given the scenarios where the serving cell measurement offloading/relaxation occurs, rapid movement within a cell might cause inaccurate behavior due to reliance solely on measurement-based thresholds.

For instance, consider high-speed moving devices like those on vehicles or trains. These UEs aren't ideal candidates for serving cell measurement offloading/relaxation because they'll quickly transition back to regular MR paging due to their high-speed. Consequently, frequent toggling between these states leads to ping-pong and even increased energy usage of the device.

To address this issue, we propose to integrate a "low mobility" criterion as an optional feature that networks can configure according to specific requirements. This addition aims to enhance efficiency and reduce unnecessary power consumption during LP-WUS operations.

And of course, considering NR neighbour RRM measurement relaxation, it is not so strange to introduce the low mobility criterion.

Proposal 2: The entry/exit conditions of serving cell measurement offloading and serving cell RRM measurement relaxation, may optionally include low mobility criterion based on serving cell quality from MR (FFS LR)

And, as agreed for LP-WUS monitoring in idle/ inactive mode, we propose to include - ‘Separate thresholds can be configured for OFDM-based and OOK-based WUR if a cell supports both types of LRs’ for the serving cell measurement offloading and relaxation.

Proposal 3. For the entry/ exit conditions of serving cell measurement offloading and serving cell RRM measurement relaxation,

- Separate thresholds can be configured for OFDM-based and OOK-based WUR if a cell supports both types of LRs.

2.2 Neighbor cell RRM measurement relaxation

For neighbour cell MR RRM measurement relaxation, RAN2 made below working assumpiton:

For neighbor cell measurement relaxation for UEs capable of LP-WUS, do not define additional MR-based criterion over the R16 criteria. RAN2 assume ‘UE not at cell edge’ is reused, FFS on ‘UE with low mobility’. FFS (if needed) on enhancements based on R16 criteria (e.g., based on the LR measurements) for the case when MR serving cell measurement results are not available.

Here we would like to re-confirm that the existing Rel16 criteria is already there, standardized, and there is no need of FFS on ‘UE with low mobility’.

Proposal 4: Clarify the existing ‘working assumption’ on neighbour cell RRM measurement relaxation as follows:

- For MR neighbour cell measurement relaxation for UEs capable of LP-WUS, MR ‘UE not at cell edge’ and MR ‘UE with low mobility’ criteria have already been supported since Rel-16, because there is no involvement of LR at all.

And for LR, we would like to extend the criteria as follows:

Proposal 5: For MR neighbour cell RRM measurement relaxation for UEs capable of LP-WUS, the LR-based ‘UE not at cell edge’ and ‘UE low mobility’ criteria are supported.

3 Conclusion

RAN2 is requested to discuss and agree to the following proposals:

Observation 1. Conditions of serving cell measurement offloading when UE is in RRC connected mode

Entry condition(s)

- If the serving cell quality from MR (or, from both MR and LR) is above threshold(s)

- The metrics for serving cell quality includes (LP-)RSRP and optional (LP-)RSRQ

Exit condition(s)

- If the serving cell quality from LR is below a threshold

- The metrics for serving cell quality includes LP-RSRP and optional LP-RSRQ

Observation 2. Conditions of MR serving cell RRM measurement relaxation when UE is in RRC connected mode

Entry condition(s)

- If the serving cell quality from MR (FFS LR) is above threshold(s)

- The metrics for serving cell quality includes RSRP and optional RSRQ

Exit condition(s)

- FFS on exit condition for serving cell RRM relaxation

Proposal 1: Similar to serving cell offloading, the exit conditions of MR serving cell RRM measurement relaxation are:

    - If the serving cell quality from LR is below a threshold

    - FFS low mobility criterion

Proposal 2: For the entry/exit conditions of serving cell measurement offloading and serving cell RRM measurement relaxation, introduce ‘low mobility’ criterion based on serving cell quality from MR (FFS LR)

Proposal 3. For the entry/ exit conditions of serving cell measurement offloading and serving cell RRM measurement relaxation,

- Separate thresholds can be configured for OFDM-based and OOK-based WUR if a cell supports both types of LRs.

Proposal 4: Clarify the existing ‘working assumption’ on neighbour cell RRM measurement relaxation as follows:

- For MR neighbour cell measurement relaxation for UEs capable of LP-WUS, MR ‘UE not at cell edge’ and MR ‘UE with low mobility’ criteria have already been supported since Rel-16, because there is no involvement of LR at all.

Proposal 5: For MR neighbour cell RRM measurement relaxation for UEs capable of LP-WUS, the LR-based ‘UE not at cell edge’ and ‘UE low mobility’ criteria are supported.

References

[1] RP-234056 Low-power wake-up signal and receiver for NR (LP-WUS/WUR), CMCC