3GPP TSG-RAN WG2 #129bis                                             	    R2-2502018

Wuhan, China, April 7th – 11th, 2025

Agenda item:	   8.6.4

Source: 		Qualcomm Incorporated

Title: 	Conditional intra-CU LTM

Document for:	Discussion

# 1	Introduction

RAN2#129 agreed the following regarding conditional LTM [1]:

Agreements on RRC issues: - A maximum number of 8 candidate configurations (LTM plus CLTM candidate configurations) can be configured. - The LTM-Candidate IE is re-used to model an CLTM candidate cell configuration. - Re-use existing L1 events for CLTM. Agreements on MAC issues: - CLTM TAT is started/re-started when UE receives TA value for the candidate cell sent by the MAC CE from current serving cell. - If the CLTM TAT associated with one candidate cell is running, UE considers the TA for the candidate cell is valid; if CLTM TAT associated with one candidate cell expires, UE considers the TA for the candidate cell is invalid. - If UE has valid TA value at the time CLTM execution, UE performs RACH-less CLTM, otherwise UE performs RACH base CLTM. - The CLTM TAT is per candidate configuration. - FFS on whether the received CLTM TAs of other candidate cells is released or still valid at UE side upon CLTM execution. - Absolute TA value is included in the MAC CE for CLTM early TA acquisition. - The candidate configuration ID is included in the MAC CE for CLTM early TA acquisition. And one configuration ID for one TA in one MAC CE. Agreements on C-LTM: - From a given source cell, network configures either L1 execution condition or L3 execution condition for a CLTM candidate cell. - The UE starts execution condition evaluation once it receives CLTM configuration including C-LTM execution condition. - For conditional LTM based on L1 measurements (LTM3/5-like event), the evaluation is per candidate beam. For conditional LTM based on L3 measurements (condA3/condA5 events), the evaluation is per candidate cell (i.e., with consolidation of multiple beams of the candidate cell). - When the C-LTM execution condition is satisfied with a candidate beam (any beam from the candidate RS set) / cell, UE performs RACH-less C-LTM (with that triggered beam for L1 based C-LTM case) if UE has a valid TA for the associated candidate cell. Otherwise, UE performs RACH-based C-LTM. - For L1 based C-LTM (assuming single beam fulfils C-LTM execution condition), beam selection for RACH-less LTM with CG is based on the beam meets C-LTM execution condition. - For L1 based C-LTM events, the C-LTM execution is triggered when at least one beam fulfills the C-LTM event condition. When multiple candidate beams satisfy the C-LTM condition, it is up to UE implementation to select a beam and perform C-LTM. - Do not support DG-based first UL transmission for RACH-less conditional LTM. - For RACH-based conditional LTM procedure, CFRA can be supported when CFRA resource is included in candidate cell configurations, otherwise CBRA is performed. - For intra-CU CLTM, it is up to NW implementation to ensure that only the candidate cells belonging to the same CU as the current serving cell are provided with the execution conditions. - Support the CLTM fast failure recovery, which reuses R18 LTM failure recovery mechanism (i.e. based on CBRA).

# 1	RACH-less C-LTM recovery

In RAN2#129, it was agreed to reuse Rel-18 LTM failure recovery mechanism, i.e., CBRA-based LTM switch procedure. While Rel-18 CBRA-based LTM fast recovery can be naturally supported for C-LTM, the possibility of utilizing CG resources for RACH-less C-LTM fast recovery should be considered.

Rel-18 LTM fast recovery had to be RACH-based LTM switch because only the target candidate cell in LTM CSC MAC CE activates the reception of CG resources and all the other candidate cells wouldn’t know about LTM switch of UE.

In Rel-19 C-LTM, however, the serving cell would have to inform several candidate cells (which UE may try C-LTM switch to) of activating CG resource receptions because it doesn’t know which candidate cell will be chosen by UE. As also agreed in RAN2#129, UE would try CG-based RACH-less C-LTM when at least one candidate beam fulfils the event condition and UE has a valid TA (either through TA MAC CE or UE-measured TA). Therefore, it is possible that one or more candidate cells still monitor CG resources when UE tries C-LTM fast recovery, in which case utilizing the CG resource could further reduce the recovery latency compared to RACH-based C-LTM fast recovery.

Observation 1: Unlike Rel-18 LTM, there could be more than one candidate cell that are indicated by serving cell to monitor CG resources for potential C-LTM execution by UE and they have to keep monitoring CG resources until they receive UL signal or another indication from original or new serving cell. Then it would be beneficial to utilize those CG resources for RACH-less C-LTM fast recovery.

Proposal 1: RAN2 agree to support RACH-less C-LTM recovery for Rel-19 intra-CU C-LTM when the following conditions are met:

- The selected cell is one of C-LTM candidate cell AND

- The selected C-LTM candidate cell has configured CG resource AND

- UE has a valid TA for the selected C-LTM candidate cell by either (i) TA from TA MAC CE for which TA timer is running or (ii) UE-measured TA.

# 2	Early C-LTM target cell notification for RACH-less C-LTM

When it comes to CG-based first UL transmission on target cell for RACH-less C-LTM, due to the unknown C-LTM cell switch timing to both serving cell and multiple candidate cells, CG time/frequency resources would have to be reserved earlier than actual C-LTM timing and most of them would be left unused, i.e., those CG resources cannot be used for other PUSCH allocations due to the potential collision with C-LTM UE’s first UL transmission. As more potential target candidate cells configure such CG resources, the overall resource inefficiency could become even more severe.

Therefore, it would be beneficial to introduce a new signaling procedure (“early target cell notification”) for UE to notify the potential target cell(s) and beam(s) for C-LTM to the serving cell such that the candidate cells can start monitoring CG resources only for a certain time period after receiving the notification message from the serving cell. This procedure would allow other candidate cells (which were not selected) to reuse the configured CG resources for other PUSCH allocations without any collision issue.

Observation 2: CG resource reservations from multiple target cells cany lead to significant inefficiency in time/frequency resources due to a large number of resource reservations.

Proposal 2: For RACH-less C-LTM, introduce an “early target cell notification” signaling procedure where UE informs the serving cell of potential target C-LTM cell(s) and beam(s).

# 3	Early TA signalling and maintenance

## 3.1	Candidate cell TA group

For the successful RACH-less C-LTM, it is necessary for UE to have a valid TA value for the target candidate cell. Through measurement reports, serving cell would be able to roughly know which candidate cells are potentially target candidate cells for C-LTM. However, if the number of those candidate cells is not small enough, these multiple PDCCH ordered RACH procedures would become overhead such that DL and/or UL performance would degrade due to interruptions.

To alleviate the PDCCH ordered RACH overhead issue, the candidate cell TA group can be considered where UE can assume an acquired TA value from one candidate cell (either from new TA MAC CE or UE-measured TA) can be applied to all other candidate cells within the TA group. By adopting this TA group, serving cell can trigger only a few PDCCH ordered RACH procedures for all candidate cells within the TA group, hence the RACH overhead could be significantly reduced.

Observation 3: For successful RACH-less C-LTM, serving cell may have to trigger PDCCH ordered RACH procedures for all potential target candidate cells, which could become huge overhead and interruptions in data transmission and reception at UE.

Proposal 3: RAN2 to support the candidate cell TA group where UE can assume that a TA value from one candidate cell (acquired either from new TA MAC CE or UE-measured TA) can be applied to all other candidate cells in that TA group.

## 3.2	TA validation procedure

In RAN2#129, it was agreed the TA timer-based TA validation procedure, i.e., if C-LTM TA timer starts/re-starts when UE receives TA value for the candidate cell via MAC CE, and as long as the TA timer associated with the candidate cell is running UE considers the TA for the candidate cell is valid.

Since the purpose of TA adjustment in uplink transmissions is to compensate for the additional propagation delay, the TA validation procedure should be able to track the propagation delay change, which can be also identified by RSRP change (reflecting a pathloss change).

The TA validation procedure at UE has been discussed in other topics before such as CG-SDT transmission or SRS transmission for positioning, and the existing validation procedure can be used as a baseline. According to Section 5.72.2 in TS 38.321 MAC specification, the following procedure is defined for the TA validation for CG-SDT.

The MAC entity shall consider the TA of the initial CG-SDT transmission with CCCH message to be valid when the following conditions are fulfilled: 1> The RSRP values for the stored downlink pathloss reference and the current downlink pathloss reference are valid according to TS 38.133 [11]; and 1> Compared to the stored downlink pathloss reference RSRP value, the current RSRP value of the downlink pathloss reference calculated as specified in TS 38.133 [11] has not increased/decreased by more than cg-SDT-RSRP-ChangeThreshold, if configured; and 1> cg-SDT-TimeAlignmentTimer is running

From above three conditions, the second (RSRP change threshold) and the third (TA timer) would be relevant for C-LTM. The first condition (valid RSRP) is to cover UE in RRC_INACTIVE state, so it is not necessary for C-LTM.

In addition to the TA timer that has been already agreed, checking the RSRP change (L1 or L3) can be used to track UE’s pathloss change which impacts the propagation latency.

Observation 4: The TA validation procedure for CG-SDT can be used as a reference for the TA maintenance procedure in C-LTM execution.

Proposal 4: RAN2 to agree the following as baseline for TA maintenance at UE:

- The TA timer value and RSRP change threshold are configured in RRC (which can be candidate cell specific or candidate cell group specific).

- When UE successfully receives the new TA MAC CE or measures UE-measured TA

  - Start the TA timer

  - Store the RSRP value of the candidate beam of the candidate cell

- The TA value is considered to be valid when the following conditions are true:

  - The RSRP change between the stored value and the current value has not increased/decreased by more than RSRP threshold; AND

  - The TA timer is running.

## 3.3	Candidate cell TA value handling after C-LTM execution

In RAN2#129, the UE behaviour of handling the received TA values for other candidate cells (except for the target candidate cell for C-LTM) upon C-LTM execution was left as FFS. Unlike Rel-18 LTM in which the serving cell can decide the target candidate cell first and then the early DL/UL synchronization can be performed for that selected candidate cell only, in C-LTM, UE determines the target candidate cell based on the event configuration, therefore the early DL/UL synchronization procedures would need to be performed towards multiple candidate cells due to the uncertainty at the serving cell. Considering the overhead for early TA acquisition (i.e., DL/UL scheduling interruption, MAC CE signaling), if UE has received multiple TA values for candidate cells from the serving cell and the corresponding TA timers are running, then it would be beneficial for UE to keep maintaining them for any future use (e.g., subsequent C-LTM or RACH-less C-LTM recovery).

Observation 5: In C-LTM, multiple early TA acquisition procedures may be required for successful RACH-less cell switch, and the overhead (DL/UL interruption and MAC CE signaling) of such multiple procedures may not be negligible. Therefore, it would be beneficial for UE to keep the TA value and TA timer for the future use.

Proposal 5: Upon C-LTM execution, UE performs the following for received TA values and TA timers for other candidate cells than the target candidate cell:

- Maintain the TA value.

- Keep the TA timer running.

# Conclusion

This contribution discussed aspects related to conditional intra-gNB LTM. The following observations and proposals have been made:

Observation 1: Unlike Rel-18 LTM, there could be more than one candidate cell that are indicated by serving cell to monitor CG resources for potential C-LTM execution by UE and they have to keep monitoring CG resources until they receive UL signal or another indication from original or new serving cell. Then it would be beneficial to utilize those CG resources for RACH-less C-LTM fast recovery.

Proposal 1: RAN2 agree to support RACH-less C-LTM recovery for Rel-19 intra-CU C-LTM when the following conditions are met:

- The selected cell is one of C-LTM candidate cell AND

- The selected C-LTM candidate cell has configured CG resource AND

- UE has a valid TA for the selected C-LTM candidate cell by either (i) TA from TA MAC CE for which TA timer is running or (ii) UE-measured TA.Observation 2: CG resource reservations from multiple target cells cany lead to significant inefficiency in time/frequency resources due to a large number of resource reservations.

Proposal 2: For RACH-less C-LTM, introduce an “early target cell notification” signaling procedure where UE informs the serving cell of potential target C-LTM cell(s) and beam(s).

Observation 3: For successful RACH-less C-LTM, serving cell may have to trigger PDCCH ordered RACH procedures for all potential target candidate cells, which could become huge overhead and interruptions in data transmission and reception at UE.

Proposal 3: RAN2 to support the candidate cell TA group where UE can assume that a TA value from one candidate cell (acquired either from new TA MAC CE or UE-measured TA) can be applied to all other candidate cells in that TA group.

Observation 4: The TA validation procedure for CG-SDT can be used as a reference for the TA maintenance procedure in C-LTM execution.

Proposal 4: RAN2 to agree the following as baseline for TA maintenance at UE:

- The TA timer value and RSRP change threshold are configured in RRC (which can be candidate cell specific or candidate cell group specific).

- When UE successfully receives the new TA MAC CE or measures UE-measured TA

  - Start the TA timer

  - Store the RSRP value of the candidate beam of the candidate cell

- The TA value is considered to be valid when the following conditions are true:

  - The RSRP change between the stored value and the current value has not increased/decreased by more than RSRP threshold; AND

  - The TA timer is running.

Observation 5: In C-LTM, multiple early TA acquisition procedures may be required for successful RACH-less cell switch, and the overhead (DL/UL interruption and MAC CE signaling) of such multiple procedures may not be negligible. Therefore, it would be beneficial for UE to keep the TA value and TA timer for the future use.

Proposal 5: Upon C-LTM execution, UE performs the following for received TA values and TA timers for other candidate cells than the target candidate cell:

- Maintain the TA value.

- Keep the TA timer running.

# References

- Chairman Notes; TSG RAN WG2 Meeting #129