3GPP TSG-RAN WG2 Meeting #129bis	R2-2502048

Wuhan, China, Apr. 7th – 11th, 2025

Agenda item:		8.8.2 Downlink coverage enhancements

Title:		Discussion on downlink coverage enhancements in NR NTN

Source:		ETRI

Document for:		Discussion

# 1. Introduction

During the 103th meeting of RAN, a new WI on NTN for NR phase 3 was approved, including the following objective:

1. Study and specify if beneficial downlink coverage enhancements targeting support for additional reference satellite payload parameters covering both GSO and NGSO constellations operating in FR1-NTN or FR2-NTN [RAN1, RAN2, RAN4] - Define additional reference satellite payload parameters assuming power sharing among satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint, such that satellite beams may not all be simultaneously active or may be active below the nominal EIRP density per satellite beam (see section 6.1.1 in TR 38.821) due to limited power and limited feeder link bandwidth. - Define the corresponding power sharing assumptions and necessary link level and system level evaluation methodology and relevant KPIs for evaluations of the coverage, to allow for identification of physical channels/signals and system-level aspects that need enhancements and the corresponding needed improvements. - Study and if needed specify solutions, including link level enhancements for FR1-NTN (e.g. for PDCCH, PDSCH) and/or system level enhancements for FR1-NTN and/or FR2-NTN, allowing dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint. - Notes for this objective: - SSB channel enhancement is not considered - Antenna gain of UE shall be assumed to be -5.5dBi in case of smartphone in FR1-NTN, the UE is assumed to be a full duplex UE, and at least 2Rx are considered at the UE - NGSO to be considered in priority: LEO Set-1 @ 600 km - Rel-18 network energy saving techniques should be considered as baseline in the system level study

The first discussion regarding the new WI took place during 125bisth meeting of RAN WG2 and then, the following agreements were made for downlink coverage enhancements in NTN.

Agreements: 1. With regard to link level enhancement, RAN2 waits for RAN1 agreement on the DL channels to enhance before starting any RAN2 work. 2. We will continue the discussion on RAN2 aspects of DL coverage enhancements (e.g. cell level / beam level DTX/DRX mechanism, etc.) in the next meetings, trying to identify questions to RAN1 for aspects where we need their input

During the 126th meeting of RAN WG2, the following agreements were made:

Agreements: 1. Based on the solution being investigated in RAN1, RAN2 will further discuss whether/how legacy UEs might operate in a cell supporting DL coverage enhancements. 2. RAN2 assumes that both EFC and EMC are supported

During the 127th meeting of RAN WG2, the following agreements were additionally made:

Agreements: 1. From RAN2 point of view, if the SSB periodicity is no larger than 160ms, there is no RAN2 impact on SSB configuration (there might still be impacts on DTX aspects) 2. From RAN2 point of view, If the SSB periodicity is larger than 160ms, for example ssb-PeriodicityServingCell, measurement gap periodicity, SMTC configuration, ssb-Periodicity-r17 for NonCellDefiningSSB-r17 may need to be extended. And the field description of nAndPagingFrameOffset may need to be enhanced to consider the SSB periodicity higher than 160ms. 3. RAN2 can further consider SMTC impacts due to beam-hopping / larger SSB periodicity 4. If there is a need to bar pre-Rel19 NTN UEs from accessing a cell operating with DL coverage enhancement (e.g. because of extreme SSB periodicity) the existing NTN bar bit can be used. FFS about the behaviour for Rel-19 UEs not supporting DL coverage enhancement when the existing NTN bar bit is set.

During the 127bisth meeting of RAN WG2, the following agreements were additionally made:

Agreements: 1. If it turns out that there is a need to bar UEs not supporting DL-CE, Rel-19 UEs not supporting DL-CE can be barred from accessing a cell operating with DL-CE using the existing NTN bar bit, in the same way as pre-Rel19 NTN UEs (this is an extension of the previous agreement to include also Rel-19 UE not supporting DL-CE) 2. If it turns out that there is a need to bar UEs supporting DL-CE, then we need to introduce a barring mechanism to control access of UEs supporting Rel19 NTN DL-CE. FFS on the details. (This also implies that UEs supporting Rel19 NTN DL-CE will not consider the existing NTN barring bit) 3. (also depending on the details of the RAN1 solution) we can further consider methods to allow UEs not supporting DL CE to down-prioritize or prevent re-selection to cells operating with DL CE.

During the 128th meeting of RAN WG2, the following agreements were additionally made:

Agreements: 1. If we need to bar UEs not supporting DL-CE (via legacy mechanism), we introduce a new barring in SIB1 to be able to selectively bar “UEs supporting DL-CE” (FFS if we will finally refer to “UEs supporting extended SSB periodicity” instead). FFS on the details (e.g. how many bits, etc.)

During the 129th meeting of RAN WG2, the following agreements were additionally made:

Agreements: 1. RAN2 assumes it will be possible to have different SSB periodicity among neighbour cells in the same frequency layer 2. RAN2 assumes that in a NR NTN cell, SSB beam sweeping in different spatial directions is possible as in a NR TN cell: the whole cell is covered by the different SSB beams in half-frame 3. RAN2 also assumes that, with the current status of RAN1 discussion, if one cell is defined by multiple “satellite beams”, the satellite beams are all simultaneously active or inactive (“beam hopping” applies equally to all the satellite beams of a given cell) 4. The number of SMTC/gaps a UE needs to consider at any time will not be increased further.

In this paper, we would like to provide our views on the highlighted aspects as well as remaining issues on downlink coverage enhancements in NTN.

# 2. Discussion

2.1	SMTC impacts due to beam hopping

The maximum number of SMTCs

During the 129th meeting, RAN2 assumed that support for both clustered-cells illumination and scattered-cells illumination scenarios could be considered for further discussions regarding SMTC. It was observed that if simultaneously active cells are clustered, the existing SMTC mechanism (with a maximum of four SMTCs per frequency) may be sufficient. However, if simultaneously active cells are scattered, especially in intra-frequency neighbour-cell scenarios, the neighbouring cells may transmit their SSBs at different times, potentially exceeding the existing SMTC mechanism's capabilities. Additionally, during the same meeting, RAN2 agreed to assume the feasibility of having different SSB periodicities among neighbour cells within the same frequency layer.

In several last RAN2 meetings, one of main issues for support of beam hopping in the scope of Rel-19 has been whether or not the maximum number of SMTCs needs to be increased. ETRI believes that the answer depends on various beam hopping scenarios (e.g., clustered-cells illumination vs scattered-cells illumination as well as different SSB periodicities among neighbour cells).

Figure 1 shows the number of SMTCs required in several clustered-cells illumination scenarios. As shown in Fig 1, the maximum numbers of SMTCs required to avoid inter-beam interference can exceed four, depending on how many cells are clustered.

Figure 1 Required number of SMTC in clustered-cells illumination scenarios

For example, Figures 1(a) and 1(b) illustrate scenarios where 7 and 4 active cells form single clusters, respectively. All beams within each cluster can be supported using just one SMTC window, as they are active simultaneously. However, separate SMTC windows are needed for other cluster groups due to differing active durations. When the UE is located at the center of the red circle, the network needs to configure a maximum of 3 and 4 SMTC windows in Figures 1(a) and 1(b), respectively. These scenarios can be supported within the legacy multiple SMTC configurations. In contrast, Figures 1(c) and 1(d) depict scenarios with 3 and 2 simultaneous active cells per cluster, respectively. In these cases, the network must configure a maximum of 5 and 6 SMTC windows, respectively. Therefore, these beam-hopping scenarios cannot be supported within the existing configuration limit of 4 SMTCs.

Observation 1: Even when simultaneously active cells are clustered, the existing SMTC mechanism—which allows a maximum of four SMTCs per frequency—might still be insufficient, potentially exceeding its current capabilities.

Proposal 1: RAN2 to assume that existing SMTC mechanisms may be insufficient even for Option A (clustered-cells illumination scenarios).

Figure 2 illustrates SMTC configurations in scattered-cell illumination scenarios. As shown in Fig. 2(a), the number of SMTCs required to prevent inter-beam interference could exceed the current maximum of four because neighbouring cells may transmit their SSBs at randomly different times. Alternatively, Figure 2(b) shows that group-based scattered-cells illumination could support beam hopping within the existing four-SMTC limit, if only one beam per group is active and neighbour groups transmit SSBs simultaneously. However, RAN1 simulation assumptions (Set 1-2) indicate that neighbouring scatter groups may transmit SSBs at different times, as a single satellite may serve up to 1058 beam footprints with only 16 active beams. Therefore, ETRI concludes that the four SMTCs may not be sufficient for various scattered-cells scenarios.

Figure 2 Required number of SMTC in scattered-cells illumination scenarios

Observation 2: The legacy four SMTCs may be insufficient for various scattered-cell illumination scenarios

Proposal 2: RAN2 to confirm that existing SMTC mechanisms may be insufficient for Option B (Scattered-cells illumination scenarios).

Proposal 3: RAN2 to study a unified solution applicable to both Option A (clustered-cells illumination) and Option B (scattered-cells illumination), assuming the existing SMTC mechanism is insufficient

SMTC configurations

By the way, during the last RAN2 meeting, it was agreed that the maximum number of SMTCs or measurement gaps a UE needs to consider simultaneously would not be increased further, even in beam-hopping scenarios where the number of candidate SMTCs exceeds this limit. Based on this agreement, two types of solutions can be considered.

The first is a UE location-based SMTC selection, performed on the UE side. In this approach, the maximum number of configured SMTCs may be increased (e.g., up to 7), and the network configures multiple SMTCs up to this maximum. Each SMTC is associated with a reference location (e.g., the centre of a cell where the UE is expected to measure SSBs using that SMTC). The network provides the UE with these multiple SMTC configurations, each linked to a reference location. Upon receiving this information, the UE selects a subset of the configured SMTCs based on the distance between its current location (e.g., obtained via GNSS) and the reference locations, for example, in ascending order of distance. To optimize power consumption, the UE may select only one SMTC or up to the maximum number of SMTCs it can monitor simultaneously.

The second is a UE location-based SMTC configuration, performed on the network side. In this approach, the maximum number of configured SMTCs is not increased (e.g., remains at 4), and the network configures only one SMTC or up to the maximum number that the UE can monitor simultaneously, depending on power consumption considerations. The network associates each candidate SMTC with a reference location (e.g., the centre of a cell where the UE is expected to perform SSB measurement using that SMTC) and provides the UE with these reference locations. The network may also request the UE to select a single reference location or a subset of them based on their proximity to the UE’s current location (e.g., in ascending order of distance, with location obtained via GNSS). Upon receiving the request, the UE selects a subset of reference locations according to this distance-based criterion and reports the selection back to the network. Based on the UE's response, the network configures one or more SMTCs (up to the allowed maximum), taking UE power consumption optimization into account.

Proposal 4: RAN2 to introduce a new SMTC mechanism based on either UE location-based SMTC selection (UE side) or UE location-based SMTC configuration (network side).

SMTC window adjustments

In legacy NR-NTN, SMTC window adjustment mechanism was introduced to account for propagation delay differences caused by multiple satellites. To support this, the UE estimates its location (e.g., via GNSS).

Before evaluating the need for such SMTC window adjustment in beam hopping operations, the following neighbouring cell scenarios should be considered:

- Case 1: All neighbouring cells are formed by multiple beams from a single satellite.

- Case 2: Some neighbouring cells are formed by beams from different satellites.

In Case 1, multiple SMTCs for neighbouring beams from a single satellite have fixed window timing offsets from the UE’s perspective. Therefore, adjustment may not be needed.

Observation 3: SMTC window adjustment, as introduced in legacy NR-NTN, may not be necessary for neighbouring beams from a single satellite.

In Case 2, as illustrated in Figure 3, a serving cell may have neighbouring cells from multiple satellites. SSBs from neighbouring cells of the same satellite share the same propagation delay, while those from different satellites may differ. Without location-based SMTC window adjustment, the UE may not receive SSBs from these neighbouring cells.

Figure 3 Example of SMTC configurations with no SMTC window adjustment

Observation 4: Without location-based SMTC window adjustment, the UE may fail to receive SSBs from neighbouring beam-hopping cells originating from other satellites.

Proposal 5: RAN2 to consider omitting SMTC window adjustment for neighbouring beam-hopping cells from the serving satellite, while applying SMTC window adjustment—similar to legacy NR-NTN—for neighbouring beam-hopping cells from different satellites.

2.2	Barring issues

During the 127th bis meeting, it was agreed that, if there is a need to bar pre-Rel-19 NTN UEs and Rel-19 UEs not supporting DL-CE from accessing a cell operating with DL-CE, the existing NTN bar bit can be used. It was also agreed that, if there is a need to bar UEs supporting DL-CE, a new barring mechanism should be introduced to control access for UEs supporting Rel-19 NTN DL-CE. However, the specifics of this mechanism are still under discussion (FFS). In this regard, the following explicit and implicit solutions are possible for consideration:

- Explicit solution: Define a cellBarredNTN-DL-CE field with values {bared, notBared}.

  - The value "barred" indicates that the cell is barred for UEs supporting DL-CE, while "notBarred" indicates that the cell is accessible to UEs supporting DL-CE. If not present, the UE assumes the cell is not allowed for connectivity to UEs supporting DL-CE.

  - When both cellBarred and cellBarredNTN are set to "barred," UEs supporting DL-CE ignore the "barred" value.

- Implicit solution: Use a SIB to convey system information related to NTN DL-CE cell configurations (e.g., DTX/DRX pattern, SSB periodicity extension, etc.).

  - When both cellBarred and cellBarredNTN are set to "barred," UEs supporting DL-CE check for the presence of a SIB conveying system information related to NTN DL-CE cell configuration. If the SIB including information related to NTN DL-CE is present, the cell is allowed for connectivity to UEs supporting DL-CE; if absent, the UE assumes the cell is not allowed for connectivity to UEs supporting DL-CE.

If a barring mechanism for UEs supporting DL-CE is needed, ETRI prefers the explicit barring mechanism, as it provides a simple and clear solution, whereas the implicit solution may impose additional burdens on the UE to monitor other SIBs in addition to SIB1.

Observation 5: If there is a need to bar UEs supporting DL-CE, an explicit barring mechanism provides a simple and clear solution, while the implicit mechanism may impose additional burdens on UEs by requiring them to monitor SIBs beyond SIB1.

Proposal 6: RAN2 to consider an explicit barring mechanism to control access for UEs supporting Rel-19 DL-CE, if there is a need to bar UEs supporting DL-CE.

- Define a cellBarredNTN-DL-CE field with values {barred, notBarred}.

  - The value "barred" indicates that the cell is barred for connectivity to UEs supporting DL-CE, while "notBarred" means that the cell is allowed for connectivity to UEs supporting DL-CE. If not present, the UE assumes the cell is not allowed for connectivity to UEs supporting DL-CE.

  - When both cellBarred and cellBarredNTN are set to "barred," UEs supporting DL-CE ignore the "barred" value.

# 3. Conclusions

Observation 1: Even when simultaneously active cells are clustered, the existing SMTC mechanism—which allows a maximum of four SMTCs per frequency—might still be insufficient, potentially exceeding its current capabilities.

Proposal 1: RAN2 to assume that existing SMTC mechanisms may be insufficient even for Option A (clustered-cell illumination scenarios).

Observation 2: The existing configuration limit of four SMTCs may be insufficient to adequately support various scattered-cells illumination scenarios.

Proposal 2: RAN2 to confirm that existing SMTC mechanisms may be insufficient for Option B (Scattered-cell illumination scenarios).

Proposal 3: RAN2 to study a single solution that can be applied to both Option A (clustered-cell illumination) and Option B (scattered-cell illumination), under the assumption that the existing SMTC mechanism may be insufficient for both Options A and B.

Proposal 4: RAN2 to introduce a new SMTC mechanism based on a UE location-based SMTC selection in UE side or a UE location-based SMTC configuration in network side.

Observation 3: A SMTC window adjustment mechanism introduced in legacy NTN may not be necessary for neighbouring hopping beams made from a single satellite.

Observation 4: If an SMTC window adjustment mechanism based on the UE’s location is not applied, the UE may fail to receive SSBs from neighbouring beam-hopping cells originating from other satellites.

Proposal 5: RAN2 to consider omitting SMTC window adjustment for neighbouring beam-hopping cells from the serving satellite, while applying SMTC window adjustment—similar to legacy NR-NTN—for neighbouring beam-hopping cells originating from different satellites.

Observation 5: If there is a need to bar UEs supporting DL-CE, an explicit barring mechanism provides a simple and clear solution, while the implicit mechanism may impose additional burdens on UEs by requiring them to monitor SIBs beyond SIB1.

Proposal 6: RAN2 to consider an explicit barring mechanism to control access for UEs supporting Rel-19 DL-CE, if there is a need to bar UEs supporting DL-CE.

- Define a cellBarredNTN-DL-CE field with values {barred, notBarred}.

  - The value "barred" indicates that the cell is barred for connectivity to UEs supporting DL-CE, while "notBarred" means that the cell is allowed for connectivity to UEs supporting DL-CE. If not present, the UE assumes the cell is not allowed for connectivity to UEs supporting DL-CE.

  - When both cellBarred and cellBarredNTN are set to "barred," UEs supporting DL-CE ignore the "barred" value.