3GPP TSG RAN WG2#129bis	2502071

Wuhan, China, April 7th – 11th, 2025

Agenda Item:	8.17

Source:	OPPO

Title:	Discussion on IoT NTN TDD mode

Document for:	Discussion, Agreement

Introduction

In RAN#105 meeting, a new WID on IoT NTN TDD mode was approved, which was further revised to [1] in RAN#106 meeting. This work item aims to enable Iridium’s TDD band to provide NB-IoT service leveraging commonalities with half-duplex NB-IoT FDD NTN, by defining a new NB-IoT TDD mode for NTN based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures. The objectives include the following.

- Specify a new NB-IoT TDD NTN mode based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures, including: - Definition, configuration (if needed) and signaling (if needed) of the periodic pattern, necessary adaptation (if needed) and associated UE procedures [RAN1, RAN2] - Support a pattern with a period of 9 radio frames for the target MSS allocated band, where D=U=8 with a fixed guard period.RAN1 to consider whether there is a need for a mechanism to achieve an adjustable guard period for the purpose of allowing deployment with the TDD frame structure of the legacy system operating in the target MSS allocated band. - Other necessary impacts on higher layers [RAN2] - RRM and RF core requirements [RAN4] - Specify a new NB-IoT TDD operating NTN band for the MSS allocation spanning 1616-1626.5 MHz for DL and UL, based on the outcome of the study, to be used as example band for this WI [RAN4]. - Specify band numbering - Specify SAN and UE RF characteristics - Specify DL and UL channelization. - Specify channel bandwidth as 200 kHz - Note1: No NTN-NTN coexistence study needed. - Note2: Leverage existing work as much as possible for TN-NTN coexistence of adjacent bands

In this contribution, we will discuss TDD support for NB IoT NTN from RAN2’s perspective and present our view.

Discussion

In RAN2#129 meeting, RAN2 discussed the access control for NTN cell working in the new TDD mode, and agreed that legacy barring bit will be used, but whether to use cellBarred or cellBarred-NTN is FFS. Technically, either cellBarred or cellBarred-NTN is feasible since legacy NB-IoT UEs in either TN or NTN anyway would not camp on a cell in 1616-1626.5 MHz MSS band. The description for cellBarred and cellBarred-NTN are captured in spec as below.

cellBarred Barred means the cell is barred for connectivity to EPC, as defined in TS 36.304 [4].

cellBarred-NTN Barred means the cell is barred for connectivity to NTN, as defined in TS 36.304 [4]. E-UTRAN always includes cellBarred-NTN and sets cellBarred to 'barred' in an NTN cell.

According to above, if cellBarred is used, some modification for the description of cellBarred-NTN is needed, e.g. the highlighted part can be revised as “E-UTRAN always includes cellBarred-NTN and sets cellBarred to 'barred' in an NTN cell expect for an NTN cell in the TDD mode”. If cellBarred-NTN is used, there would be no spec impact. Thus, we prefer to use cellBarred-NTN

Reuse the legacy barred bit cellBarred-NTN in SIB1 to control whether a R19 UE supporting TDD mode is allowed to access a cell in the 1616-1626.5 MHz MSS allocated band.

- IoT NTN TDD mode has been discussed in the last RAN1 meetings, and the following agreements have been made in RAN1#119 meeting.

Agreement Regarding operation within the same band as the TDD frame structure of legacy system in the 1.6GHz MSS band (shown below), RAN1 work in Rel-19 considers the following design constraints: - At the satellite, all downlink NB-IoT channels/signals in a cell can only use one of the downlink slots in the TDD frame structure (DL1, DL2, DL3 or DL4) across 90ms periods. - The same downlink slot is used in all the 90ms periods. - At the satellite, all uplink NB-IoT channels in a cell can only use one of the uplink slots in the TDD frame structure (UL1, UL2, UL3 or UL4) across 90ms periods. - The same uplink slot is used in all the 90ms periods. - The one uplink slot and one downlink slot in the TDD frame structure have the same index (DL1 & UL1, DL2 & UL2, DL3 & UL3, or DL4 & UL4). - NOTE: this does not imply that the only configuration(s) to be specified are according to these constraints. Observation RAN1 concludes that at least D=8 and U=8 is feasible with N=9 from point of view of the design constraints (from earlier agreement) imposed by the TDD frame structure of the legacy system in the 1616-1626.5 MHz MSS band.

- Based on above RAN1 agreements, the TDD frame structure constraints that all the downlink and uplink transmission can only use one downlink slot and one uplink slot, respectively, with the same index across 90 ms periods. And it has been approved in RAN#106 meeting that D=U=8 and N=9.

- In RAN2#129 meeting, the support of eDRX in NTN TDD mode was discussed, and the following agreements were reached.

Agreements:

- RAN2 will continue studying paging aspects based, on RAN1 progress.

- RAN2 confirms that idle mode eDRX is supported in IoT-NTN TDD network.

- RAN2 thinks that a change of H-SFN duration (Option 1-1) and/or H-SFN total number (option 2-2) will impact RAN2 and SA2 specification regarding the support of idle mode eDRX in IoT-NTN TDD network and the impact should be evaluated.

- Meanwhile, RAN1 made the following conclusion on H-SFN wrapround issue.

Conclusion For determining the offset between the 90ms TDD structure and the 10240ms H-SFN: the UE may derive the 90ms TDD structure based on observations of downlink signals. No specification impact.

- According to above agreements, eDRX can be supported in NTN TDD mode with no impact on H-SFN. Regarding i-DRX for NB-IoT, UE derives T as follows:

- For NB-IoT: If UE specific DRX value is allocated by upper layers and minimum UE specific DRX value is broadcast in system information, T = min (default DRX value, max (UE specific DRX value, minimum UE specific DRX value broadcast in system information)). If UE specific DRX is not configured by upper layers or if the minimum UE specific DRX value is not broadcast in system information, the default DRX value is applied.

- As highlighted above, if UE is configured by CN with UE specific DRX cycle and minimum UE specific DRX value is broadcasted, T is derived based on default DRX value, UE specific DRX value, minimum UE specific DRX value. Currently, the value range for all these parameters cannot match with the new NTN TDD pattern with a periodicity of 90ms, which would make the PF/PO be located in the non-D subframes. If we just consider to postpone the PO(s) located outside the D subframes the next D subframes, there may be multiple such POs in the same 90ms period postponed to the same 8 D subframes, we should define a PO mapping rule, which would be too complicated. In addition, it is not conducive for network resource planning. In order to avoid this, also considering that the new TDD mode is not back compatible, it would be most simple and straightforward to introduce new periodicities for DRX cycle to align with the TDD pattern, i.e. with a periodicity value of multiple of 90ms. To achieve this, new values for default DRX value and minimum UE specific DRX value should be introduced. For UE-specific DRX cycle, it is configured by CN, so it should be up to SA2 to decide how to deal with it. We need to send a LS to check with SA2.

Introduce new values for defaultPagingCycle and ue-SpecificDRX-CycleMin, i.e. with a periodicity value of multiple of 90ms.

Send a LS to check with SA2 how to handle UE-specific DRX cycle for IoT NTN TDD mode.

- In RAN1#120, RAN1 made the following conclusion regarding scheduling of SI message.

Conclusion It is RAN1’s understanding that the issue of scheduling and transmitting SI messages in the presence of non-D subframes will be solved mainly (or completely) by RAN2 specifications. RAN1 may discuss potential impact on RAN1 specifications, if any.

- Based on above RAN1 conclusion, the SI message issue should be within RAN2 scope. A UE acquires a SI message within the corresponding SI window. According to the current spec, each SI window is determined by the start time point and the window length, and SI windows for respective SI messages are placed one by one. For NTN TDD mode, similar to paging, by introducing new SI window periodicity with values of multiple of 90ms, it can enable the start of SI window for the first SI message in SystemInformationBlockType1-NB to always locate at the D NB-IoT subframe, which would facilitate network to manage the radio resource. Regarding the SI window length (i.e. si-WindowLength), currently, it is common for all the SI messages and refers to absolute length in time. In the new TDD mode, only the D NB-IoT subframes within the SI window can be used for SI message transmission. If the SI transmission within SI window overlaps with non-D NB-IoT subframes since the SI transmission duration may probably be larger than 8ms, we think it should be postponed to the next D NB-IoT subframes. In order to ensure enough time for SI message transmission and ensure that all the SI message have the same time length for actual transmission as well, we should either extend the value of si-WindowLength or specify that si-WindowLength refers to the accumulated usable NB-IoT DL subframes. This can also enable the start of all SI windows to be located t at the D NB-IoT subframe.

In NTN TDD mode, introduce new periodicities for SI window with values of multiple of 90ms.

From RAN2 perspective, SI transmission within SI window overlapping with non-D NB-IoT subframes are postponed to the next D NB-IoT subframes.

In NTN TDD mode, for si-WindowLength, RAN2 considers the following options:

- Option 1: Extend the si-WindowLength with values for multiples of 90ms.

- Option 2: Specify that si-WindowLength refers to the accumulated usable DL subframes.

- For UL, currently, both RACH periodicity and UL SPS periodicity cannot match with the new NTN TDD pattern, similar to i-DRX cycle and SI window periodicity, we should introduce new periodicities for RACH and UL SPS to align with the TDD pattern, i.e. with a periodicity value of multiple of 90ms, to ensure that the start of NPRACH and NPUSCH transmission can be located in the U NB-IoT subframe. For NPRACH and NPUSCH transmission with repetition, if the transmission overlaps with non-U NB-IoT subframe, it should be postponed to the next U NB-IoT subframe.

Introduce new periodicities for RACH and UL SPS with values of multiple of 90ms

From RAN2 perspective, NPRACH and NPUSCH overlapping with non-U NB-IoT subframes are postponed to the next U NB-IoT subframes.

- Considering that postpone of PDSCH or PRACH/PUSCH transmission overlapping with non-D or non-U NB-IoT subframe may also been discussed in RAN1, we suggest to confirm our RAN2’s understanding with RAN1.

Send an LS to RAN1 to confirm our understanding on P5 and P8.

- In TS36.321, a note on scheduling restriction is given as follow.

NOTE 4: For NB-IoT, for operation in FDD mode, and for operation in TDD mode with a single HARQ process when the associated HARQ RTT timer has been started, DL and UL transmissions will not be scheduled in parallel, i.e. if a DL transmission has been scheduled an UL transmission will not be scheduled until HARQ RTT Timer of the DL HARQ process has expired (and vice versa).

- According to the above note, for a NB-IoT UE configured with a single HARQ process, the UE is not required to monitor PDCCH since a DL or UL transmission has been scheduled. This has also been reflected in the start condition of onDurationTimer as highlighted below.

- if the Long DRX Cycle is used and [(SFN * 10) + subframe number] modulo (longDRX-Cycle) = drxStartOffset: - if NB-IoT: - if there is at least one HARQ process for which neither HARQ RTT Timer nor UL HARQ RTT Timer is running, start onDurationTimer. - else: - start onDurationTimer.

- In NTN TDD mode, for UL or DL transmission with repetition, if transmission duration is longer than 8ms, the transmission may be segmented. For UL segmented transmission, there will be DL NB-IoT subframes located between successive UL segmented transmissions. Following the principle in the above note, UE should not monitor PDCCH during these DL NB-IoT subframes, and this should be considered as an additional start condition restriction for onDurationTimer

In NTN TDD mode, UE doesn’t start onDurationTimer within the UL segmented transmission gap.

- Conclusion

Based on the discussion we propose the following:

- Reuse the legacy barred bit cellBarred-NTN in SIB1 to control whether a R19 UE supporting TDD mode is allowed to access a cell in the 1616-1626.5 MHz MSS allocated band.

Introduce new values for defaultPagingCycle and ue-SpecificDRX-CycleMin, i.e. with a periodicity value of multiple of 90ms.

Send a LS to check with SA2 how to handle UE-specific DRX cycle for IoT NTN TDD mode.

In NTN TDD mode, introduce new periodicities for SI window with values of multiple of 90ms.

From RAN2 perspective, SI transmission within SI window overlapping with non-D NB-IoT subframes are postponed to the next D NB-IoT subframes.

In NTN TDD mode, for si-WindowLength, RAN2 considers the following options:

- Option 1: Extend the si-WindowLength with values for multiples of 90ms.

- Option 2: Specify that si-WindowLength refers to the accumulated usable DL subframes.

Introduce new periodicities for RACH and UL SPS with values of multiple of 90ms

From RAN2 perspective, NPRACH and NPUSCH overlapping with non-U NB-IoT subframes are postponed to the next U NB-IoT subframes.

Send an LS to RAN1 to confirm our understanding on P5 and P8.

In NTN TDD mode, UE doesn’t start onDurationTimer within the UL segmented transmission gap.

- Reference

[1] RP-243293, Revised WID on introduction of IoT-NTN TDD mode