3GPP TSG-RAN WG2 #129bis	R2-2502073

Wuhan, China, April 7th – 11th 2025

Source:                    	DENSO CORPORATION

Title:  	Discussion on Store & Forward operation

Document for:        	Discussion and decision

Agenda Item:         	8.9.2		Support of Store & Forward

- Introduction

In Rel-19, the objectives for Store & Forward operation of IoT-NTN are described in the WID of Rel-19 IoT-NTN [1] as follows:

- Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore:

  - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2]

    - At least specify necessary enhancements e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3]

Note: Strive to minimise UE impact.

Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required

Based on the chair notes for last meeting RAN2#129[2], the following agreements are made for S&F operation:

Agreements:

1.	RAN2 currently assumes that a S&F mode changes does not necessarily imply a MME change

2.	The already agreed time information on transition from S&F operation mode to normal mode is provided in SIB31 (can work on signalling optimizations in case the new time information and T-service would carry the same information)

3.	Use absolute UTC time to broadcast the (S&F-> normal mode) time information. When this information is broadcast and indicates a time in the future the UE can assume the NW is currently operating in S&F mode (until the indicated S&F-> normal mode transition time).

4.	A change to the (S&F-> normal mode) time indication shall not result in system information change notification. Legacy rules for acquiring SIB31 apply

This paper attempts to investigate about consideration on S&F operation based on the above objectives and agreements.

- Discussion

2.1 Time information of current operation mode

In RAN2 #129, it was agreed that the time information on transition from S&F mode to normal mode is provided in SIB31. As mentioned in the past discussions, Rel-19 UE could use this information at least to delay some NAS procedures until the feeder link is resumed. Besides the transition from S&F mode to normal mode, we think time information on transition from normal mode to S&F mode could be useful to trigger cell reselection when the remaining time of normal mode becomes short.

Observation 1: Time information on transition from normal mode to S&F mode could be useful to trigger cell reselection procedure to select a cell with a longer remaining time in normal mode.

From perspective of the relationship between t-Service and the time information on transition from normal mode to S&F mode, if the time information is informed by using t-Service, UE may not be able to distinguish whether satellite is about to stop NTN service or going to S&F mode. Therefore, we would like to propose the following:

Observation 2: UE may not be able to distinguish whether satellite is about to stop NTN service or going to S&F mode if the time information on transition from normal mode to S&F mode is informed by using t-Service.

Proposal 1: Time information on transition from normal mode to S&F mode could also be provided to UE in SIB31, and the time information could be separated from t-Service.

Additionally, to search for a cell with a longer remaining time in normal mode, the UE may need to know the current operation mode and remaining time of neighbor satellites. Therefore, we would like to propose that S&F related information for neighbor satellites (current operation mode, transition time) could be provided to UE.

Proposal 2: S&F related information for neighbor satellites (current operation mode, transition time) could be provided to UE.

2.2 Existing RRC connection at the mode switch

It has been already agreed that UE not supporting S&F capability should be barred on “S&F mode”. In the case where the UE not supporting S&F already has established RRC connection before switching to “S&F mode”, we think the RRC connection should be released when the satellite is going to “S&F mode” from “real-time mode”, since the UE not supporting S&F most likely does not expect the significant delay on S&F operation.

Proposal 3: RRC connection of UE not supporting S&F should be released with RRCConnectionRelease message when the satellite is going to “S&F mode” from “real-time mode”.

Furthermore, during the last meeting, it is still FFS whether a new release cause will be introduced for the case where the network wants to release a Rel-19 UE because the feeder link becomes unavailable. We think the new release cause is not necessary although it could be used to avoid re-connection during mode switching. Instead of the new release cause, waitTime in RRCConnectionRelease could be used to avoid re-connection until the completion of mode switching. The network may not be able to estimate the exact timing when the feeder link becomes unavailable actually, however, the network could change the S&F indication in SIB1 to “S&F mode” in advance and set waitTime to the time duration until next the SIB1 with new S&F indication.

Observation 3: To avoid re-connection during mode switching, waitTime could work by setting the time duration until the next SIB1 with new S&F mode indication.

Proposal 4: The new release cause would not be necessary to avoid re-connection during mode switching.

- Summary and proposal

This paper discussed about consideration on S&F operation and proposed the following:

Observation 1: Time information on transition from normal mode to S&F mode could be useful to trigger cell reselection procedure to select a cell with a longer remaining time in normal mode.

Observation 2: UE may not be able to distinguish whether satellite is about to stop NTN service or going to S&F mode if the time information on transition from normal mode to S&F mode is informed by using t-Service.

Proposal 1: Time information on transition from normal mode to S&F mode could also be provided to UE in SIB31, and the time information could be separated from t-Service.

Proposal 2: S&F related information for neighbor satellites (current operation mode, transition time) could be provided to UE.

Proposal 3: RRC connection of UE not supporting S&F should be released with RRCConnectionRelease message when the satellite is going to “S&F mode” from “real-time mode”.

Observation 3: To avoid re-connection during mode switching, waitTime could work by setting the time duration until the next SIB1 with new S&F mode indication.

Proposal 4: The new release cause would not be necessary to avoid re-connection during mode switching.

- References

[1] RP-242397, “Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3”

[2] R2-2501333, “Report from Break-out session on NR-NTN and IoT-NTN”