3GPP TSG-RAN WG2 Meeting #129bis                                                               R2-2502144

Wuhan, China, 7th - 11th April 2025

Source:	CATT

Title:	RRM Relaxation and Offloading in RRC_IDLE/INACTIVE

Agenda Item:	8.4.3

Document for:	Discussion and Decision

Introduction

In this contribution, we provide our considerations on FFSs of RRM relaxation and offloading in RRC_IDLE/INACTIVE.

Discussion

Common remaining open issues

It is FFS on the terminology LP-WUS UE. The possible options include:

- Option 1: A UE supporting LP-WUS features, i.e., LP-WUS monitoring, MR measurement relaxation and/or serving cell measurement offloading, in RRC_IDLE/INACTIVE.

- Option 2: A UE supporting LP-WUS monitoring in RRC_IDLE/INACTIVE.

- Option 3: A UE supporting MR measurement relaxation/serving cell measurement offloading for LP-WUS in RRC_IDLE/INACTIVE.

- Option 4: A UE configured with LP-WUS in RRC_IDLE/INACTIVE.

- Option 5: A UE enabling LP-WUS function(s), e.g., LP-WUS monitoring, and/or MR measurement relaxation/serving cell measurement offloading.

In RRC_IDLE/INACTIVE, configurations are broadcasted via system information. It is a bit strange to say that the UE in RRC_IDLE/INACTIVE is configured with LP-WUS.

In order to reduce UE power consumption as much as possible, a smart UE would supports both LP-WUS monitoring and MR measurement relaxation/serving cell measurement offloading, and enables LP-WUS monitoring and MR measurement relaxation/serving cell measurement offloading functions simultaneously. But it is a bit strange that a UE must support LP-WUS monitoring in RRC_IDLE/INACTIVE if it determines criteria of MR measurement relaxation/serving cell measurement offloading are fulfilled or not. And it is a bit strict that a UE must enable LP-WUS function(s), e.g. starting LP-WUS monitoring if it determines criteria of MR measurement relaxation/serving cell measurement offloading are fulfilled or not. But we are ok to discuss option 5 if majority companies support it. Hence, we slightly prefer option 1 and option 3. Whether option 1 or option 3 is adopted may depend on UE capability discussion for LP-WUS in RRC_IDLE/INACTIVE.

Proposal 1: RAN2 discuss which following alternative is adopted for the terminology LP-WUS UE:

- Alt1: A UE supporting LP-WUS features in RRC_IDLE/INACTIVE; or

- Alt2: A UE supporting MR measurement relaxation/serving cell measurement offloading for LP-WUS in RRC_IDLE/INACTIVE; or

- Alt3: A UE enabling LP-WUS function(s), e.g., LP-WUS monitoring, and/or MR measurement relaxation/serving cell measurement offloading.

It is FFS for the metrics of MR measurement relaxation and serving cell measurement offloading. Similar as LP-WUS monitoring, the metrics for serving cell quality measured by MR/LR of MR measurement relaxation and serving cell measurement offloading include (LP-)RSRP and optional (LP-)RSRQ.

Proposal 2: The metrics for serving cell quality measured by MR/LR of MR measurement relaxation and serving cell measurement offloading include (LP-)RSRP and optional (LP-)RSRQ.

And similar as LP-WUS monitoring, separate thresholds for conditions of MR measurement relaxation and serving cell measurement offloading are configured for OFDM-based and OOK-based WUR if a cell supports both types of LRs.

Proposal 3: Separate thresholds for conditions of MR measurement relaxation and serving cell measurement offloading are configured for OFDM-based and OOK-based WUR if a cell supports both types of LRs.

MR RRM measurement relaxation

- Criteria for MR serving cell measurement relaxation

In RAN2#129 meeting, agreements on MR serving cell measurement relaxation included:

The entry condition for MR serving cell RRM relaxation can include both MR and LR measurements. If LR threshold is configured, the entry condition is when both MR and LR measurement are above the configured thresholds.

And it is still FFS on exit condition for MR serving cell measurement relaxation, e.g., whether a separate exit condition other than ‘not fulfilling the entry condition’ is needed, or whether exit condition includes MR and/or LR-based measurements.

The possible options for exit condition of MR serving cell measurement relaxation include:

- Option 1: No separate exit condition.

- Option 2: Only based on LR serving cell quality.

- Option 3: Only based on MR serving cell quality.

During MR serving cell measurement relaxation period, MR serving cell quality are relaxed with longer measurement cycle. MR serving cell quality cannot be updated timely. Hence, it is not reasonable to consider MR serving cell quality only for exit condition of MR serving cell measurement relaxation. Option 3 is not preferred. Option 1 is similar as R16 RRM relaxation criteria. Option 2 is similar as criteria for both LP-WUS monitoring and serving cell measurement offloading. Both options can work. We propose RAN2 discuss which option is adopted.

Proposal 4: RAN2 discuss which option is adopted for the exit condition of MR serving cell measurement relaxation:

- Option 1: No separate exit condition, i.e, not fulfilling the entry condition.

- Option 2: Only based on LR serving cell quality.

- Same or separate criteria for MR serving cell and neighbor cell measurement relaxation

It is FFS if the entry condition for serving cell RRM measurement relaxation is the same as neighbour cell RRM measurement relaxation. In RAN4#114 meeting, it was agreed:

Agreement: - MR RRM relaxation for serving cell/neighbour cell: scaling factor is 16.

As the same relaxation factor(s) applies, it is straightforward to use the same criteria for both MR serving cell measurement relaxation and neighbor cell measurement relaxation. In addition, relaxed RRM results of MR serving cell and neighbor cell are used for cell reselection. In order to reduce UE power consumption, there is no motivation to achieve normal MR serving cell measurement results and relaxed MR neighbor cell measurement results for cell reselection and vice versa .

Hence, we propose:

Proposal 5: Same relaxation criteria are applied to both MR serving cell and neighbor cell measurement relaxation for UEs capable of LP-WUS.

MR Serving cell measurement offloading

In legacy, if the MR serving cell is good enough (e.g. in cell center), the UE may not perform neighbor cell measurements including [1]:

-	If the serving cell fulfills Srxlev > SIntraSearchP (i.e. s-IntraSearchP broadcast in SIB2) and Squal > SIntraSearchQ (i.e. s-IntraSearchQ broadcast in SIB2), the UE may not perform intra-frequency measurements.

-	If the serving cell fulfills Srxlev > SnonIntraSearchP (i.e. s-NonIntraSearchP broadcast in SIB2) and Squal > SnonIntraSearchQ (i.e. s-NonIntraSearchQ broadcast in SIB2), the UE may choose not to perform measurements of NR inter-frequency cells of equal or lower priority, or inter-RAT frequency cells of lower priority.

In RAN2#127 meeting, RAN2 agreed that entry conditions for MR serving cell measurement offloading (i.e., there is no serving cell measurement by MR) can be defined as at least MR greater than a certain RSRP threshold, and LR could also be considered. Entry conditions for serving cell measurement offloading should not be worse than conditions for legacy MR neighbor cell measurement off on intra-frequency, NR inter-frequency cells of equal or lower priority, or inter-RAT frequency cells of lower priority, considering serving cell measurement offloading should be careful. Therefore, MR RSRP threshold in entry conditions for serving cell measurement offloading should be better than or equal to s-IntraSearchP and s-NonIntraSearchP, as shown in Figure 1. Similarly, MR RSRQ threshold in entry conditions for serving cell measurement offloading should be better than or equal to s-IntraSearchQ and s-NonIntraSearchQ.

Proposal 6: MR RSRP threshold in the entry conditions for serving cell measurement offloading (i.e., there is no serving cell measurement by MR) should be better than or equal to s-IntraSearchP and s-NonIntraSearchP.

Proposal 7: MR RSRQ threshold in the entry conditions for serving cell measurement offloading (i.e., there is no serving cell measurement by MR) should be better than or equal to s-IntraSearchQ and s-NonIntraSearchQ.

Before entry conditions for serving cell measurement offloading are fulfilled, the UE can follow legacy criteria for MR neighbor cell measurement off no matter if MR serving cell measurement is relaxed or not, i.e. if the MR serving cell fulfills Srxlev > SIntraSearchP (i.e. s-IntraSearchP broadcast in SIB2) and Squal > SIntraSearchQ (i.e. s-IntraSearchQ broadcast in SIB2), the UE may not perform MR intra-frequency measurements; If the MR serving cell fulfills Srxlev > SnonIntraSearchP (i.e. s-NonIntraSearchP broadcast in SIB2) and Squal > SnonIntraSearchQ (i.e. s-NonIntraSearchQ broadcast in SIB2), the UE may choose not to perform MR measurements of NR inter-frequency cells of equal or lower priority, or inter-RAT frequency cells of lower priority.

Proposal 8: Before entry conditions for serving cell measurement offloading are fulfilled, the UE can follow legacy criteria for stopping MR neighbor cell measurement on intra-frequency, NR inter-frequency cells of equal or lower priority, and inter-RAT frequency cells of lower priority.

Figure 1: One example of the serving cell measurement offloading coverage

Relationships among conditions

According to RAN4 agreements, the following cases were agreed:

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #1 Fully offloading case | Off | Off: FFS the condition and the details | ON |
| #3 Relaxed case b | On with relaxation measurement | On with relaxation measurement | ON |

Until now, RAN2 has discussed LP-WUS monitoring conditions, MR RRM measurement relaxation (i.e. case #3 Relaxed cased b in RAN4) conditions, and MR serving cell measurement offloading (i.e. case #1 Fully offloading case in RAN4) conditions. And we have made the following progress:

- For LP-WUS monitoring conditions, we have agreed:

  - Entry condition: based on MR and optional LR.

  - Exit condition: based on LR.

- For MR serving cell measurement offloading conditions, we have agreed:

  - Entry condition: based on MR and optional LR.

  - Exit condition: based on LR.

If conditions for these cases, including LP-WUS monitoring, MR RRM measurement relaxation and MR serving cell measurement offloading, are configured in a cell, we would like to clarify their relationships at first. It is straightforward that conditions for MR serving cell measurement offloading should be better than those for MR RRM measurement relaxation. During previous RRM relaxation discussion, MR serving cell relaxation was not introduced as it may seriously impact the user’s experience. In R19, MR serving cell relaxation is discussing with LP-WUS capable UEs. If the UE doesn’t monitor LP-WUS, it is dangerous to relax MR serving cell. In worst case the UE maybe out of coverage and paging maybe lost. Therefore, conditions for MR RRM measurement relaxation should be better or equal to those for LP-WUS monitoring. One example of relationships among these conditions is shown in Figure 2.

Figure 2 One example of relationships among different conditions

Proposal 9: Conditions for MR RRM measurement relaxation should be better or equal to those for LP-WUS monitoring, and conditions for MR serving cell measurement offloading should be better than those for MR RRM measurement relaxation.

Conclusion

According to the analysis in Section 2, we propose:

Common remaining open issues

Proposal 1: RAN2 discuss which following alternative is adopted for the terminology LP-WUS UE:

- Alt1: A UE supporting LP-WUS features in RRC_IDLE/INACTIVE; or

- Alt2: A UE supporting MR measurement relaxation/serving cell measurement offloading for LP-WUS in RRC_IDLE/INACTIVE; or

- Alt3: A UE enabling LP-WUS function(s), e.g., LP-WUS monitoring, and/or MR measurement relaxation/serving cell measurement offloading.

Proposal 2: The metrics for serving cell quality measured by MR/LR of MR measurement relaxation and serving cell measurement offloading include (LP-)RSRP and optional (LP-)RSRQ.

Proposal 3: Separate thresholds for conditions of MR measurement relaxation and serving cell measurement offloading are configured for OFDM-based and OOK-based WUR if a cell supports both types of LRs.

MR RRM measurement relaxation

Proposal 4: RAN2 discuss which option is adopted for the exit condition of MR serving cell measurement relaxation:

- Option 1: No separate exit condition, i.e, not fulfilling the entry condition.

- Option 2: Only based on LR serving cell quality.

Proposal 5: Same relaxation criteria are applied to both MR serving cell and neighbor cell measurement relaxation for UEs capable of LP-WUS.

MR Serving cell measurement offloading

Proposal 6: MR RSRP threshold in the entry conditions for serving cell measurement offloading (i.e., there is no serving cell measurement by MR) should be better than or equal to s-IntraSearchP and s-NonIntraSearchP.

Proposal 7: MR RSRQ threshold in the entry conditions for serving cell measurement offloading (i.e., there is no serving cell measurement by MR) should be better than or equal to s-IntraSearchQ and s-NonIntraSearchQ.

Proposal 8: Before entry conditions for serving cell measurement offloading are fulfilled, the UE can follow legacy criteria for stopping MR neighbor cell measurement on intra-frequency, NR inter-frequency cells of equal or lower priority, and inter-RAT frequency cells of lower priority.

Relationships among conditions

Proposal 9: Conditions for MR RRM measurement relaxation should be better or equal to those for LP-WUS monitoring, and conditions for MR serving cell measurement offloading should be better than those for MR RRM measurement relaxation.

Reference

- 3GPP TS 38.304 V18.2.0, User Equipment (UE) procedures in Idle mode and RRC Inactive state.