3GPP TSG-RAN WG2 Meeting #129bis	R2-2502151

Wuhan, China, 07 – 11 April 2025

Agenda item:	8.2.3

Source:	Nokia

Title:	Aspects on random access for AIoT

WID/SID:	Ambient_IoT_solutions - Release 19

Document for:	Discussion and Decision

# 1	Introduction

This contribution handles the open issues within AIoT, in particular;

- Contention free access procedure

- Signalling of msg1 resource(s)

- General signalling of D2R resource(s)

- Procedures for re-access

# 2	Discussion

## 2.1	CF(R)A

For contention-free access, we propose to assume the UE may use the 3-steps procedure as baseline i.e.

- paging, by the reader

- random ID transmission, by the device

- echoing of random ID, by the reader

- full ID transmission, full ID by the device

This is to ensure that security procedure is still in place even for contention free access. In case less steps are needed, RAN2 should look at signaling optimization i.e. the unified solution. One may argue that the random ID is wasteful transmission since it is not used for contention resolution, but it could later be used for identifying the device for a command, or similar data. Hence, for the sake of forward compatibility, and simple device implementation, we believe this is good enough.

Proposal 1: CF(R)A signaling should mimic CBRA signaling to ensure security procedures (e.g. key exchanges) are equally applicable to both access methods.

## 2.2	Msg1 resources

RAN1 agreed that a set of msg1 resources can be provided by a preceding R2D message, and RAN2 has agreed that this could be either paging or another QueryRep like message as in RFID.

- Agreements The A-IoT paging message can include a number of msg1 resources From RAN2 perspective, after initial paging message, the R2D transmission which determines the Msg1 resource(s), can be achieved by one of the below two ways, unless RAN1 concludes to use L1 signaling later: - Way-1: introducing new R2D message other than the paging message, e.g., QueryRep-like; or - Way-2: reusing the same paging message, using field(s) to indicate it is only to determine the Msg1 resource(s) and omitting the paging identifier (device ID/group ID) field … RAN2 assumes that device randomly selects among FDMA occasions as the baseline.

For the first agreement, the number of resources is used to determine the random function within the device i.e.

nresource=mod(randN,Nresources)

With nresource being the resource number selected, randN is the random number created by the device, and Nresources the number of resources signalled by the reader. Note that here, the random number should be different from what is provided in msg1 to the reader as otherwise the reader selecting similar resource will provide the same ID.

Observation 1: If the same random number is used as the random ID for msg1 and the resource selection, the IDs will also collide and ambiguity will happen in resource grant.

Proposal 2: The randomly selected resource should not be based on the same random number as the random access ID.

In our understanding, the device will then based on this number, the device should select a time and frequency slot of the provided resources within the indication of the paging, or QueryRep-like message. Additionally, in RAN2#129 FDMA resource selection were briefly discussed, leading to the agreement that a device will select the provided FDMA resources at random as baseline.

The simplest solution seems to be that the randomness is the one incorporated within a combined frequency/time resource selection process, and thus the time and frequency resource will be selected as.

ntimeslot= floor(Nresourcesnresource)

nfrequency slot=mod(Nresourcesnresource)

With ntimeslot being the selected timeslot number, and nfrequency slot the frequency resource.

Proposal 3: The number of resources provided by the paging message covers the number of time slots multiplied by the number of frequency resources.

As of the above analysis, the paging providing “a number of msg1 resources” actually means both a number of timeslots, and the number of frequency resources per timeslot.

Proposal 4: The R2D transmission determining the msg1 resources will indicate both the timeslot(s) and number of frequencies per timeslot to the device.

In order to enable the reader to indicate contention is resolved and a resource is given to one or multiple devices within a given timeslot, it should be able to indicate to multiple devices within msg2. Allowing multiplexing of multiple device IDs within a single message instead of a dedicated unicast message, enables optimal resource usage.

The simplest way of allowing access to a device within a given time/frequency slot would be the reader providing a list of RN16’s which won the contention, and the device then uses the frequency resource which it randomly selected to transmit msg3 and subsequent messages. However, this may bring multiple issues such as;

- The device would need to store the selected frequency slot to be used for transmission of msg3.

- In some cases, two devices may select the same frequency slot but the reader may be able to resolve contention anyway in case not all frequency slots within the timeslot were contented. In such cases it may be beneficial to allow the reader to provide one of the devices with one of the free frequency slots instead of relying on re-access

- Due to various gNB implemented procedures, it may determine that different frequencies are better for the device to transmit on in order to optimise performance. Such goals could be to;

  - Interference mitigation by spacing devices to accommodate for the bad internal filters,

  - Frequency hopping due to security reasons, or others.

Proposal 5: The device should not be expected to store the frequency slot in which it sent msg1 i.e. the device should follow reader indication on the frequency slot provided within a given timeslot.

How the reader indicates such frequency slot may be further discussed, but some options are;

- An ordered list of RN16s of the devices, in which each entry indicates a dedicated frequency slot.

  - The disadvantage is that padding must occur in case fewer devices has requested access within the timeslot than how many frequency slots are available. Furthermore, in case a reader may choose to only provide a subset of frequencies of what is granted, no padding means ambiguity on which slot is referred.

- An RN16 followed by a frequency slot number or bitmap indicating the frequency slot

  - For few frequency slots, this may cause more overhead, but for a larger number of frequency slots this will perform better than padding.

Proposal 6: RAN2 to discuss how to assign a dedicated frequency resource for a device within msg1.

Lastly, a question arises whether the device should use the same frequency during the entire procedure in case a command is indicated. In our view, this would again require the device to store which frequency resource should be used, and thus we believe that any R2D resource carrying an D2R should also carry the frequency index/bitmap.

Proposal 7: RAN2 to discuss whether the frequency index provided in msg1 is persistent for all subsequent D2R communication within the transaction or whether each R2D transmission providing a D2R resource will provide also a frequency index.

## 2.3	Signaling of D2R resources

RAN1 is currently discussing how to provide the msg1 resources for the device, and the latest feature lead proposal is to only allow a maximum value of 2 time resources in advance to be provided. The main argument for avoiding large X is that there would have to be increasingly larger guard times between the X time resources to accommodate the wide range of frequency offsets of the multiple devices. Although, X>2 could also be agreed, RAN2 should be able to assume that the X<<<<number of devices.

Observation 2: RAN2 can assume that only a few D2R timeslot resources can be provided in advance by the R2D signalling.

FL3 High Priority Proposal 2.1.1-1b For following according to TR 38.769 for Msg1, if X>1 is supported. - The maximum value of X is 2. - RAN1 targets to decide on the support of X>1 at RAN1#120bis A R2D transmission triggering random access determines X time domain resource(s) for D2R transmission(s) for Msg1, where each D2R transmission for Msg1 occurs in one time domain resource of the X time domain resource(s). The study includes: - X=1 and X>1 and X>=1. The maximum value of X>1 should be set considering the device implementation complexity, device power consumption, the resource usage efficiency affected at least by SFO, and inventory latency..

As a result of the few timeslot resources being provided in advance, RAN2 may also safely assume that the device will regularly get an R2D message indicating D2R resources at least for msg1. As a device may loose one or more messages, it may become desynchronised with the resource selection process i.e. not being aware of the current timeslot number. Thus, we think that the message providing the msg1 resource should also include a msg1 resource index.

Proposal 8: The message providing the msg1 resources should also include the timeslot count.

## 2.4	Feedback indication for re-access determination

RAN2 agreed a NACK based solution to determine re-access for at least msg3, but it was also agreed that in case segmentation was used, this should be always controlled by the reader. At least the FFS on whether segmentation applies to msg3 is contradictory to the agreement on the NACK based solution, and may result in devices with very little energy to waste it waiting for the segmentation message.

In case a device receives a NACK, it may perform re-access procedure, but there is no requirement that it must. In some cases, a device may choose to, or accidentally, power-off after the first transmission of msg3 and then not attempt re-access. In order to comply with this procedure, a device should not be obliged to reply to any msg2 retransmissions

Proposal 9: Segmentation should not apply to msg3 at least in Rel-19, FFS whether this has spec impact i.e. the device may be allowed not to reply on subsequent segment retransmission messages.

Agreements

NACK based mechanism is supported for D2R messages to determine re-access for at least msg3.  FFS details including whether we need a timer or explicit message and when reader sends feedback

- Agreements on segmentation

To support segmentation, a 1 bit indication is introduced to indicate whether there is more data or not, if SA2 indicates that CN can provide an estimated expected D2R message size.   If not possible, FFS if the 1 bit is sufficient.

Segment retransmission is supported.

For segment retransmission, reader explicitly indicates an offset in the MAC layer– e.g. number of bits successfully received so far (from the start).  FFS This implies that unsegmented packet can also be retransmitted.   FFS if this applies to msg3

# 3	Conclusion

This document has made the following observations and proposals:

Proposal 1: CF(R)A signaling should mimic CBRA signaling to ensure security procedures (e.g. key exchanges) are equally applicable to both access methods.

Observation 1: If the same random number is used as the random ID for msg1 and the resource selection, the IDs will also collide and ambiguity will happen in resource grant.

Proposal 2: The randomly selected resource should not be based on the same random number as the random access ID.

Proposal 3: The number of resources provided by the paging message covers the number of time slots multiplied by the number of frequency resources.

Proposal 4: The R2D transmission determining the msg1 resources will indicate both the timeslot(s) and number of frequencies per timeslot to the device.

Proposal 5: The device should not be expected to store the frequency slot in which it sent msg1 i.e. the device should follow reader indication on the frequency slot provided within a given timeslot.

Proposal 6: RAN2 to discuss how to assign a dedicated frequency resource for a device within msg1.

Proposal 7: RAN2 to discuss whether the frequency index provided in msg1 is persistent for all subsequent D2R communication within the transaction or whether each R2D transmission providing a D2R resource will provide also a frequency index.

Observation 2: RAN2 can assume that only a few D2R timeslot resources can be provided in advance by the R2D signalling.

Proposal 8: The message providing the msg1 resources should also include the timeslot count.

Proposal 9: Segmentation should not apply to msg3 at least in Rel-19, FFS whether this has spec impact i.e. the device may be allowed not to reply on subsequent segment retransmission messages.