3GPP TSG-RAN2#129bis										            R2-2502203

Wuhan, China, April  07 – 11, 2025

Source: 			ZTE Corporation, Sanechips

Title: 	RLC enhancements for XR

Agenda item:	8.7.5

Document for: Discussion and Decision

# Introduction

Rel-19 XR WI[1] has an objective for RAN2 to specify RLC re-transmission related enhancements for operation of RLC Acknowledged Mode (AM) with small packet delay budget. .

In RAN2#127bis meeting, the following agreements have been made[2]:

Agreements on RLC timely retransmissions - RAN2 confirm that existing mechanisms are insufficient to resolve the timely RLC retransmission problem and RLC enhancements for timely RLC retransmission are investigated in Rel-19. - Exclude enhanced status reporting. - Focus the discussion on autonomous retransmission and polling enhancements, e.g. we need to understand how each option affects the capacity and packet delay. Agreements on avoiding unnecessary retransmissions - RAN2 confirm the previous baseline assumption: the RLC receiving window always advances to any given RLC SN before the transmitting window does. - RAN2 will adopt a “combined” approach for avoiding unnecessary RLC retransmissions, i.e. - TX side stops transmissions of an outdated SDU - RX side abandons the SDU based on a local timer - Rx informs Tx side about the abandoned SDUs, as a baseline we assume existing SR can be reused unless issues are identified - FFS if some C-PDU handling is needed to avoid C-PDU discard - FFS if some indication is sent from Tx to Rx. The assumption is this is not a full status report, but something simple (if needed)

In RAN2#128 meeting, the following agreements have been made[3]:

- Special handling to avoid PDCP control PDU discard is not needed. - A new RLC timer at the Rx is introduced to determine obsolete RLC SDUs. The timer starts when the gap is detected at RLC layer. - The abandoned RLC SDUs determined by a new RLC timer are positively acknowledged in the STATUS report. Timely RLC retransmissions – autonomous versus polling - Timely RLC retransmission solution covers both autonomous retransmission and polling enhancement and NW can configure either or both of them. Agreements on unnecessary RLC retransmissions avoidance - There is no clear understanding on how the indication would look like or what problem it would solve that cannot be solved by the local timer - Unless critical issue is identified, no Tx to Rx indication will be introduced

In RAN2#129 meeting, the following agreements have been made[5]:

Autonomous retransmissions and polling enhancements - Autonomous retransmission and/or polling should be triggered when the remaining time of an RLC SDU falls below a specified threshold. FFS if remaining time is determined based on discardTimer at PDCP or new timer at RLC - Only a single autonomous retransmission will be triggered per RLC SDU. - There is no dynamic activation/deactivation of the autonomous retransmission mechanism. - We have separate thresholds for autonomous reTx and for polling Unnecessary retransmissions avoidance - When the TX RLC entity receives a discard indication of the SDU from PDCP, the TX RLC entity considers the SDU as an outdated SDU. The TX RLC entity does not perform any transmission and retransmission of such SDU/SDU segment. - A new RLC timer at the TX is not introduced to determine outdated RLC SDUs. - The new RLC timer at the RX is per RLC entity - The duration of the new RLC timer is not lower than that of t-reassembly

In this contribution, we will further discuss the RLC re-transmission related enhancements and give our proposals.

# Discussion

In RAN2#129 meeting, it has been agreed that “Autonomous retransmission and/or polling should be triggered when the remaining time of an RLC SDU falls below a specified threshold. FFS if remaining time is determined based on discardTimer at PDCP or new timer at RLC”. Since the discardTimer is maintained in PDCP entity, but the autonomous retransmission and polling is triggered in RLC entity, either PDCP entity sends the remaining time to RLC entity(e.g. RLC entity will maintain a timer with value of remaining time), or PDCP entity sends an indication to indicate that the remaining time is less than the configured remaining time threshold to RLC entity. Similar as the remaining time threshold for LCP, the remaining time threshold for Autonomous retransmission and/or polling should also be configured per LCH. Considering that LCH is not directly visible in PDCP, we prefer that PDCP entity notify RLC entity the remaining time of each RLC SDU, and RLC entity determines whether the remaining time of an SDU is below the remaining time threshold for autonomous retransmission and/or polling.

Proposal 1a: The remaining time thresholds for Autonomous retransmission and/or polling are configured per LCH.

Proposal 1b: PDCP entity notifies the RLC entity the remaining time of each RLC SDU, and RLC entity determines whether the remaining time of an SDU is below the remaining time threshold for autonomous retransmission and/or polling.

Considering that timely RLC retransmission and avoiding unnecessary RLC retransmission is beneficial for both Uplink transmission and downlink transmission, the RLC related enhancements should apply to both uplink and downlink.

Proposal 2: RAN 2 confirms that the RLC related enhancements (including autonomous retransmission based on remaining time, polling based on remaining time, SDU discarding based RX side local timer) apply for both uplink and downlink.

# Conclusion

The following proposals are made:

Proposal 1a: The remaining time thresholds for Autonomous retransmission and/or polling are configured per LCH.

Proposal 1b: PDCP entity notifies the RLC entity the remaining time of each RLC SDU, and RLC entity determines whether the remaining time of an SDU is below the remaining time threshold for autonomous retransmission and/or polling.

Proposal 2: RAN 2 confirms that the RLC related enhancements (including autonomous retransmission based on remaining time, polling based on remaining time, SDU discarding based RX side local timer) apply for both uplink and downlink.

# References

- RP-243318, Revised WID on XR (eXtended Reality) for NR Phase 3, RAN#106

- Draft_RAN2_127b_Meeting_Report_v1

- Draft_RAN2_128_Meeting_Report_v1

- 3GPP TS 38.322, V18.2.0 (2024-12), Technical Specification Group Radio Access Network; NR; Radio Link Control (RLC) protocol specification(Release 18)

- Draft_RAN2_129_Meeting_Report_v2