3GPP TSG-RAN WG2 Meeting #129bis	R2-2502230

Wuhan, China, Apr. 7th – 11th , 2025

Source:	NTT DOCOMO, INC.

Title:	Evaluations on measurement event prediction

Agenda Item:	8.3.3

Document for: 	Discussion and Decisions

- Introduction

During the RAN#102 meeting, the SID on “Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR”[1] was approved. The SID describes the study scope of the AI/ML-aided mobility enhancements as below,

Study and evaluate potential benefits and gains of AI/ML aided mobility for network triggered L3-based handover, considering the following aspects:

- AI/ML based RRM measurement and event prediction,

  - Cell-level measurement prediction including intra and inter-frequency (UE sided and NW sided model) [RAN2]

    - Inter-cell Beam-level measurement prediction for L3 Mobility (UE sided and NW sided model) [RAN2]

  - HO failure/RLF prediction (UE sided model) [RAN2]

  - Measurement events prediction (UE sided model) [RAN2]

- Study the need/benefits of any other UE assistance information for the network side model [RAN2]

- The evaluation of the AI/ML aided mobility benefits should consider HO performance KPIs (e.g., Ping-pong HO, HOF/RLF, Time of stay, Handover interruption, prediction accuracy, and measurement reduction) etc.) and complexity tradeoffs [RAN2]

  - NOTE: Simulation assumption and methodology can leverage TR 38.901, 38.843 and 36.839. And leave the detail discussion to RAN2

- Potential AI mobility specific enhancement should be based on the Rel19 AI/ML-air interface WID general framework (e.g. LCM, performance monitoring etc) [RAN2]

  - NOTE: This would only be treated after sufficient progress is made in the Rel-19 AI/ML air interface WID

- Potential specification impacts of AI/ML aided mobility [RAN2]

- Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4]

This paper will discuss the detailed evaluations and results for the measurement event predictions.

- Evaluation results on measurement event predictions

  - Simulation setup and assumptions

In this paper, we evaluate the intermediate and system-level performance of two scenarios, which are

- Case A (corresponding to Scenario 4, FR2-to-FR2 temporal domain prediction Case A)

- Case B (corresponding to Scenario 2, FR1-to-FR1 temporal domain prediction Case B)

Following the agreements at RAN2 #126 and RAN2 #127, the common simulation assumptions are listed in the Appendix, and the simulation assumptions specific to the measurement event prediction are presented in Table 1. We consider the indirect measurement event prediction for the evaluations submitted in this paper.

For Case A, the target is to enhance the HO performance with AI/ML-based measurement event predictions. At the RAN2 #128 meeting, the following options were agreed upon as the candidate schemes of the HO performance enhancement [4].

For measurement event prediction for temporal domain case A, company focus on option 2 or option 3. Option 2: network transmit handover command purely based on actual measurement event regardless whether an actual measurement result(@t2) is earlier or later than predicted measurement event((@t1)) Option 3: For AI mobility, HO preparation starts when an event is predicted to happen (i.e., t0), and HO command is sent when A3 entering conditions are met based on actual/real measurement and an event is predicted to be met for the duration of TTT.

We adopted Option 3 as a HO performance enhancement scheme to verify the performance gain from the predicted measurement events. This option utilized the predicted measurement event for the early HO preparation to avoid the too-late HO. When an A3 event is entering and the AI/ML predictor indicates that this event will be triggered, the NW can send the HO command directly to avoid the too-late HO. A typical HO procedure of Option 3 is shown in Figure 1.

Figure 1: A typical procedure of Option 3 for HO performance enhancements

The target of Case 2 is to reduce measurement overhead. Therefore, the HO procedures are not impacted. We reuse the legacy HO model and the HO definition in TR36.839 [3] to evaluate the system-level KPIs. The only difference is that the predictions replace some of the Layer 3 measurement results.

Table 1 Simulation assumptions for measurement event prediction

| Parameter | Value |
| --- | --- |
| A3 event offset (dB) | 2 |
| TTT (ms) | 80/320 |
| UE speed (km/h) | 30, 60, 90 |
| OW length (ms) | N/A |
| PW length (ms) | 40ms (sliding) |
| Max ETD (ms) | 80 |
| MRRT (Case B) | 50% |

The details of the following evaluation results can be found in the following files:

- ME_Case A_Indirect_129bis_DCM_2502230_V1.xlsx

- ME_Case B_Indirect_129bis_DCM_2502230_V1.xlsx

  - Case A (FR2-to-FR2 temporal domain prediction)

The intermediate and SLS evaluation results are shown in Table 2 and Table 3 for different TTT settings, respectively.

Table 2: Evaluation results when TTT = 80 ms

|  | UE speed | Intermediate KPI | SLS |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Average RSRP difference [dB] | HoF rate | HO attempt rate [per second per UE] | Total HoF [per second per UE] |
| Baseline (w/o AI/ML) | 60 kmph | N/A | 22% | 0.41 | 0.09 |
|  | 90 kmph | N/A | 24% | 0.55 | 0.13 |
|  | 120 kmph | N/A | 25% | 0.68 | 0.17 |
| With AI/ML predictions | 60 kmph | 0.91 | 16% | 0.45 | 0.07 |
|  | 90 kmph | 1.06 | 18% | 0.62 | 0.11 |
|  | 120 kmph | 1.11 | 19% | 0.78 | 0.15 |

Table 3: Evaluation results TTT = 320 ms

|  | UE speed | Intermediate KPI | SLS |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Average RSRP difference [dB] | HoF rate | HO attempt rate [per second per UE] | Total HoF [per second per UE] |
| Baseline (w/o AI/ML) | 60 kmph | N/A | 24% | 0.34 | 0.08 |
|  | 90 kmph | N/A | 29% | 0.45 | 0.13 |
|  | 120 kmph | N/A | 31% | 0.54 | 0.17 |
| With AI/ML predictions | 60 kmph | 0.91 | 17% | 0.44 | 0.08 |
|  | 90 kmph | 1.06 | 19% | 0.60 | 0.11 |
|  | 120 kmph | 1.11 | 19% | 0.75 | 0.15 |

As shown in Figure 1, the enhanced HO scheme (Option 3) tends to trigger the HO earlier. Typically, the HO command can be sent with TTT and HO preparation duration earlier than the legacy command, which results in a higher HO attempt rate than that of legacy HO schemes. The HO performance is improved from the earlier HO with prediction. A significant reduction of the HoF rate can be observed, and the total HoF per second per UE is reduced as well.

Observation 1

- For Case A with HO procedure Option 3, the HO performance is improved with the AI/ML-predicted measurement events.

  - The significant reduction in the HoF rate can be observed with the increase in the HO attempt rate.

  - Considering the newly introduced KPI, total HoF per UE per second, the HO performance is still improved by the AI/ML predictions.

  - Case B (FR1-to-FR1 temporal domain prediction)

The intermediate and SLS evaluation results are shown in Table 4 and Table 5 for different TTT settings, respectively.

Table 4: Evaluation results when TTT = 80 ms

|  | UE speed | Intermediate KPI | SLS |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Average RSRP difference [dB] | HoF rate | HO attempt rate [per second per UE] | Total HoF [per second per UE] |
| Baseline (w/o AI/ML) | 30 kmph | N/A | 9% | 0.11 | 0.010 |
|  | 60 kmph | N/A | 14% | 0.21 | 0.029 |
|  | 90 kmph | N/A | 18% | 0.31 | 0.056 |
| With AI/ML predictions | 30 kmph | 0.07 | 8% | 0.11 | 0.010 |
|  | 60 kmph | 0.09 | 14% | 0.21 | 0.029 |
|  | 90 kmph | 0.10 | 18% | 0.31 | 0.056 |

Table 5: Evaluation results when TTT = 320 ms

|  | UE speed | Intermediate KPI | SLS |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Average RSRP difference [dB] | HoF rate | HO attempt rate [per second per UE] | Total HoF [per second per UE] |
| Baseline (w/o AI/ML) | 30 kmph | N/A | 15% | 0.11 | 0.015 |
|  | 60 kmph | N/A | 20% | 0.19 | 0.037 |
|  | 90 kmph | N/A | 27% | 0.27 | 0.072 |
| With AI/ML predictions | 30 kmph | 0.07 | 16% | 0.10 | 0.016 |
|  | 60 kmph | 0.09 | 23% | 0.19 | 0.042 |
|  | 90 kmph | 0.10 | 30% | 0.27 | 0.079 |

The evaluations show similar performance between the baseline (all measurements are actually measured) and AI/ML predictions. We observed that AI/ML-based predictions can be used to replace UE measurements, thereby reducing measurement efforts.

Observation 2

- For Case B, when measurement events are generated based on predicted measurements with MRRT = 50%, there is a minor performance loss regarding the HoF rate and the total HoF per second per UE.

  - No impact is observed on the HO attempt rate per second per UE.

Based on these evaluations, we have the following proposal,

Proposal 1

- Capture Observation 1 and Observation 2 into the TR for system-level performance of Case A and Case B, respectively.

- Conclusion

In this contribution, we have the following observations,

Observation 1

- For Case A with HO procedure Option 3, the HO performance is improved with the AI/ML-predicted measurement events.

  - The significant reduction in the HoF rate can be observed with the increase in the HO attempt rate.

  - Considering the newly introduced KPI, total HoF per UE per second, the HO performance is still improved by the AI/ML predictions.

Observation 2

- For Case B, when measurement events are generated based on predicted measurements with MRRT = 50%, there is a minor performance loss regarding the HoF rate and the total HoF per second per UE.

  - No impact is observed on the HO attempt rate per second per UE.

Proposal 1

- Capture Observation 1 and Observation 2 into the TR for system-level performance of Case A and Case B, respectively.

Appendix: Simulation assumptions

Table  A1 Common simulation assumptions

| Parameter | FR1 Band #1 (common) | FR1 Band #2 (inter-frequency) | FR2 |
| --- | --- | --- | --- |
| Carrier frequency | 4 GHz (TDD) | 2 GHz (FDD) | 30 GHz |
| SCS | 30 kHz | 15 kHz | 120 kHz |
| Deployment | 2-tier with wrap around (7 sites, 3 sectors per site) |  | 2-tier with wrap around (7 sites, 3 sectors per site) |
| ISD | 500 m |  | 200 m |
| Channel model | UMa with distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901. Inter-frequency correlation model: Section 7.6.5 in TR38.901. |  | UMi w/ distance-dependent LoS probability function defined in Table 7.4.2-1 of TR38.901. |
| System bandwidth | 20 MHz |  | 80 MHz |
| Sample period | 40 ms |  | 80 ms |
| UE speed | 90 kmph |  | 60, 90, 120 kmph |
| UE distribution/dropping | 100% outdoor |  | 100% outdoor |
| BS antenna configuration | 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ Number of BS beams: 4 |  | (4, 8, 2, 1, 1, 1, 1), (dV, dH) = (0.5, 0.5) λ Number of BS beams: 32 |
| BS antenna radiation pattern | 3-sector antenna radiation pattern, 8 dBi |  | TR 38.802 Table A.2.1-6 |
| UE antenna configuration | 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) |  | (1, 4, 2, 1, 2, 1, 1), 2 panels (left, right) Number of UE beams: 4 |
| UE antenna radiation pattern | Omni-direction |  | TR 38.802 Table A.2.1-8 |
| BS Tx power | 44 dBm for 20 MHz bandwidth |  | 40 dBm |
| Maximum UE Tx power | 23 dBm |  | 23 dBm |
| BS receiver noise figure | 5 dB |  | 7 dB |
| UE receiver noise figure | 9 dB |  | 10 dB |
| BS antenna height | 25 m |  | 10 m |
| UE antenna height | 1.5 m |  | 1.5 m |
| Spatial consistency | Procedure A in TR38.901 |  | Procedure A in TR38.901 |
| UE trajectory model | Option 3 |  | Option 3 |
| UE boundary model | Option 3 |  | Option 3 |

The RRC parameters are configured following the post-RAN2 #126 email agreements in the following tables.

Table  A2 RRC parameters for the Layer 3 filtering

| L3 filtering parameter | value |
| --- | --- |
| FR1 Filter Coefficient | 4 |
| FR2 Filter Coefficient | 4 |

Table  A3 RRC parameters for the measurement period

| Measurement period | value |
| --- | --- |
| FR1 to FR1 intra-frequency w.o. gap | 200 ms |
| FR1 to FR1 inter-frequency with gap | 200 ms |
| FR2 to FR2 intra-frequency w.o. gap | 400 ms |

Table  A4 RRC parameters for beam consolidation/selection

| Consolidation parameter | value |
| --- | --- |
| nrofSS-BlocksToAverage for FR1 | 1 |
| nrofSS-BlocksToAverage for FR2 | 3 |
| absThreshSS-BlocksConsolidation for FR1 | -110 dBm |
| absThreshSS-BlocksConsolidation for FR2 | -110 dBm |

References

- Moderator (CMCC), RP-234055, “Study on AI (Artificial Intelligence)/ML (Machine Learning) for mobility in NR,” Dec. 2023.

- RAN2 Chair (InterDigital), “RAN2 #128 Chair Notes”, Nov., 2024.

- 3GPP TR36.839 v11.1.0, Mobility enhancements in heterogeneous networks (Release 11), Dec., 2012.

- 3GPP TR 38.843 v2.0.0, Study on Artificial Intelligence (AI)/Machine Learning(ML) for NR air interface (Release 18), Dec., 2023.

- RAN2 Chair (InterDigital), “RAN2 #128 Chair Notes”, Nov, 2024.

- RAN2 Chair (InterDigital), “RAN2 #129 Chair Notes”, Feb, 2025.