3GPP TSG-RAN WG2 Meeting #129bis                                               	 R2-2502280

Wuhan, China, April 7th – 11st, 2025

Agenda Item:	8.1.2.2 LCM for UE-sided model for Beam Management use case

Source:	NEC

Title:	Discussion on signalling procedure of supporting applicability report Option B

Document for:	Discussion, Decision

Introduction

RAN2#129 meeting reached following agreements on applicability reporting.

Agreements

- Inference configuration/parameters can be signalled in step 3 and/or Inference configuration can be signalled in step 5 (i.e. option a and option b from RAN1).

- The full inference configuration is sent in CSI-ReportConfig.

- Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability.

- FFS signaling details for option B (e.g. whether it is signaling in CSI-Report Config or otherconfig)

- Support the explicit reporting of applicability/inapplicability in initial report and subsequent reporting it reports only applicability it changed. FFS if we report explicit cause.

- If option A is configured in Step 3, for periodic CSI reporting, the UE autonomously activate the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete in step 4 (i.e. without need to wait RRCReconfiguration in Step 5).

- The provided periodic CSI configuration should be consistent with reported UE capabilities.

- FFS option B.

- Semi-persistent and aperiodic CSI reporting of applicable functionality is activated following legacy CSI framework:
Semi-persistent reporting, activated by MAC CE/DCI
Aperiodic CSI reporting, activated by DCI.

This contribution discusses signalling details of Option B.

Discussion

Motivations to support Option B

Before talking about the motivation to support Option B, let’s firstly clarify the difference between Option A and Option B for periodic CSI reporting. Procedure of both options based on RAN1 LS reply [1] is summarized below (Common Step 1 and 2 are not included). Note that the blue part below is based on RAN2#129 agreement. In this contribution, we focus solely on periodic CSI reports and will not specifically mention this focus further.

Table 1 Procedure comparison of Option A and Option B for periodic CSI reporting based on [1]

|  | Option A | Option B |
| --- | --- | --- |
| Step 3 | one or more of CSI-ReportConfig for inference configuration | One set or multiple sets of inference related parameters for applicability report only (not for inference) |
| Step 4 | UE reports applicability for all the above one or more CSI-ReportConfig | UE reports applicability for all the set(s) of inference related parameters |
|  | the UE autonomously activate the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete in step 4 (i.e. without need to wait RRCReconfiguration in Step 5). |  |
| Step 5 | (optional) NW can configure CSI-ReportConfig for inference configuration in RRCReconfiguration, | NW configures CSI-ReportConfig for inference configuration in RRCReconfiguration, |
|  |  | periodic CSI Report is considered as activated after RRCReconfigurationComplete. |

As summarized above, for Option A, NW needs to blindly configure one or more of CSI-ReportConfig for inference configuration in Step 3 without the UE applicability related information. Therefore, both applicable and inapplicable configurations could be configured in Step 3.

For Option A, NW needs to blindly configure one or more of CSI-ReportConfig for inference configuration in Step 3 without the UE applicability related information. Both applicable and inapplicable configurations could be configured in Step 3.

In the legacy CSI-ReportFramework, there is a UE capability on the maximum number of periodic CSI report setting per BWP for CSI report (shown below).

Assuming a similar UE capability is introduced later, and based on the RAN2#129 agreement that states, “The provided periodic CSI configuration should be consistent with the reported UE capabilities”, we observe that the maximum number of periodic CSI reports (including both applicable and inapplicable) configured in Step 3 must align with the reported UE capability. Therefore, when the number of inference configuration candidates on the network side exceeds the reported UE capability, the network must perform Step 3 multiple times to obtain the UE’s applicability/inapplicability report for all inference configuration candidates. This results in increased overhead and configuration delays.

For Option A, when the number of inference configuration candidates on the network side exceeds the reported UE capability, the network must perform Step 3 multiple times to obtain the UE’s applicability/inapplicability report for all inference configuration candidates. This results in increased overhead and configuration delays.

The drawback of Option A (as mentioned in Observation 1) can be avoided by using Option B. Under Option B, the sets of inference-related parameters used solely for the applicability report (and not for inference) configured in Step 3 will not be activated on the UE side upon reporting the applicability for all the sets of inference-related parameters in Step 4. Consequently, for Option B, the maximum number of periodic CSI reports (including both applicable and inapplicable) configured in Step 3 does not need to align with the reported UE capability. In other words, when the number of inference configuration candidates on the network side exceeds the reported UE capability, it is sufficient for the network to perform Step 3 just once.

For Option B, when the number of inference configuration candidates on the network side exceeds the reported UE capability, it is sufficient for the network to perform Step 3 just once.

Another drawback of Option A is the significant resource wastage associated with the transmission of inapplicable periodic CSI reference signals. In Option A, since the UE autonomously activates the applicable functionalities upon reporting applicability via RRCReconfigurationComplete in Step 4 (i.e., without waiting for RRCReconfiguration in Step 5), the network must transmit reference signals for all CSI-ReportConfig configured in Step 3. Because Step 3 under Option A includes both applicable and inapplicable configurations, this leads to a substantial resource wastage related to inapplicable periodic CSI reference signal transmissions.

Another drawback of Option A is the significant resource wastage associated with the transmission of inapplicable periodic CSI reference signals.

This drawback of Option A (as mentioned in Observation 3) can be avoided by using Option B. Under Option B, the sets of inference-related parameters used solely for the applicability report (and not for inference) configured in Step 3 will not be activated on the UE side upon reporting the applicability for all the sets of inference-related parameters in Step 4. Instead, the UE activates the periodic CSI Report after receiving the applicable inference configurations in Step 4. This means that the network only needs to transmit reference signals for all applicable CSI-ReportConfig configured in Step 4, eliminating any resource wastage associated with the transmission of inapplicable periodic CSI reference signals under Option A.

For Option B, the network only needs to transmit reference signals for all applicable CSI-ReportConfig configured in Step 4, eliminating any resource wastage associated with the transmission of inapplicable periodic CSI reference signals under Option A.

In addition to the above, Option B also offers significant benefits by providing simple and signaling-efficient control when the network aims to concentrate inference configurations to a certain CSI-ReportConfig(s). This is possible because Option B allows the network to configure commonly applicable CSI-ReportConfig(s) in Step 5, based on applicability reports from different UEs in Step 4.

Option B also offers significant benefits by providing simple, signaling-efficient control when the network aims to concentrate inference configurations to a certain CSI-ReportConfig(s).

Based on Observations 1-6, we think the motivation to support Option B is quite clear.

RAN2 confirms applicable functionality reporting Option B is supported in Rel-19.

Signaling details for option B

Option B in RAN1 LS reply [1] is defined as below:

Option B: One set or multiple sets of inference related parameters for applicability report only (not for inference)

- It is up to RAN2 to design the container.

- The set of inference related parameters selected from the IEs in/or the IEs referred by CSI-ReportConfig as a starting point, e.g.,

  - the associated ID
Note: this doesn’t imply the associated ID is mandatory

  - Set A related information

  - Set B related information

  - Report content related information

  - For BM-Case 2, 
Time instances related information for measurements
Time instances related information for prediction

Referring to signalling structure in the draft running RRC CR for AIML PHY (shown below), we observe that inference related parameters (except associated ID) for applicability report only (not for inference) of Option B listed in [1] are already in CSI-ReportConfig.

Regarding the remaining parameter, associated ID, it can be observed from RAN1#120 meeting agreement (copied below) that associated ID is also in CSI-ReportConfig.

Agreement

For UE-sided model, in CSI-ReportConfig for inference

•	One or two associated IDs can be configured in CSI-ReportConfig

o	When Set B is equal or a subset of set A (i.e., NZP-CSI-RS-ResourceId/SSB-Index in the resource set for Set B is within the NZP-CSI-RS-ResourceId/SSB-Index in the resource set for set A), one associated ID is configured,

o	Otherwise, one associated ID is configured for set A and another one associated ID is configured for Set B

All parameters of the set of inference related parameters listed in the RAN1 LS reply are in CSI-ReportConfig.

The first issue regarding the signaling details for option B is whether it is signaling in CSI-Report Config or otherconfig. Based on the Observation 7, we believe it is straightforward and signaling-efficient to use CSI-ReportConfig to configure one set or multiple sets of inference-related parameters solely for the applicability report (not for inference) of Option B. In other words, signaling Option B via Otherconfig requires the introduction of new IEs to configure parameters that can already be configured through the existing CSI-ReportConfig, resulting in an increase in signaling overhead. Additionally, signaling Option B via Otherconfig necessitates introducing a new procedure for the initial applicability report, apart from the initial applicability report in RRCReconfigurationComplete, leading to complicated UE behavior and significant specification impacts.

Signaling Option B via Otherconfig requires the introduction of new IEs to configure parameters that can already be configured through the existing CSI-ReportConfig, resulting in an increase in signaling overhead.

Additionally, signaling Option B via Otherconfig necessitates introducing a new procedure for the initial applicability report, apart from the initial applicability report in RRCReconfigurationComplete, leading to complicated UE behavior and significant specification impacts.

For Option B, one set or multiple sets of inference related parameters for applicability report only (not for inference) is signalled by CSI-ReportConfig.

For Option A, RAN2#129 meeting has agreed that “The full inference configuration is sent in CSI-ReportConfig”, then next issue is how to differentiate the CSI-ReportConfig is for either Option A or Option B. The main difference of UE behaviour upon receiving the Option A CSI-ReportConfig and Option B CSI-ReportConfig is whether to autonomously activate the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete in step 4 (i.e. without need to wait RRCReconfiguration in Step 5). During RAN2#129 e-mail discussion [AT129][026][AI Phy] Signaling for BM, one potential solution is to introduce a flag in CSI-ReportConfig to indicate whether a full inference configuration or a set of inference related parameters is contained in that CSI-ReportConfig, i.e., Option 2 in [3].

We think the new flag within CSI-ReportConfig is sufficient for differentiating Option A and Option B. An illustration of the new flag summarized as Option 2 in [3] is copied below.

Figure 1 An illustration of the new flag (Option 2 in [3])

Support a new flag in CSI-ReportConfig to indicate whether a full inference configuration or a set of inference related parameters is contained in that CSI-ReportConfig.

With the new flag, procedure of Option A and Option B for periodic CSI reporting is summarized again in Table 2 assuming that the new flag of “full inference” is an optional IE with only one value, “false”. Details of “full inference” flag can be left to stage 3.

Table 2 Procedure of Option A and B for periodic CSI reporting (with explicit indication of prediction status)

|  | Option A | Option B |
| --- | --- | --- |
| Step 3 | one or more of CSI-ReportConfig for inference configuration | One or more of CSI-ReportConfig for applicability report only (not for inference) (the new flag of “full inference configuration: False” is configured) |
| Step 4 | UE reports applicability for all the above one or more CSI-ReportConfig | UE reports applicability for all the set(s) of inference related parameters |
|  | the UE autonomously activate the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete in step 4 (i.e. without need to wait RRCReconfiguration in Step 5). |  |
| Step 5 | (optional) NW can configure CSI-ReportConfig for inference configuration in RRCReconfiguration | NW configures CSI-ReportConfig for inference configuration in RRCReconfiguration. |
|  | ・・・ | periodic CSI Report is considered as activated after RRCReconfigurationComplete. |

Signaling Option B via CSI-ReportConfig, along with a new flag in CSI-ReportConfig to indicate whether a full inference configuration or a set of inference related parameters is contained in that CSI-ReportConfig, achieves a unified signalling procedure of supporting applicability report Option A and Option B.

Conclusion

This contribution discussed signalling details of Option B.

Observations:

Observation 1	For Option A, NW needs to blindly configure one or more of CSI-ReportConfig for inference configuration in Step 3 without the UE applicability related information. Both applicable and inapplicable configurations could be configured in Step 3.

Observation 2	For Option A, when the number of inference configuration candidates on the network side exceeds the reported UE capability, the network must perform Step 3 multiple times to obtain the UE’s applicability/inapplicability report for all inference configuration candidates. This results in increased overhead and configuration delays.

Observation 3	For Option B, when the number of inference configuration candidates on the network side exceeds the reported UE capability, it is sufficient for the network to perform Step 3 just once.

Observation 4	Another drawback of Option A is the significant resource wastage associated with the transmission of inapplicable periodic CSI reference signals.

Observation 5	For Option B, the network only needs to transmit reference signals for all applicable CSI-ReportConfig configured in Step 4, eliminating any resource wastage associated with the transmission of inapplicable periodic CSI reference signals under Option A.

Observation 6	Option B also offers significant benefits by providing simple, signaling-efficient control when the network aims to concentrate inference configurations to a certain CSI-ReportConfig(s).

Observation 7	All parameters of the set of inference related parameters listed in the RAN1 LS reply are in CSI-ReportConfig.

Observation 8	Signaling Option B via Otherconfig requires the introduction of new IEs to configure parameters that can already be configured through the existing CSI-ReportConfig, resulting in an increase in signaling overhead.

Observation 9	Additionally, signaling Option B via Otherconfig necessitates introducing a new procedure for the initial applicability report, apart from the initial applicability report in RRCReconfigurationComplete, leading to complicated UE behavior and significant specification impacts.

Observation 10 Signaling Option B via CSI-ReportConfig, along with a new flag in CSI-ReportConfig to indicate whether a full inference configuration or a set of inference related parameters is contained in that CSI-ReportConfig, achieves a unified signalling procedure of supporting applicability report Option A and Option B.

Proposals:

Proposal 1	RAN2 confirms applicable functionality reporting Option B is supported in Rel-19.

Proposal 2	For Option B, one set or multiple sets of inference related parameters for applicability report only (not for inference) is signalled by CSI-ReportConfig.

Proposal 3	Support a new flag in CSI-ReportConfig to indicate whether a full inference configuration or a set of inference related parameters is contained in that CSI-ReportConfig.

- References

[1] R2-2407848

[2] R1-2410898

[3] R2-2501517